Supplementary Information for

Space-frequency-polarization-division multiplexed wireless communication system using anisotropic space-time-coding digital metasurface

Jun Chen Ke1,2, Xiangyu Chen3, Wankai Tang3, Ming Zheng Chen1,2, Lei Zhang1,2, Li Wang1,2, Jun Yan Dai1,2, Jin Yang4, Jun Wei Zhang1,2, Lijie Wu1,2, Qiang Cheng1,2,*, Shi Jin3,*, and Tie Jun Cui1,2,*

1 Institute of Electromagnetic Space, Southeast University, Nanjing 210096, China

2 State Key Laboratory of Millimeter Waves, Southeast University, Nanjing 210096, China

3 National Mobile Communications Research Laboratory, Southeast University, Nanjing 210096, China

4 Southeast University Wuxi Campus, Wuxi 214061, China

* Corresponding authors.

E-mail: qiangcheng@seu.edu.cn; jinshi@seu.edu.cn; tjcui@seu.edu.cn

Supplementary Note 1: Derivation of the reflected wave Erω for periodic Γ(t) with linear phase

When the phase of time-varying reflection coefficient Γ(t) is a periodically linear function, as shown in Eq. (4), the reflection spectrum distribution, Erω, can be derived as

|  | Erω=k=-∞+∞akEiω-kω0, | (S1) |
| --- | --- | --- |

where ω0=2πT represents the modulation angular frequency, and ak is the kth Fourier-series coefficient of Γt given by[1]

ak=Asinc(∆φ2-kπ)ej[φ1+∆φ2-kπ+mod∆φ2π-k,2⋅π+ε(2kπ-∆φ)⋅π],	(S2)

where ⋅ is the rounding down function, and sinc(⋅), mod(⋅), and ε(⋅) are the sinc, mod, and step functions, respectively. In accordance with the amplitude described in Eq. (S2), the +1st harmonic is dominant when ∆φ>π. On the contrary, if the phase of the reflection coefficient decreases linearly and satisfies ∆φ<-π, the highest peak of the reflection spectrum is located at the −1st harmonic.

Supplementary Note 2: Allocation strategy of the modulation signal sets for two polarization channels

To realize frequency-polarization-division multiplexed MPSK modulations, we should assign modulation signal sets Sx-pol. and Sy-pol. to the corresponding polarization channels. In general, if the MPSK modulations in both x and y polarization channels contain m possible symbols, the corresponding harmonics must have log2m-bit phase-switchable properties. According to Fig. 2d, the phases of the +1st and −1st harmonics vary linearly with the time delay td. To this end, we set unit time delay t0=Tm to define all states of the log2m-bit harmonic phases, obtaining the modulation signal sets Sx-pol. and Sy-pol. as:

Sx-pol.=Γxxt,Γxxt-t0,…,Γxx[t-(m-1)t0],	(S3a)

Sy-pol.={Γyyt,Γyyt-t0,…,Γyy[t-(m-1)t0]},	(S3b)

where

Γxxt-nt0=Axej∆φ(1-t+T-nt0T),   0≤t≤nt0Axej∆φ(1-t-nt0T),       nt0<t≤T,   n=0, 1, …, m-1,	(S3c)

and

Γyyt-nt0=Ayej∆φt+T-nt0T,   0≤t≤nt0Ayej∆φt-nt0T,       nt0<t≤T,   n=0, 1, …, m-1.	(S3d)

Each element in Sx-pol. and Sy-pol. corresponds to the phase state of the corresponding harmonic, which can transmit at most log2m bits of information.

For example, the corresponding Sx-pol. and Sy-pol.for the frequency-polarization-division multiplexed QPSK modulations can be expressed as

Sx-pol.=Γxxt,Γxxt-T4,Γxxt-T2,Γxxt-3T4,	(S4a)

Sy-pol.=Γyyt,Γyyt-T4,Γyyt-T2,Γyyt-3T4.	(S4b)

For the frequency-polarization-division multiplexed 16PSK modulations, we have

Sx-pol.=Γxxt,Γxxt-T16,Γxxt-T8,Γxxt-3T16,Γxxt-T4,Γxxt-5T16,Γxxt-3T8,Γxxt-7T16,Γxxt-T2,Γxxt-9T16,Γxxt-5T8,Γxxt-11T16,Γxxt-3T4,Γxxt-13T16,Γxxt-7T8,Γxxt-15T16,	(S5a)

Sy-pol.=Γyyt,Γyyt-T16,Γyyt-T8,Γyyt-3T16,Γyyt-T4,Γyyt-5T16,Γyyt-3T8,Γyyt-7T16,Γyyt-T2,Γyyt-9T16,Γyyt-5T8,Γyyt-11T16,Γyyt-3T4,Γyyt-13T16,Γyyt-7T8,Γyyt-15T16.	(S5b)

Supplementary Note 3: Effective circuit model and parameters of varactor

During the EM simulations, the varactors can be equivalent to the R-L-C series circuits, as shown in Supplementary Fig. S1. The effective circuit parameter values under different reverse biasing voltages are listed in Supplementary Table S1.[2]

Supplementary Figure S1 | The effective R-L-C series circuit of varactor.

Supplementary Table S1 | Effective circuit parameter values under different reverse biasing voltages.

| Biasing Voltage (V) | L (nH) | C (pF) | R (Ω) |
| --- | --- | --- | --- |
| 0 | 0.70 | 2.65 | 0.63 |
| -1 | 0.70 | 1.85 | 0.57 |
| -2 | 0.70 | 1.54 | 0.53 |
| -3 | 0.70 | 1.36 | 0.50 |
| -4 | 0.70 | 1.24 | 0.47 |
| -5 | 0.70 | 1.17 | 0.45 |
| -6 | 0.70 | 1.09 | 0.43 |
| -7 | 0.70 | 1.04 | 0.41 |
| -8 | 0.70 | 1.00 | 0.40 |
| -9 | 0.70 | 0.97 | 0.38 |
| -10 | 0.70 | 0.95 | 0.38 |
| -11 | 0.70 | 0.91 | 0.37 |
| -12 | 0.70 | 0.87 | 0.36 |
| -13 | 0.70 | 0.85 | 0.35 |
| -14 | 0.70 | 0.83 | 0.34 |
| -15 | 0.70 | 0.81 | 0.33 |
| -16 | 0.70 | 0.79 | 0.33 |
| -17 | 0.70 | 0.78 | 0.32 |
| -18 | 0.70 | 0.76 | 0.32 |
| -19 | 0.70 | 0.76 | 0.31 |

Supplementary Note 4: Frequency-polarization-division multiplexed MPSK wireless communication system

The schematic diagram of the frequency-polarization-division multiplexed MPSK wireless communication system is sllustrated in Supplementary Fig. S2. Here, Computer 1 in the transmitting end (transmitter in Fig. 5a) translates two different information sources (e.g., video files) into two sets of binary bit streams and sends them to FPGA (NI PXIe-7976R) of the control platform (NI PXIe-1082). By pre-programming the mapping relation in FPGA, the bit streams are reconstructed to symbol sequences. The two symbol sequences are separately converted into biasing voltage sequences through digital-to-analog converter (DAC, NI 5783) and external amplifier circuits. These biasing voltage sequences are then injected into the corresponding feed terminals of the anisotropic STC digital metasurface to drive the varactor diodes. Under the radiation of a 45° linearly polarized plane wave at 2.7 GHz, the metasurface reflects the −1st x-polarized and +1st y-polarized harmonics simultaneously. At the receiving end (receiver in Fig. 5a) placed at 3.5 m away, a dual-polarized horn antenna will receive the two modulated waves with different polaizations simultaneously and transmit them to the corresponding channels of a software-defined radio platform (NI USRP-2943R) with high isolation (at least 20 dB). After sequentially completing down-conversion, analog-to-digital conversion, and baseband signal processing, the modulation waves are converted to the bit streams. Finally, in Computer 2, the bit streams are reconstructed to recover the original information. During the information transmission, the interactive communication and clock synchronization among the control platform, microwave signal generator (RIGOL DSG3060), and USRP-2943R are performed using the phase stable cables.

Supplementary Figure S2 | Schematic diagram of the frequency-polarization-division multiplexed MPSK wireless communication system.

Supplementary Note 5: Experimental setup of dynamic dual-harmonic beamforming on two polarization directions based on the anisotropic STC digital metasurface

To experimentally validate the dynamic dual-harmonic beamforming along two polarization directions, we measure the harmonic far-field scattering patterns for different control signals. The measurement setup for beamforming on the xoz plane is presented in Supplementary Fig. S3a, where a microwave signal generator (Agilent E8267D) and spectrum analyzer (Agilent E4447A) are used to provide the incident wave at 2.7 GHz and receive the scattering harmonic energy, respectively, with the help of single-polarized horn antennas connected with the phase stable cables in microwave anechoic chamber. The metasurface is fixed to a swivel table rotating on the horizontal plane, and a customized control platform provides the control signal sequences for each column. All instruments are synchronized by the phase stable cables, and the metasurface sample and two horn antennas are placed at the same height. For measuring the beamforming on the yoz plane (Supplementary Fig. S3b), we simply flipped the metasurface by 90° and loaded the corresponding control signals towards each row of the metasurface, while the other configurations remain unchanged.

Supplementary Figure S3 | Experimental setup for measuring the dynamic dual-harmonic beamforming along two polarization directions. Measurement setups for beamforming on the (a) xoz, and (b) yoz planes.

Supplementary Note 6: Constellation diagrams and BERs for Users 1-4

The measured results of the constellation diagrams and BERs for Users 1-4 are presented in Supplementary Fig. S4. From the results, we observe that Users 1 and 3 can demodulate nearly perfect QPSK constellation points and have lower BERs; however, Users 2 and 4 have deteriorate constellation points and larger BERs. As a consequence, Users 1 and 3 can receive excellent communication quality, while Users 2 and 4 receive relatively good communication quality.

Supplementary Figure S4 | BERs for Users 1-4 according to the measured transmitting powers through the space-frequency-polarization-division multiplexed wireless communication system, and the corresponding constellation diagrams (insets). (a) Results for Users 1 and 2. (b) Results for Users 3 and 4.

Supplementary References

[1]	Dai, J. Y. et al. High-efficiency synthesizer for spatial waves based on space-time-coding digital metasurface. Laser Photon. Rev. 14, 1900133 (2020).

[2]	https://www.skyworksinc.com/-/media/SkyWorks/Documents/Products/101-200/SMV 1405_1430_Series_200068W.pdf.