Supplementary material

S1. Batch adsorption equations

The adsorption capacity at any time (qt, mg g-1), adsorption capacity at the equilibrium (qe, mg g-1), and removal percentage (R, %) values were calculated by the Equations (1), (2), and (3), respectively:

| qt = C0–Ct Vm | (1) |
| --- | --- |
| qe = C0–Ce Vm | (2) |
| R = C0–CtC0 100 | (3) |

Where: C0 is the initial adsorbate concentration in the liquid phase (mg L–1), Ce is the equilibrium concentration of the adsorbate in the liquid phase (mg L–1), Ct is the adsorbate concentration in the liquid phase at any time (mg L–1), m is the mass of adsorbent (g), and V is the volume of solution (L).

S.2 Kinetic equations

The pseudo-first order model (Eq.4), pseudo-second order model (Eq.6), Elovich (Eq. 7), and Avrami (Eq. 8)

| qt=q1 (1-exp(-k1t)) | (4) |
| --- | --- |
| qt=t1k2q22+tq2 | (5) |
| qt = 1aln1+abt | (6) |
| qt=qAv (1-exp(-kAvt)nAv) | (7) |

Being, k1 (min-1), k2 (g mg-1 min-1) and kav (min-1) as pseudo-first-order, pseudo-second-order, and Avrami speed constants, respectively; ho the initial adsorption rate (mg g-1min-1); q1, q2, and qav, the theoretical values for the adsorption capacity (mg g-1); the initial speed (dq/dt) when qt = 0 (mg g-1 min-1), b desorption constant of the Elovich model (g mg-1) and t the time (min); nav the exponent in the Avrami model.

S3. Isotherm models

The Langmuir model (Eq. 8), Freundlich model (Eq. 9), and Dubinin–Radushkevich model (Eq. 10) are presented as follows:

| qe = qLKLCe1+KLCe | (8) |
| --- | --- |
| qe = KFCe1nf | (9) |
| qe = qmDRexp-βε2 | (10) |
| ε = RTlnCSCe | (11.a) |

Where: qL is the maximum adsorption capacity of Langmuir model (mg g-1), KL is the Langmuir constant (L mg-1); KF is the Freundlich constant ((mg g–1) (mg L–1)–1/n); 1/nf (dimensionless) is the heterogeneity factor; β is the Dubinin–Radushkevich constant (mol2 kJ-2); ε is the Polanyi potential; R is the universal gas constant (8,314 J mol-1 K-1); CS is the naproxen (NPX) solubility; T is the temperature (K); a is the Tempkin isotherm constant (L mg-1); b is the heat of adsorption (J mol-1).

S4. Thermodynamics estimation

The Gibbs free energy (ΔG0, kJ mol-1), enthalpy change (ΔH0, kJ mol-1), and entropy change (ΔS0, kJ mol-1 K-1) were calculated by the following Equations:

| Ke = KMWγXΓ | (12) |
| --- | --- |
| ∆G0 = -RTlnKe | (13) |
| ∆G0 = ∆H0-T∆S0 | (14) |
| lnKe = ∆S0R-∆H0RT | (15) |

K is the constant parameter from the most suitable isotherm fit (L mg-1), Ke is the equilibrium constant (dimensionless), Mw is the methylene blue molecular weight (mg mol-1), γX  is the activity coefficient of the diuron (dimensionless, assuming γX=1), Γ is the unitary activity coefficient of the diuron (1 mol L-1), T is the temperature (K), and R is the universal gas constant (8.31 × 10-3 kJ mol-1 K-1).

S5 Dynamic models

The dynamic models were selected to predict the breakthrough curve. The Yoon–Nelson (Eq. 16), and Thomas (Eq. 17):

| CtC0=1+expkYNτ-t-1 | (16) |
| --- | --- |
| CtC0=1+expmadskthqeqQ-kthC0t-1 | (17) |

where kYN is the constant rate of Yoon–Nelson (min–1), τ is the time required for 50% adsorbate breakthrough from Yoon–Nelson model (min), kth is the constant of Thomas (mL mg–1 min–1), qeq is the adsorption capacity of the Thomas model (mg g–1).

S6. Statistical evaluation

The determination coefficient (R2) (Eq. 18), adjusted determination coefficient (R2adj) (Eq. 19), average relative error (ARE) (Eq. 20), and mean square error (MSR) (Eq. 21) are given by the following Equations:

| R2 = 1-i=1nyexp-ypred2i=1nyexp-yexp2 | (18) |
| --- | --- |
| Radj2 = 1-1-R2n-1n-p-1 | (19) |
| ARE = 100%ni=1nyexp-ypredyexp | (20) |
| MSR = 1n-pi=1nyexp-ypred2 | (21) |

where: yexp is the experimental value; ypred is the predicted value; n is the number of experimental values; p is the number of parameters according to the model.

Table S1

| Inorganic dyes and compounds | Concentration (mg L-1) | λmáx (nm) |
| --- | --- | --- |
| Methylene blue | 20 | 664 |
| Crystal violet | 10 | 590 |
| Basic fuchsin | 10 | 525 |
| Red Procion | 10 | 544 |
| Rhodamine b | 10 | 610 |
| Malachite green | 10 | 615 |
| Bright blue | 10 | 630 |
| Red Ponceau | 10 | 508 |
| Sodium chloride (NaCl) | 100 |  |
| Sodium carbonate (Na2CO3) | 100 |  |

Table S2

| Adsorvent | pH | T (K) | qmáx (mg g-1) | Reference |
| --- | --- | --- | --- | --- |
| Porous activated kaolinite (AKaol) | 10 | 353 | 171.0 | [1] |
| Barley malt bagasse biochar | 6.5 | 298 | 161 | [2] |
| Glycerin waste as the precursor for activated carbon production | 7 | 297 | 370 | [3] |
| Mesoporous silica from coal gasification fine Slag | 7.88 | 293 | 140.57 | [4] |
| Wheat straw biochar | 7 | 298 | 62.5 | [5] |
| Hydrolyzed polyacrylamide modified diatomite waste | 9 | 303 | 37.12 | [6] |
| Kaolin | 6 | 323 | 52.76 | [7] |
| Natural clay | 7 | 323 | 100 | [8] |
| Ti in vanadium doped anatase titania | 7 | 293 | 56.64 | [9] |
| Algerian palygorskite | 7 | 298 | 57.47 | [10] |
| Foliar powder of the species Sansevieria trifasciata | 10 | 298 | 225.8 | This work |

References

[1]	S. Asuha, F. Fei, W. Wurendaodi, S. Zhao, H. Wu, X. Zhuang, Activation of kaolinite by a low-temperature chemical method and its effect on methylene blue adsorption, Powder Technol. 361 (2020) 624–632. https://doi.org/10.1016/j.powtec.2019.11.068.

[2]	M.A. Franciski, E.C. Peres, M. Godinho, D. Perondi, E.L. Foletto, G.C. Collazzo, G.L. Dotto, Development of CO2 activated biochar from solid wastes of a beer industry and its application for methylene blue adsorption, Waste Manag. 78 (2018) 630–638. https://doi.org/10.1016/j.wasman.2018.06.040.

[3]	M. Gonçalves, C.S. Castro, I.K.V. Boas, F.C. Soler, E.D.C. Pinto, R.L. Lavall, W.A. Carvalho, Glycerin waste as sustainable precursor for activated carbon production: Adsorption properties and application in supercapacitors, J. Environ. Chem. Eng. 7 (2019) 103059. https://doi.org/10.1016/j.jece.2019.103059.

[4]	S. Liu, X. Chen, W. Ai, C. Wei, A new method to prepare mesoporous silica from coal gasification fine slag and its application in methylene blue adsorption, J. Clean. Prod. 212 (2019) 1062–1071. https://doi.org/10.1016/j.jclepro.2018.12.060.

[5]	G. Li, W. Zhu, C. Zhang, S. Zhang, L. Liu, L. Zhu, W. Zhao, Effect of a magnetic field on the adsorptive removal of methylene blue onto wheat straw biochar, Bioresour. Technol. 206 (2016) 16–22. https://doi.org/10.1016/j.biortech.2015.12.087.

[6]	T. Ma, Y. Wu, N. Liu, Y. Wu, Hydrolyzed polyacrylamide modified diatomite waste as a novel adsorbent for organic dye removal: Adsorption performance and mechanism studies, Polyhedron. 175 (2020) 114227. https://doi.org/10.1016/j.poly.2019.114227.

[7]	L. Mouni, L. Belkhiri, J.C. Bollinger, A. Bouzaza, A. Assadi, A. Tirri, F. Dahmoune, K. Madani, H. Remini, Removal of Methylene Blue from aqueous solutions by adsorption on Kaolin: Kinetic and equilibrium studies, Appl. Clay Sci. 153 (2018) 38–45. https://doi.org/10.1016/j.clay.2017.11.034.

[8]	O. Sakin Omer, M.A. Hussein, B.H.M. Hussein, A. Mgaidi, Adsorption thermodynamics of cationic dyes (methylene blue and crystal violet) to a natural clay mineral from aqueous solution between 293.15 and 323.15 K, Arab. J. Chem. 11 (2018) 615–623. https://doi.org/10.1016/J.ARABJC.2017.10.007.

[9]	K. Pal, K. Ghorai, S. Aggrawal, T.K. Mandal, P. Mohanty, M.M. Seikh, A. Gayen, Remarkable Ti-promotion in vanadium doped anatase titania for methylene blue adsorption in aqueous medium, J. Environ. Chem. Eng. 6 (2018) 5212–5220. https://doi.org/10.1016/j.jece.2018.08.015.

[10]	L. Dali Youcef, L.S. Belaroui, A. López-Galindo, Adsorption of a cationic methylene blue dye on an Algerian palygorskite, Appl. Clay Sci. 179 (2019) 105145. https://doi.org/10.1016/j.clay.2019.105145.

Table S3

| Temperature (K) | ∆G0 (kJ mol–1) | ∆H0 (kJ mol–1) | ∆S0 (kJ mol–1 K–1) |
| --- | --- | --- | --- |
| 298 | -22.4046 | -3.7304 | 0.0626 |
| 308 | -23.0309 |  |  |
| 318 | -23.6573 |  |  |
| 328 | -24.2836 |  |  |