ELISA protocol

To detect the serum, 0.1 μg of DHAV-1 or 0.1 μg of DHAV-1s individually, in 100 μL of a bicarbonate carbonate buffer (pH 9.6), and plates incubated for 16 h at 4 °C. After three washes with PBS-T, the plates were blocked by 10 mM phosphate buffered saline solution containing 0.05% Tween-20 (PBST, pH 7.4)   plus   5% skimmed milk for 2 h at 37°C. Then, the plates were washed three times with PBST, and 100 μL of duck serum (1:100 diluted in PBST + 5% skimmed milk) was added in duplicate. After 1 h of incubation at 37°C and three washes with PBST, 100 μL of HRP-conjugated Anti-Duck IgG (H+L) Antibody (Kirkegaard & Perry Labs; Gaithersburg, MD, USA) diluted at 1:4000 in PBST + 5% skimmed milk, were added to the plates for 1 h at 37°C. The reaction was developed with 100 μL of a Developing-Coloring Solution ( Boster Biological Technology co.ltd) at room temperature in the dark for 15 min. Subsequently, the reaction was stopped with 100 μL of 0.5 M H2SO4 solution. Absorbance was measured at 450 nm using a microtiter plate reader (ELx; BioTek). The corresponding positive value (P) was approximately 1.0, the negative value (N) was less than 0.2. The serum sample with the P/N less than 2.1 was considered to be DAV-1 or DHAV-1s-negative.