Supplementary Section

1. MR imaging acquisition

Table S1 Parameters of MRI scanning from seven hospitals

| Hospitals | Sequences | Vendor | MRI strength | B value(sec/mm2) | Slice thickness (mm) | Spacing between slices (mm) | Echo time(s) | Repetition time(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUH 1st | T2WI, DWI, ADC | Siemens Skyra | 3T | 50/70/1500/2000 | 3 | 3/3.45 | 60/104 | 6540/7590 |
| SUH 2nd | T2WI, DWI, ADC | Philips Ingenia | 3T | 10/20/50/100/200/1000/2000 | 1.5/2/3/3.4/3.5/3.7/ 3.8/3.9/4/4.1/4.2/4.3/5 | 1.65/3/3.2/3.3/3.4/3.5/3.7/ 3.8/3.9/2.2/4/4.1/4.2/4.3/5 | 77/78/100 | 4542/4828/4898/ 4733/4972/ 6000 |
| ZJGH | T2WI, DWI, ADC | Philips achieva | 3T | 0/400/800/1500 | 3 | 3.3 | 62/75 | 20004342//6118/ 6173/6498/6489 |
| SKH | T2WI, DWI, ADC | Siemens Skyra | 3T | 400/800/1200 | 3.5/4 | 3.5/4 | 72/97 | 3900/4342/6118/ 6173/6489/6498/ 7500 |
| TZH | T2WI, DWI, ADC | Siemens Skyra and Vero | 3T | 0/50/800/1000/1500 | 3.5/4/5/5.5 | 3.5/4/4.8/6/6.6 | 62/64/74/97/104 | 4480/5000/5100/ 7500/8600 |
| SQH | T2WI, DWI, ADC | Philips Ingenia | 3T | 0/500/1000/1400/2000 | 3/4 | 3.5/4.4 | 68/80/110 | 4473/4176/4564/ 6600/6700 |
| CSH | T2WI, DWI, ADC | Philips Achieva TX | 3T | 0, 1000, 2000 | 3 | 3 | 76/80 | 2750/3000 |

Abbreviation: T2WI, T2-weighted imaging; DWI, diffusion-weighted imaging; ADC, Apparent diffusion coefficient; SUH 1st, the First Affiliated Hospital of Soochow University; SUH 2nd, the Second Affiliated Hospital of Soochow University; ZJGH, the Affiliated Zhangjiagang Hospital of Soochow University; SKH, Suzhou Kowloon Hospital; TZH, the People's Hospital of Taizhou; SQH, the People's Hospital of Suqian; CSH, Changshu NO.1 People's Hospital.

2. PI-RADS assessment

All prostate MRI examinations were divided into five groups and performed on the index lesions based on T2WI, DWI/ADC and DCE imaging according to Prostate Imaging and Reporting and Data System (PI-RADS) version 2.1 [1] respectively by five board-certified radiologists from SUH 1st and SUH 2nd (reader 1 [J.B.] with 5 years of experience, reader 2 [X.M.Q.] with 3 years of experience, reader 3 [P.F.J.] with 4 years of experience, reader 4 [Y.Y.Z.] with 5 years of experience, reader 5 [Y.T.J.] with 5 years of experience). They all accessed to clinical details including age, PSA level, digital rectal examination, and other risk factors like family history and routine habits. Then, all index lesions were re-divided into two groups, for which the results of PI-RADS assessment of the five radiologists were checked respectively by two expert-level radiologists from SUH 1st and SUH 2nd (reader 6 [X.M.W.] with 18 years of experience; reader 7 [J.K.S.] with 22 years of experience). The expert-level radiologists also accessed the same clinical details as those accessed by the five radiologists. Any disagreement between the five radiologists and the two expert-level radiologists was discussed until a consensus was reached. All readers were the members of the institution’s prostate disease management team with 3-22 years’ experience in prostatic MRI and had read more than 1000 prostatic MRI scans using a PI-RADS scoring system by the time of the present study. All readers were blind to the pathological diagnosis information.

3. Histological review

Needle biopsies obtained by TRUS and MRGB guidance were independently reviewed by experienced urological pathologists in respective hospitals. In this procedure, a “two-step-checking” scheme was used to avoid potential bias, wherein a junior pathologist first read the prostate specimens, and then the reports were checked by a senior pathologist. If there was disagreement, the reports were discussed until consensus was reached. Each core containing cancer was assigned a primary and a secondary Gleason grade group. For the patients with radical prostatectomy (RP), the prostatic specimens were uniformly processed and submitted for histological investigation.

4. MRI annotation

Entire volumes of interest (VOIs) of lesion was manually delineated on T2WI using the open-source ITK-SNAP software (http://www.itksnap.org/pmwiki/pmwiki.php, version 3.8.0) based on histopathologic-imaging matching by the above mentioned five board-certified radiologists. For each of patient with RP, first, histopathological specimens were assembled into pseudo-whole-mount sections. Then, urological pathologists marked the location of the lesion on the sections, which was then coregistered to the patients’ MRI images. In this way, regions of annotated PCa via histological examination were mapped onto the MRI images to produce the ground truth maps. And the references standard for Gleason grade was based on the RP finding. For patients without RP, the reference standard for Gleason grade group was based on the biopsy findings using MRI/TRUS-fusion targeted biopsy followed by 11-gauge core systematic needle biopsy. A challenge in image labeling is the presence of ambiguous regions, where the true tumor boundary cannot be deduced from the MRI. To fill this gap, the VOI of each lesion was drawn twice by each of the five board-certified independent radiologists. When patients had multiple lesions, only the index lesion with the highest Gleason score or with the largest size (if same Gleason score) was assessed.

5. Data preprocessing

According to the purpose of the present study, image preprocessing was performed on T2WI, high b value DWI images and ADC maps derived from DWI images.

(1) Data de-identification

The origin format of data in the present study was Digital Imaging and Communications in Medicine (DICOM), which contained patients’ private information such as name and address. The patients’ specific sensitive information was removed by converting data to Neuroimaging Informatics Technology Initiative (NIFTI) format, abiding by the requirement of data de-identification in the U.S. Health Insurance Portability and Accountability Act (HIPAA), and the E.U. General Data Protection Regulation (GDPR).

(2) Image registration

In the present study, for each patient, both DWI images and ADC maps were registered to T2WI images, respectively. The aim of the registration was to reduce the motion-induced misalignments and remove the difference in the spatial resolution such as voxel size and slice thickness between T2WI and DWI images. The image registration was performed with the SimpleElastix toolbox (http://simpleelastix.github.io/) by applying translation registration method based on the Advanced Mattes Mutual Information criterion.

(3) Multicentre data harmonization

In the present retrospective study, the multicentre data were acquired with different parameters and by the MRI scanners of different manufactures, leading to the inter-center variation in the image spatial resolution and image intensity. The present study employed a data harmonization method to reduce this inter-center image difference. Specifically, first, 3D-ROI containing intra-tumoral and part of peritumoral tissues in each sequence were extracted as an independent image (3D-ROI image). The peritumoral information included some of tumor microenvironment such as tumor neovascularization, lymphatic vessels, and tumor-related stroma or cells. All 3D-ROI images were interpolated into ones with the common voxel size which met the input requirements of the deep learning network. Then, for each patient, the 3D-ROI images were z-score normalized by subtracting their means and then being divided by their standard deviations, respectively.

(4) Data augmentation

Data augmentation was performed only for the images of the training cohort except tuning dataset to enrich data diversity at training step, therefore alleviating the overfitting and improving the generalization of the deep learning models. In the present study, the basic augmentation operations for images comprised of translation in random direction and rotation at random angle. In the training cohort, the augmented images replaced their respective original images to train the deep learning models. In contrast, the remaining tuning dataset in the training cohort and the external validation cohorts without data augmentation were used for the model tuning and testing, respectively.

6. Construction of deep learning signature of PIDL-CS

In the present study, details of the AUC, accuracy, sensitivity and specificity for DL-CS models in tuning dataset were summarized in Table S2. The DL-CS model based on ResNet3D (DL-CS-Res) shows higher AUC, accuracy, sensitivity and specificity values than those of the models respectively based on other networks. Then, we constructed an integrated model (PIDL-CS) combining the deep learning signature of DL-CS-Res and PI-RADS score using logistic regression for the detection of csPCa.

Table S2 Performance of DL-CS models in the tuning dataset (probability threshold = 0.5)

| DL-CS models | AUC (95%CI) | Accuracy (%) | Sensitivity (%) | Specificity (%) |
| --- | --- | --- | --- | --- |
| ResNet3D | 0.922(0.868,0.975) | 90.2(110/122) | 89.5(34/38) | 90.5(76/84) |
| DenseNet3D | 0.893(0.828,0.957) | 84.4(103/122) | 78.9(30/38) | 86.9(73/84) |
| ShuffleNet3D | 0.856(0.776,0.935) | 83.6(102/122) | 81.6(31/38) | 84.5(71/84) |
| MobileNet3D | 0.922(0.869,0.974) | 89.3(109/122) | 86.8(33/38) | 90.5(76/84) |

Abbreviation: AUC, area under receiver operating characteristic curve; DL-CS, deep learning models for the classification between clinically significant and non-clinically significant prostate cancer.

7. Selection of the probability threshold values of DL-CS-Res and PIDL-CS

In the present study, the PI-RADS threshold for the detection of csPCa was ≥ 3. To compare the specificities of the DL-CS-Res and PIDL-CS with that of PI-RADS assessment, the probability thresholds of these classification models were set, so that the absolute difference in the sensitivity between the classification model (i.e., DL-CS-Res or PIDL-CS) and PI-RADS assessment with threshold ≥ 3 was the smallest, whereas the specificity of this model was the largest under this sensitivity in the tuning dataset.

As shown in Table S3, in tuning dataset, PI-RADS assessment had a sensitivity of 100% (38/38) and specificity of 44.0% (37/84) for the detection of csPCa with the threshold of PI-RADS score ≥ 3. Corresponding to this threshold, the probability threshold of the DL-CS-Res for the detection of csPCa was set as ≥ 0.27, with which the DL-CS-Res showed the same sensitivity as that of PI-RADS assessment 100.0% (38/38), and at the same time had the largest specificity 50.0% (42/84) in the tuning dataset. Fig S1(a) showed the threshold points of the detection of csPCa for PI-RADS and DL-CS-Res in the tuning dataset, respectively. This probability threshold was used to evaluate the performances of DL-CS-Res in each external validation cohort.

As shown in Table S3, in tuning dataset, PIDL-CS showed the same sensitivity as that of PI-RADS assessment 100.0% (38/38), and at the same time had the largest specificity 76.2% (64/84) in the tuning dataset. Fig S1(b) showed the threshold points of the detection of csPCa for PI-RADS and PIDL-CS in the tuning dataset, respectively. This probability threshold was used to evaluate the performances of PIDL-CS in each external validation cohort.

Table S3 Sensitivity and specificity of DL-CS-Res, PIDL-CS and PI-RADS assessment for the detection of csPCa in the tuning dataset

| Model and model threshold | Sensitivity (%) | P value | Specificity (%) | P value |
| --- | --- | --- | --- | --- |
| Radiologists PI-RADS ≥ 3 | 100.0(38/38) | Reference | 44.0(37/84) | Reference |
| DL-CS-Res Threshold ≥ 0.27† | 100.0(38/38) | > 0.99 | 50.0(42/84) | 0.49 |
| PIDL-CS Threshold ≥ 0.35† | 100.0(38/38) | > 0.99 | 76.2(64/84) | < 0.001* |

Note: * Significant (P < 0.05)

† Indicates comparison to PI-RADS with threshold of ≥ 3

Abbreviation: DL-CS-Res, the deep learning model based on ResNet3D network for the classification between clinically significant and non-clinically significant prostate cancer; PI-RADS, Prostate Imaging and Reporting and Data System; PIDL-CS, integrated model combining DL-CS-Res and PI-RADS score.

Fig. S1 Threshold points of DL-CS-Res, PIDL-CS and PI-RADS assessment for the detection of csPCa in the tuning dataset. a The threshold points of DL-CS-Res (green circle) and PI-RADS assessment (green star) for the detection of csPCa in the tuning dataset; b The threshold points of PIDL-CS (green circle) and PI-RADS assessment (green star) for the detection of csPCa in the tuning dataset. Receiver operating characteristics curves of DL-CS-Res and PIDL-CS are red lines, and that of PI-RADS assessment are blue lines.

* Significant (P < 0.05)

Abbreviation: DL-CS-Res, the deep learning model based on ResNet3D network for the classification between clinically significant and non-clinically significant prostate cancer; PI-RADS, Prostate Imaging and Reporting and Data System; PIDL-CS, integrated model combining DL-CS-Res and PI-RADS score.

8. Comparison of performance between deep learning models in external validation cohorts

The results of the paired comparisons of the ROC curves among the DL-BM models (Table S4) and those among the DL-CS models (Table S5) in each external validation cohort. Not significant difference (Ps > 0.05, except ShuffleNet3D > MobileNet3D in TZH external validation [P = 0.02]) was observed for each paired comparison among the DL-BM models for the detection of PCa (Table S4). Not significant difference (Ps > 0.05, except ResNet3D > DenseNet3D in TZH external validation [P = 0.04]) was observed for each paired comparison among the DL-CS models for the detection of csPCa (Table S5).

Table S4 Paired comparisons of the ROC curves among the DL-BM models in each external validation cohort

| The pairs of comparison |  | P value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SKH | TZH | CSH+SQH |
| ShuffleNet3D vs ResNet3D |  | 0.41 | 0.47 | 0.64 |
| ShuffleNet3D vs DenseNet3D |  | 0.88 | 0.06 | 0.27 |
| ShuffleNet3D vs MobileNet3D |  | 0.35 | 0.02* | 0.24 |
| ResNet3D vs DenseNet3D |  | 0.23 | 0.30 | 0.06 |
| ResNet3D vs MobileNet3D |  | 0.93 | 0.37 | 0.16 |
| DenseNet3D vs MobileNet3D |  | 0.43 | 0.98 | > 0.99 |

Note: P value were calculated by Delong test

* Significant (P < 0.05)

Abbreviation: DL-BM, deep learning models for the classification between benign and malignant lesions; SKH, Suzhou Kowloon Hospital; TZH, the People's Hospital of Taizhou; SQH, the People's Hospital of Suqian; CSH, Changshu NO.1 People's Hospital; ROC, receiver operating characteristics.

Table S5 Paired comparisons of the ROC curves among the DL-CS models in each external validation cohort

| The pairs of comparison |  | P value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SKH | TZH | CSH+SQH |
| ResNet3D vs DenseNet3D |  | 0.70 | 0.04* | 0.86 |
| ResNet3D vs ShuffleNet3D |  | 0.45 | 0.89 | 0.13 |
| ResNet3D vs MobileNet3D |  | 0.50 | 0.38 | 0.20 |
| DenseNet3D vs ShuffleNet3D |  | 0.26 | 0.14 | 0.10 |
| DenseNet3D vs MobileNet3D |  | 0.64 | 0.56 | 0.17 |
| ShuffleNet3D vs MobileNet3D |  | 0.15 | 0.46 | 0.43 |

Note: P value were calculated by Delong test

* Significant (P < 0.05)

Abbreviation: DL-CS, deep learning models for the classification between clinically significant and non-clinically significant prostate cancer; SKH, Suzhou Kowloon Hospital; TZH, the People's Hospital of Taizhou; SQH, the People's Hospital of Suqian; CSH, Changshu NO.1 People's Hospital; ROC, receiver operating characteristics.

References

1.	Turkbey B, Rosenkrantz AB, Haider MA, Padhani AR, Villeirs G, Macura KJ, et al. Prostate Imaging Reporting and Data System Version 2.1: 2019 Update of Prostate Imaging Reporting and Data System Version 2. Eur Urol. 2019;76:340-51. https://doi.org/10.1016/j.eururo.2019.02.033.