SUPPORTING INFORMATION:

Techno-economic and Life Cycle Analysis of Mixalco® Processes for Mixed Alcohol Production from Brown Algae

Abhay Athaley1, Yue Zhang2, Eleftherios T. Papoutsakis1, and Marianthi Ierapetritou*1

1. Department of Chemical & Bio-molecular Engineering, University of Delaware, Newark, Delaware, United States

2. Department of Chemical Engineering, University of California, Davis, California, United States

Total Supp. Pages: 14 Total Supp. Tables: 4 Total Supp. Figures: 4

TABLE OF CONTENTS – Sections

- Detailed Reactor Information							S02

- LCA input data summary							S03

- Capital and Operating Cost for the processes					S04

TABLE OF CONTENTS – Tables

Table S1			S02

Table S2			S03

Table S3			S04

Table S4			S05

Table S5			S06

Table S6			S07

Table S1: Summary of reactor details

| KR |  |  |  |
| --- | --- | --- | --- |
| Name | Residence time (min) | Flow rate (L/min) | Reactor Capacity (liter) |
| Fermentation R1 | 7200.0 | 9869.5 | 852724800.0 |
| Ketone R1 | 1800.0 | 961.6 | 1730853.7 |
| Hydrogenation R2 | 9.2 | 543.0 | 83.6 |
| Hydrogenation R3 | 13.0 | 588.2 | 127.8 |
| Hydrogenation R4 | 12.5 | 609.5 | 127.0 |
| ER |  |  |  |
| Fermentation R1 | 7200.0 | 9704.5 | 838468800.0 |
| Ester R1 | 5.0 | 1630.3 | 8151.5 |
| Hydrogen R2 | 5.0 | 1320.0 | 6600.0 |
| KR+SBC |  |  |  |
| Fermentation R1 | 7200.0 | 8600.0 | 743040000.0 |
| Ketone R1 | 1800.0 | 2265.1 | 4077174.6 |
| Hydrogenation R2 | 9.2 | 654.0 | 100.6 |
| Hydrogenation R3 | 13.0 | 791.0 | 171.8 |
| Hydrogenation R4 | 12.5 | 885.0 | 184.4 |
| NF R1 | 4440.0 | 1239.0 | 66013920.0 |
| NF R2 | 9600.0 | 1589.0 | 183052800.0 |
| ER+SBC |  |  |  |
| Fermentation R1 | 120.0 | 9700.0 | 13968000.0 |
| Ester R1 | 5.0 | 1630.0 | 8150.0 |
| Hydrogen R2 | 5.0 | 1320.0 | 6600.0 |
| NF R1 | 4440.0 | 2578.7 | 137393136.0 |
| NF R2 | 9600.0 | 4097.4 | 472020480.0 |

Table S2: LCA input data summary

| Route | KR | KR+NF | ER | ER+NF |
| --- | --- | --- | --- | --- |
| Input (kg/hr) |  |  |  |  |
| Biomass | 62500.0 | 62500.0 | 62500.0 | 62500.0 |
| CaCO3 | 1000.0 | 2501.3 | 0.0 | 0.0 |
| H2O | 60031.1 | 870535.0 | 81914.6 | 1159010.1 |
| H2 | 338.7 | 1968.5 | 1095.2 | 5100.0 |
| Ethanol | 9.7 | 19845.6 | 9.7 | 33243.0 |
| Iodoform | 0.2 | 0.2 | 0.2 | 0.2 |
| Heptanol | 0.0 | 0.0 | 197.8 | 390.0 |
| ABC | 0.0 | 0.0 | 3999.9 | 5505.5 |
| Output (kg/hr) |  |  |  |  |
| Mixed Alcohol | 11262.5 | 28288.7 | 16519.2 | 53599.2 |
| WasteWater | 6109.4 | 18976.4 | 30762.6 | 71349.0 |
| CO2 to water | 44.3 | 322.6 | 401.9 | 2766.3 |
| H2 | 400.8 | 22.0 | 401.9 | 37.6 |
| CO2 to air | 16179.0 | 1357.0 | 32.2 | 4042.6 |
| Utilities (kJ/hr) |  |  |  |  |
| Air | 857338376.4 | 4161444.1 | 0.0 | 0.0 |
| Cooling Water | 221089075.1 | 4706011951.7 | 1141145756 | 2108395794.4 |
| Fired Heat (1000) | 16060.0 | 46818431.6 | 29292037.72 | 181877266.4 |
| HP Steam | 334579617.8 | 0.0 | 843489144.8 | 1359892712 |
| Refrigerant 1 | 261471.2 | 285144319.1 | 0 | 285144319.1 |
| Electricity (KWh) | 14585047.2 | 29159103.6 | 5915307.6 | 37216159.2 |

Table S3: Capital and Operating cost summary for KR process

| Item | Cost (million $) | Item | Cost (million $/yr.) |
| --- | --- | --- | --- |
| Purchased Equipment | 41.15 | Algae | 39.99 |
| Heaters | 7.42 | CaCO3 | 16.00 |
| Equipment Setting | 0.05 | Ethanol | 0.07 |
| Piping | 3.36 | H2 | 4.06 |
| Civil | 2.31 | Catalyst | 13.98 |
| Steel | 1.05 | Heat Utility | 17.68 |
| Instrumentation | 3.20 | Electricity | 3.99 |
| Electrical | 2.76 | Other Operating cost (labor, maintenance) | 12.82 |
| Insulation | 1.96 | Total Operating cost | 108.59 |
| Paint | 0.18 |  |  |
| Other | 13.62 |  |  |
| GandA Overheads | 1.98 |  |  |
| Contract Fee | 2.00 |  |  |
| Contingencies | 13.25 |  |  |
| Initial catalyst | 6.99 |  |  |
| Total Project Cost | 101.28 |  |  |

Table S4: Capital and Operating cost summary for KR+SBC process

| Item | Cost (million $) | Item | Cost (million $/yr.) |
| --- | --- | --- | --- |
| Purchased Equipment | 44.75 | Algae | 39.99 |
| Heaters | 7.69 | CaCO3 | 40.01 |
| Equipment Setting | 0.07 | Ethanol | 137.95 |
| Piping | 3.76 | Enzyme | 0.22 |
| Civil | 2.53 | H2 | 23.62 |
| Steel | 0.98 | Catalyst | 32.74 |
| Instrumentation | 3.17 | Heat Utility | 15.06 |
| Electrical | 2.76 | Electricity | 5.00 |
| Insulation | 1.97 | Other Operating cost (labor, maintenance) | 27.97 |
| Paint | 0.18 | Total Operating cost | 322.56 |
| Other | 14.71 |  |  |
| G and A Overheads | 2.12 |  |  |
| Contract Fee | 2.13 |  |  |
| Contingencies | 14.25 |  |  |
| Indicial catalyst | 16.37 |  |  |
| Total Project Cost | 117.44 |  |  |

Table S5: Capital and Operating cost summary for ER process

| Item | Cost (million $) | Item | Cost (million $/yr.) |
| --- | --- | --- | --- |
| Purchased Equipment | 48.89 | Algae | 39.99 |
| Heaters | 22.88 | Ethanol | 0.07 |
| Equipment Setting | 0.06 | H2 | 13.14 |
| Piping | 3.50 | Catalyst | 17.72 |
| Civil | 2.21 | Heat Utility | 18.13 |
| Steel | 0.97 | Electricity | 3.64 |
| Instrumentation | 3.00 | Other Operating cost (labor, maintenance) | 8.73 |
| Electrical | 2.68 | Total Operating cost | 101.42 |
| Insulation | 1.89 |  |  |
| Paint | 0.18 |  |  |
| Other | 13.15 |  |  |
| G and A Overheads | 1.85 |  |  |
| Contract Fee | 1.90 |  |  |
| Contingencies | 12.45 |  |  |
| Indicial catalyst | 8.86 |  |  |
| Total Project Cost | 124.48 |  |  |

Table S5: Capital and Operating cost summary for ER+SBC process

| Item | Cost (million $) | Item | Cost (million $/yr.) |
| --- | --- | --- | --- |
| Purchased Equipment | 77.72 | Algae | 39.99 |
| Heaters | 37.82 | Enzyme | 0.52 |
| Equipment Setting | 0.09 | Ethanol | 255.26 |
| Piping | 5.51 | H2 | 61.19 |
| Civil | 3.78 | Catalyst | 56.02 |
| Steel | 1.56 | Heat Utility | 35.20 |
| Instrumentation | 4.17 | Electricity | 6.41 |
| Electrical | 3.31 | Other Operating cost (labor, maintenance) | 40.94 |
| Insulation | 3.10 | Total Operating cost | 495.53 |
| Paint | 0.28 |  |  |
| Other | 20.33 |  |  |
| G and A Overheads | 3.11 |  |  |
| Contract Fee | 2.97 |  |  |
| Contingencies | 20.67 |  |  |
| Indicial catalyst | 28.01 |  |  |
| Total Project Cost | 212.43 |  |  |