Additional File 1: Code S1. Arduino code for hardware of the thermoacoustic sensor-based system for detection of respiratory diseases in chicken

#include <Wire.h>                //initiates wire library and allows communication with I2C/TWI devices

#include <LiquidCrystal_I2C.h>      // it’s a library for I2C display

#include "SoftwareSerial.h"             //is a library that enables serial communication with a digital pin other than          the serial port

SoftwareSerial esp(10, 11);// RX3, TX2

String uri_short = "/cathy/v1/post/hardware.php?data=1";

String uri;

// Set the LCD address to 0x27 for 16 chars and 2 line display

LiquidCrystal_I2C lcd(0x27, 16, 2);

#include <OneWire.h>

#include <DallasTemperature.h>      // arduino library for Dallas Temperature ICs

#define ONE_WIRE_BUS 2          // data wire is connected to the arduino digital pin 2

OneWire oneWire(ONE_WIRE_BUS);

int buzer=12;

DallasTemperature sensors(&oneWire);

float 	Celcius=0;

float 	Fahrenheit=0; void setup(void)

{   lcd.begin();

// 	Turn 	on 	the 	blacklight 	and 	print 	a 	message.   lcd.backlight();

lcd.print("Sensor Bsed Resp");   lcd.setCursor(0,1);

lcd.print("DDS IN POULTRY");  esp.begin(9600);

Serial.begin(9600);  connectWifi();   sensors.begin();

pinMode(buzer,OUTPUT);

}

void loop(void)

{

String state;   int sound = analogRead(A6);   sensors.requestTemperatures();    Celcius=sensors.getTempCByIndex(0);   Fahrenheit=sensors.toFahrenheit(Celcius);   lcd.clear(); //  if(Celcius>35){

//    Serial.println("sick .......................");

//    }

//    else{

//      Serial.println("not sick .......................");

//      }

//

if(sound>410 && sound<570 && Celcius<35){

Serial.println("not sick .......................");   state="Not 	Sick";

digitalWrite(buzer,LOW);

}   else{

Serial.println("sick .......................");     state="Sick";     digitalWrite(buzer,HIGH);

}

Serial.println(sound);

lcd.print(" 	T: 	");

lcd.print(Celcius);   lcd.setCursor(0,1);

lcd.print(" 	S: 	");

lcd.print(sound);    lcd.setCursor(8,1);   lcd.print(state);   // String state="Not Sick";

uri = uri_short +"&node1="+ Celcius +"&node2="+ sound;

Serial.println(uri);    httppost ();   delay(1000);

} void connectWifi()

{ 	String 	ssid

="Cathy";

String password="cathy224";

Serial.println("Trying 	to 	connect");     esp.println("AT+CWMODE=3");     delay(100);

String cmd = "AT+CWJAP=\"" +ssid+"\",\"" + password + "\"";     esp.println(cmd);     esp.println("AT");      String inData = esp.readStringUntil('\r');

Serial.println("Got reponse from ESP8266: " + inData);

delay(4000);      if(esp.find("OK")) { Serial.println("Connected!");

}     else {

// connectWifi();

Serial.println("Cannot connect to wifi");

}

}

void httppost () {

String data;

String server ="192.168.43.163";  //ip adress replace with www.sostechmakers.com

Serial.println("Starting Conn");   esp.println("AT+CIPSTART=\"TCP\",\"" + server + "\",80");//start a TCP connection.

if( esp.find("OK")) {

Serial.println("TCP connection ready");

}

delay(1000);

String postRequest =

"POST " + uri +

" HTTP/1.0\r\n" +

"Host: " + server + "\r\n" +

"Accept: *" + "/" + "*\r\n" +

"Content-Length: " + data.length() + "\r\n" +

"Content-Type: application/x-www-form-urlencoded\r\n" +

"\r\n" + data;

String sendCmd = "AT+CIPSEND=";//determine the number of caracters to be sent.

esp.print(sendCmd);

esp.println(postRequest.length() );

delay(500);

if(esp.find(">")) {

Serial.println("Sending..");      esp.print(postRequest);

if( esp.find("SEND OK")) {

Serial.println("Packet sent");

while (esp.available()) {

String tmpResp = esp.readString();

Serial.println("tmpResp=");

Serial.println(tmpResp);

}

esp.println("AT+CIPCLOSE"); // close the connection

}

}

}