Appendix A

Table A1 Geological or geotechnical mapping parameters

|  | Symbol | Units |
| --- | --- | --- |
| Sites | S |  |
| Joint Set | Jn |  |
| Joint Spacing | JS | mm |
| Joint Persistence | JP | m |
| Joint Aperture | JA | mm |
| Joint Roughness Coefficient | JRC |  |
| Joint Infilling | JI | kPa |
| Joint Weathering | JW |  |
| Rock Mass Rating class | RMR |  |

Appendix B

Table B1. Rock Mass Rating resulting using geological mapping data

| Sites | Set | Spacing | Persistence (m) | Aperture (mm) | JRC | Infilling | Weathering | RMR Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ST1 | J1 ( 108/48) | Close (60-200) | Medium (3-10) | Moderate (2.5-10) | 2-4 | Soft | Moderate | 52 III Fair Rock |
|  | J2 ( 32/57) | Very close (20-60) | Medium (3-10) | Very tight (<0.1) | 4-6 | Soft | Moderate |  |
|  | J3 (293/40) | Very close (20-60) | High (10-20) | Very tight (<0.1) | 8-12 | None | slight |  |
|  | J4 (213/57) | Moderate ( 200-600) | Medium (3-10) | Very tight (<0.1) | 10-12 | None | Slight |  |
| ST2 | J1 (248/88) | Very close (20-60) | Medium (3-10) | Partly open (0.25-0.5) | 4-6 | None | Slight | 56 III Fair Rock |
|  | J2 (54/78) | Very close (20-60) | Medium (3-10) | Very tight (<0.1) | 2-4 | None | Slight |  |
|  | J3 (107/15) | Moderate ( 200-600) | Very High (>20) | Very tight (<0.1) | 2-4 | None | Slight |  |
|  | J4 (192/45) | Moderate ( 200-600) | High (10-20) | Very tight (<0.1) | 8-12 | None | Slight |  |
|  | J5 (72/60) | Moderate ( 200-600) | Medium (3-10) | Moderate (2.5-10) | 2-4 | None | Slight |  |
| ST3 | J1 ( 73/35) | Close (60-200) | Medium (3-10) | Moderate (2.5-10) | 8-12 | Soft | Moderate | 56 III Fair Rock |
|  | J2 ( 101/17) | Close (60-200) | Very High (>20) | Moderate (2.5-10) | 8-12 | None | Slight |  |
|  | J3 (146/54) | Moderate ( 200-600) | Medium (3-10) | Very tight (<0.1) | 4-6 | None | Slight |  |
|  | J4 (116/54) | Moderate ( 200-600) | Medium (3-10) | Very tight (<0.1) | 2-4 | None | Slight |  |
|  | J5 (210/75) | Moderate ( 200-600) | Medium (3-10) | Very tight (<0.1) | 2-4 | None | Slight |  |
|  | J6 (315/46) | Close (60-200) | Medium (3-10) | Very tight (<0.1) | 2-4 | None | moderate |  |
|  | J7 (250/48) | Very Close (20-60) | High (10-20) | Moderate (2.5-10) | 4-6 | None | Slight |  |
|  | J8 (293/63) | Close (60-200) | Medium (3-10) | Partly open (0.25-0.5) | 8-12 | None | Slight |  |
|  | J9 (356/36) | Moderate (200-600) | Medium (3-10) | Moderate (2.5-10) | 2-4 | Soft | moderate |  |
| ST4 | J1 (270/63) | Close (60-200) | Medium (3-10) | Moderate (2.5-10) | 2-4 | Soft | Slight | 53 III Fair Rock |
|  | J2 (37/65) | Moderate (200-600) | Medium (3-10) | Moderate (2.5-10) | 4-6 | None | Moderate |  |
|  | J3 (153/41) | Moderate (200-600) | Very High (>20) | Moderate (2.5-10) | 4-6 | None | Moderate |  |
|  | J4 (222/76) | Close (60-200) | High (10-20) | Moderate (2.5-10) | 4-6 | None | Slight |  |
|  | J5 (332/58) | Close (60-200) | High (10-20) | Very tight (<0.1) | 4-6 | None | Slight |  |
| ST5 | J1 (243/78) | Moderate (200-600) | High (10-20) | Very tight (<0.1) | 2-4 | Soft | Slight | 56 III Fair Rock |
|  | J2 (79/62) | Very Close (20-60) | Medium (3-10) | Moderate (2.5-10) | 2-4 | None | Slight |  |
|  | J3 (36/54) | Very Close (20-60) | Medium (3-10) | Partly open (0.25-0.5) | 8-12 | None | Slight |  |
|  | J4 (163/55) | Close (60-200) | Medium (3-10) | Moderate (2.5-10) | 8-12 | None | Slight |  |
|  | J5 (302/18) | Close (60-200) | Medium (3-10) | Moderate (2.5-10) | 2-4 | None | Slight |  |
| ST6 | J1 (226/57) | Moderate (200-600) | Medium (3-10) | Very tight (<0.1) | 2-4 | Soft | Moderate | 56 III Fair Rock |
|  | J2 (348/77) | Moderate (200-600) | Medium (3-10) | Very tight (<0.1) | 4-6 | None | Slight |  |
|  | J3 (301/56) | Close (60-200) | Very High (>20) | Very tight (<0.1) | 8-12 | None | Slight |  |
|  | J4 (25/21) | Close (60-200) | Medium (3-10) | Moderate (2.5-10) | 4-6 | None | Slight |  |
|  | J5 (53/68) | Very Close (20-60) | Medium (3-10) | Moderate (2.5-10) | 2-4 | None | Slight |  |
|  | J6 (104/43) | Moderate (200-600) | Medium (3-10) | Partly open (0.25-0.5) | 2-4 | None | Slight |  |
|  | J7 (161/75) | Moderate (200-600) | Medium (3-10) | Moderate (2.5-10) | 2-4 | None | Moderate |  |
|  | J8 (126/67) | Very close (20-60) | Medium (3-10) | Moderate (2.5-10) | 4-6 | None | Slight |  |
|  | J9 (35/71) | Close (60-200) | High (10-20) | Moderate (2.5-10) | 8-12 | Soft | Slight |  |