Supplementary file 3. Processing and storage of blood samples

Self-collected fingerprick blood samples are collected in 0.5 µl Greiner Minicollect tubes (#450533) and returned in a pre-printed and addressed safety-bag envelope the same day. Upon arrival, blood samples are immediately centrifuged. Serum is subsequently separated, aliquoted and stored at -80°C. The serum samples are analyzed with the Elecsys Anti-SARS-CoV-2 S and Anti-SARS-CoV-2 assays on the Cobas e801 (Roche Diagnostics, Mannheim, Germany), which are electrochemiluminescence immunoassays measuring Ig levels against respectively the receptor binding domain of the Spike (S1) protein (S-antibodies) and the Nucleoprotein (N-antibodies) of SARS-COV-2. S-antibody concentrations are reported in Binding Antibody Units (BAU/ml). Samples with higher S-antibody concentrations than the measuring range (up to 12,500 BAU/ml) are diluted 1:900, re-measured and quantified up to 225,000 BAU/ml. For N-antibody concentrations, the qualitative cut-off index is converted to numeric results in BAU/ml using batch-specific, linear calibration-lines obtained with a dilution range of the NIBSC 20/136 WHO standard (NIBSC)[19]. The cut-off for N-positivity is set by converting cut-off index 1.0 to corresponding BAU/ml using these calibration lines.