Supplementary Material

Comprehensive environmental risk assessment of heavy metals in an abandoned dye factory of China

using SERA methodology based on source apportionment

3 Figures

3 Tables

Fig. S1. Sampling sites of heavy metals in soils surround the dye factory

Fig. S2. Enrichment factor of different heavy metals at the sampling points.

Fig. S3. Factor profiles of soil heavy metals in the study area from PMF model.

Table S1

Toxicity coefficient of different heavy metals in soil.

| Element | As | Cd | Cr | Hg | Pb |
| --- | --- | --- | --- | --- | --- |
| Tri | 10 | 30 | 2 | 40 | 5 |

Table S2

The relationship between ecological risk index and classification of heavy metal environmental pollution.

| Scope of potential ecological risk index(Eri) | Ecological risk level of single-factor pollution | Slope of potential toxicity index(RI) | General level of potential ecological risk |
| --- | --- | --- | --- |
| Eri < 40 | Low | RI < 150 | Low-grade |
| 40 ≤ Eri < 80 | Moderate | 150 ≤ RI < 300 | Moderate |
| 80 ≤ Eri < 160 | High | 30 0 ≤ RI < 600 | Severe |
| 160 ≤ Eri < 320 | Higher | 600 ≤ RI | Serious |
| 320 ≤ Eri | Serous |  |  |

Table S3

The other parameters of human health risk model.

| Symbol | Instructions | Reference value |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | children | adults |  |
| IngR | ingestion rate | 200 | 100 | mg/d |
| InhR | inhalation rate | 0.012 | 0.03 | kg/ d |
| EF | exposure frequency | 350 |  | d/ y |
| ED | exposure duration | 6 | 30 | y |
| AT | averaging time | 3650 | 25550 | d |
| PEF | emission factor | 1.36×109 |  | m3/kg |
| SL | skin adherence factor | 0.2 | 0.7 | mg·cm2/ kg |
| SA | exposed skin surface area | 2800 | 5700 | cm2 |
| ABS | dermal absorption factor | 10-4 |  | - |

Note: - represent non-dimensional parameter