Supplementary Tables

Table S1 Information of 27 sub-watersheds

| Sub | Population (Person) | Population density (Person/km2) | Area (km2) | Slope (%) | Impervious area ratio (%) |
| --- | --- | --- | --- | --- | --- |
| S12 | 174,017 | 20,155.8 | 8.63 | 7.12 | 56 |
| S13 | 5 | 8.9 | 0.60 | 8.81 | 9 |
| S13-1 | 16,445 | 7,670.9 | 2.14 | 7.24 | 49 |
| S14 | 47,839 | 21,217.2 | 2.25 | 5.37 | 75 |
| S15 | 13,531 | 15,517.2 | 0.87 | 7.23 | 58 |
| S16 | 42,275 | 28,540.5 | 1.48 | 6.81 | 70 |
| S17 | 20,434 | 30,095.3 | 0.68 | 6.68 | 66 |
| S18 | 11,582 | 32,127.9 | 0.36 | 4.98 | 90 |
| S19 | 4,895 | 2,501.8 | 1.96 | 6.96 | 26 |
| S20 | 7,009 | 5,546.5 | 1.26 | 7.71 | 21 |
| S21 | 1,581 | 625.0 | 2.53 | 6.90 | 11 |
| S22 | 41,766 | 32,536.4 | 1.28 | 5.43 | 84 |
| S23 | 15,646 | 44,489.7 | 0.35 | 2.76 | 96 |
| S24 | 31,793 | 29,522.4 | 1.08 | 10.49 | 59 |
| S25 | 7,956 | 6,821.2 | 1.17 | 9.04 | 18 |
| S25-1 | 481 | 213.8 | 2.25 | 4.94 | 13 |
| S26 | 677 | 532.4 | 1.27 | 5.66 | 21 |
| S27 | 435 | 223.7 | 1.95 | 6.48 | 19 |
| S28 | 512 | 375.1 | 1.36 | 7.48 | 28 |
| S29 | 293 | 375.9 | 0.78 | 2.06 | 19 |
| S30 | 16 | 14.1 | 1.14 | 15.76 | 8 |
| S31 | 927 | 160.2 | 5.79 | 5.99 | 23 |
| S32 | 394 | 226.2 | 1.74 | 5.20 | 20 |
| S33 | 930 | 274.8 | 3.38 | 5.57 | 11 |
| S33-1 | 474 | 150.9 | 3.14 | 11.73 | 8 |
| S34 | 83 | 85.5 | 0.97 | 7.99 | 16 |
| S34-1 | 2,287 | 414.2 | 5.52 | 7.90 | 18 |

Table S2 Information of CMIP6 GCMs used in this study

| No | Institution | Model | Resolution (lon × lat) (km) |
| --- | --- | --- | --- |
| 1 | Commonwealth Scientific and Industrial Research Organisation, Australia | ACCESS-ESM1-5 | 192 x 145 |
| 2 | Canadian Centre for Climate Modelling and Analysis, Canada | CanESM5 | 128 x 64 |
| 3 | Euro-Mediterranean Centre on Climate Change, Italy | CMCC-ESM2 | 288 x 192 |
| 4 | NOAA Geophysical Fluid Dynamics Laboratory, USA | GFDL-ESM4 | 360 x 180 |
| 5 | Institute for Numerical Mathematics, Russia | INM-CM4-8 | 180 x 120 |
| 6 |  | INM-CM5-0 |  |
| 7 | Institut Pierre-Simon Laplace, France | IPSL-CM6A-LR | 144 x 143 |
| 8 | Korea Institute of Ocean Science and Technology, South Korea | KIOST-ESM | 192 x 96 |
| 9 | Japan Agency for Marine-Earth Science and Technology, Atmosphere and Ocean Research Institute for Environmental Studies, Japan | MIROC6 | 256 x 128 |
| 10 | Max Planck Institute for Meteorology, Germany | MPI-ESM1-2-HR | 384 x 192 |
| 11 |  | MPI-ESM1-2-LR | 192 x 96 |
| 12 | Meteorological Research Institute, Japan | MRI-ESM2-0 | 320 x 160 |
| 13 | Nanjing University of Information Science and Technology, China | NESM3 | 192 x 96 |
| 14 | Norwegian Climate Centre, Norway | NorESM2-LM | 144 x 96 |
| 15 |  | NorESM2-MM | 288 x 192 |

Table S3 TFN weight according to water quantity evaluation criteria

| DPSIR | Criteria | Component | Min | Mod | Max |
| --- | --- | --- | --- | --- | --- |
| Driving force | D1 | Population | 0.470 | 0.523 | 0.575 |
|  | D2 | Population density | 0.348 | 0.386 | 0.425 |
| Pressure | P1 | Percent of impervious area | 0.253 | 0.281 | 0.309 |
|  | P2 | Watershed slope (%) | 0.078 | 0.087 | 0.096 |
|  | P3a | Annual water supply amount | 0.487 | 0.541 | 0.595 |
| State | S1a | Total runoff | 0.217 | 0.241 | 0.265 |
|  | S2a | Total infiltration | 0.512 | 0.569 | 0.626 |
|  | S3a | Average flowrate during dry season | 0.089 | 0.099 | 0.109 |
| Impact | I1a | Total runoff (future) | 0.173 | 0.193 | 0.212 |
|  | I2a | Total infiltration (future) | 0.513 | 0.570 | 0.627 |
|  | I3a | Average flowrate during dry season (future) | 0.132 | 0.147 | 0.161 |
| Response | R1a | Permeable pavement performance in total runoff | 0.357 | 0.397 | 0.436 |
|  | R2a | Permeable pavement performance in total infiltration | 0.115 | 0.128 | 0.141 |
|  | R3a | Permeable pavement Performance in flowrate during dry season | 0.271 | 0.301 | 0.331 |
|  | R4a | Cost for installation permeable pavement | 0.075 | 0.083 | 0.092 |

Table S4 TFN weight according to water quality evaluation criteria

| DPSIR | Criteria | Component | Min | Mod | Max |
| --- | --- | --- | --- | --- | --- |
| Driving force | D1 | Population | 0.471 | 0.523 | 0.575 |
|  | D2 | Population density | 0.347 | 0.386 | 0.425 |
| Pressure | P1 | Percent of impervious area | 0.185 | 0.206 | 0.227 |
|  | P2 | Watershed slope (%) | 0.045 | 0.050 | 0.055 |
|  | P3b | Wastewater discharge | 0.588 | 0.653 | 0.718 |
| State | S1b | Load of BOD | 0.119 | 0.132 | 0.146 |
|  | S2b | Load of DO | 0.260 | 0.289 | 0.318 |
|  | S3b | Load of TN | 0.127 | 0.141 | 0.155 |
|  | S4b | Load of TP | 0.292 | 0.325 | 0.357 |
|  | S5b | Load of SS | 0.020 | 0.022 | 0.024 |
| Impact | I1b | Load of BOD (future) | 0.160 | 0.178 | 0.196 |
|  | I2b | Load of DO (future) | 0.202 | 0.224 | 0.247 |
|  | I3b | Load of TN (future) | 0.165 | 0.184 | 0.202 |
|  | I4b | Load of TP (future) | 0.264 | 0.293 | 0.322 |
|  | I5b | Load of SS (future) | 0.027 | 0.030 | 0.033 |
| Response | R1b | Permeable pavement Performance in BOD | 0.125 | 0.139 | 0.153 |
|  | R2b | Permeable pavement Performance in DO | 0.057 | 0.064 | 0.070 |
|  | R3a | Permeable pavement Performance in TN | 0.129 | 0.143 | 0.157 |
|  | R4a | Permeable pavement Performance in TP | 0.134 | 0.149 | 0.164 |
|  | R5a | Permeable pavement Performance in SS | 0.111 | 0.124 | 0.136 |
|  | R6a | Cost to installation permeable pavement facility | 0.262 | 0.291 | 0.320 |

Supplementary Equations

- Supplementary description of Spatial interpolation and bias correction method

The inverse distance weighting (IDW) method is an interpolation method using the concept of Tobler’s first law (Tobler, 1970) that values at relatively close distances are more related than values at far distance. Eq. (S1) is used to perform spatial interpolation on climate data in GCM grid format using the IDW method.

Pi=i=1Nwsxi=1NwsxPi(xs)                                                  (S1)

Here, Pi represents the climate variable of study area, wsx is the interpolation weight, and Pi(xs) is the GCM simulated climate variables around the study area.

For bias correction, the quantile mapping method that correct the error using the cumulative probability distribution of the observed and simulated values (Hashino et al., 2007) was used. The bias corrected data by the quantile mapping method is obtained using Eq. (2).

xm,pt=Fo,h-Fm,hxm,p(t)                                               (S2)

where xm,pt is the bias-corrected data, xm,p(t) is the simulated value at time t in period p, Fm,h is cumulative distribution function (CDF) of simulated values, and Fo,h- is the inverse function of the CDF of observed values.

- Supplementary description of Fuzzy TOPSIS

The standardized fuzzy matrix R=rij for applying triangular fuzzy number (TFN) to TOPSIS is expressed as Eq. (S3) for the benefit criterion and Eq. (S4) for the cost criterion.

rij=aijcj*,bijcj*,cijcj*,  cj*=maxi{cij}                                             (S3)

rij=aj*cij,aj*bij,aj*aij,  aj*=mini{aij}                                            (S4)

Here, aij, bij, and cij represent minimum, mode, maximum value of TFN matrix. When rij=aij,bij,cij, fuzzy positive ideal solution (FPIS) A+ and fuzzy negative ideal solution, FNIS A- are defined as in Eq. (S5) based on the weighted normalized fuzzy matrix V=[vij].

A+=(v1+,v2+,…vn+); A-=(v1-,v2-,…,vn-)                                 (S5)

Here, vij=rij×wj, vj+=maxi{aij}, and vj-=mini{cij}. Also, the distance between the two TFNs m=(m1,m2,m3) and n=(n1,n2,n3) is calculated through Eq. (S6). Accordingly, the distance between A+(FPIS) & A-(FNIS) and each alternative (i) is calculated through Eq. (S7). Finally, the relative closeness coefficient (CCi*) of each alternative is calculated through Eq. (S8).

dm, n=13m1-n12+m2-n22+m3-n32                          (S6)

di+=j=1nd(vij,vj+),     di-=j=1nd(vij,vj-)                                 (S7)

CCi*=di-(di++di-)                                                             (S8)

- Supplementary description of VIKOR

When deriving a compromise solution using the VIKOR method, the best value (fj*) and the worst value (fj-) for the values according to the evaluation criteria of alternatives are first calculated through Eq. (S9) on the benefic criteria.

fj*=maxifij,             fj-=minifij                                             (S9)

Based on the calculated fj*, fj- values, Si and Ri values are derived through Eqs. (S10) ~ (S11).

Si=j=1nwjfj*-fij/fj*-fj-                                           (S10)

Ri=maxjwjfj*-fij/(fj*-fj-),                                          (S11)

where wj denotes the weights of the criteria, expressing their relative importance. Finally, the Qi value is calculated through Eqs. (S12) ~ (S14).

Qi=v(Si-S*)/(S—S*)+(1-v)(Ri-R*)/(R—R*)                       (S12)

Here,

S*=miniSi, S-=maxiSi                                                   (S13)

R*=miniRi, R-=maxiRi                                                  (S14)

v is introduced as the weight of the strategy of ‘‘the majority of criteria” (or ‘‘the maximum group utility”). Here, we assume that v = 0.5.

References

Tobler, W. (1970). “A computer movie simulating urban growth in the Detroit region”, Economic Geography, Vol. 46, No. 2, pp. 234~240.

Hashino, T., Bradley, A.A., and Schwartz, S.S. (2007). “Evaluation of bias-correction methods for ensemble streamflow volume forecasts”, Hydrology and Earth System Sciences, Vol. 11, pp. 939~950.