SUPPLEMENTARY INFORMATION

Platinum Shortage Threatens China's Hydrogen Ambition for Carbon Neutrality

Peng Wang, Fan Yang, Chenyang Wang, Klaus Hubacek, Laixiang Sun, Wei-Qiang Chen, Kuishuang Feng & Jiashuo Li

Summary Information:

Number of SI Pages: 38

Number of SI Figures: 14

Number of SI Tables: 15

TABLE OF CONTENTS

S1.	Abbreviated letter index table	2

S2.	Comparison with the results of previous studies	3

S3.	Data and method details	7

S3.1.	Data used for evaluating Pt demand	7

S3.2.	Method of mapping the platinum cycle in the future	8

S4.	Scenarios instruction	11

S4.1.	Description of climate scenarios	11

S4.2.	Description of circular economy scenarios	11

S5.	Supporting results	13

S5.1.	China's Pt demand trend associated with hydrogen development	13

S5.2.	China's cumulative Pt demand structure associated with H2 development	14

S5.3.	China’s Pt import map in 2019	15

S5.4.	Pt supplier countries’ pressure caused by China's hydrogen infrastructure	16

S5.5.	Benefits of circular measures under each scenario	17

S5.6.	China’s platinum supply and consumption mix from 2021 to 2060	20

S5.7.	Pt cost driven by the rapidly growing Pt demand under the CN scenario	23

S5.8.	Share of Pt cost in the total cost of each hydrogen technology in 2060	25

S6.	Parament setting	26

S6.1.	Carbon emissions pathway of each climate scenario	26

S6.2.	Classification and technical parameters of hydrogen production technology	27

S6.3.	Cost evolution of hydrogen production technology	28

S6.4.	Cost evolution of road transport technology	29

S6.5.	Market share of fuel cell technologies in industry energy and CHP use area	31

S6.6.	Lifetime setting of Pt-containing hydrogen equipment	32

S6.7.	Recycling patterns for key hydrogen energy infrastructure	33

S6.8.	Parament settings of different recycling patterns	34

S6.9.	Pt intensity evolution of key hydrogen technologies	35

Supplemental References	36

- Abbreviated letter index table

Table S1 List of abbreviations.

| Abbreviation | Full Name |
| --- | --- |
| H2 | Hydrogen |
| Pt | Platinum |
| LDV | Light-duty Vehicles |
| CN | Carbon Neutrality |
| BAU | Business-As-Usual |
| CCS | Carbon Capture and Storage |
| FC | Fuel Cell |
| FCEV | Fuel Cell Electric Vehicle |
| BEV | Battery Electric Vehicle |
| PEM | Polymer Electrolyte Membrane |
| PAFC | Phosphoric Acid Fuel Cell |
| SOFC | Solid Oxide Fuel Cell |
| HRS | Hydrogen Refuelling Station |
| MCFC | Molten Carbonate Fuel Cell |
| PEMFC | Proton Exchange Membrane Fuel Cell |
| CHP | Combined Heat and Power |
| CR | collection rate |
| ZA | South Africa |
| RUS | Russia |
| RoW | Rest of World |
| EOL-RR | end-of-life recycling rate |

- Comparison with the results of previous studies

Table S2 Comparison of our hydrogen demand forecast with previous studies

| Reference | H2 demand | Region | Climate target | H2 application | End-use sector coverage |
| --- | --- | --- | --- | --- | --- |
| Hydrogen council1 | 200 Mt in 2050 | China | 1.5℃ | H2 as fuels and H2 for industrial applications | Building (20%) Industry (50%) Power (10%) Transport (40%) |
| International Energy Agency2 | 90 Mt in 2060 | China | Carbon neutrality | H2 fuels and H2 for industrial applications | Industry (40%) Refinery (20%) Transport (25%) Others (15%) |
| China hydrogen alliance3 | 130 Mt in 2060 | China | Carbon neutrality | H2 as fuels and H2 for industrial applications | Building (5.5%) Industry (60%) Power (4.5%) Transport (30%) |
| Shell4 | 25.5 Mt in 2060 | China | 2℃ | H2 as fuels | Heavy industry (25%) Transport (22%) Others (53%) |
| BP5 | 93 Mt in 2050 | China | 1.5℃ | H2 as fuels and H2 for industrial applications | Not Available |
| CN scenario, this study | 20.0 Mt in 2060 | China | Carbon neutrality | H2 as fuels | Industry (37.6%) Transport (62.4%) |
| 1.5℃ scenario, this study | 32.1 Mt in 2060 | China | 1.5℃ | H2 as fuels | Industry (27.6%) Transport (72.4%) |
| BAU scenario, this study | 38.0 Mt in 2060 | China | No climate target | H2 as fuels | Industry (6.7%) Transport (93.3%) |

Table S3 Review on the key Pt demand results from previous studies

| Reference | Pt demand* | Technology Scope | Key assumption | Key results |
| --- | --- | --- | --- | --- |
| (Raymond, Sterck and Clifford, 2022)6 | 31.1 t (1M oz) p.a. by 2030 124.4t (4M oz) p.a. by 2040 40.4t (1.3M oz) p.a. by 2028 208.4t (6.7M oz) p.a. by 2040 | FCEVs | Policy-driven scenario: FCEV adoption is driven by government and regional subsidies, incentives, and legislated targets. FCEVs reaches 2m in 2030 and 11m in 2040. Commercially enhanced adoption scenario: government and regional policies have engendered infrastructure critical mass. FCEVs reaches 3m in 2030 and 15m in 2040. | Supply without increases can cause platinum scarcity, which would hinder FCEV growth rates, while Pt production can be expanded with time to satisfy the demand growth. |
| (Tong et al., 2022)7 | nearly 280-350 t p.a. in 2050 | Transportation (FCEVs, ICEVs) | Energy scenario: carbon neutrality within half a century. Technology assumption: S1: Technological Substitution–substituting ICEs with FCVs and BEVs. S2: Diversified niches-considering the path-dependency of technological change. S3: Landscape change–combined with a systematic change in mobility landscape | There is an imbalance between supply and demand for Pt between regions, calling for a global coordination to enhance the material efficiency for Pt |
| (Hao et al., 2019)8 | 33.8 kt(D1) 93.2 kt(D2) 56.1 kt(D3) 2010-2100 cumulatively | Transportation (FCEVs, ICEVs) | Population and economic growth: SSP2. Vehicle sales, scrappage, and stock: the global vehicle sales will increase from the current level of 97 million to 144 million in 2030 (49% increase), 192 million in 2050 (98% increase), and 220 million in 2100 (127% increase). Technology assumption: D1: FCVs penetrating only the HDV segment. D2: FCVs into the HDV & LDV segment. D3: Lowering PGM loading of fuel cells. | PGMs are not likely to be a constraint for the mass deployment of FCEVs at the global level, there could be significant supply risks due to resource location. |
| (Alonso, Field and Kirchain, 2012)9 | 59 kt 2010-2050 cumulatively | All platinum-using industries | Energy scenario: IEA projected fleet for meeting 450 greenhouse gas goals. Technology assumption: simulate historical platinum usage and its growth rate for individual Pt-using industries, and FCEVs reach 12% in 2035. | The demand can only be met by the primary supply curve （projected based on 5% exponential growth rate）. |
| (Rasmussen et al., 2019)10 | 51.4 kt (the most aggressive fuel cell scenario) 2016-2050 cumulatively | All platinum-using industries | FCEVS: bottom-up approach, fuel cell rapid development scenario Electrolyser: extrapolate Danish to all other countries Other sectors: assumption based on their historical patterns | Only in high demand scenarios and when fuel cell market penetration is higher than the expected, the aggregate demand to 2050 will exceed the 2016 global platinum reserves. |
| (Elshkaki, 2013)11 | over 1.4 kt p.a. in 2050 over 8.0 kt p.a. in 2100 | All platinum-using industries | Fuel Cells are modelled based on the demand for vehicles, the share of Fuel Cell Vehicles (FCVs). FCEVs market share reaches 50% by the year 2050 and 100% by the year 2100. Catalytic Converters are modelled based on the demand for vehicles, FCVs market share, CC market share. Other Applications are modelled based on socio-economic variables, metal price and time. | Platinum identified resources are expected to deplete with or without the introduction of FCVs before the end of the century. |

Note: Those values are obtained roughly based on their result presentations.

- Data and method details

  - Data used for evaluating Pt demand

Table S4 Data used for evaluating Pt demand

| Variable | Value | Unit | Source | Process |
| --- | --- | --- | --- | --- |
| China's Pt primary production | 2.5 | ton | BGS12 | the average of data during 2015-2020 |
| Global Pt primary production | 180.3 | ton | USGS13 | the average of data during 2013-2020 |
| China's current Pt gross demand | 72.4 | ton | SFA Oxford14 | the average of data during 2013-2020 |
| China's recycling | 13.94 | ton | SFA Oxford14 | the average of data during 2013-2020 |
| Pt content of PGM mines (Pt ratio) | 51.3 | % | (Nassar, 2015)15 (Rasmussen et al., 2019)16 | the average of 21 PGM mines |
| China's Pt reserve | 65 | ton | China Mineral Resources17 | Pt ratio * PGMs reserve of 2020 |
| Global Pt reserve | 35397 | ton | USGS13 | Pt ratio * PGMs reserve of 2020 |
| Pt reserve of ZA | 32319 | ton | USGS13 | Pt ratio * PGMs reserve of 2020 |
| Pt reserve of RUS | 2000.7 | ton | USGS13 | Pt ratio * PGMs reserve of 2020 |
| Pt reserve of RoW | 1077.3 | ton | USGS13 | Pt ratio * PGMs reserve of 2020 |
| Pt price (1) | 1064.6 | USD/troy ounce | LBMA18 | the average of data during 2013-2021 |
| Pt import share of ZA (2) | 60.3 | % | UN Comtrade19 | Share sourced from South Africa of the imported Pt |
| Pt import share of RUS (2) | 10.2 | % | UN Comtrade19 | Share sourced from Russia of the imported Pt |
| Pt import share of RoW (2) | 29.4 | % | UN Comtrade19 | Share sourced from RoW of the imported Pt |

Note:	(1) The figure is in 2020 dollars.

(2) The selected year is 2019. Trade pattern in 2019 is more convincing, considering the Covid-19’s effect to the Pt supply chain20.

  - Method of mapping the platinum cycle in the future

Figure S1 Framework of China's Pt cycle in the future

Table S5 Detailed explanation of the calculation of each flow in China's Pt cycle.

| Flow | Description | Calculation | Data source |
| --- | --- | --- | --- |
| D1 | Pt demand for Industry FC | Stock-driven model | Calculation in this study |
| D2 | Pt demand for H2 Production | Stock-driven model | Calculation in this study |
| D3 | Pt demand for LDV | Stock-driven model | Calculation in this study |
| D4 | Pt demand for Bus | Stock-driven model | Calculation in this study |
| D5 | Pt demand for Truck | Stock-driven model | Calculation in this study |
| D6 | Pt demand for Jewellery | Statistics | SFA Oxford14 |
| D7 | Pt demand for Glass | Statistics | SFA Oxford14 |
| D8 | Pt demand for Chemical | Statistics | SFA Oxford14 |
| D9 | Pt demand for Autocatalyst | Statistics | SFA Oxford14 |
| D10 | Pt demand for Petroleum | Statistics | SFA Oxford14 |
| D11 | Pt demand for Other sector | Statistics | SFA Oxford14 |
| D_sum | Total Pt demand | D_sum=D1+…+D11 | Mass balance |
| R1 | Recycling of Industry FC | Stock-driven model | Calculation in this study |
| R2 | Recycling of H2 Production | Stock-driven model | Calculation in this study |
| R3 | Recycling of LDV | Stock-driven model | Calculation in this study |
| R4 | Recycling of Bus | Stock-driven model | Calculation in this study |
| R5 | Recycling of Truck | Stock-driven model | Calculation in this study |
| R6 | Pt demand for Jewellery | Statistics | SFA Oxford14 |
| R9 | Pt demand for Autocatalyst | Statistics | SFA Oxford14 |
| R_tra | Recycling from traditional sector | Statistics | SFA Oxford14 |
| R_H2 | Recycling from H2 sector | R_H2=R1+R2+R3+R4+R5 | Mass balance |
| R_sum | Pt demand from Recycling | R_sum=R_tra+R_H2 | Mass balance |
| S_sum | Pt demand from virgin ores | S_sum=D_sum-R_sum | Mass balance |
| S1 | China's domestic Pt primary production | Statistics | BGS12 |
| S5 | Primary Pt sourced from overseas | S5=S_sum-S1 | Mass balance |
| S2 | Primary Pt sourced from South Africa | S2=S5*Import share_ZA | Mass balance |
| S3 | Primary Pt sourced from Russia | S3=S5*Import share_RUS | Mass balance |
| S4 | Primary Pt sourced from Rest of World | S4=S5*Import share_RoW | Mass balance |

- Scenarios instruction

  - Description of climate scenarios

Table S6 Description of climate scenarios

| Scenario | Social Economic Parament | Climate Target | Temperature Increase in 2060 |
| --- | --- | --- | --- |
| BAU | SSP2 | No climate policy constraints | 2.341°C |
| 1.5°C | SSP1 | No more than 1.5°C temperature increase in 2100 | 1.819°C |
| CN | SSP1 | Carbon emissions are reduced to zero in 2060 | 1.684°C |

  - Description of circular economy scenarios

Material efficiency improvement strategies such as increasing recycling, product redesign and material substitution can improve Pt supply resilience in the face of future uncertainties21–23. Here, we quantify the potential benefits of four sets of Pt material-based efficiency strategies across Pt life cycle, which are: (1) Pt-free technology substitution, (2) Pt service life extension, (3) Pt recycling enhancement, and (4) Reducing Pt content. Strategy (1) lies in the planning stage and aims to avoid Pt usage in H2 value chain by selecting alternative Pt-free technologies. For instance, in the industrial use of hydrogen energy, those emerging Pt-free technologies (e.g., SOFC and MCFC) are assumed to quickly replace PAFC and PEMFC and fully occupy the market by 2045. Strategy (2) focuses on the in-use stage to extend key Pt technology service lifetime by ensuring asset durability and optimising operation. Strategy (3) incorporates various end-of-life strategies for better management of end-of-life products and high efficiency of the Pt recovery process. Strategy (4) complies with the principle of material reduction for the same hydrogen service, mainly through technology breakthroughs and R&D to reduce the platinum content in hydrogen equipment.

Table S7 Description of circular economy scenarios

| Strategy | Technology market share |  | Life time |  | Recycling |  | Material intensity |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | II | I | II | I | II | I | II |
| No policy | √ |  | √ |  | √ |  | √ |  |
| (1) Pt-free technology substitution |  | √ | √ |  | √ |  | √ |  |
| (2) Pt service life extension | √ |  |  | √ | √ |  | √ |  |
| (3) Pt recycling enhancement | √ |  | √ |  |  | √ | √ |  |
| (4) Pt basic reduction | √ |  | √ |  | √ |  |  | √ |

Note:

- Technology market share: Type I means general technology mix; Type II means No-Pt technology mix.

- Life time: Type I means general lifetime, Type II means the extended lifetime.

- Pt recycling enhancement: Type I adopts current recycling setting; Type II adopts enhanced recycling setting.

- Material intensity: Type I means the current advanced intensity; Type II means the basic reduction intensity.

- Supporting results

  - China's Pt demand trend associated with hydrogen development

Figure S2 China's Pt demand trend associated with hydrogen development from 2021 to 2060. (a) the CN scenario, (b) the 1.5°C scenario, and (c) the BAU scenario

  - China's cumulative Pt demand structure associated with H2 development

Figure S3 China's cumulative Pt demand structure associated with hydrogen development during 2021-2060. (a) the CN scenario, (b) the 1.5°C scenario, and (c) the BAU scenario

  - China’s Pt import map in 2019

Figure S4 China’s Pt import map in 2019 (Unite: ton/year)

  - Pt supplier countries’ pressure caused by China's hydrogen infrastructure

Figure S5 Pt supplier countries’ pressure caused by China's hydrogen infrastructure. (a) the CN scenario, (b) the 1.5°C scenario, and (c) the BAU scenario

  - Benefits of circular measures under each scenario

Figure S6 Benefits of circular measures under the CN scenario. (a) Pt-free technology substitution; (b) Pt service life extension; (c) Pt recycling enhancement, and (d) Reducing Pt content.

Figure S7 Benefits of circular measures under the 1.5°C scenario. (a) Pt-free technology substitution; (b) Pt service life extension; (c) Pt recycling enhancement, and (d) Reducing Pt content.

Figure S8 Benefits of circular measures under the BAU scenario. (a) Pt-free technology substitution; (b) Pt service life extension; (c) Pt recycling enhancement, and (d) Reducing Pt content.

  - China’s platinum supply and consumption mix from 2021 to 2060

Figure S9 China’s platinum supply and consumption mix from 2021 to 2060 under the carbon neutrality (CN) scenario

Figure S10 China’s platinum supply and consumption mix from 2021 to 2060 under the 1.5°C scenario

Figure S11 China’s platinum supply and consumption mix from 2021 to 2060 under the BAU scenario

  - Pt cost driven by the rapidly growing Pt demand under the CN scenario

Table S8 Pt cost driven by the rapidly growing Pt demand under the CN scenario (Unite: $/g)

| Scenario | CM |  |  | CA |  |  | RR |  |  | AR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discount rate | D1 | D2 | D3 | D1 | D2 | D3 | D1 | D2 | D3 | D1 | D2 | D3 |
| 2021 | 48.60 | 68.12 | 70.91 | 48.56 | 67.89 | 70.64 | 48.56 | 67.88 | 70.63 | 48.54 | 67.77 | 70.50 |
| 2022 | 49.91 | 76.98 | 81.62 | 49.76 | 75.74 | 80.09 | 49.75 | 75.70 | 80.04 | 49.64 | 74.86 | 79.01 |
| 2023 | 50.79 | 84.95 | 91.72 | 50.50 | 82.12 | 88.08 | 50.49 | 82.03 | 87.96 | 50.26 | 79.84 | 85.19 |
| 2024 | 51.36 | 91.67 | 100.62 | 50.98 | 87.03 | 94.44 | 50.97 | 86.93 | 94.30 | 50.62 | 83.27 | 89.55 |
| 2025 | 51.73 | 96.92 | 107.83 | 51.28 | 90.61 | 99.19 | 51.28 | 90.61 | 99.19 | 50.89 | 86.07 | 93.18 |
| 2026 | 52.30 | 107.02 | 122.41 | 51.73 | 96.85 | 107.74 | 51.74 | 97.01 | 107.96 | 51.29 | 90.74 | 99.37 |
| 2027 | 52.87 | 121.44 | 145.06 | 52.30 | 106.90 | 122.24 | 52.20 | 104.97 | 119.38 | 51.67 | 96.01 | 106.56 |
| 2028 | 53.20 | 132.89 | 164.81 | 52.66 | 115.39 | 135.27 | 52.47 | 110.64 | 127.89 | 51.92 | 99.94 | 112.10 |
| 2029 | 53.37 | 141.09 | 180.07 | 52.90 | 122.10 | 146.15 | 52.64 | 114.79 | 134.33 | 52.11 | 103.33 | 116.98 |
| 2030 | 53.47 | 146.01 | 189.74 | 53.04 | 126.95 | 154.36 | 52.76 | 118.19 | 139.76 | 52.29 | 106.83 | 122.13 |
| 2031 | 53.74 | 165.28 | 231.79 | 53.37 | 140.80 | 179.53 | 53.07 | 128.04 | 156.24 | 52.57 | 113.13 | 131.73 |
| 2032 | 53.96 | 192.22 | 305.38 | 53.68 | 160.88 | 221.52 | 53.39 | 141.66 | 181.17 | 52.86 | 120.84 | 144.07 |
| 2033 | 54.04 | 208.11 | 360.31 | 53.83 | 174.82 | 255.56 | 53.55 | 151.01 | 199.97 | 53.04 | 126.98 | 154.40 |
| 2034 | 54.07 | 215.89 | 391.50 | 53.90 | 183.10 | 278.13 | 53.63 | 156.55 | 211.83 | 53.17 | 131.74 | 162.75 |
| 2035 | 54.07 | 217.49 | 398.34 | 53.92 | 186.29 | 287.33 | 53.66 | 158.75 | 216.71 | 53.25 | 135.34 | 169.26 |
| 2036 | 54.08 | 220.10 | 409.86 | 53.95 | 190.32 | 299.46 | 53.69 | 161.56 | 223.10 | 53.34 | 139.21 | 176.48 |
| 2037 | 54.10 | 226.94 | 442.09 | 53.99 | 198.13 | 324.61 | 53.75 | 166.80 | 235.42 | 53.43 | 144.00 | 185.74 |
| 2038 | 54.11 | 233.02 | 473.83 | 54.03 | 205.55 | 350.74 | 53.80 | 171.80 | 247.79 | 53.51 | 148.79 | 195.37 |
| 2039 | 54.12 | 238.35 | 504.35 | 54.05 | 212.48 | 377.46 | 53.85 | 176.56 | 260.13 | 53.59 | 153.59 | 205.42 |
| 2040 | 54.13 | 242.93 | 533.04 | 54.07 | 218.86 | 404.34 | 53.88 | 181.06 | 272.38 | 53.65 | 158.45 | 216.05 |
| 2041 | 54.13 | 247.12 | 561.61 | 54.09 | 225.04 | 432.82 | 53.92 | 185.61 | 285.34 | 53.72 | 163.45 | 227.47 |
| 2042 | 54.14 | 250.87 | 589.35 | 54.11 | 230.95 | 462.66 | 53.95 | 190.21 | 299.13 | 53.77 | 168.52 | 239.61 |
| 2043 | 54.14 | 253.93 | 613.76 | 54.12 | 236.21 | 491.73 | 53.97 | 194.63 | 313.05 | 53.82 | 173.55 | 252.26 |
| 2044 | 54.14 | 256.38 | 634.76 | 54.12 | 240.83 | 519.58 | 53.99 | 198.88 | 327.13 | 53.86 | 178.52 | 265.41 |
| 2045 | 54.14 | 258.32 | 652.39 | 54.13 | 244.84 | 545.76 | 54.01 | 202.98 | 341.39 | 53.90 | 183.42 | 279.02 |
| 2046 | 54.14 | 260.69 | 675.39 | 54.14 | 250.07 | 583.22 | 54.04 | 208.88 | 363.26 | 53.94 | 189.28 | 296.30 |
| 2047 | 54.14 | 262.26 | 691.72 | 54.14 | 254.23 | 616.26 | 54.06 | 214.55 | 385.89 | 53.97 | 195.09 | 314.54 |
| 2048 | 54.14 | 262.80 | 697.66 | 54.14 | 256.05 | 631.84 | 54.07 | 217.69 | 399.21 | 54.00 | 199.36 | 328.78 |
| 2049 | 54.14 | 262.81 | 697.83 | 54.14 | 256.37 | 634.66 | 54.07 | 218.78 | 403.99 | 54.01 | 202.23 | 338.75 |
| 2050 | 54.14 | 262.30 | 692.14 | 54.14 | 255.28 | 625.17 | 54.07 | 218.07 | 400.89 | 54.02 | 203.80 | 344.33 |
| 2051 | 54.14 | 261.65 | 685.28 | 54.14 | 254.08 | 615.03 | 54.07 | 217.74 | 399.45 | 54.03 | 205.43 | 350.29 |
| 2052 | 54.14 | 261.60 | 684.71 | 54.14 | 254.22 | 616.20 | 54.08 | 219.30 | 406.25 | 54.04 | 208.13 | 360.38 |
| 2053 | 54.14 | 261.73 | 686.13 | 54.14 | 254.73 | 620.49 | 54.08 | 221.35 | 415.51 | 54.05 | 211.04 | 371.67 |
| 2054 | 54.14 | 262.05 | 689.46 | 54.14 | 255.59 | 627.87 | 54.09 | 223.85 | 427.11 | 54.06 | 214.15 | 384.24 |
| 2055 | 54.14 | 262.49 | 694.24 | 54.14 | 256.74 | 637.96 | 54.10 | 226.71 | 440.97 | 54.07 | 217.43 | 398.10 |
| 2056 | 54.14 | 262.90 | 698.75 | 54.14 | 257.85 | 647.96 | 54.10 | 229.59 | 455.53 | 54.08 | 220.68 | 412.44 |
| 2057 | 54.14 | 263.18 | 701.96 | 54.14 | 258.68 | 655.70 | 54.11 | 232.18 | 469.23 | 54.09 | 223.68 | 426.31 |
| 2058 | 54.14 | 263.40 | 704.52 | 54.14 | 259.36 | 662.26 | 54.11 | 234.61 | 482.62 | 54.10 | 226.52 | 440.01 |
| 2059 | 54.14 | 263.58 | 706.53 | 54.14 | 259.93 | 667.74 | 54.12 | 236.87 | 495.61 | 54.10 | 229.17 | 453.39 |
| 2060 | 54.14 | 263.71 | 708.14 | 54.14 | 260.40 | 672.42 | 54.12 | 238.98 | 508.15 | 54.11 | 231.64 | 466.34 |

Note: D1=1.4; D2=5.4%; D3=8%.

  - Share of Pt cost in the total cost of each hydrogen technology in 2060

Figure S12 Share of platinum cost in the total cost of individual hydrogen technologies in 2060. (a) Cost share of platinum in different hydrogen technologies with varying assumptions of platinum intensity (CM = Current Mainstream, CA = Current Advance, RR = Reference Reduction, AR = Advanced Reduction); (b) – (d) Probability distributions of platinum cost share in hydrogen technologies.

- Parament setting

  - Carbon emissions pathway of each climate scenario

Table S9 Carbon emissions pathway of each climate scenario
(Unite: Mt carbon/year)

| Year | BAU | 1.5°C | CN |
| --- | --- | --- | --- |
| 2020 | 3203.15 | 3207.16 | 3125.24 |
| 2025 | 3504.89 | 3510.71 | 2734.59 |
| 2030 | 3700.54 | 3707.34 | 2343.93 |
| 2035 | 3805.97 | 3813.61 | 1953.28 |
| 2040 | 3848.96 | 2709.80 | 1562.62 |
| 2045 | 3841.63 | 2063.31 | 1171.97 |
| 2050 | 3801.75 | 1257.86 | 781.31 |
| 2055 | 3702.83 | 616.80 | 390.66 |
| 2060 | 3595.11 | 172.94 | 0.00 |

  - Classification and technical parameters of hydrogen production technology

Table S10 Classification and technical parameters of hydrogen production technology

| Technology | Type | Input | Plant Lifetime (year) | Stack Lifetime (year) | Capacity factor |
| --- | --- | --- | --- | --- | --- |
| Biomass to H2 | Central production | biomass | 40 | NA | 90% |
| Biomass to H2 CCS |  |  | 40 | NA | 90% |
| Coal chemical |  | coal | 30 | NA | 90% |
| Coal chemical CCS |  |  | 30 | NA | 90% |
| Electrolysis |  | electricity | 40 | 7 | 97% |
| Natural gas steam reforming |  | gas | 40 | NA | 90% |
| Natural gas steam reforming CCS |  |  | 40 | NA | 90% |
| Thermal splitting |  | nuclear | 40 | 7 | 90% |
| Electrolysis |  | solar | 40 | 7 | 50% |
| Electrolysis |  | wind | 40 | 7 | 50% |
| Electrolysis | Forecourt production | electricity | 20 | 7 | 86% |
| Natural gas steam reforming |  | gas | 20 | NA | 86% |

  - Cost evolution of hydrogen production technology

Table S11 Cost evolution of hydrogen production technology from 2020 to 2060 (Unite: $/GJ)

| Technology | 2020 | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 | 2055 | 2060 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Biomass to H2 | 7.11 | 7.03 | 6.90 | 6.79 | 6.81 | 6.80 | 6.76 | 6.72 | 6.67 |
| Biomass to H2 CCS | 20988.10 | 20758.00 | 20803.40 | 20579.70 | 20360.80 | 20365.40 | 20153.30 | 19945.60 | 19995.10 |
| Coal chemical | 4.11 | 4.06 | 4.03 | 3.97 | 3.92 | 3.87 | 3.83 | 3.79 | 3.76 |
| Coal chemical CCS | 19170.60 | 18976.30 | 18992.30 | 18803.60 | 18618.70 | 18607.40 | 18428.20 | 18252.40 | 18245.60 |
| Electrolysis | 20.95 | 20.34 | 18.87 | 17.73 | 17.31 | 17.13 | 16.99 | 16.77 | 16.54 |
| Natural gas steam reforming | 3.57 | 3.66 | 3.63 | 3.66 | 3.76 | 3.85 | 3.99 | 4.08 | 4.10 |
| Natural gas steam reforming CCS | 8909.44 | 8828.80 | 8858.83 | 8779.89 | 8715.26 | 8732.77 | 8669.56 | 8594.82 | 8625.17 |
| Thermal splitting | 7.30 | 7.31 | 7.32 | 7.33 | 7.35 | 7.37 | 7.39 | 6.56 | 5.72 |
| Electrolysis | 7.40 | 6.42 | 4.94 | 3.83 | 3.46 | 3.37 | 3.27 | 3.18 | 3.09 |
| Electrolysis | 9.44 | 8.66 | 7.38 | 6.44 | 6.16 | 6.15 | 6.12 | 5.99 | 5.81 |
| Electrolysis | 24.83 | 24.11 | 22.57 | 21.31 | 20.77 | 20.48 | 20.23 | 19.89 | 19.57 |
| Natural gas steam reforming | 13.91 | 14.13 | 13.84 | 13.77 | 13.95 | 14.06 | 14.31 | 14.33 | 14.04 |

Data Source: Reference 24,25

  - Cost evolution of road transport technology

Table S12 Cost evolution of different road transport technology from 2020 to 2060 (Unite: $/vehicle*km)

| Sector | Scenario | Subsector | Technology | 2020 | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 | 2055 | 2060 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDV | BAU | Mini Car | BEV | 0.096 | 0.087 | 0.078 | 0.069 | 0.069 | 0.069 | 0.068 | 0.068 | 0.068 |
|  |  | Mini Car | FCEV | 0.109 | 0.099 | 0.089 | 0.079 | 0.079 | 0.079 | 0.079 | 0.079 | 0.079 |
|  |  | Subcompact Car | BEV | 0.156 | 0.140 | 0.124 | 0.107 | 0.107 | 0.106 | 0.105 | 0.105 | 0.105 |
|  |  | Subcompact Car | FCEV | 0.157 | 0.144 | 0.131 | 0.118 | 0.117 | 0.117 | 0.117 | 0.117 | 0.117 |
|  |  | Compact Car | BEV | 0.233 | 0.215 | 0.197 | 0.179 | 0.178 | 0.177 | 0.176 | 0.175 | 0.175 |
|  |  | Compact Car | FCEV | 0.242 | 0.223 | 0.204 | 0.185 | 0.184 | 0.183 | 0.183 | 0.183 | 0.183 |
|  |  | Multipurpose Vehicle | BEV | 0.206 | 0.205 | 0.203 | 0.202 | 0.202 | 0.201 | 0.200 | 0.196 | 0.192 |
|  |  | Multipurpose Vehicle | FCEV | 0.162 | 0.164 | 0.166 | 0.167 | 0.168 | 0.170 | 0.172 | 0.171 | 0.170 |
|  |  | Large Car& SUV | BEV | 0.390 | 0.360 | 0.329 | 0.299 | 0.298 | 0.296 | 0.294 | 0.294 | 0.293 |
|  |  | Large Car& SUV | FCEV | 0.378 | 0.359 | 0.340 | 0.313 | 0.308 | 0.304 | 0.308 | 0.308 | 0.308 |

| Sector | Scenario | Subsector | Technology | 2020 | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 | 2055 | 2060 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Truck | BAU | Heavy Truck | BEV | 0.530 | 1.400 | 2.270 | 3.139 | 4.009 | 4.878 | 5.748 | 6.617 | 7.487 |
|  |  | Heavy Truck | FCEV | 0.739 | 0.739 | 0.739 | 0.739 | 0.739 | 0.739 | 0.739 | 0.739 | 0.739 |
|  |  | Middle Truck | BEV | 0.508 | 1.342 | 2.175 | 3.008 | 3.842 | 4.675 | 5.508 | 6.342 | 7.175 |
|  |  | Middle Truck | FCEV | 0.708 | 0.708 | 0.708 | 0.708 | 0.708 | 0.708 | 0.708 | 0.708 | 0.708 |
|  |  | Light Truck | BEV | 0.610 | 1.610 | 2.610 | 3.610 | 4.610 | 5.610 | 6.610 | 7.610 | 8.610 |
|  |  | Light Truck | FCEV | 0.850 | 0.850 | 0.850 | 0.850 | 0.850 | 0.850 | 0.850 | 0.850 | 0.850 |
|  | 1.5°C&CN | Heavy Truck | BEV | 0.530 | 0.374 | 0.317 | 0.261 | 0.229 | 0.197 | 0.165 | 0.165 | 0.165 |
|  |  | Heavy Truck | FCEV | 0.739 | 0.417 | 0.330 | 0.243 | 0.212 | 0.180 | 0.148 | 0.148 | 0.148 |
|  |  | Middle Truck | BEV | 0.508 | 0.358 | 0.304 | 0.250 | 0.220 | 0.189 | 0.158 | 0.158 | 0.158 |
|  |  | Middle Truck | FCEV | 0.708 | 0.400 | 0.317 | 0.233 | 0.203 | 0.172 | 0.142 | 0.142 | 0.142 |
|  |  | Light Truck | BEV | 0.610 | 0.430 | 0.365 | 0.300 | 0.263 | 0.227 | 0.190 | 0.190 | 0.190 |
|  |  | Light Truck | FCEV | 0.850 | 0.480 | 0.380 | 0.280 | 0.243 | 0.207 | 0.170 | 0.170 | 0.170 |

| Sector | Scenario | Subsector | Technology | 2020 | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 | 2055 | 2060 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bus | BAU | Heavy Bus | BEV | 1.083 | 1.083 | 1.083 | 1.083 | 1.083 | 1.083 | 1.083 | 1.083 | 1.083 |
|  |  | Heavy Bus | FCEV | 1.289 | 1.289 | 1.289 | 1.289 | 1.289 | 1.289 | 1.289 | 1.289 | 1.289 |
|  |  | Light Bus | BEV | 0.967 | 0.967 | 0.967 | 0.967 | 0.967 | 0.967 | 0.967 | 0.967 | 0.967 |
|  |  | Light Bus | FCEV | 1.151 | 1.151 | 1.151 | 1.151 | 1.151 | 1.151 | 1.151 | 1.151 | 1.151 |
|  | 1.5°C&CN | Heavy Bus | BEV | 1.083 | 0.934 | 0.672 | 0.659 | 0.646 | 0.633 | 0.633 | 0.633 | 0.633 |
|  |  | Heavy Bus | FCEV | 1.289 | 1.112 | 0.800 | 0.659 | 0.587 | 0.551 | 0.551 | 0.541 | 0.537 |
|  |  | Light Bus | BEV | 0.967 | 0.834 | 0.600 | 0.588 | 0.577 | 0.565 | 0.565 | 0.565 | 0.565 |
|  |  | Light Bus | FCEV | 1.151 | 0.992 | 0.714 | 0.588 | 0.524 | 0.492 | 0.492 | 0.483 | 0.479 |

Data Source: 25,26

  - Market share of fuel cell technologies in industry energy and CHP use area

There is currently no statistic about the fuel cells type in industry energy and CHP use area. this study collects historical shipment and megawatts data of the fuel cells by type from E4Tech report27, and provides a market share estimate worthy of reference in this area.

Figure S13 Market share of fuel cell technologies in industry energy and CHP use

  - Lifetime setting of Pt-containing hydrogen equipment

Table S13 Lifetime setting of Pt-containing hydrogen equipment

| Sector | Subsector | General |  | Extended |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Plant Lifetime (year) | Stack Lifetime (year) | Plant Lifetime (year) | Stack Lifetime (year) |
| Industry energy use & CHP | hydrogen | 20 | 10 | 20 | 15 |
|  | hydrogen | 20 | 10 | 20 | 15 |
| LDV | Compact Car | 15 | 8 | 15 | 10 |
|  | Large Car and SUV | 15 | 8 | 15 | 10 |
|  | Mini Car | 15 | 8 | 15 | 10 |
|  | Multipurpose Vehicle | 15 | 8 | 15 | 10 |
|  | Subcompact Car | 15 | 8 | 15 | 10 |
| Truck | Light Truck | 15 | 8 | 15 | 10 |
|  | Middle Truck | 15 | 8 | 15 | 10 |
|  | Heavy Truck | 15 | 8 | 15 | 10 |
| Bus | Heavy Bus | 15 | 8 | 15 | 10 |
|  | Light Bus | 15 | 8 | 15 | 10 |
| H2 Production | Electrolysis | 20 | 7 | 20 | 10 |

Note: Extended stack life time are collected from literature 28,29.

  - Recycling patterns for key hydrogen energy infrastructure

Table S14 Recycling patterns for key hydrogen energy infrastructure

| Sector | Subsector | Recycling Pattern |
| --- | --- | --- |
| Industry energy use & CHP | hydrogen | Closed-Loop |
|  | hydrogen |  |
| H2 Production-Electrolysis | H2 central production |  |
|  | H2 central production |  |
|  | H2 central production |  |
|  | H2 forecourt production | Open-Loop |
| LDV | Compact Car |  |
|  | Large Car and SUV |  |
|  | Mini Car |  |
|  | Multipurpose Vehicle |  |
|  | Subcompact Car |  |
| Truck | Light Truck |  |
|  | Middle Truck |  |
|  | Heavy Truck |  |
| Bus | Heavy Bus |  |
|  | Light Bus |  |

  - Parament settings of different recycling patterns

Table S15 Parament settings of different recycling patterns

| Recycling Indicator | General |  | Enhancement |  |
| --- | --- | --- | --- | --- |
|  | Closed -loop | Open -loop | Closed -loop | Open -loop |
| CR15,30,31 | 100.0% | 70.0% | 100.0% | 99.0% |
| Recovery rate32 | 78.0% | 78.0% | 95.0% | 95.0% |
| EOL-RR | 78.0% | 54.6% | 95.0% | 94.1% |

Note31:

- CR is the abbreviation of old scrap collection rate, and describes the amount of the end-of -life metals which is collected and sent to the recycling chain.

- Recovery rate is also known as recycling process efficiency rate, and describe the efficiency in the given recycling process.

- EOL-RR is the abbreviation of end-of-life recycling rate, and refers to functional recycling, and includes recycling as a pure metal and as an alloy.

- The relationship between these three variables are as follows:

EOL-RR = CR × Recovery rate.

  - Pt intensity evolution of key hydrogen technologies

Pt intensities of those hydrogen technologies are collected from extensive literature review16,28,33–35. The evolution of Pt intensity of PEM electrolyser was collected from NREL36. For PEMFC, its Pt intensity during 2020-2060 is assumed to follow the trajectory calibrated by Ministry of Industry and Information Technology of China8,37,38. For PAFC, its Pt intensity was 19 times that of PEMFC in the same year39,40. This study assumed the same ratio between Pt intensity of PAFC and that of PEMFC throughout the study period.

Figure S14 Pt intensity evolution of key hydrogen technologies

Supplemental References

1.	Hydrogen Council. Hydrogen for Net Zero-A Critical Cost-competitive Energy Vector. https://hydrogencouncil.com/wp-content/uploads/2021/11/Hydrogen-for-Net-Zero_Full-Report.pdf (2021).

2.	IEA. An Energy Sector Roadmap to Carbon Neutrality in China. https://www.iea.org/reports/an-energy-sector-roadmap-to-carbon-neutrality-in-china (2021).

3.	China Hydrogen Alliance. White Paper on China Hydrogen and Fuel Cell Industry(2020). (2021).

4.	Shell. Shell Scenarios: Sky - Meeting the goals of the Paris agreement. 72 (2018).

5.	BP. BP Energy Outlook 2022. https://www.bp.com/content/dam/bp/business-sites/en/global/corporate/pdfs/energy-economics/energy-outlook/bp-energy-outlook-2022.pdf (2021).

6.	Raymond, T., Sterck, E. & Clifford, B. PLATINUM ESSENTIALS: Fuel cell electric vehicles are forecast to drive material long-term demand growth for platinum. https://platinuminvestment.com/files/438859/WPIC_Platinum_Essentials_March_2022 (Potential FCEV demand).pdf (2022).

7.	Tong, X., Dai, H., Lu, P., Zhang, A. & Ma, T. Saving global platinum demand while achieving carbon neutrality in the passenger transport sector: linking material flow analysis with integrated assessment model. Resour. Conserv. Recycl. 179, 106110 (2022).

8.	Hao, H. et al. Securing Platinum-Group Metals for Transport Low-Carbon Transition. One Earth 1, 117–125 (2019).

9.	Alonso, E., Field, F. R. & Kirchain, R. E. Platinum Availability for Future Automotive Technologies. Environ. Sci. Technol. 46, 12986–12993 (2012).

10.	Rasmussen, K. D., Wenzel, H., Bangs, C., Petavratzi, E. & Liu, G. Platinum Demand and Potential Bottlenecks in the Global Green Transition: A Dynamic Material Flow Analysis. Environ. Sci. Technol. 53, 11541–11551 (2019).

11.	Elshkaki, A. An analysis of future platinum resources, emissions and waste streams using a system dynamic model of its intentional and non-intentional flows and stocks. Resour. Policy 38, 241–251 (2013).

12.	Idoine, N. E. et al. World Mineral Production 2016-20. (British Geological Survey, 2022).

13.	U.S. Geological Survey. Mineral commodity summaries 2022. https://doi.org/10.3133/mcs2022 (2022).

14.	SFA Oxford. Platinum Standard. https://www.sfa-oxford.com/media/bgqp0zlh/sfa-oxford-edition-10-the-platinum-standard-may-2021.pdf (2021).

15.	Nassar, N. T. Global stocks and flows, losses, and recoveries of platinum-group elements. (2015).

16.	Rasmussen, K. D., Wenzel, H., Bangs, C., Petavratzi, E. & Liu, G. Platinum Demand and Potential Bottlenecks in the Global Green Transition: A Dynamic Material Flow Analysis. Environ. Sci. Technol. (2019) doi:10.1021/acs.est.9b01912.

17.	Ministry of Natural Resources, P. China Mineral Resources. (GEOLOGICAL PUBLISHING HOUSE, 2021).

18.	LBMA. LBMA Precious Metal Prices: Platinum. https://www.lbma.org.uk/cn/prices-and-data#/%0A.

19.	UN Comtrade. United Nations commodity trade statistics database. https://comtrade.un.org/.

20.	Nansai, K. et al. Global Flows of Critical Metals Necessary for Low-Carbon Technologies: The Case of Neodymium, Cobalt, and Platinum. Environ. Sci. Technol. 48, 1391−1400 (2014).

21.	Catlow, C. R., Davidson, M., Hutchings, G. J. & Mulholland, A. Science to enable the circular economy. Philos. Trans. R. Soc. A Math. Phys. Eng. Sci. 378, 20200060 (2020).

22.	Khan, S., Nontso, Z., Magweregwede, F., Godfrey, L. & Sereme, B. Placing the South African mining sector in the context of a circular economy transition. Circ. Econ. AS Dev. Oppor. 9 (2021).

23.	Burlakovs, J. et al. Platinum Group Elements in Geosphere and Anthroposphere: Interplay among the Global Reserves, Urban Ores, Markets and Circular Economy. Minerals  vol. 10 (2020).

24.	Abdelkareem, M. A. et al. Environmental aspects of fuel cells: A review. Sci. Total Environ. 752, (2021).

25.	Calvin, K. et al. GCAM v5. 1: representing the linkages between energy, water, land, climate, and economic systems. Geosci. Model Dev. 12, 677–698 (2019).

26.	Zhang, Y., Zhang, Z., Miao, Y., Liu, X. & Wu, Y. Report on the development of China’s hydrogen industry (in Chinese). http://pg.jrj.com.cn/acc/Res/CN_RES/INDUS/2020/10/22/73726abb-c2db-4c81-b0d7-1f6aa3c36707.pdf (2020).

27.	E4Tech. The Fuel Cell Industry Review 2020. https://fuelcellindustryreview.com/ (2021).

28.	Bareiß, K., de la Rua, C., Möckl, M. & Hamacher, T. Life cycle assessment of hydrogen from proton exchange membrane water electrolysis in future energy systems. Appl. Energy 237, 862–872 (2019).

29.	Staffell, I. et al. The role of hydrogen and fuel cells in the global energy system. Energy Environ. Sci. 12, 463–491 (2019).

30.	Graedel, T. E. et al. What do we know about metal recycling rates? J. Ind. Ecol. 15, 355–366 (2011).

31.	Graedel, T. E. et al. Recycling rates of metals: a status report. (United Nations Environment Programme, 2011).

32.	Duclos, L. et al. Closing the loop: Life cycle assessment and optimization of a PEMFC platinum-based catalyst recycling process. Green Chem. 22, 1919–1933 (2020).

33.	Carmo, M., Fritz, D. L., Mergel, J. & Stolten, D. A comprehensive review on PEM water electrolysis. Int. J. Hydrogen Energy 38, 4901–4934 (2013).

34.	Smolinka, T., Wiebke, N., Sterchele, P., Lehner, F. & Jansen, M. Sudie_IndWEDe-Industrialisierung der Wasserelektrolyse in Deutschland: Chancen und Herausforderungen für nachhaltigen Wasserstoff für Verkehr, Strom und Wärme. (NOW GmbH, 2018).

35.	Bernt, M., Siebel, A. & Gasteiger, H. A. Analysis of voltage losses in PEM water electrolyzers with low platinum group metal loadings. J. Electrochem. Soc. 165, F305 (2018).

36.	Mayyas, A. T., Ruth, M. F., Pivovar, B. S., Bender, G. & Wipke, K. B. Manufacturing cost analysis for proton exchange membrane water electrolyzers. (2019).

37.	Ministry of Industry and Information Technology. Technology Roadmap for Energy-Saving and New Energy Vehicles. (China Machine Press, 2016).

38.	New Energy and Industrial Technology Development Organization. Fuel cell and hydrogen technology roadmap. (China Machine Press, 2017).

39.	Remick, R. & Wheeler, D. Molten Carbonate and Phosphoric Acid Stationary Fuel Cells: Overview and Gap Analysis. https://www.osti.gov/biblio/990108 (2010) doi:10.2172/990108.

40.	Staffell, I. Zero carbon infinite COP heat from fuel cell CHP. Appl. Energy 147, 373–385 (2015).