Supplementary materials for

Preclinical evaluation of new -radionuclide therapy targeting LAT1:

2-[211At]astato--methyl-phenylalanine in tumor-bearing model

Yasuhiro Ohshima1,†, Hiroyuki Suzuki2, Hirofumi Hanaoka3, Ichiro Sasaki1, Shigeki Watanabe1, Hiromitsu Haba4, Yasushi Arano2, Yoshito Tsushima3, Noriko S. Ishioka1

Affiliations

1Department of Radiation-Applied Biology Research, Quantum Beam Science Research Directorate, National Institute for Quantum and Radiological Science and Technology, Takasaki, Japan.

2Graduate School of Pharmaceutical Science, Chiba University, Chiba, Japan

3Gunma University Graduate School of Medicine, Maebashi, Japan

4Nishina Center for Accelerator-Based Science, RIKEN, Wako, Japan

†Corresponding author:

Yasuhiro Ohshima, PhD.

Quantum Beam Science Directorate, National Institute for Quantum and Radiological Science and Technology, 1233 Watanuki, Takasaki, Gunma, 370-1292, Japan

Tel: +81 27 346 9461

Fax: +81 27 346 9688

E-mail: ohshima.yasuhiro@qst.go.jp

Detailed Materials and Methods

Production of 211At

211At was produced via the 209Bi(,2n)211At reaction [1]. 209Bi foil (size: 10 mm × 10 mm × 0.25 mm thickness, purity: 99.999%, Goodfellow Corporation (Coraopolis, PA, USA)) was irradiated with -particles (28.1 MeV, current: 3.5 µA) for 3 h using an azimuthally varying-field (AVF) cyclotron AVF930 (Sumitomo Heavy Industry) at the Takasaki Advanced Radiation Research Institute, QST. 211At was recovered by the dry distillation method reported previously [1]. Briefly, an irradiated target was heated on a quartz boat at 650°C for 30 min under streaming of He gas to distill 211At, and the evacuated 211At was trapped into a polyetheretherketone (PEEK) tube cooled by liquid nitrogen. 211At was also produced at the RIKEN RI Beam Factory in a similar manner [2]. A metallic 209Bi target (chemical purity: >99.999%, thickness: 25.5 mg/cm2) prepared by a vacuum evaporation on an aluminum plate (thickness: 1 mm) was irradiated with -particles (29.1-MeV, current: 6.1 µA) delivered from the RIKEN AVF cyclotron. The -particles first passed through a Be vacuum window (thickness: 18.1 μm), He cooling gas (length: 65 mm; pressure: 1.08 bar), and finally hit the 209Bi target at 15 degrees. Thus, the effective thickness of the 209Bi target was 98.0 mg/cm2. The -particle energy was calculated to be 28.2 MeV on the center of the target with the SRIM program. After the 8.15-h irradiation, 211At was extracted from the target by heating at 850oC for 15 min under streaming of O2 gas (10 mL/min) and trapped into a perfluoroalkoxy (PFA) tube (i.d.= 1 mm, length = 100 cm) which was cooled with ethanol (-96°C) in a cooling apparatus (EYELA UT-2000). Trapped 211At was harvested with CHCl3 and dried with N2 gas. Radioactivity and radionuclidic purity of 211At were evaluated with a 687-keV γ-ray using a high-purity germanium (HP-Ge) detector coupled to a multichannel analyzer (Seiko EG&G MCA 7700).

Cell culture

SKOV3 was routinely maintained in Dulbecco's modified Eagle's medium (Wako Pure Chemical Industries, Osaka, Japan) containing 10% heat-inactivated fetal bovine serum (Serum Source International, Charlotte, NC, USA), penicillin (100 units/mL), streptomycin (100 g/mL), and L-glutamine (2 mM) at 37°C in 5% CO2, 95% air.

Fig. S1 TLC radiochromatograms after the first-step reaction preparing Prot-[211At]-2-AAMP (left) and after the following deprotection reaction (right). The Rf value of Prot-[211At]-2-AAMP was 0.47 on TLC developed with a mixture of hexane and ethyl acetate (4/1, v/v) (left), and that of [211At]-2-AAMP was 0.57 on TLC developed with a mixture of 1-butanol, acetic acid and water (4/1/1, v/v/v) (right).

Fig. S2 Body weight change after intravenously injection of [211At]-2-AAMP (0.5, 1, 1.5 and 2 MBq/head) to the mice once at Day 0.

Reference

- Vaidyanathan G, Zalutsky MR. 1-(m-[211At]astatobenzyl)guanidine: synthesis via astato demetalation and preliminary in vitro and in vivo evaluation. Bioconjug Chem. 1992;3:499-503.

- Fujiki K, Kanayama Y, Yano S, Sato N, Yokokita T, Ahmadi P, et al. 211At-labeled immunoconjugate via a one-pot three-component double click strategy: practical access to α-emission cancer radiotherapeutics. Chem Sci. 2019;10:1936-1944.