Supplementary file 4. Definition of OCT/OCTA Metrics

| Parameters | Unit | Definition |
| --- | --- | --- |
| Foveal Avascular Zone |  |  |
| Area | mm² | Area within manually traced FAZ. |
| Perimeter | mm | Perimeter of manually traced FAZ |
| Shape Descriptions | Unitless |  |
| Circularity |  | Irregularity of the FAZ perimeter when compared to the perimeter of a perfect circle of equal area. Mathematically, it is defined by the formula: 4π*[Area]/[Perimeter]²，with a value of 1.0 indicating a perfect circle. |
| Aspect ratio |  | Aspect ratio measures length-to-width ratio, and is calculated by taking the ratio of the major to minor axis of a fitted ellipse ：[Major Axis]/[Minor Axis]. |
| Roundness |  | Roundness is the inverse of aspect ratio, and a completely round circle would have a value of one:4*[Area]/(π*[Major Axis]². |
| Solidity |  | Solidity measures area divided by the convex hull area; solidity values close to one describe circle with little to no serrations.: [Area]/ [Convex area]. |
| DRIL | Grade | Unidentifiable boundaries of the ganglion cell layer (GCL)—inner plexiform layer (IPL) complex, inner nuclear layer (INL), and outer plexiform layer (OPL) within the central 1500 μm region.Grade0=none；Grade1=single Side of the FAZ；Grade2=both Sides of the FAZ. |
| Outer layer fracture | Yes | Outer layer disruption was defined as horizontal length of disruptions involving the external limiting membrane 、Ellipsoid zone(EZ) and Retinal pigment epithelium（RPE）. |
| DMI | Grade | Grade 1 = contour not smooth, round or oval, visible irregularities, but not necessarily abnormal. Grade 2 = obvious damage, <180°; Grade 3 = obvious damage, >180°, partial remnants; Grade 4 = complete damage to the contour. |
| DME | Yes | Defined as a CRT > 300μm. The presence of intraretinal cysts was defined as the presence of localized hypo reflective areas. |
| Vessel Density | % | Ratio of white pixels (vessels/flow) to total pixels in an image. |
| Non-perfusion Area | mm² | Defined as absence of small retinal arteries and or capillaries and can be demonstrated as contours through adjacent small arteries，Including the single NPA area greater than 0.05mm² into the statistics. |