Table S1. Selected ions were applied for the quantification and qualification of PAH analytes by GC-MS equipment (SIM mode).

| Ion group | Analyte (PAHs) | Time window (min.) | Confirmation ions (m/z) | Quantification ion (m/z) |
| --- | --- | --- | --- | --- |
| 1 | I.S. (Biphenyl) | 8–13 | 153, 152 | 154 |
| 1 | NA (Naphthalene) | 6–13 | 128, 127 | 128 |
| 1 | Ace (Acenaphthylene) | 8–13 | 153, 151 | 152 |
| 1 | Ac (Acenaphthene) | 8–13 | 154, 152 | 153 |
| 2 | F (Fluorene) | 13–15 | 165, 167 | 166 |
| 3 | Pa (Phenanthrene) | 15–17 | 179, 176 | 178 |
| 3 | A (Anthracene) | 15–17 | 179, 176 | 178 |
| 4 | Fl (Fluoranthene) | 17–20 | 203, 101 | 202 |
| 4 | P (Pyrene) | 17–20 | 203, 101 | 202 |
| 5 | BaA (Benzo[a] anthracene) | 20–23 | 226, 229 | 228 |
| 5 | Ch (Chrysene) | 20–23 | 226, 229 | 228 |
| 6 | BbF (Benzo[b] fluoranthene) | 23–28 | 253, 126 | 252 |
| 6 | BkF (Benzo[k] fluoranthene) | 23–28 | 253, 126 | 252 |
| 6 | BaP (Benzo[a]pyrene) | 23–28 | 253, 126 | 252 |
| 7 | IP (Indeno[1,2,3– cd]pyrene) | 28–31 | 277, 138 | 276 |
| 7 | DhA (Dibenzo[a,h] anthracene) | 28–31 | 279,139 | 278 |
| 7 | BgP (Benzo[g,h,i] perylene) | 28–31 | 277,138 | 276 |

Table S2: PAHs and their toxic equivalent factors (TEFs).

| PAHs | TEF | PAHs | TEF |
| --- | --- | --- | --- |
| Benzo(a)pyrene (BaP) | 1 | Anthracene (A) | 0.01 |
| Dibenz(a,h)anthracene (DahA) | 1 | Naphthalene (NA) | 0.001 |
| Benzo(k)fluoranthene (BkF) | 0.1 | Acenaphthylene (AC) | 0.001 |
| Indeno(l,2,3-cd)pyrene (IcdP) | 0.1 | Acenaphthene (ACE) | 0.001 |
| Benz(a)anthracene (BaA) | 0.1 | Phenanthrene (PHE) | 0.001 |
| Benzo(b)fluoranthene (BbF) | 0.1 | Fluorine (FLO) | 0.001 |
| Chrysene (CHR) | 0.01 | Pyrene (PYR) | 0.001 |
| Benzo(g,h,i)perylene (BghiP) | 0.01 | Fluoranthene (FL) | 0.001 |

Table S3. Reproducibility relative standard deviation (RSDR; n = 6), repeatability relative standard deviation (RSDr; n = 6), Recoveries, Linear range, LOQ, LOD, and Coefficient of estimation (r2).

| Target compound | Linear range (µg/L) | Limit of detection (LOD) (µg/L) | Limit of quantification (LOQ) (µg/L) | Coefficient of estimation (r2) | Recoveries (%) | Repeatability (RSDr) (%) | Reproducibility (RSDR) (%)* |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Na | 0.005-10 | 0.08 | 0.24 | 0.988 | 100.4 | 8.2 | 8, 9, 12 |
| Ace | 0.005-10 | 0.04 | 0.12 | 0.995 | 94.8 | 8.9 | 8, 12, 16 |
| Ac | 0.005-10 | 0.07 | 0.21 | 0.992 | 98.2 | 7.3 | 6, 11,15 |
| F | 0.005-10 | 0.06 | 0.18 | 0.997 | 100.5 | 6.8 | 5, 10, 12 |
| Ph | 0.005-10 | 0.04 | 0.12 | 0.989 | 98.6 | 6.2 | 6, 9, 11 |
| A | 0.005-10 | 0.03 | 0.09 | 0.990 | 93.3 | 9.8 | 9, 13, 17 |
| Fl | 0.005-10 | 0.05 | 0.15 | 0.996 | 99.6 | 6.9 | 6, 8, 11 |
| P | 0.005-10 | 0.04 | 0.12 | 0.987 | 102.4 | 7.5 | 7, 12, 13 |
| B(a)A | 0.005-10 | 0.09 | 0.27 | 0.991 | 97.2 | 8 | 7, 9, 12 |
| Ch | 0.005-10 | 0.16 | 0.48 | 0.994 | 97.7 | 8.7 | 8, 11, 17 |
| B(b)F | 0.005-10 | 0.13 | 0.39 | 0.995 | 100.8 | 8.9 | 9, 10, 14 |
| B(k)F | 0.005-10 | 0.08 | 0.24 | 0.990 | 96.6 | 8.1 | 7, 13, 17 |
| B(a)P | 0.005-10 | 0.06 | 0.18 | 0.992 | 101.6 | 10.3 | 10, 14, 17 |
| D(h)A | 0.005-10 | 0.14 | 0.42 | 0.989 | 99.3 | 9.5 | 9, 13, 16 |
| B(g)P | 0.005-10 | 0.28 | 0.84 | 0.991 | 94.8 | 8.8 | 8, 10, 14 |
| I(cd)P | 0.005-10 | 0.21 | 0.63 | 0.993 | 96.1 | 8.7 | 8, 11, 14 |
| *RSDR of 1 µg/L, 5 µg/L, and 10 µg/L standard value (n = 6) |  |  |  |  |  |  |  |