Additional file 2: Supplementary Tables

S1 Table: Summarised technical characteristics of RNA extraction methods from PAXgene® and Tempus™ Blood RNA Tubes.

| Blood stabilization tube type | PAXgene® Blood RNA Tubes |  | Tempus™ Blood RNA tubes |  |
| --- | --- | --- | --- | --- |
| RNA isolation protocol | PAXgene® Blood RNA Kit | MagMAX™ for Stabilized Blood Tubes RNA Isolation Kit, compatible with PAXgene® Blood RNA tubes | Tempus™ Spin RNA Isolation Kit | MagMAX™ for Stabilized Blood Tubes RNA Isolation Kit, compatible with Tempus™ Blood RNA tubes |
| Blood volume | 2.5mL | 2.5mL | 3mL | 3mL |
| RNA isolation method | spin columns | magnetic beads | spin columns | magnetic beads |
| RNA processing method | manual | manual | manual | manual |
| Proteinase K treatment requirement | yes | yes | no | yes |
| DNase digestion implemented in Kit | yes | yes | no (optional) | yes |
| Elution volume (µL) | 40 | 20-80 | 90 | 20-80 |
| Expected RNA yield | >3μg | 3-18μg | 6-25μg | 3-25μg |

S2 Table: List of primers used in this study for quantitative real-time PCR and their efficiencies. 
Regression line fit to raw Cq values against log2 diluted cDNA concentrations. R2 values were calculated using Pearson’s correlation. Efficiencies were calculated as per MIQE guidelines (E = 10^((-1/slope)-1)) using the slope of the regression line previously described.

| Gene | GenBank Accession | Primer Bank ID | Primer Sequence | Amplicon Length (bp) | Standard Curve linear equation | R2 | Efficiency (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBP (Short) |  |  | Forward |  |  |  |  |
|  | NM_003194 | 285026518c1 | CCACTCACAGACTCTCACAAC | 127 | y = -3.203x + 23.385 | 0.995 | 105.21 |
|  |  |  | Reverse |  |  |  |  |
|  |  |  | CTGCGGTACAATCCCAGAACT |  |  |  |  |
| TBP (Mid) |  |  | Forward |  |  |  |  |
|  | NM_003194 | 285026518c2 | CCACTCACAGACTCTCACAAC | 274 | y = -3.0639x + 24.876 | 0.984 | 112.02 |
|  |  |  | Reverse |  |  |  |  |
|  |  |  | AATCAGTGCCGTGGTTCGTG |  |  |  |  |
| SDHA (Short) |  |  | Forward |  |  |  |  |
|  | NM_004168 | 156416002c1 | CAAACAGGAACCCGAGGTTTT | 201 | y = -3.2634x + 20.886 | 0.977 | 102.46 |
|  |  |  | Reverse |  |  |  |  |
|  |  |  | CAGCTTGGTAACACATGCTGTAT |  |  |  |  |
| SDHA (Mid) |  |  | Forward |  |  |  |  |
|  | NM_004168 | 156416002c2 | CAAACAGGAACCCGAGGTTTT | 301 | y = -3.4536x + 22.792 | 0.993 | 94.78 |
|  |  |  | Reverse |  |  |  |  |
|  |  |  | GGTGTCGTAGAAATGCCACCT |  |  |  |  |
| 18s (Short) |  |  | Forward |  |  |  |  |
|  | NM_022551 | 14165467c1 | GCGGCGGAAAATAGCCTTTG | 139 | y = -3.3503x + 17.163 | 0.996 | 98.83 |
|  |  |  | Reverse |  |  |  |  |
|  |  |  | GATCACACGTTCCACCTCATC |  |  |  |  |
| 18s (Mid) |  |  | Forward |  |  |  |  |
|  | NM_022551 | 14165467c2 | GCGGCGGAAAATAGCCTTTG | 229 | y = -3.3656x + 17.715 | 0.993 | 98.21 |
|  |  |  | Reverse |  |  |  |  |
|  |  |  | GACCTGGCTGTATTTTCCATCC |  |  |  |  |

S3 Table: Multiple linear regression model outputs for RNA concentration, A260/A280 and RIN values at each temperature condition.

| RNA concentration (ng/μL) - Spectrophotometer |  |  |  |  |
| --- | --- | --- | --- | --- |
| 25°C (adj-R2 = 0.832 (p = 3.494e-05) |  |  |  |  |
|  | Estimate | Std. Error | Statistic | p value |
| (Intercept) | 73.893 | 16.145 | 4.577 | 0.0006*** |
| Tube type: Tempus | 98.962 | 13.182 | 7.507 | 7.16e-06*** |
| Days7 | 0.988 | 16.145 | 0.061 | 0.952 |
| Days10 | -15.822 | 16.145 | -0.980 | 0.346 |
| Subject B | -31.642 | 16.145 | -1.960 | 0.074 |
| Subject C | 57.528 | 16.145 | 3.563 | 0.003900** |
| 30°C (adj-R2 = 0.761 (p = 0.0003) |  |  |  |  |
| (Intercept) | 50.483 | 17.482 | 2.888 | 0.014* |
| Tube type: Tempus | 69.487 | 14.274 | 4.868 | 0.0004*** |
| Days7 | -0.655 | 17.482 | -0.037 | 0.971 |
| Days10 | 3.330 | 17.482 | 0.190 | 0.852 |
| Subject B | -34.077 | 17.482 | -1.949 | 0.075 |
| Subject C | 67.992 | 17.482 | 3.889 | 0.002** |
| 35°C (adj-R2 = 0.484 (p = 0.020) |  |  |  |  |
| (Intercept) | 79.980 | 23.110 | 3.460 | 0.005*** |
| Tube type: Tempus | 45.300 | 18.870 | 2.401 | 0.033* |
| Days7 | -25.120 | 23.110 | -1.087 | 0.298 |
| Days10 | -39.210 | 23.110 | -1.696 | 0.116 |
| Subject B | -30.830 | 23.110 | -1.334 | 0.207 |
| Subject C | 49.340 | 23.110 | 2.135 | 0.054 |
| 40°C (adj-R2 = 0.630 (p = 0.003) |  |  |  |  |
| (Intercept) | 74.480 | 13.910 | 5.355 | 0.0002*** |
| Tube type: Tempus | 29.530 | 11.360 | 2.600 | 0.023* |
| Days7 | -29.340 | 13.910 | -2.109 | 0.057 |
| Days10 | -19.930 | 13.910 | -1.433 | 0.177 |
| Subject B | -38.470 | 13.910 | -2.766 | 0.017* |
| Subject C | 27.260 | 13.910 | 1.960 | 0.074 |
| A260/A280 |  |  |  |  |
| 25°C (adj-R2 =-0.186 (p = 0.794) |  |  |  |  |
|  | Estimate | Std.error | Statistic | p value |
| (Intercept) | 2.196 | 0.022 | 101.726 | <2e-16*** |
| Tube type: Tempus | -0.018 | 0.018 | -1.020 | 0.328 |
| Days7 | -0.006 | 0.022 | -0.294 | 0.774 |
| Days10 | -0.006 | 0.022 | -0.263 | 0.797 |
| Subject B | 0.006 | 0.022 | 0.271 | 0.791 |
| Subject C | -0.017 | 0.022 | -0.781 | 0.450 |
| 30°C (adj-R2 = 0.469 (p = 0.023) |  |  |  |  |
| (Intercept) | 2.204 | 0.020 | 111.518 | <2e-16*** |
| Tube type: Tempus | -0.053 | 0.016 | -3.277 | 0.007** |
| Days7 | -0.010 | 0.020 | -0.506 | 0.622 |
| Days10 | -0.024 | 0.020 | -1.197 | 0.254 |
| Subject B | -0.054 | 0.020 | -2.732 | 0.018* |
| Subject C | -0.017 | 0.020 | -0.856 | 0.409 |
| 35°C (adj-R2 = 0.372 (p = 0.055) |  |  |  |  |
| (Intercept) | 2.189 | 0.029 | 74.899 | <2e-16*** |
| Tube type: Tempus | -0.072 | 0.024 | -3.022 | 0.011* |
| Days7 | -0.026 | 0.029 | -0.898 | 0.387 |
| Days10 | -0.018 | 0.029 | -0.605 | 0.557 |
| Subject B | -0.056 | 0.029 | -1.916 | 0.079 |
| Subject C | 0.002 | 0.029 | 0.071 | 0.944 |
| 40°C (adj-R2 = 0.706 (p = 0.001) |  |  |  |  |
| (Intercept) | 2.172 | 0.018 | 117.830 | <2e-16*** |
| Tube type: Tempus | -0.093 | 0.015 | -6.157 | 4.89e-05*** |
| Days7 | -0.018 | 0.018 | -0.968 | 0.352 |
| Days10 | -0.027 | 0.018 | -1.469 | 0.167 |
| Subject B | -0.020 | 0.018 | -1.058 | 0.311 |
| Subject C | 0.024 | 0.018 | 1.307 | 0.216 |
| log2(RIN) |  |  |  |  |
| 25°C (adj-R2 = 0.666 (p = 0.002) |  |  |  |  |
|  | Estimate | Std.error | Statistic | p value |
| (Intercept) | 2.608 | 0.081 | 32.135 | 5.21e-13*** |
| Tube type: Tempus | 0.018 | 0.066 | 0.276 | 0.787 |
| Days7 | -0.241 | 0.081 | -2.967 | 0.012* |
| Days10 | -0.483 | 0.081 | -5.947 | 6.75e-05*** |
| Subject B | -0.025 | 0.081 | -0.303 | 0.767 |
| Subject C | -0.142 | 0.081 | -1.746 | 0.106 |
| 30°C (adj-R2 = 0.744 (p = 0.0004) |  |  |  |  |
| (Intercept) | 1.868 | 0.106 | 17.642 | 6e-10*** |
| Tube type: Tempus | 0.472 | 0.086 | 5.458 | 0.0001*** |
| Days7 | -0.111 | 0.106 | -1.043 | 0.317 |
| Days10 | -0.474 | 0.106 | -4.477 | 0.001*** |
| Subject B | 0.016 | 0.106 | 0.147 | 0.885 |
| Subject C | -0.138 | 0.106 | -1.306 | 0.216 |
| 35°C (adj-R2 = 0.623 (p = 0.004) |  |  |  |  |
| (Intercept) | 1.737 | 0.138 | 12.630 | 2.73e-08*** |
| Tube type: Tempus | 0.496 | 0.112 | 4.416 | 0.001*** |
| Days7 | -0.383 | 0.138 | -2.783 | 0.017* |
| Days10 | -0.437 | 0.138 | -3.177 | 0.008** |
| Subject B | -0.113 | 0.138 | -0.823 | 0.427 |
| Subject C | -0.170 | 0.138 | -1.239 | 0.239 |
| 40°C (adj-R2 = 0.579 (p = 0.01) |  |  |  |  |
| (Intercept) | 1.500 | 0.063 | 23.676 | 1.93e-11*** |
| Tube type: Tempus | 0.045 | 0.052 | 0.875 | 0.399 |
| Days7 | -0.179 | 0.063 | -2.822 | 0.015* |
| Days10 | -0.317 | 0.063 | -5.007 | 0.0003*** |
| Subject B | 0.023 | 0.063 | 0.355 | 0.729 |
| Subject C | -0.072 | 0.063 | -1.135 | 0.278 |

S4 Table: Multiple linear regression model for RNA concentration measured by Agilent 2100 Bioanalyzer

| Agilent 2100 Bioanalyzer | RNA concentration (ng/μL) |  |  |  |
| --- | --- | --- | --- | --- |
| Explanatory variable | Estimate | Std. Error | t value | p value |
| (Intercept) | 57.627 | 214.498 | 0.269 | 0.789 |
| Tube type: Tempus | 438.154 | 128.773 | 3.403 | 0.001** |
| Days | 14.905 | 27.193 | 0.548 | 0.586 |
| Temperature | 1.462 | 6.413 | 0.228 | 0.82 |
| Tube type Tempus: Days | -7.112 | 9.09 | -0.782 | 0.437 |
| Tube type Tempus: Temperature | -7.8 | 3.341 | -2.335 | 0.023* |
| Days: Temperature | -0.507 | 0.813 | -0.624 | 0.535 |
| Adjusted R-squared: 0.603 (p value: 9.591e-12) |  |  |  |  |
| 25°C (adj-R2 = 0.584 (p = 0.006) |  |  |  |  |
|  | Estimate | Std.error | Statistic | p value |
| (Intercept) | 163.830 | 51.120 | 3.205 | 0.008*** |
| Tube type: Tempus | 182.220 | 41.740 | 4.366 | 0.001*** |
| Days7 | 41.830 | 51.120 | 0.818 | 0.429 |
| Days10 | -15.670 | 51.120 | -0.306 | 0.765 |
| Subject B | -111.670 | 51.120 | -2.184 | 0.050* |
| Subject C | 29.330 | 51.120 | 0.574 | 0.005** |
| 30°C (adj-R2 = 0.724 (p = 0.001) |  |  |  |  |
| (Intercept) | 79.170 | 42.050 | 1.882 | 0.084 |
| Tube type: Tempus | 180.330 | 34.340 | 5.252 | 0.0002*** |
| Days7 | -34.170 | 42.050 | -0.812 | 0.432 |
| Days10 | 26.670 | 42.050 | 0.634 | 0.538 |
| Subject B | -72.830 | 42.050 | -1.732 | 0.109 |
| Subject C | 112.830 | 42.050 | 2.683 | 0.020 |
| 35°C (adj-R2 = 0.451 (p = 0.027) |  |  |  |  |
| (Intercept) | 68.280 | 48.030 | 1.421 | 0.181 |
| Tube type: Tempus | 82.670 | 39.220 | 2.108 | 0.057 |
| Days7 | 22.000 | 48.030 | 0.458 | 0.655 |
| Days10 | -82.330 | 48.030 | -1.714 | 0.112 |
| Subject B | 22.000 | 48.030 | 0.458 | 0.655 |
| Subject C | 136.170 | 48.030 | 2.835 | 0.015* |
| 40°C (adj-R2 = 0.596 (p = 0.005) |  |  |  |  |
| (Intercept) | 86.000 | 35.557 | 2.419 | 0.032* |
| Tube type: Tempus | 84.778 | 29.032 | 2.920 | 0.013* |
| Days7 | -3.833 | 35.557 | -0.108 | 0.916 |
| Days10 | -22.833 | 35.557 | -0.642 | 0.533 |
| Subject B | -38.167 | 35.557 | -1.073 | 0.304 |
| Subject C | 118.500 | 35.557 | 3.333 | 0.006** |

*** p< 0.001, **p< 0.01, *p< 0.05 from separate multiple linear regression models with total RNA and RIN values as dependent variables and tube type, day and subjects as the independent variables for each temperature point.

S6 Table: Two-way ANOVA results with multiple comparisons outputs for control and test conditions on PAXgene® and Tempus™ tube types on Ct values obtained for 18s, SDHA and TBP. **** p< 0.0001, *** p< 0.001, **p< 0.01, *p< 0.05, ns – non-significant

| Bonferroni’s multiple comparisons test | Predicted (LS) mean diff. | 95.00% CI of diff. | Below threshold? | Summary | Adjusted p value |
| --- | --- | --- | --- | --- | --- |
| 18s short (100-200bp) |  |  |  |  |  |
| PAXgene® |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 0.216 | -1.394 to 1.826 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 7 | 0.455 | -1.155 to 2.065 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | 0.303 | -1.307 to 1.913 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 0.734 | -0.876 to 2.344 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | -0.058 | -1.668 to 1.552 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | -0.600 | -2.210 to 1.010 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 5 | 0.038 | -1.572 to 1.648 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | -0.384 | -1.994 to 1.226 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 10 | -0.088 | -1.698 to 1.522 | No | ns | >0.999 |
| 25 - 1 vs. 40- 5 | -0.087 | -1.697 to 1.523 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 7 | -0.626 | -2.236 to 0.984 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 10 | -1.678 | -3.288 to -0.068 | Yes | * | 0.035 |
| Tempus™ |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 0.497 | -1.113 to 2.107 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 7 | 0.371 | -1.239 to 1.981 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | 0.354 | -1.446 to 2.154 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | -0.589 | -2.199 to 1.021 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | 0.458 | -1.152 to 2.068 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | 0.503 | -1.107 to 2.113 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 5 | 0.288 | -1.322 to 1.898 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | 0.017 | -1.593 to 1.627 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 10 | 0.025 | -1.585 to 1.635 | No | ns | >0.999 |
| 25 - 1 vs. 40- 5 | -0.825 | -2.435 to 0.785 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 7 | 0.123 | -1.487 to 1.733 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 10 | -0.543 | -2.153 to 1.067 | No | ns | >0.999 |
| 18s medium (200-300bp) |  |  |  |  |  |
| PAXgene® |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | -0.332 | -3.247 to 2.583 | No | ns | 0.953 |
| 25 - 1 vs. 25 - 7 | -0.059 | -5.358 to 5.241 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | -0.220 | -5.786 to 5.347 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 0.142 | -5.013 to 5.297 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | -0.944 | -8.888 to 6.999 | No | ns | 0.942 |
| 25 - 1 vs. 30 - 10 | -1.627 | -7.669 to 4.415 | No | ns | 0.489 |
| 25 - 1 vs. 35 - 5 | -0.590 | -10.91 to 9.726 | No | ns | 1.000 |
| 25 - 1 vs. 35 - 7 | -1.093 | -9.542 to 7.357 | No | ns | 0.917 |
| 25 - 1 vs. 35 - 10 | -1.036 | -13.51 to 11.44 | No | ns | 0.992 |
| 25 - 1 vs. 40- 5 | -1.014 | -10.91 to 8.883 | No | ns | 0.972 |
| 25 - 1 vs. 40 - 7 | -1.567 | -6.057 to 2.924 | No | ns | 0.336 |
| 25 - 1 vs. 40 - 10 | -3.090 | -7.650 to 1.470 | No | ns | 0.105 |
| Tempus™ |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 0.575 | -3.909 to 5.060 | No | ns | 0.919 |
| 25 - 1 vs. 25 - 7 | 0.458 | -4.390 to 5.306 | No | ns | 0.983 |
| 25 - 1 vs. 25 - 10 | 0.115 | -80.75 to 80.98 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 0.169 | -10.41 to 10.75 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | 0.397 | -8.594 to 9.389 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | 0.284 | -2.694 to 3.262 | No | ns | 0.982 |
| 25 - 1 vs. 35 - 5 | -0.646 | -14.10 to 12.81 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | -0.086 | -7.871 to 7.699 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 10 | -0.130 | -5.603 to 5.343 | No | ns | >0.999 |
| 25 - 1 vs. 40- 5 | -0.962 | -18.28 to 16.36 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 7 | -0.093 | -5.292 to 5.107 | No | ns | >0.9999 |
| 25 - 1 vs. 40 - 10 | -0.979 | -10.28 to 8.320 | No | ns | 0.968 |
| SDHA short (100-200bp) |  |  |  |  |  |
| PAXgene® |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | -2.491 | -24.83 to 19.85 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 7 | -2.014 | -7.929 to 3.901 | No | ns | 0.412 |
| 25 - 1 vs. 25 - 10 | -2.829 | -7.956 to 2.298 | No | ns | 0.162 |
| 25 - 1 vs. 30 - 5 | -2.863 | -7.071 to 1.345 | No | ns | 0.107 |
| 25 - 1 vs. 30 - 7 | -3.270 | -6.294 to -0.246 | Yes | * | 0.043 |
| 25 - 1 vs. 30 - 10 | -3.832 | -4.927 to -2.738 | Yes | ** | 0.004 |
| 25 - 1 vs. 35 - 5 | -3.030 | -5.341 to -0.718 | Yes | * | 0.029 |
| 25 - 1 vs. 35 - 7 | -3.624 | -6.454 to -0.794 | Yes | * | 0.031 |
| 25 - 1 vs. 35 - 10 | -3.649 | -9.402 to 2.104 | No | ns | 0.123 |
| 25 - 1 vs. 40- 5 | -3.626 | -7.994 to 0.7429 | No | ns | 0.072 |
| 25 - 1 vs. 40 - 7 | -3.948 | -4.529 to -3.367 | Yes | ** | 0.001 |
| 25 - 1 vs. 40 - 10 | -4.670 | -6.432 to -2.908 | Yes | ** | 0.007 |
| Tempus™ |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 1.209 | -9.472 to 11.890 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 7 | 0.926 | -9.730 to 11.580 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | 0.859 | -14.830 to 16.540 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 1.503 | -7.459 to 10.460 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | 0.856 | -9.513 to 11.230 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | 0.850 | -7.348 to 9.048 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 5 | 0.316 | -3.551 to 4.184 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | -0.039 | -8.257 to 8.179 | No | ns | >0.9999 |
| 25 - 1 vs. 35 - 10 | 0.083 | -65.390 to 65.560 | No | ns | >0.9999 |
| 25 - 1 vs. 40- 5 | -0.179 | -7.152 to 6.794 | No | ns | >0.9999 |
| 25 - 1 vs. 40 - 7 | -0.120 | -7.101 to 6.860 | No | ns | >0.9999 |
| 25 - 1 vs. 40 - 10 | -2.802 | -37.880 to 32.280 | No | ns | >0.9999 |
| SDHA medium (200-300bp) |  |  |  |  |  |
| PAXgene® |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | -3.096 | -4.734 to -1.458 | Yes | **** | <0.0001 |
| 25 - 1 vs. 25 - 7 | -2.592 | -4.230 to -0.954 | Yes | *** | 0.0002 |
| 25 - 1 vs. 25 - 10 | -3.905 | -5.543 to -2.267 | Yes | **** | <0.0001 |
| 25 - 1 vs. 30 - 5 | -4.165 | -5.803 to -2.527 | Yes | **** | <0.0001 |
| 25 - 1 vs. 30 - 7 | -4.810 | -6.448 to -3.172 | Yes | **** | <0.0001 |
| 25 - 1 vs. 30 - 10 | -5.814 | -7.452 to -4.176 | Yes | **** | <0.0001 |
| 25 - 1 vs. 35 - 5 | -4.893 | -6.531 to -3.255 | Yes | **** | <0.0001 |
| 25 - 1 vs. 35 - 7 | -5.387 | -7.025 to -3.749 | Yes | **** | <0.0001 |
| 25 - 1 vs. 35 - 10 | -6.029 | -7.667 to -4.391 | Yes | **** | <0.0001 |
| 25 - 1 vs. 40- 5 | -5.667 | -7.305 to -4.029 | Yes | **** | <0.0001 |
| 25 - 1 vs. 40 - 7 | -6.650 | -8.288 to -5.012 | Yes | **** | <0.0001 |
| 25 - 1 vs. 40 - 10 | -8.037 | -9.675 to -6.399 | Yes | **** | <0.0001 |
| Tempus™ |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 1.039 | -0.599 to 2.677 | No | ns | 0.747 |
| 25 - 1 vs. 25 - 7 | 0.336 | -1.302 to 1.974 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | 0.526 | -1.112 to 2.164 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 0.976 | -0.662 to 2.614 | No | ns | 0.950 |
| 25 - 1 vs. 30 - 7 | 0.641 | -0.997 to 2.279 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | 0.103 | -1.535 to 1.741 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 5 | 0.260 | -1.378 to 1.898 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | -0.495 | -2.133 to 1.143 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 10 | -0.760 | -2.587 to 1.066 | No | ns | >0.999 |
| 25 - 1 vs. 40- 5 | -0.601 | -2.239 to 1.037 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 7 | -0.851 | -2.489 to 0.787 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 10 | -2.941 | -4.579 to -1.303 | Yes | **** | <0.0001 |
| TBP short (100-200bp) |  |  |  |  |  |
| PAXgene® |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | -2.338 | -4.725 to 0.049 | No | ns | 0.059 |
| 25 - 1 vs. 25 - 7 | -2.337 | -4.724 to 0.050 | No | ns | 0.059 |
| 25 - 1 vs. 25 - 10 | -2.657 | -5.044 to -0.270 | Yes | * | 0.021 |
| 25 - 1 vs. 30 - 5 | -2.931 | -5.318 to -0.544 | Yes | ** | 0.008 |
| 25 - 1 vs. 30 - 7 | -3.110 | -5.497 to -0.723 | Yes | ** | 0.005 |
| 25 - 1 vs. 30 - 10 | -3.350 | -5.737 to -0.963 | Yes | ** | 0.002 |
| 25 - 1 vs. 35 - 5 | -2.859 | -5.246 to -0.472 | Yes | * | 0.011 |
| 25 - 1 vs. 35 - 7 | -3.328 | -5.715 to -0.941 | Yes | ** | 0.002 |
| 25 - 1 vs. 35 - 10 | -3.027 | -5.414 to -0.640 | Yes | ** | 0.006 |
| 25 - 1 vs. 40- 5 | -3.337 | -5.724 to -0.950 | Yes | ** | 0.002 |
| 25 - 1 vs. 40 - 7 | -3.178 | -5.565 to -0.791 | Yes | ** | 0.004 |
| 25 - 1 vs. 40 - 10 | -3.713 | -6.100 to -1.326 | Yes | *** | 0.001 |
| Tempus™ |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 1.060 | -1.327 to 3.447 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 7 | 0.506 | -1.881 to 2.893 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | 1.020 | -1.367 to 3.407 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 1.338 | -1.049 to 3.725 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | 0.402 | -1.985 to 2.789 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | 0.264 | -2.123 to 2.651 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 5 | -0.084 | -2.471 to 2.303 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | -0.172 | -2.559 to 2.215 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 10 | -1.934 | -4.321 to 0.453 | No | ns | 0.204 |
| 25 - 1 vs. 40- 5 | -0.251 | -2.638 to 2.136 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 7 | -0.096 | -2.483 to 2.291 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 10 | -0.796 | -3.183 to 1.591 | No | ns | >0.999 |
| TBP medium (200-300bp) |  |  |  |  |  |
| PAXgene® |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | -2.490 | -5.256 to 0.277 | No | ns | 0.106 |
| 25 - 1 vs. 25 - 7 | -2.597 | -5.364 to 0.169 | No | ns | 0.080 |
| 25 - 1 vs. 25 - 10 | -3.327 | -6.094 to -0.560 | Yes | * | 0.010 |
| 25 - 1 vs. 30 - 5 | -3.600 | -6.367 to -0.833 | Yes | ** | 0.005 |
| 25 - 1 vs. 30 - 7 | -4.976 | -7.743 to -2.209 | Yes | **** | <0.0001 |
| 25 - 1 vs. 30 - 10 | -6.284 | -9.050 to -3.517 | Yes | **** | <0.0001 |
| 25 - 1 vs. 35 - 5 | -4.347 | -7.114 to -1.581 | Yes | *** | 0.001 |
| 25 - 1 vs. 35 - 7 | -5.561 | -8.328 to -2.795 | Yes | **** | <0.0001 |
| 25 - 1 vs. 35 - 10 | -5.537 | -8.303 to -2.770 | Yes | **** | <0.0001 |
| 25 - 1 vs. 40- 5 | -5.139 | -7.906 to -2.373 | Yes | **** | <0.0001 |
| 25 - 1 vs. 40 - 7 | -5.217 | -7.984 to -2.451 | Yes | **** | <0.0001 |
| 25 - 1 vs. 40 - 10 | -5.859 | -8.626 to -3.093 | Yes | **** | <0.0001 |
| Tempus™ |  |  |  |  |  |
| 25 - 1 vs. 25 - 5 | 1.211 | -1.556 to 3.977 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 7 | 0.644 | -2.123 to 3.411 | No | ns | >0.999 |
| 25 - 1 vs. 25 - 10 | 0.910 | -1.857 to 3.676 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 5 | 1.320 | -1.447 to 4.086 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 7 | 0.311 | -2.456 to 3.078 | No | ns | >0.999 |
| 25 - 1 vs. 30 - 10 | -0.068 | -2.834 to 2.699 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 5 | -0.067 | -2.834 to 2.699 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 7 | -0.635 | -3.401 to 2.132 | No | ns | >0.999 |
| 25 - 1 vs. 35 - 10 | -3.655 | -6.422 to -0.888 | Yes | ** | 0.004 |
| 25 - 1 vs. 40- 5 | -0.621 | -3.388 to 2.146 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 7 | -1.094 | -3.861 to 1.672 | No | ns | >0.999 |
| 25 - 1 vs. 40 - 10 | -2.440 | -5.207 to 0.326 | No | ns | 0.121 |