Supplementary Information

Online Resource 1

Article title: Prefeasibility analysis of low-scale biorefineries: annatto and açai case

Journal name: Food and Bioprocess Technology

Author names: Jhonny Alejandro Poveda-Giraldo1, Sara Piedrahita-Rodríguez1, Natalia Salgado Aristizabal1, Haminton Salas Moreno2, Carlos Ariel Cardona Alzate1

Affiliation: 1Institute of Biotechnology and Agribusiness, Department of Chemical Engineering, Universidad Nacional de Colombia sede Manizales, Manizales, Colombia.

2Biology Department, Faculty of Natural Sciences, Universidad Tecnológica del Chocó, Quibdó, Chocó.

e-mail address of the corresponding author: ccardonaal@unal.edu.co

SI1. Process description for the simulations

Scenario 1 (Sc1): Annatto dye paste production (stand-alone)

The production of the annatto dye paste consists of extracting the dye using water (see Fig1). The process began with conditioning the annatto seed (size reduction using ground annatto) and carried to an industrial pulper or maceration machine. Then, the dye extraction process was performed at 25°C and 1 atm in sequential extractions, three and four ground and natural seed extractors, respectively. Subsequently, the extract streams were filtered and evaporated. For this case, the experimental results of the annatto seed dye extractions define which simulation will be carried out, whether with natural seed or ground seed, based on extraction yields.

Fig1 Annatto dye paste production

Scenario 2 (Sc2): Açai pulp production (stand-alone)

The açai pulp production is shown in Fig2. The fruit was disinfected with sodium hypochlorite (100 ppm) for 20 min at 0.75 L of water per kilogram of fruit. Then, the açai was washed with water at a rate of 2 L for each kilogram of açai. This water is finally removed by filtration. The fruit is then softened with hot water (40°C) for 20 min (2.5 L of water per 4 kg of açai) before pulping. After this time, the fruit was carried to a sequential pulping of 4 and 1 mm mesh, where a pulp with 11-14% solids was obtained. This stream underwent a pasteurization process at 85-92°C and was subjected to a thermal shock.

Fig2 Açai pulp production

Scenario 3 (Sc3): Açai pulp production and biogas (biorefinery)

This scenario considers the production of annatto seed dye (Sc1) and the valorization of the waste stream towards biogas production (see Fig3). This section takes the exhausted annatto seed stream to an anaerobic digestion process to obtain a stream rich in biogas (mainly CH4 and CO2) and a residue called digestate. The anaerobic digestion was performed at 37°C and 1 atm.

Fig3 Annatto dye and biogas production

Scenario 4 (Sc4): Annatto dye paste production and biogas (biorefinery)

Similar to Sc3, this scenario considers the production of açai pulp (Sc2) and biogas production (see Fig4). The slurry stream is taken to an anaerobic digestion process (37°C and 1 atm) to obtain biogas and digestate.

Fig4 Açai pulp and biogas production

Scenario 5 (Sc5): Integration scheme (biorefinery)

Sc5 integrates raw materials in the same processing scheme (see Fig5). In this sense, açai and annatto are processed to obtain açai pulp and annatto dye paste following the same operating conditions mentioned in the previous scenarios. The waste streams (slurry and exhausted annatto seed) are integrated to be processed by anaerobic digestion for biogas generation. The integration methodology applied in the experimental section was also applied in the simulation stage (Piedrahita-Rodriguez, 2020). If the experimental results indicate that the integration ratio needs to be readjusted, the modification will be made to simulate a more appropriate one. Thus, the scheme of this scenario consists of a multiple feedstock biorefinery.

Fig5 Integration scheme to açai and annatto use

Reference

Piedrahita-Rodriguez, S. (2020). Design of biorefineries with multiple raw materials for the use of agroindustrial waste in post-conflict zones in Colombia - Master thesis. Universidad Nacional de Colombia sede Manizales.