Supplementary Materials and Methods.

Appendix 1. Culture methods and treatment details

Cell culture

The cancer cells and HIBEpiC cell were cultured as adherent monolayer in RPMI culture medium with 10% fetal bovine serum and 1% penicillin/streptomycin (PS). All cells were maintained at 37˚C in 5% CO2 in a humidified incubator until they reached 80% confluence, after which they were harvested for passaging or experiments.

Irreversible Electroporation details

After the harvest as above, cells were re-suspended in the fresh culture medium for IRE treatment. All cells were treated with pulses (70 pulses, 90 µs pulse duration, 1 HZ) under varying electric field strength (500V/cm, 750 V/cm, 1000 V/cm, 1250 V/cm, 1500 V/cm, 1750 V/cm, 2000 V/cm) using ECM830 pulse generator (Harvard Apparatus Inc., Holliston, MA, USA). Electric pulses were delivered to the cells using electroporation cuvette (Harvard Apparatus, Holliston, MA, USA) with a gap of 2 mm.

Appendix 2. Design and construction of bile duct numerical model

Numerical models of bile duct and electrode

To simplify the calculation, a two-dimensional model of bile duct wall established using Comsol Multiphysics software (Comsol Inc., USA). The length of the model was selected to be 40.0mm and the wall thickness was 1.0 mm. The thicknesses of the epithelium layer (EL) and sub-epithelium connective tissue layer (SL) were 0.3mm and 0.7mm respectively. The plate electrode was used in this study, the length and thickness of the electrode were set to 7mm and 0.05mm respectively.

Electrical and thermal properties of bile duct tissue were obtained from an open database [32]. The electric field distribution from pulses of fixed parameters (1500 V/cm, 90 µs pulse length, 70 pulses, 1 Hz) was determined by solving the Laplace equation, as described in literature [23]. Pennes bioheat transfer equation was selected to estimate the tissue temperature changes from pulse application [23].