Supplementary material

Detailed scanning parameters in the six centers:

ALL the patients were kept in the supine position with both hands near the head. The images were acquired during full expiration and without injection of any contrast.

Upon CT examination, Image acquisition was performed from the level of the thoracic inlet to inferior to the costophrenic angle. Images were obtained with mediastinal window (width, 400 HU; level, 30 HU) and lung window (width, 1500 HU; level, − 600 HU) settings.

Center 1:

The scanning parameters were as follows: 120 kilovolt peak, 146 mA, 0.28 s gantry rotation, 80-mm collimation, a 360-mm field of view and 512-mm matrix. Images were reconstructed with a high-spatial-resolution algorithm for parenchymal analysis.

Center 2:

The scanning parameters were as follows: 120 kilovolt peak, 34 mA, 0.5 s gantry rotation. Images were reconstructed with a high-spatial-resolution algorithm for parenchymal analysis.

Center 3-5

The scanning parameters were as follows: tube voltage, 140kVp; tube current calculated according to individuals’ weight, height, and body mass index; rotation time, 0.5s; 0.6 mm-collimation.

Center 6

The scanning parameters were as follows: 120 kilovolt peak, 143 mA, 0.75s gantry rotation, 16×1.5 collimation, a 330-mm field of view and 512-mm matrix. Images were reconstructed with a high-spatial-resolution algorithm for parenchymal analysis.