Supplementary Information for

Secondary organic aerosol formation from mixed volatile organic compounds: Effect of RO2 chemistry and precursor concentration

Tianzeng Chena, Peng Zhanga, c, Biwu Chua, b, c, Qingxin Maa, b, c, Yanli Ged, Jun Liua, c, Hong Hea, b, c, *

a State Key Joint Laboratory of Environment Simulation and Pollution Control, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China

b Center for Excellence in Regional Atmospheric Environment, Institute of Urban Environment, Chinese Academy of Sciences, Xiamen 361021, China

c University of Chinese Academy of Sciences, Beijing 100049, China

d State Key Joint Laboratory of Environmental Simulation and Pollution Control, Beijing Innovation Center for Engineering Science and Advanced Technology, College of Environmental Sciences and Engineering, Peking University, Beijing 100871, China

Corresponding author: honghe@rcees.ac.cn (Hong He)

# S1. SOA yield derived from individual VOCs

Two sets of additional photochemical experiments in the presence of NOx were carried out in our 30 m3 indoor smog chamber using m-xylene and 1,3,5-trimethylbenzene, respectively. The initial concentrations of m-xylene and 1,3,5-trimethylbenzene ranged from 205–656 ppb and 368–570 ppb, respectively. The detailed experimental conditions are listed in Table S1. According to the relationship between the aerosol mass concentration (Mo) and SOA yield derived from these additional experiments, SOA yield curves of m-xylene and 1,3,5-trimethylbenzene were obtained as shown in Figure S6. They are comparable to the results reported in previous studies1-7. Further, the SOA yields of m-xylene and 1,3,5-trimethylbenzene were estimated according to the SOA yield curves and the aerosol mass concentration formed in the mixed anthropogenic VOC (AVOC) scenario. As for the SOA yield of isoprene, an SOA yield of 0.01 obtained from a review article8 was used.

# S2. Method to estimate the volatility distributions of gas-phase products

According to the method mentioned by Bianchi et al.9, a revised set of 2D-VBS composition activity parameters was adopted to calculate the volatility of gas-phase products as follows:

where =25, =0.475, =0.2, and =+0.9.  and  are the number of carbon and oxygen atoms in the compound, respectively. As for the presence of N-containing organics, mainly in the form of organo-nitrates containing −ONO2 groups, the effect of each group is assumed to lower  by 2.5 decades, during which any NO3- associated with chemical ionization via nitrate clustering was excluded.

# S3. Method to estimate the contribution of intermediate products to SOA formation

To estimate the mass concentration (Mi) of these intermediate products, the formic acid sensitivity was applied to all ions using the following equation10:

where MWi is the molecular weight of species i, Ii is the signal of specific ion i in Hz, R is the ideal gas constant, T is the temperature in Kelvin, and SFA is the sensitivity of formic acid. In our study, the calibration of iodide-CIMS was performed using different concentration gradients of formic acid by flowing N2 over a constant temperature permeation tube filled with formic acid. The sensitivity of formic acid was calculated to be 0.98±0.25 ion counts s-1 ppt-1, which is similar to the sensitivity of 1.2 ion counts s-1 ppt-1 reported by Bannan et al.11.

To further evaluate the contribution of these intermediate products to SOA formation, the mass fraction of species i (ξi) that partition into the condensed phase needs to be determined using the following equation12:

where Ci* is the effective saturation concentration of species i, COA is the organic aerosol concentration formed during the photo-oxidation reaction.

The contribution of these intermediate products to SOA formation is then calculated using the following equation:

Considering that all detected species are assumed to have the same sensitivity as formic acid, the effective saturation concentration of species was estimated according to the speculative molecular composition rather than the exact measured value of individual species; therefore, some uncertainties exist in estimating the contribution of gas-phase organic intermediate products to SOA formation. Nevertheless, this method provides a practical way for estimating the integration properties10. More quantitative data are needed to constrain the instrumental sensitivity for species with different carbon numbers, functional groups and oxidation states to obtain more accurate estimates. Additionally, it is urgent to develop more instruments to simultaneously measure the organic gas- and particle-phase information at the molecular level.

Table S1. Summary of additional experimental conditions in this study.

| Exp. # | RH (%) | T (°C) | HC0 (ppb) | NOx,0 (ppb) | HC0/NOx,0 (ppb/ppb) | ΔHC (ppb) | ΔHC (μg/m3) | ΔMo (μg/m3) | SOA yield c |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mXy01 a | 5.0±1 | 26±1 | 656 | 95.6 | 6.86 | 368 | 1596.4 | 136.4 | 0.0855 |
| mXy02 | 5.0±1 | 26±1 | 529 | 76.7 | 6.90 | 294 | 1276.5 | 103.3 | 0.0809 |
| mXy03 | 5.0±1 | 26±1 | 424 | 55.2 | 7.68 | 250 | 1083.7 | 74.9 | 0.0691 |
| mXy04 | 5.0±1 | 26±1 | 459 | 66.8 | 6.87 | 257 | 1114.7 | 88.8 | 0.0796 |
| mXy05 | 5.0±1 | 26±1 | 375 | 52.7 | 7.12 | 210 | 910.4 | 80.8 | 0.0888 |
| mXy06 | 5.0±1 | 26±1 | 280 | 38.1 | 7.35 | 157 | 680.8 | 46.6 | 0.0684 |
| mXy07 | 5.0±1 | 26±1 | 205 | 31.2 | 6.59 | 115 | 498.3 | 36.4 | 0.0731 |
| TMB01 b | 5.0±1 | 26±1 | 570 | 85.5 | 6.66 | 430 | 2115.3 | 100.0 | 0.0473 |
| TMB 02 | 5.0±1 | 26±1 | 476 | 69.8 | 6.83 | 367 | 1806.3 | 71.6 | 0.0396 |
| TMB 03 | 5.0±1 | 26±1 | 368 | 54.9 | 6.71 | 288 | 1416.4 | 49.6 | 0.0351 |

a mXy is m-Xylene; b TMB is 1,3,5-trimethylbenzene; c SOA yield was calculated as the mass concentration ratio of corrected SOA to reacted hydrocarbon.

Figure S1. Typical dimeric products from the self- and cross-reactions of bicyclic peroxy radicals (BPRs) in the photo-oxidation of different mixed VOCs scenarios (i.e., individual toluene and mixed toluene, m-xylene, and 1,3,5-trimethylbenzene).

Figure S2. Percentage contribution of self- and cross-reactions of RO2 from different VOCs in the photo-oxidation of mixed anthropogenic VOCs (AVOCs) scenario.

Figure S3. Volatility distributions of gas-phase products formed from the photo-oxidation of different mixed VOC scenarios (i.e., toluene, mixed toluene and isoprene, as well as mixed toluene, m-xylene, and 1,3,5-trimethylbenzene) in the presence of NOx (~20 ppb) at (a) higher and (b) lower VOC concentration levels. The intensity normalized to the ion source iodide was used. The intensities of LVOC and ELVOC were magnified by a factor of 10 to make the difference easier to distinguish.

Figure S4. Estimated OH concentration during photo-oxidation in different mixed VOC scenarios (i.e., toluene, mixed toluene and isoprene, as well as mixed toluene, m-xylene, and 1,3,5-trimethylbenzene) in the presence of NOx (~20 ppb) at (a) higher and (b) lower VOC concentration levels.

Figure S5. SOA yield curve obtained in different mixed VOC scenarios with higher precursor concentrations and the SOA yield determined in the mixed AVOC scenario with lower precursor concentrations. Details can be found in Table 1.

Figure S6. SOA yield curves of (a) m-xylene and (b) 1,3,5-trimethylbenzene derived from our study and previous studies1-7.

# References

1	Odum, J. R. et al. Gas/particle partitioning and secondary organic aerosol yields. Environ. Sci. Technol. 30, 2580-2585, doi:10.1021/es950943+ (1996).

2	Ng, N. L. et al. Secondary organic aerosol formation from m-xylene, toluene, and benzene. Atmos. Chem. Phys. 7, 3909-3922, doi:10.5194/acp-7-3909-2007 (2007).

3	Cocker, D. R., Mader, B. T., Kalberer, M., Flagan, R. C. & Seinfeld, J. H. The effect of water on gas-particle partitioning of secondary organic aerosol: II. m-xylene and 1,3,5-trimethylbenzene photooxidation systems. Atmos. Environ. 35, 6073-6085, doi:10.1016/s1352-2310(01)00405-8 (2001).

4	Song, C., Na, K. & Cocker, D. R. Impact of the hydrocarbon to NOx ratio on secondary organic aerosol formation. Environ. Sci. Technol. 39, 3143-3149, doi:10.1021/es0493244 (2005).

5	Song, C., Na, K., Warren, B., Malloy, Q. & Cocker, D. R. Impact of propene on secondary organic aerosol formation from m-xylene. Environ. Sci. Technol. 41, 6990-6995, doi:10.1021/es062279a (2007).

6	Healy, R. M. et al. Gas/particle partitioning of carbonyls in the photooxidation of isoprene and 1,3,5-trimethylbenzene. Atmos. Chem. Phys. 8, 3215-3230, doi:10.5194/acp-8-3215-2008 (2008).

7	Sato, K. et al. AMS and LC/MS analyses of SOA from the photooxidation of benzene and 1,3,5-trimethylbenzene in the presence of NOx: effects of chemical structure on SOA aging. Atmos. Chem. Phys. 12, 4667-4682, doi:10.5194/acp-12-4667-2012 (2012).

8	Carlton, A. G., Wiedinmyer, C. & Kroll, J. H. A review of secondary organic aerosol (SOA) formation from isoprene. Atmos. Chem. Phys. 9, 4987-5005, doi:10.5194/acp-9-4987-2009 (2009).

9	Bianchi, F. et al. Highly oxygenated organic molecules (HOM) from gas-phase autoxidation involving peroxy radicals: a key contributor to atmospheric aerosol. Chem. Rev. 119, 3472-3509, doi:10.1021/acs.chemrev.8b00395 (2019).

10	Chhabra, P. S. et al. Application of high-resolution time-of-flight chemical ionization mass spectrometry measurements to estimate volatility distributions of α-pinene and naphthalene oxidation products. Atmos. Meas. Tech. 8, 1-18, doi:10.5194/amt-8-1-2015 (2015).

11	Bannan, T. J. et al. Seasonality of formic acid (HCOOH) in London during the ClearfLo Campaign. J. Geophys. Res. Atmos. 122, 12488-12498, doi:10.1002/2017jd027064 (2017).

12	Donahue, N. M., Robinson, A. L., Stanier, C. O. & Pandis, S. N. Coupled partitioning, dilution, and chemical aging of semivolatile organics. Environ. Sci. Technol. 40, 2635-2643, doi:10.1021/es052297c (2006).