Supplementary Materials for

Ranking of Empirical Evapotranspiration Models in Different Climate Zones of Pakistan

Mohammed Magdy Hamed1,2, Najeebullah Khan3, Shamsuddin Shahid2, Mohd Khairul Idlan Muhammad2

Table S-1. List of the empirical ET equations and applications (after Muhammad et al., 2018)

| - No | Model (Year) | Equation | Application |
| --- | --- | --- | --- |
| - 1 | Dalton (1802) | ET = 0.3648+0.07223(u)es-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 2 | Trabert (1896) | ET = 0.3075ues-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 3 | Meyer (1926) | ET = 0.375+0.05026(u)es-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 4 | Rohwer (1931) | ET = (3.3+0.891u)es-ea | Valipour (2014), Rezaei (2016) |
| 5 | Penman (1948) | ET = (2.625+0.000479/u)es-ea | Valipour (2014), Rezaei (2016) |
| 6 | Albrecht (1950) | ET = 0.1005+0.297(u)es-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 7 | Makkink (1957) | ET = 0.61ΔΔ+γRs58.5-0.12 | Lu et al (2005), Niaghi et al (2013), Rahimikhoob et al (2012) |
| 8 | Ivanov (1961) | ET = 0.0000625+Tmean2100-RH | Valipour (2014), Rezaei (2016) |
| 9 | Turc (1961) | ET = 0.013TmeanTmean+15Rs+50 | Heydari et al (2013), Lu et al (2005), Xystrakis and Matzarakis (2010) |
| 10 | Hamon (1963) | ET = 0.1651LdRHOSAT x KPEC RHOSAT = 216.7ESATTmean+273.3 ESAT = 6.108exp17.269×TmeanTmean+273.3 | Lu et al (2005) |
| 11 | Jensen Haise (1963) | ET = Rsλ0.025Tmean+0.08 | Xystrakis and Matzarakis (2010); Heydari et al (2014) |
| 12 | Brockamp & Wenner (1963) | ET = 0.543(u0.456)es-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 13 | Papadakis (1966) | ET = 2.5ema-ea | Valipour (2014), Rezaei (2016) |
| 14 | WMO (1966) | ET = 0.1298+0.0934(u)es-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 15 | Schendel (1967) | ET = 16TmeanRH | Bormann (2011), Djaman (2015) |
| 16 | Mahringer (1970) | ET = 0.150723.6ues-ea | Bormann (2011), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 17 | Priestley Taylor (1972) | ET = αΔΔ+γRnλ | Bormann (2011), Niaghi et al (2013), Rahimikhoob et al (2012) |
| 18 | McGuinness & Bordne (1972) | ET = 0.0082Tmean-0.19Rs1500(2.54) | Tabari et al (2013), Heydari et al (2014) |
| 19 | Szasz (1973) | ET = 0.00536Tmean+2121-RH10023f(u) fu=0.0519u+0.905 | Racz et al (2013) |
| 20 | Caprio (1974) | ET = 6.1106Rs1.8Tmean+1.0 | Oudin et al (2005) |
| 21 | Blaney Criddle (1977) | ET = k×p(0.46Tmean+8.13) | Niaghi et al (2013, Heydari et al (2014), Josilva et al (2016), Poyen et al (2018) |
| 22 | Linacre (1977) | ET=700(Tmean±0.006z)100-L+15Tmean-Td80-Tmean | Josilva (2016) |
| 23 | Kharuffa (1985) | ET = 0.34pTmean1.30 | Heydari et al (2014) |
| 24 | Hargreaves Samani (1985) | ET = 0.0023Ra2.45TD0.5Tmean+17.8 | Heydari et al (2013), Oudin et al (2005), Rahimikhoob et al (2012), Xu and Singh (2002), Xystrakis and Matzarakis (2010), Josilva et al (2016) |
| 25 | Ritchie (1990) | ET=α1(3.87×10-3)( Rs(0.6Tmax+0.4Tmin+29)) | Tabari et al (2013) |
| 26 | Abtew (1996) | ET = 0.53Rsλ | Abtew, 1996 |
| 27 | FAO56 PM (1998a) | ET=0.408Rn-G+γ900Tmean+273ues-ea∆+γ(1+0.34u) | Heydari et al (2013), Niaghi et al (2013), Rahimikhoob et al (2012), Tabari et al (2013), Valipour (2014), Rezaei (2016) |
| 28 | Irmak-Rs (2003) | ET = -0.611+0.149Rs+0.079Tmean | Heydari et al (2013), Niaghi et al (2013), Tabari et al (2013), Heydari et al (2014) |
| 29 | Irmak-Rn (2003 | ET = 0.489+0.289Rn+0.023Tmean | Heydari et al (2013), Niaghi et al (2013), Tabari et al (2013), Heydari et al (2014) |
| 30 | Trajkovic (2007) | ET = 0.0023RaTD0.424Tmean+17.8 | Djaman (2015) |
| 31 | Ravazzani (2012) | ET=0.817+0.00022z0.0023Ra(TD0.5)Tmean+17.8 | Djaman (2015) |

ET is the potential evapotranspiration (mm day−1). ET is in mm day−1 in all equations except Ritchie and McGuinness & Bordne models, ET is in cm day−1. Rn is the net radiation (MJ m−2 day−1). G is the soil heat flux (MJ m−2 day−1). Ra is the extraterrestrial radiation (MJ m−2 day−1). γ is the psychrometric constant (kPa/°C). es is the saturation vapour pressure (hPa). ea is the actual vapour pressure (hPa). es and ea are in hPa in all equations except Papadakis, Rohwer, Penman and FAO56 Penman-Monteith models, es and ea are in kPa. Δ is the slope of the saturation vapour pressure–temperature curve (kPa/°C). λ is latent heat of evaporation (MJ/kg). Tmean is the average daily air temperature (°C). Tmean is in °C in all equations except McGuinness & Bordne model, Tmean is in degree Fahrenheit. U is the mean daily wind speed at 2 m (m s−1). f(u) is function of wind speed. Z is the elevation (m). L is local latitude (degrees). Td is dew point temperature (◦C). Tmin is the minimum air temperature (°C). Tmax is the maximum air temperature (°C). TD is maximum and minimum temperature difference (°C). RH is the average relative humidity (%). Rs is the solar radiation. Rs is in MJ m−2 day−1 in all equations except Turc, Makkink, Ritchie and McGuinness & Bordne models, Rs is in Cal/m2 day and Caprio model, Rs is in kJ/m2 day. ema is the saturation vapour pressure at the monthly mean daily maximum temperature (kPa). k is monthly consumptive use coefficient which depends on location, season and vegetation type. The average value of k is 0.8 which also requires local calibration (Poyen et al, 2018). p is the mean annual percentage of daytime hours for different latitudes that can be obtained from Doorenbos and Pruitt (1977). p is expressed as constant (0.274) in Josilva et al (2016). Ld is daytime length in multiples of 12 h. RHOSAT is saturated vapor density (g m−3). ESAT is saturated vapor pressure (mbar). KPEC is calibration coefficient (1.2). α is constant (1.26). α1 is constant (1.1).