Appendix

  - DNA Analysis

Technique: Real—Time PCR using the Taqman® SNP genotyping Assays

Apparatus: Samples to be analysed (DNA template), P2 micro pipette with tips, Tris HCl, EDTA, DNase-free water, sterile filtered paper, 96 well plates, vortex, thermo cycler, centrifuge Taqman® genotyping mastermix 2X.

Taqman® genotyping assay 40X: (Purchased from Life Technologies®; a part of Thermo Fisher Scientific®)

Primer: APOL1

Probe: rs73885319, rs71785313

Principle: Two allele-specific minor groove binding (MGB) probes that are labelled with distinct fluorescent dyes to allow for allelic discrimination. The assays are predesigned, and include PCR primer pairs in addition to the probes which allow for specific amplification of the target sequence prior to genotyping. Detection is achieved through real time PCR equipment, and relies on the 5’ exonuclease activity of the Taq enzyme for cleavage of the allele-specific 5’ signal to allow for generation of the permanent assay signal. This signal is detected only after extension by Taq Polymerase due to the nonfluorescent quencher (NFQ) attached at the 3’ end of the probe to prevent nonspecific signal being detected, subsequently improving the sensitivity and specificity of the assay.

Procedure:

- Sample Preparation: Wet DNA samples

- The DNA sample in each tube was diluted with DNase-free water to deliver a final DNA mass in the range of 1 to 20ng/well

- All wells were carefully filled to make sure for the same amount of the samples and non-template controls.

- A volume of 11.25µl of samples was used as template.

- Diluting the SNP Genotyping assays:

  - Preparation of 100ml 1X Tris EDTA buffer solution: In a beaker containing 1ml of Tris-HCl (brought to pH 8.0), 0.2ml of 0.5M EDTA was added. Distilled water was added up to the 100ml mark.

  - In the tube containing the 40X genotyping assay, the previously made solution was added to double the original amount.

  - This mixture was the vortexed and centrifuged for 30 seconds and 2minutes at 10 000Xg respectively. It was then aliquoted into 3 different recipients and stored at -200C.

- Preparing the Reaction mix

  - The number of reactions to be performed for each assay was calculated (at least two non-template controls (NTC) were included for each plate

  - The volumes of the different components of the mixture were added as seen in the table below.

Table S1: Reagent volumes for the PCR reaction mix

| Reagent | Concentration | Volume for 1 sample (µl) | Volume for 1 sample(µl) | Volume for 337 samples (µl) |
| --- | --- | --- | --- | --- |
| Taqman universal PCR master mix (2X), No AmpErase UNG | 2X | 12.5 | 10 | 3370 |
| 20X working stock of SNP genotyping assay | 20X | 1.25 | 1.0 | 337 |
| DNase-free water |  | / | / |  |
| Sample | 1-50ng | 11.25 | 9 |  |
| Total |  | 25 | 20 |  |

- The bottle containing Taqman® Universal PCR mastermix (2X) (abbreviated as UMM in subsequent steps), no AmpErase UNG was gently swirled. The UMM was well mixed before use.

- The Taqman SNP genotyping assay was then vortexed for 30seconds and centrifuged at 10 000Xg for 1 minute.

- The required amounts of UMM and 20X SNP genotyping assay was pipetted into a sterile well on the 96 well plate which was then capped and inverted severally to mix the contents.

- This was then centrifuged briefly to spin down the contents and eliminate air bubbles from the reaction mix.

- Cycling Conditions:

- The samples were run in the Real-Time PCR system using the following conditions

Table S2: Cycling Conditions for PCR reaction mix

|  | AmpiTaq Gold Enzyme Activation | PCR |  |
| --- | --- | --- | --- |
|  | HOLD | CYCLE (40 cycles) |  |
|  |  | Denature | Anneal/ Extend |
| Time | 10minutes | 15 seconds | 1minute |
| Temperature | 950C | 920C | 600C |

PCR polymerase chain reaction

- At least three replicates of each standard or sample were used to ensure that an accurate standard curve is generated.

- The range of known copy number should bracket anticipated copy numbers of the unknown samples on the same plate.

- Analysis

- ROX Passive Reference: The ROXTM Passive Reference provides an internal reference to which the reporter dye signal is normalized during data analysis on Applied Biosystems instruments. Normalization is necessary to correct for fluorescence fluctuations due to changes in concentration and volume.

- Rn Values: Normalization is accomplished by dividing the emission intensity of the reporter dye by the emission intensity of the ROX Passive Reference. The resulting ratio is the Rn (normalized reporter) for a given reaction tube.

Where

Rn = Emission Intensity of Reporter / Emission intensity of Passive

- Standard Curves

- Quantification