# Appendix

## Corrected ejection fraction

The EFc was calculated using the following regression equation17:

EFc (%) = 38.9-0.423D+2.07W-0.648z -2.07θ -0.174L (R2 = 0.856, P<0.0001).

(Eq 1)

Where D = LVIDd (mm), W=EDWT (mm), z = longitudinal shortening, θ = midwall circumferential shortening. To calculate the EFc, we inputted D and W using the mean of the normal control cohort i.e. D = 52 mm, W = 7.6 mm and the longitudinal shortening (z), midwall circumferential shortening (θ) and length (L) were inputted as the measured values in each individual.

## Midwall circumferential shortening

Midwall circumferential shortening (θ) was estimated using the following equation:

θ %= LVIDd + EDWT– LVIDs + HLVIDd + EDWT× 100 					(Eq 2)

where H = ((LVIDd + EDWT)3 – (LVIDd)3 + (LVIDs)3)1/3 - LVIDs

## Stresses

### Longitudinal Lamé stress

σz=Piri2 - Poro2ro2-ri2  and since pericardial pressure is small the Poro2 term can be ignored, and the equation simplified to:

σz=Piri2ro2-ri2 									(Eq 3)

### Midwall circumferential (hoop) Lamé stress

σθ = Piri2 - Poro2ro2-ri2 - ri2ro2Po-Pirm2ro2-ri2 						(Eq 4)

where Pi is inner (ventricular cavity) pressure (in Pa), Po is outer (pericardial) pressure and rm is midwall, ro is outer (epicardial), and ri is inner (luminal) LV radii. We assumed Po to be atmospheric, i.e., Po = 0 - gauge pressure).

### Laplace longitudinal stress

Laplace longitudianal  stress=P×D4t, 					(Eq 5)

where P is systolic pressure, D is internal diameter, t is mean wall thickness.

### Cardiomyocyte stress

Longitudinal force per shell=cos(θ) × cardiomyocyte stress × π(S2+2rS) 	(Eq 6)

where r=inner edge of shell radius, S = shell thickness, θ = helical angle in radians.

Assuming a 10-shell model S=t/10 where t=end-diastolic wall thickness. Cardiomyocyte stress is then calculated so that total accumulative force for all the shells, calculated using numerical integration, is the same as that determined from longitudinal stress using the Lamé equation (Eq 3).

## Stroke work

SW = MAP × SV 								(Eq 7a)

where MAP = (SBP + 2 × DBP)/3. SBP is peak systolic blood pressure and DBP is diastolic blood pressure measure in Pa.

## LV pressure-strain loop (LV PSL) index

LV PSL=SBP× εz% in mmHg% 					(Eq 7b)

converted from mmHg% to mHg% by dividing by 1,000.

## Strain energy density and strain energy

GLASED was calculated using the longitudinal Lamé derived stress (σz) and longitudinal shortening (εz), such that:

GLASED = ½ × σz × εz .							(Eq 8)

CASED was calculated using the midwall circumferential (hoop) Lamé derived stress (σθ) and midwall circumferential shortening θ, such that:

CASED = ½ × σθ × θ 							(Eq 9)

GLASE and CASE were calculated from M× GLASED and M× CASED  respectively, where M= left ventricular muscle volume derived by MRI.

## The Laplace stress-strain product (SSP)

Laplace longitudinal stress strain product was calculated as follows:

SSP=½*εz  ×Laplace stress

=εz × P×D8t, 							(Eq 10)

where εz is longitudinal shortening, P is systolic pressure, D is internal diameter, t is mean wall thickness.

## Worked example showing the calculation of GLASED

GLASED =  ½ × σz × εz

= ½ × Piri2ro2-ri2 × εz

Let Pi=16,665.3 Pa (SBP 125 mmHg), ri= 0.26 m (half of LVIDD = 52 mm), ro= 0.336 m (ri+EDWT= 26+7.6 mm) and εz= 15.2%

=½× 16,665 × 26.0233.62 - 26.02 × 15.2%

=½ × 16,665 × 67611,29 - 676 × 15.2%

=½ × 11,265,540453 × 15.2%

=½ × 24,871 × 15.2%

GLASED = 1.93 KJ/m3

## Active strain energy density: comparison of non-linear and linear methods

A typical pressure-strain curve was derived from a study by Loncaric and colleagues (Figure A1).44 The curve was divided into 13 points adjusting mural thickness and ventricular diameter between mitral valve closure (MVC) and aortic valve closure (AVC) and a stress-strain curve plotted (Figure A2). The individual longitudinal stress for each of the 12 periods were calculated using the Lamé equation and the longitudinal ASED (GLASED) was calculated from the sum of the ASED for each period and giving the area under the curve between MVC and AVC (Figure A3).

MVC

MVC

AVC

A2

AVC

A1

σz

εz

A3

A4

In this example, the GLASED was 2.80 KPa/m3 (non-linear method) and using Eq 8 was 2.71 KPa/m3 (linear method - blue triangle A4) suggesting that using the ½ factor is a reasonable compromise for routine use in clinical practice. Geometrically, the ½ factor is half the area of the rectangle with height (σz , peak stress based on SBP) and width (εz, peak strain) with area of (σz × εz, dashed box). Note that this compromise is achieved because the vertical hatched area is similar to the horizontal hatched area in Figure A4.

## Strain energy density proof

SED can be derived from the definition of Work i.e. F × l

where F = force, l = displacement/distance, V=volume, A = cross sectional area, L, original length.

Energy work=½×F×l, where F is the peak force (see Figure A4).

=½×F×lV×V, where V is volume.

=½×F×lA×L×V, where A is cross sectional area, L is original length.

=½×FA×lL×V

Substituting  σ= FA   and ϵ=  lL, gives

Strain energy (SE)  =½×σ×ϵ× V, and

Strain energy density (SED) =SE/V

=½×σ×ϵ

# Reproducibility: Interclass correlation Class 2, Type 1 and Alpha 0.05 in 20 individuals from control group

| Measurement | ICC | Lower | Upper |
| --- | --- | --- | --- |
| LV ED length 4C | 0.984 | 0.960 | 0.990 |
| LV ED length 3C | 0.983 | 0.959 | 0.993 |
| LV ED length 2C | 0.990 | 0.974 | 1.000 |
| LV ES length 4C | 0.991 | 0.978 | 0.996 |
| LV ES length 3C | 0.990 | 0.975 | 0.996 |
| LV ES length 2C | 0.984 | 0.960 | 0.994 |
| EDWT | 0.981 | 0.952 | 0.992 |

ED, end-diastolic; ES, end-systolic; 4C, 4 chamber view; 3C, 3 chamber view; 2C, 2 chamber view; EDWT, end-diastolic wall thickness.