# Supplementary

Detailed comparison between experimental and numerical results in terms of crater width and bubble height is tabulated in Table S1. Results are presented for various burial depths of C-4 mine, different experimental shot IDs, and specific time intervals along with their respective error margins.

Table S1 Comparison of soil crater and bubble height dimensions obtained from numerical simulations with experimental values [14]

| Shot# | DOB (mm) | Time (ms) | Crater width (mm) |  |  | Height of detonation cloud (mm) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Experiment [14] | Simulation | Error | Experiment [14] | Simulation | Error |
| 20-2 | 0 | 0.051 | 136 | 139 | 2.2% | - | - | - |
| 19-2 | 0 | 0.102 | 170 | 179 | 5.0% | - | - | - |
| 17-1 | 30 | 0.126 | 199 | 197 | 1.0% | 88 | 84 | 4.5% |
| 17-2 | 30 | 0.201 | 246 | 244 | 0.8% | 165 | 158 | 4.2% |
| 12-2 | 80 | 0.301 | 229 | 232 | 1.3% | 43 | 44 | 2.3% |
| 13-2 | 80 | 0.401 | 291 | 290 | 0.3% | 62 | 65 | 4.8% |

Comparison of In-Air and In-Soil peak overpressure along with time of arrival of the blast wave are presented in Table S2 and Table S3 respectively. Results are presented for three burial depths of 0mm, 30mm and 80mm for different experimental shot IDs and stand-off distances from the mine.

Table S2 Comparison of peak over pressure (In-Air Pressure Transducer) obtained from numerical simulations with experimental values [14]

| In-Air Pressure Transducer |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shot# | DOB (mm) | Stand-off distance (mm) | Peak overpressure (MPa) |  |  | Time of arrival (ms) |  |  |
|  |  |  | Experiment [14] | Simulation | Error | Experiment [14] | Simulation | Error |
| 18 | 0 | 300 | 4.32 | 4.29 | 0.69% | 0.090 | 0.089 | 1.11% |
| 15 | 30 | 330 | 1.39 | 1.38 | 0.72% | 0.270 | 0.265 | 1.85% |
| 14 | 80 | 380 | 0.79 | 0.78 | 1.27% | 0.830 | 0.810 | 2.17% |

Table S3 Comparison of peak over pressure (In-Soil Carbon resistance gauges) obtained from numerical simulations with experimental values [14]

| In-Soil Carbon resistance gauges |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shot# | DOB (mm) | Stand-off distance (mm) | Peak overpressure (MPa) |  |  | Time of arrival (ms) |  |  |
|  |  |  | Experiment [14] | Simulation | Error | Experiment [14] | Simulation | Error |
| 19 | 0 | 87.3 | 68.80 | 68.10 | 1.16% | 0.075 | 0.074 | 0.67% |
| 19 | 0 | 102.3 | 29.53 | 30.23 | 2.37% | 0.112 | 0.108 | 3.93% |
| 19 | 0 | 137.3 | 15.96 | 15.72 | 1.50% | 0.175 | 0.170 | 3.13% |
| 15 | 30 | 87.3 | 55.37 | 55.10 | 0.49% | 0.078 | 0.078 | 0.26% |
| 15 | 30 | 102.3 | 30.73 | 30.84 | 0.36% | 0.120 | 0.116 | 3.34% |
| 17 | 30 | 137.3 | 18.40 | 18.52 | 0.65% | 0.185 | 0.180 | 2.91% |
| 12 | 80 | 87.3 | 53.87 | 53.93 | 0.11% | 0.076 | 0.076 | 0.66% |
| 12 | 80 | 102.3 | 38.22 | 38.19 | 0.08% | 0.118 | 0.117 | 0.93% |
| 12 | 80 | 137.3 | 21.94 | 21.79 | 0.68% | 0.174 | 0.162 | 6.94% |