# MATERIALS AND METHODS

2.3 MRI Data Acquisition

All participants had brain resting-state functional magnetic resonance imaging (rs-fMRI) with a 3T Verio Siemens scanner which with a standard birdcage head coil in order to minimize head movement. During the scan participants were asked to remain as still as possible, close eyes, and remain awake without thinking of anything. T1-weighted images for image registration and functional localization were acquired by 3D spoiled gradent echo (SPGR）, parameters were following: repetition time (TR) =2530 ms ; echo time (TE) =3.34 ms; flip angle (FA) = 7 degrees; number of slices = 128; slice thickness = 1.33 mm; matrix = 256 × 192; field of view FOV = 256 mm × 256 mm. Functional images were acquired with a gradient-recalled echo-planar (GRE-EPI) imaging pulse sequence which included 240 volumes. The parameters were: TR = 2,000 ms; TE = 30 ms; FA=90 degrees; number of slices =31; slice thickness =3.5 mm; matrix = 64× 64; FOV = 220 mm × 220 mm.

  - Image Preprocessing

All fMRI data were preprocessed by Data Processing and Analysis for (Resting-State) Brain Imaging (DPABI 4.3, http://rfmri.org/dpabi), which is based on MATLAB 2014b platform(http://www.mathworks.com/products/matlab). The first 10 volumes of each image were discarded for signal equilibrium, then, corrected the inter-volume motion effects and difference of intra-volume acquisition time between slices. For quality assurance, subject with cumulative translation or rotation >3.0 mm or 3.0◦ was excluded. Structural and functional images were then co-registered. After that, framewise displacement (FD) of all resting state images was calculated. To control the effects of head motion, we used FD as a covariate in further analysis. Then, the Diffeomorphic Anatomical Registration Through Exponentiated Lie Algebra (DARTEL) algorithm was used to normalize and segment the T1 images into gray matter (GM), white matter (WM) and cerebrospinal fluid (CSF) partitions (Ashburner & Friston, 2009). Friston 24-parameter model was used to regress out head motion effects from the realigned data. Besides, in nuisance covariate regression, we selected linear trends, global signal(Fox, Zhang, Snyder, & Raichle, 2009; Gotts, Gilmore, & Martin, 2020), WM and CSF signals as nuisance covariates. Next, functional images were normalized by DARTEL into Montreal Neurological Institute (MNI) space and resampled to a 3 mm × 3 mm × 3 mm voxel size. Notably, GM volume of each subject, created by DARTEL, was needed in further analysis. What follows were spatial smoothing with a 6-mm full-width at half-maximum (FWHM) Gaussian kernel and temporal band-pass filtering (0.01-0.08 Hz).

References

Ashburner, J., & Friston, K. J. (2009). Computing average shaped tissue probability templates. Neuroimage, 45(2), 333-341. doi:10.1016/j.neuroimage.2008.12.008

Fox, M. D., Zhang, D., Snyder, A. Z., & Raichle, M. E. (2009). The global signal and observed anticorrelated resting state brain networks. J Neurophysiol, 101(6), 3270-3283. doi:10.1152/jn.90777.2008

Gotts, S. J., Gilmore, A. W., & Martin, A. (2020). Brain networks, dimensionality, and global signal averaging in resting-state fMRI: Hierarchical network structure results in low-dimensional spatiotemporal dynamics. Neuroimage, 205, 116289. doi:10.1016/j.neuroimage.2019.116289

TABLE. S1 Mean coordinates of insular subdivisions in MNI152 space

| regions | Mean MNI coordinates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | x | y | Z |  |  |  |
| Left ventral anterior insula (vAI_L) | -33 | 13 |  | -7 |  |  |
| Right ventral anterior insula (vAI_R) | 32 | 10 |  | -6 |  |  |
| Left dorsal anterior insula (dAI_L) | -38 | 6 |  | 2 |  |  |
| Right dorsal anterior insula (dAI_R) | 35 | 7 |  | 3 |  |  |
| Left posterior insula (PI_L) | -38 | -6 |  | 5 |  |  |
| Right posterior insula (PI_R) | 35 | -11 |  | 6 |  |  |