# Supplementary Information

Grain size, nutrient, chemical and microbiological testing methods used in the study.

| Test | Method | Limit of detection |
| --- | --- | --- |
| Dry matter for grainsize samples (sieved) | Drying for 16 h at 103°C, gravimetry (free water removed before analysis). | 0.10 g/100 g |
| Grain size profile (fraction ≥ 2 mm) | Wet sieving with dispersant, as received, 2.00 mm sieve, gravimetry. | 0.1 g/100 g dry weight |
| Fraction < 2 mm, ≥ 63 μm | Wet sieving using dispersant, as received, 2.00 mm and 63 μm sieves, gravimetry (calculation by difference). | 0.1 g/100 g dry weight |
| Fraction < 63 μm | Wet sieving with dispersant, as received, 63 μm sieve, gravimetry (calculation by difference). | 0.1 g/100 g dry weight |
| Total nitrogen | Catalytic combustion (900°C, O2), separation, thermal conductivity detector [Elementar Analyser]. | 0.02 g/100 g dry weight |
| Arsenic/trace metals (As, Cd, Cr, Cu, Ni, Pb, Zn, Hg) | Dried sample, <2 mm fraction. Nitric/Hydrochloric acid digestion, ICP-MS, trace level. | 0.010–0.4 mg/kg dry weight |
| E. coli | APHA 5th Edition, Chapter 9, 2015. | <30 MPN/100 g |
| Presumptive enterococci | APHA 4th Edition, 2001. | <30 MPN/100 g |