# Supplementary Information for

# -

# Towards Circular Plastics within Planetary Boundaries

Authors:

Marvin Bachmanna,*, Christian Zibunasa,*, Jan Hartmanna, Victor Tulusb, Sangwon Suhc, Gonzalo Guillén-Gosálbezb, and André Bardowd

Corresponding author:

André Bardow: abardow@ethz.ch

Affiliations:

a - Institute for Technical Thermodynamics, RWTH Aachen University, 52062 Aachen, Germany.

b - Institute for Chemical and Bioengineering, Department of Chemistry and Applied Biosciences, ETH Zürich, 8093 Zurich, Switzerland

c - Bren School of Environmental Science and Management, University of California, Santa Barbara, CA 93117, USA.

d - Energy and Process Systems Engineering, ETH Zürich, 8092 Zurich, Switzerland.

* - These authors contributed equally to this work.

## Scope definition and data sources

This study is based on a bottom-up model from Meys et al.1, representing the life cycle of >90% of global plastics. In the first section, we introduce the intermediate flows considered in the bottom-up model and all datasets of the corresponding processes. The datasets are based on detailed life cycle inventories that comprise full energy and mass balances. Over 400, mostly industrially validated, technology datasets are used in the model. For all further details on process data, we refer to Meys et al., while for details on incorporating the planetary boundary framework into the model, we refer to Section 2 of this supplementary information.

## Included intermediate flows

The bottom-up model covers chemicals, plastics, and plastic wastes. Chemicals required as intermediates for plastic production are included in the model. In summary, the model includes the following flows:

Chemicals: Acetic acid, acetone, acetonitrile, acrylic acid, acrylonitrile, adipic acid, allyl chloride, ammonia, aniline, benzene, butadiene, C4 fraction, calcium chloride, calcium oxide, ca-prolactam, carbon dioxide, carbon monoxide, caustic soda (50%), chlorine, cumene, cyclohexane, dichloropropylene, diethylene glycol, dimethyl terephthalate, dinitrotoluene, dipropylene glycol, epichlorohydrin, ethanol, ethylbenzene, ethylene, ethylene glycol, ethylene oxide, formaldehyde, glycerin, hexamethylenediamine, hydrogen, hydrogen cyanide, methanol, methyl acrylate, methylene diphenyl diisocyanate, monoethanolamine, naphtha, natural gas, nitric acid (60%), nitroben-zene, nitrogen, oleum (33%), oxygen, o-xylene, phenol, polybutadiene, polyol (polyester-based), polyol (polyether-based), propionitrile, propylene, propylene glycol, propylene oxide, p-xylene, pyrolysis gasoline, silicon carbide, sodium carbonate, sodium chloride, styrene, sulfur trioxide, sulfuric acid, synthesis gas (2:1), terephthalic acid, toluene, toluene diisocyanate, vinyl chloride, xylenes (mixed).

Plastics: Polyamide 6, polyamide 66, PET pellets (fiber-grade), PET pellets (bottle-grade), polyacrylonitrile fiber, polyethylene HD, polyethylene LD, polyethylene LLD, polypropylene, polystyrene GP, polystyrene HI, polyurethane flexible, polyurethane rigid, polyvinylchloride.

Plastic packaging wastes: Polyethylene, HD, polyethylene, LD, polyethylene, LLD, polypro-pylene, polystyrene, GP, polystyrene, HI, PET pellets (bottle-grade).

Non-packaging plastic wastes: Polyamide 6, polyamide 66, PET pellets (fiber-grade), polyacrylonitrile fiber, polyethylene, HD, polyethylene, LD, polyethylene, LLD, polypropylene, polystyrene, GP, polystyrene, HI, polyurethane, flexible, polyurethane, rigid, polyvinylchloride.

## Datasets and Mitigation Pathways

The following table (Table S1) shows all flows, production technologies and literature sources of the original bottom-up model.1 Compared to the original model, we include additional datasets for the sensitivity analysis on biomass feedstocks (Table S2) and electricity generation (Table S3). The grid mix of the International Energy Agency’s Net-zero 2050 scenario is calculated according to Table S4. For the background system, we used aggregated datasets from ecoinvent if available. If available, we used datasets at the global level (GLO). Otherwise, we used Rest-of-the-World (RoW), Europe (RER), and German (DE) data.

Table S1: Summary of flows, production technologies, and literature sources of the chemical and plastic industry's bottom-up model adapted from Meys et al.1 Comments address the availability of technologies for each GHG mitigation pathway. The numbers in the comments represent the mitigation pathways in which the technology is available: 1: fossil-based plastics, 2: bio-based plastics, 3 CCU-based plastics, 4: A combination of all technologies, except for chemical recycling to monomers, and 5: An optimistic outlook including chemical recycling to monomers.

| Name of flow | Production technologies | Source | Pathway |
| --- | --- | --- | --- |
| acetic acid | carbonylation of methanol | 2 | [1,2,3,4,5] |
| acetone | oxidation of cumene | 2 | [1,2,3,4,5] |
| acetonitrile | market for acetonitrile | 2 | [1,2,3,4,5] |
| acrylic acid, ester grade | from propylene by ammoxidation | 2 | [1,2,3,4,5] |
| acrylonitrile | propylene ammoxidation | 2 | [1,2,3,4,5] |
| acrylonitrile | monomer recovery from polyacrylonitrile wastes | 3 | [5] |
| adipic acid | benzene oxidation via cyclohexanol | 2 | [1,2,3,4,5] |
| adipic acid | monomer recovery from polyamide 66 wastes | 3 | [5] |
| allyl chloride | propylene chlorination | 2 | [1,2,3,4,5] |
| ammonia | Haber-Bosch process | 4 | [1,2,3,4,5] |
| ammonium sulfate | market for ammonium sulfate | 5 | [1,2,3,4,5] |
| aniline | reduction of nitrobenzene | 2 | [1,2,3,4,5] |
| benzene | solvent extraction from pyrolysis gasoline | 2 | [1,2,3,4,5] |
| benzene | solvent extraction from pyrolysis gasoline | 2 | [1,2,3,4,5] |
| benzene | separation of xylenes by adsorption | 2 | [1,2,3,4,5] |
| benzene | separation of xylenes by crystallization | 2 | [1,2,3,4,5] |
| benzene | methanol to aromatics | 2 | [1,2,3,4,5] |
| butadiene | market for butadiene | 5 | [1,2,3,4,5] |
| butene-1 | market for butene, mixed | 5 | [1,2,3,4,5] |
| calcium chloride | market for calcium chloride | 5 | [1,2,3,4,5] |
| calcium oxide | market for lime | 5 | [1,2,3,4,5] |
| caprolactam | production from toluene | 2 | [1,2,3,4,5] |
| caprolactam | monomer recovery from polyamide 6 wastes | 3 | [5] |
| carbon dioxide | several industrial point sources, (ammonia/ and hydrogen via Rectisol, biomass gasification via Rectisol, ethylene oxide via potassium carbonate, waste incineration via monoethanolamine, fermentation of biomass) | 2,6–13) | [3,4,5] |
| carbon dioxide | direct air capture | 14 | [3,4,5] |
| carbon monoxide | partial condensation of synthesis gas | 2 | [1,2,3,4,5] |
| chlorine | electrolysis of hydrochloric acid | 2 | [1,2,3,4,5] |
| chlorine | electrolysis via oxygen-depolarized cathodes | 2 | [1,2,3,4,5] |
| chlorine / caustic soda (50%) | electrolysis of NaCl in membrane cell | 2 | [1,2,3,4,5] |
| chlorine / caustic soda (50%) | electrolysis of NaCl in diaphragm cell | 2 | [1,2,3,4,5] |
| chlorine / caustic soda (50%) | electrolysis of NaCl in mercury cell | 2 | [1,2,3,4,5] |
| cooling water | market for water, decarbonised, at user | 15 | [1,2,3,4,5] |
| cumene | alkylation of benzene with propylene | 2 | [1,2,3,4,5] |
| cyclohexane | hydrogenation of benzene | 2 | [1,2,3,4,5] |
| deionized water | market for water, decarbonised, at user | 5 | [1,2,3,4,5] |
| dimethyl terephthalate | esterification of terephthalic acid with methanol | 2 | [1,2,3,4,5] |
| dimethyl terephthalate | monomer recovery from PET wastes | 3 | [5] |
| dinitrotoluene | nitration of toluene | 2 | [1,2,3,4,5] |
| epichlorohydrin | chlorohydrination of allyl chloride | 2 | [1,2,3,4,5] |
| ethanol | hydrolysis and fermentation | 16 | [2,4,5] |
| ethylbenzene | alkylation of benzene with ethylene (zeolite catalyst) | 2 | [1,2,3,4,5] |
| ethylene | UOP/HYDRO methanol to olefins | 2 | [1,2,3,4,5] |
| ethylene | catalytic dehydration of ethylene (adiabatic fixed-bed) | 2 | [1,2,3,4,5] |
| ethylene | catalytic dehydration of ethylene (fluidized-bed) | 2 | [1,2,3,4,5] |
| ethylene | DMTO methanol to olefins (C2/C3 = 1.5) | 2 | [1,2,3,4,5] |
| ethylene | Steam cracking of naphtha | 2 | [1,2,3,4,5] |
| ethylene | DMTO methanol to olefins (C2/C3 = 2.5) | 2 | [1,2,3,4,5] |
| ethylene | monomer recovery from polyethylene wastes | 3 | [5] |
| ethylene glycol, diethylene glycol | thermal hydration of ethylene oxide | 2 | [1,2,3,4,5] |
| ethylene glycol, diethylene glycol | monomer recovery from PET wastes | 3 | [5] |
| ethylene oxide | ethylene oxidation | 2 | [1,2,3,4,5] |
| formaldehyde | oxidation of methanol (ferric-molybdate cat.) | 2 | [1,2,3,4,5] |
| formaldehyde | oxidation of methanol (silver cat.) | 2 | [1,2,3,4,5] |
| gasoline | market for petrol, unleaded | 5 | [1,2,3,4,5] |
| glycerin | oxidation of allyl chloride via epichlorohydrin | 2 | [1,2,3,4,5] |
| hexamethylenediamine | from acrylonitrile via diponitrile | 2 | [1,2,3,4,5] |
| hexamethylenediamine | from acrylonitrile via diponitrile (electrohydrodimerization) | 2 | [1,2,3,4,5] |
| hexamethylenediamine | monomer recovery from polyamide 66 wastes | 3 | [5] |
| hydrochloric acid | market for hydrochloric acid, without water | 5 | [1,2,3,4,5] |
| hydrogen | steam methane reforming and water-gas-shift | 2 | [1,2,3,4,5] |
| hydrogen | Water-gas shift and amine separation of CO2 | 2 | [1,2,3,4,5] |
| hydrogen | low-temperature electrolysis | 17 | [1,2,3,4,5] |
| hydrogen cyanide | market for hydrogen cyanide | 5 | [1,2,3,4,5] |
| methane | from CO2 | 18,19 | [3,4,5] |
| methanol | from synthesis gas | 2 | [1,2,3,4,5] |
| methanol | from CO2 | 20 | [3,4,5] |
| methanol | monomer recovery from PET wastes | 3 | [5] |
| methyl acrylate | esterification of acrylic acid | 2 | [1,2,3,4,5] |
| methylene diphenyl diisocyanate | phosgenation of benzene | 2 | [1,2,3,4,5] |
| methylene diphenyl diisocyanate | monomer recovery from polyurethane, rigid wastes | 3 | [5] |
| miscanthus | market for miscanthus, chopped | 5 | [2,4,5] |
| monoethanolamine | market for monoethanolamine | 5 | [1,2,3,4,5] |
| naphtha | pyrolysis of all plastic waste fraction | 2 | [1,2,3,4,5] |
| naphtha | market for naphtha | 5 | [1,2,3,4,5] |
| natural gas (raw material) | market for natural gas, high pressure | 5 | [1,2,3,4,5] |
| nitric acid (60%) | from ammonia (dual pressure) | 2 | [1,2,3,4,5] |
| nitric acid (60%) | from ammonia (mono pressure) | 2 | [1,2,3,4,5] |
| nitrobenzene | nitration of benzene (adiabatic) | 2 | [1,2,3,4,5] |
| nitrobenzene | nitration of benzene (conventional) | 2 | [1,2,3,4,5] |
| nitrogen | air separation by pressure-swing adsorption | 2 | [1,2,3,4,5] |
| n-pentane | market for pentane | 5 | [1,2,3,4,5] |
| oleum (33.3%) | 33.3% oleum from sulfur trioxide and sulfuric acid | 1 | [1,2,3,4,5] |
| oxygen | cryogenic air separation | 2 | [1,2,3,4,5] |
| o-xylene | Separation of xylenes by adsorption | 2 | [1,2,3,4,5] |
| o-xylene | Separation of xylenes by crystallization | 2 | [1,2,3,4,5] |
| PET pellets (bottle-grade) | upgrading of PET fiber-grade to bottle-grade | 2 | [1,2,3,4,5] |
| PET pellets (fiber-grade) | polycondensation from dimethyl terephthalate and ethylene glycol | 2 | [1,2,3,4,5] |
| PET pellets (fiber-grade) | polycondensation from dimethyl terephthalate and ethylene glycol | 2 | [1,2,3,4,5] |
| PET pellets (fiber-grade) | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| phenol | oxidation of cumene | 2 | [1,2,3,4,5] |
| polyacrylonitrile fiber | melt extrusion | 2 | [1,2,3,4,5] |
| polyamide 6 | continous PA 6 production | 2 | [1,2,3,4,5] |
| polyamide 66 | continous PA 66 production | 2 | [1,2,3,4,5] |
| polybutadiene | solution polymerization | 2 | [1,2,3,4,5] |
| polyethylene, HD | gas-phase polymerization | 2 | [1,2,3,4,5] |
| polyethylene, HD | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| polyethylene, LD | autoclave polymerization | 2 | [1,2,3,4,5] |
| polyethylene, LD | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| polyethylene, LLD | solution polymerization | 2 | [1,2,3,4,5] |
| polyethylene, LLD | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| polypropylene | gas-phase polymerization | 2 | [1,2,3,4,5] |
| polypropylene | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| polystyrene, GP | bulk polymerization | 2 | [1,2,3,4,5] |
| polystyrene, GP | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| polystyrene, HI | bulk polymerization (incl. Polybutadiene) | 2 | [1,2,3,4,5] |
| polystyrene, HI | mechanical recycling from plastic packaging waste | 3 | [1,2,3,4,5] |
| polyol for PUR, rigid | from adipic acid and diethylene glycol | 2 | [1,2,3,4,5] |
| polyol for PUR, rigid | monomer recovery from polyurethane, rigid wastes | 3 | [5] |
| polyurethane, flexible | Continous production of flexible polyurethane | 21 | [1,2,3,4,5] |
| polyol for PUR, flexible | from propylene/ethylene oxide and glycerol | 2 | [1,2,3,4,5] |
| polyol for PUR, flexible | monomer recovery from polyurethane, flexible wastes | 3 | [5] |
| polyurethane, rigid | Continous production of flexible polyurethane foam | 21 | [1,2,3,4,5] |
| polyvinyl chloride | suspension polymerization | 2 | [1,2,3,4,5] |
| process water | market for water, decarbonised, at user | 5 | [1,2,3,4,5] |
| propylene | Lurgi methanol to propylene | 2 | [1,2,3,4,5] |
| propylene | dimerization of ethylene | 2 | [1,2,3,4,5] |
| propylene | ethylene disproportionation | 2 | [1,2,3,4,5] |
| propylene | Steam cracking of naphtha | 2 | [1,2,3,4,5] |
| propylene | UOP/HYDRO methanol to olefins | 2 | [1,2,3,4,5] |
| propylene | DMTO methanol to olefins (C2/C3 = 1.5) | 2 | [1,2,3,4,5] |
| propylene | DMTO methanol to olefins (C2/C3 = 2.5) | 2 | [1,2,3,4,5] |
| propylene | monomer recovery from polypropylene wastes | 3 | [5] |
| propylene glycol, dipropylene glycol | propylene oxide oxidation | 2 | [1,2,3,4,5] |
| propylene oxide | chlorohydrine process | 2 | [1,2,3,4,5] |
| p-xylene | Separation of xylenes by adsorption | 2 | [1,2,3,4,5] |
| p-xylene | Separation of xylenes by crystallization | 2 | [1,2,3,4,5] |
| p-xylene | methanol to aromatics | 2 | [1,2,3,4,5] |
| pyrolysis gasoline | Steam cracking of naphtha | 2 | [1,2,3,4,5] |
| silicon carbide | market for silicon carbide | 5 | [1,2,3,4,5] |
| sodium carbonate | market for sodium bicarbonate | 5 | [1,2,3,4,5] |
| sodium chloride | sodium chloride production, powder | 5 | [1,2,3,4,5] |
| steam | natural gas boiler | 2 | [1,2,3,4,5] |
| steam | electric boiler | 2 | [3,4,5] |
| steam | from biomass | 22 | [2,4,5] |
| styrene | alkylation of benzene with ethylene (liquid-phase) | 2 | [1,2,3,4,5] |
| styrene | alkylation of benzene with ethylene (gas-phase) | 2 | [1,2,3,4,5] |
| styrene | monomer recovery from polystyrene wastes | 3 | [5] |
| sulfur trioxide | market for sulfur trioxide | 5 | [1,2,3,4,5] |
| sulfuric acid | market for sulfuric acid | 5 | [1,2,3,4,5] |
| synthesis gas (2:1) | natural gas steam reforming | 2 | [1,2,3,4,5] |
| synthesis gas (2:1) | mixing of hydrogen and CO (2:1) | 1 | [1,2,3,4,5] |
| synthesis gas (2:1) | gasification of biomass | 7–11,13 | [2,4,5] |
| thermal energy | various hydrocarbons | 1 | [1,2,3,4,5] |
| thermal energy | resistance heater | 1 | [3,4,5] |
| terephthalic acid | oxidation of p-xylene | 2 | [1,2,3,4,5] |
| terephthalic acid | monomer recovery from PET wastes | 3 | [5] |
| toluene | solvent extraction from reformate | 2 | [1,2,3,4,5] |
| toluene | solvent extraction from pyrolysis gasoline | 2 | [1,2,3,4,5] |
| toluene | separation of xylenes by adsorption | 2 | [1,2,3,4,5] |
| toluene | separation of xylenes by crystallization | 2 | [1,2,3,4,5] |
| toluene | methanol to aromatics | 2 | [1,2,3,4,5] |
| toluene diisocyanate | phosgenation of toluene | 2 | [1,2,3,4,5] |
| toluene diisocyanate | monomer recovery from polyurethane, flexible wastes | 3 | [5] |
| vegetable oil | market for vegetable oil methyl ester | 5 | [1,2,3,4,5] |
| vinyl chloride | ethylene chlorination and ethylene dichloride pyrolysis | 2 | [1,2,3,4,5] |
| vinyl chloride | monomer recovery from polyvinylchloride wastes | 3 | [5] |
| xylenes, mixed | solvent extraction from reformate | 2 | [1,2,3,4,5] |
| xylenes, mixed | solvent extraction from pyrolysis gasoline | 2 | [1,2,3,4,5] |
| xylenes, mixed | methanol to aromatics | 2 | [1,2,3,4,5] |
| all plastic wastes | energy recovery | 23,24 | [1,2,3,4,5] |
| all plastic wastes | landfilling | 5 | [1,2,3,4,5] |
| all plastic packaging waste | sorting of plastic packaging waste | 3 | [1,2,3,4,5] |

Table S2: Biomass feedstocks for the sensitivity analysis.

| biomass feedstock | dataset | source |
| --- | --- | --- |
| bagasse, from sweet sorghum | market for bagasse, from sweet sorghum | 5 |
| bagasse, from sugarcane | market for bagasse, from sugarcane | 5 |
| bark chips | market for bark chips, wet, measured as dry mass | 5 |
| wood chips | market for wood chips, wet, measured as dry mass | 5 |
| wood pellets | wood pellet production | 5 |

Table S3: Electricity generation technologies for the sensitivity analysis. Photovoltaic with high full load hours (high FLH) is represented by photovoltaic in the Middle East approximated by the dataset for Saudi Arabia.

| generation technologies | dataset | source |
| --- | --- | --- |
| wind power, onshore | electricity production, wind, 1-3MW turbine, onshore | 5 |
| geothermal power | electricity production, deep geothermal | 5 |
| hydro power | electricity production, hydro, run-of-river | 5 |
| nuclear power | electricity production, nuclear, pressure water reactor | 5 |
| photovoltaic | electricity production, photovoltaic, 3kWp slanted-roof installation, multi-Si panel, mounted | 5 |
| photovoltaic, high FLH | electricity production, photovoltaic, 3kWp slanted-roof installation, multi-Si panel, mounted (SA) | 5 |

Table S4: Technology shares of the 2030’s grid mix according to the International Energy Agency’s Net-zero 2050 scenario.25 The carbon capture and storage (CCS) technologies were modeled according to Galán-Martín et al.26 For the grid mix, we omit marine power, and hydrogen and ammonia based power due to lack of data. The neglected technologies account for 2.5 % of the grid mix. Consequentially, we scaled the sum of share of the considered technologies to 100 %.

| generation technologies | dataset | share in % | source |  |
| --- | --- | --- | --- | --- |
| coal without CCS | electricity production, hard coal | 8.1 |  | 5 |  |
| coal with CCS | electricity production, hard coal | 0.8 |  | 5,26 |  |
| natural gas without CCS | electricity production, natural gas, combined cycle power plant | 17.1 |  | 5 |  |
| natural gas with CCS | electricity production, natural gas, combined cycle power plant | 0.5 |  | 5,26 |  |
| oil | electricity production, oil | 0.5 |  | 5 |  |
| nuclear | electricity production, nuclear, pressure water reactor | 10.4 |  | 5 |  |
| hydro power | electricity production, hydro, reservoir, non-alpine region | 16.1 |  | 5 |  |
| bioenergy | heat and power co-generation, wood chips, 6667 kW, state-of-the-art 2014 | 3.9 |  | 5 |  |
| wind | electricity production, wind, >3MW turbine, onshore | 22.0 |  | 5 |  |
| solar photovoltaic | electricity production, photovoltaic, 570kWp open ground installation, multi-Si | 19.2 |  | 5 |  |
| geothermal | electricity production, deep geothermal | 0.9 |  | 5 |  |
| concentrated solar power | electricity production, solar thermal parabolic trough, 50 MW | 0.6 |  | 5 |  |

## Methods

## Life cycle assessment – normalization and weighting

In current practice, life cycle assessment (LCA) is mainly used to compare environmental impacts between production pathways. Thus, burden shifting can be identified. Thereby, LCA offers two options to analyze the relevance of burden shifting by setting the burden shift into perspective: weighting and normalization.

Weighting aggregates environmental impacts across impact categories to determine an environmentally beneficial ranking of mitigation strategies. However, weighting requires value choices and, thus, is highly subjective.27 Furthermore, weighting does not help to identify mitigation strategies that yield absolute environmentally sustainable plastics since the magnitude of environmental impacts is not set in relation to a regional or planetary threshold.

Normalization compares LCA results to a reference system to understand the relative magnitude of environmental impacts. Most reference systems rely on environmental assessments of larger anthropogenic systems and regions.28 Thereby, normalization helps to grasp the magnitude of an environmental impact but so far lacks to interpret this magnitude in terms of absolute environmental sustainability. In particular, LCA lacks thresholds whose exceedance would make a system unsustainable. Thus, LCA is key for comparing alternatives but provides little insight into absolute environmental sustainability. In this study, we apply the planetary boundary framework, which normalizes impacts to planetary thresholds. Thereby, we can determine the impacts’ relevance with regard to absolute environmental sustainability.

## Adapting the planetary boundary framework

The elementary flows of the bottom-up model are defined according to ecoinvent v3.5, which includes 2080 elementary flows to the compartments air, water, soil, and from natural resources. We consider all 2080 elementary flows and determine their characterization factors for each Earth-system process by using the characterization models from Galán-Martín et al.26 for change in biosphere integrity, from D’Angelo et al.29 for the N-cycle and from Ryberg et al.30 for all further Earth-system processes. In the following, we describe the elementary flows that we additionally characterized based on the original characterization models, modifications to the already existing characterization factors, and further changes.

Climate Change:

We include all characterization factors for GHG emission from Ryberg et al.30 For CO2, they define the characterization factor by dividing the change of atmospheric CO2 concentration between the pre-industrial value (from the year 1765) and the value estimated by an RCP2.6 scenario in 2300 by the total anthropogenic CO2 emissions from the year 2000 to 2300 (based on an RCP2.6 scenario).30,31 However, the compared time horizons do not match. Therefore, we set the time horizons consistently from 1765 to 2300. The characterization factor for a change in atmospheric CO2 concentration due to a continuous CO2 emission is 1.79 ·10-11 ppm/(yr kg) and, thereby, 33% smaller compared to Ryberg et al.30 To convert the change in atmospheric CO2 concentration to radiative forcing, we adopt the characterization model from Ryberg et al. Additionally, we include all elementary flows emitted to air that follow the EU definition for NMVOC, i.e., organic compounds with a boiling point lower than or equal to 250°C.32 We conservatively consider these flows as potential CO2 precursors and calculate their characterization factors based on their carbon content.

Furthermore, we include elementary flows that represent unspecified groups of compounds from ecoinvent that do not have a specific carbon content:

- VOC, volatile organic compounds, unspecified origin

- Aldehydes, unspecified

- Hydrocarbons, aliphatic, alkanes, cyclic

- Hydrocarbons, aliphatic, alkanes, unspecified

- Hydrocarbons, aliphatic, unsaturated

- Hydrocarbons, aromatic

- Hydrocarbons, chlorinated

- Hydrocarbons, unspecified

- NMVOC, non-methane volatile organic compounds, unspecified origin

For these elementary flows, we use the characterization factor of NMVOC given by Ryberg et al.33 Moreover, we do not characterize non-fossil emissions, including CO2, CH4, and CO, which was not explicitly stated in previous literature but is in accordance with the ILCD recommendations.34

Ocean acidification

For ocean acidification, we adopt the same changes as for climate change: We match the time horizons for characterizing CO2, include CO2 precursors, approximate groups of CO2 precursors by NMVOC, and exclude non-fossil emissions. Ocean acidification is measured as the global mean saturation state of aragonite in surface seawater (Ωarag).

Change in biosphere integrity

We calculate the characterization factors for change in biosphere integrity following Galán-Martín et al.26 The first stressor of change in biosphere integrity is climate change. The calculation of climate change as a stressor is tailored to the change in biosphere integrity and thereby differs from the calculation of the planetary footprint of climate change from Ryberg et al.30 However, we consider the same elementary flows as mentioned above. For further details, the reader is referred to Galán-Martín et al.26

The second stressor is land use, represented by land occupation in ecoinvent. We adopt the method from Galán-Martín et al. to quantify the impact of land occupation based on mean species abundance loss following Hanafiah et al.35 However, Hanafiah et al. do not provide data for all land types in the ecoinvent database.5 Hence, we match the types of land occupation from Hanafiah et al. and ecoinvent v3.5 according to Table S5. We use the maximum values of each type of land occupation as a conservative approximation.

Table S5: Matching of land types between Hanafiah et al.35 and ecoinvent v3.5.5

| Classification in ecoinvent v3.5 | Classification in Hanafiah et al. | Loss in mean species abundance |
| --- | --- | --- |
| Occupation, annual crop, extensive | Permanent crop, extensive | 0.70 |
| Occupation, annual crop, intensive | Permanent crop, intenstive | 0.90 |
| Occupation, arable land, unspecified use | Arable | 0.90 |
| Occupation, arable | Arable | 0.90 |
| Occupation, bare area (non-use) | Primary vegetation | 0 |
| Occupation, cropland fallow (non-use) | Arable, non-irrigated, fallow | 0.70 |
| Occupation, dump site | Dump site | 0.95 |
| Occupation, field margin/hedgerow | Primary vegetation | 0 |
| Occupation, forest, intensive | Forest, intensive, short-cycle | 0.80 |
| Occupation, forest, primary (non-use) | Forest, extensive | 0.30 |
| Occupation, forest, secondary (non-use) | Forest | 0.50 |
| Occupation, forest, unspecified | Forest | 0.50 |
| Occupation, grassland, natural (non-use) | Primary vegetation | 0 |
| Occupation, grassland, natural, for livestock grazing | Pasture and meadow, extensive | 0.30 |
| Occupation, inland waterbody, unspecified | Water bodies, artificial | 1 |
| Occupation, lake, natural (non-use) | Sea and ocean | 0 |
| Occupation, pasture, man made | Pasture and meadow | 0.90 |
| Occupation, pasture, man made, extensive | Pasture and meadow, extensive | 0.30 |
| Occupation, pasture, man made, intensive | Pasture and meadow, intensive | 0.90 |
| Occupation, permanent crop, extensive | Permanent crop, extensive | 0.70 |
| Occupation, permanent crop, intensive | Permanent crop, intensive | 0.90 |
| Occupation, seabed | Benthos | 0 |
| Occupation, unspecified | Permanent crop, intenstive | 0.90 |
| Occupation, unspecified (non-use) | Primary vegetation | 0 |
| Occupation, wetland | Primary vegetation | 0 |
| Occupation, river, artificial | Water courses, artificial | 1 |
| Occupation, river, natural (non-use) | Sea and ocean | 0 |
| Occupation, seabed, natural (non-use) | Sea and ocean | 0 |
| Occupation, snow and ice (non-use) | Pasture and meadow, extensive | 0.30 |

Nitrogen cycle (N-cycle)

To quantify the N-cycle footprint, we use the characterization model from D’Angelo et al.29

Phosphorus cycle (P-cycle)

Ryberg et al. provide a characterization factor for phosphorus emitted to the ocean via freshwater systems.30 However, we conservatively add all phosphorus-containing substances emitted to other compartments to the assessment. To account for these compounds, we multiply the characterization factor for pure phosphorus emitted to the ocean with the mass fraction of phosphorus in the phosphorus-containing elementary flows. Furthermore, we include the transport of the phosphorus-containing compound from the compartment it was initially emitted to the ocean.

CFP-containing compound= 10-9*frp*ktransport,i

For phosphorus-containing compounds emitted to the air, we conservatively assume that 100% of the phosphorus-containing compound ultimately end up in the ocean. Thus kair-to-ocean equals 1. For the compartment of water, we further distinguish between the sub-compartments ocean and all further water bodies. The k-factor for emissions of phosphorus-containing compounds to the sub-compartment ocean is 1 by definition. In contrast, emissions to all other water bodies have a k-factor of 0.86, accounting for sedimentation based on Ryberg et al.30 Phosphorus-containing compounds emitted to soil are assumed to partially end up in freshwater due to run-off from rainfall. Thus, we multiply the characterization factors for phosphorus-containing compounds to the compartment soil with a k-factor of 0.42, which is the most conservative estimate for the run-off factors from Hart et al.36 For subsequent transport from freshwater to the ocean, we again use the k-factor of 0.86, giving a total k-factor of 0.3612.

Aerosol loading

For aerosol loading, we use all characterization factors for the elementary flows considered by Ryberg et al.30 We additionally characterize the following elementary flows to the compartment air that have not yet been characterized using the characterization factor for generic carbon from Ryberg et al. as a proxy:

- Carbon-14

- Elemental carbon

- Organic carbon

Furthermore, we include all elementary flows defined as NMVOC and the same elementary flows representing unspecified groups of compounds as for climate change (see above). Ryberg et al. define an average characterization factor for NMVOC emitted to an urban environment and one for NMVOC emitted to a rural environment. Accordingly, we use the characterization factor of NMVOC, urban, for all elementary flows emitted to the sub-compartment urban air, close to ground. In contrast, we use the characterization factor of NMVOC, rural for all other sub-compartments, i.e., lower stratosphere + upper troposphere; non-urban air or from high stacks; unspecified; and low population density, long-term.

Freshwater use

For freshwater use, we consider all elementary flows from ecoinvent v3.5, which are associated with bluewater according to the definition of Falkenmark et al.37

- Water; water; ground-

- Water; water; unspecified

- Water, cooling, unspecified natural origin; natural resource; in water

- Water, lake; natural resource; in water

- Water, river; natural resource; in water

- Water, turbine use, unspecified natural origin; natural resource; in water

- Water, unspecified natural origin; natural resource; in water

- Water, unspecified natural origin; natural resource; in ground

- Water, well, in ground; natural resource; in water

In accordance with the ecoinvent nomenclature, elementary flows sourced from the compartment of natural resources keep a positive algebraic sign, thereby indicating water consumption. In contrast, elementary flows to the compartment water get a negative algebraic sign, indicating bluewater recovery. We conservatively assess freshwater use for evaporated water, as we assume that all evaporated water is lost and does not return to freshwater reservoirs. However, Gerten et al. highlight the need to reassess the boundary value for freshwater use and human freshwater consumption.38 Furthermore, human freshwater use may not reflect all types of human interference with the global water cycle, which should be incorporated in a future reassessment.39 Thus, the results for freshwater use shown in this study should be interpreted with caution.

Stratospheric ozone depletion

We extended the model from Ryberg et al. with the following elementary flows of ozone-depleting substances emitted to the compartment air following the Montreal Protocol40:

- HCFC-124

- HCFC-21

- R10

We additionally include N2O as proposed by Algunaibet et al.41 For the compound HCFC-21, we could not find a value for the fraction release ratio and assumed it to be 100 % as a conservative assumption.

Land-system change

We considered all elementary flows from ecoinvent v3.5 for the compartment of natural resources that describe a transformation from or to forest:

- Transformation, from/to forest, extensive

- Transformation, from/to forest, intensive

- Transformation, from forest, primary (non-use)

- Transformation, from/to forest, secondary (non-use)

- Transformation, from/to forest, unspecified

In accordance with the ecoinvent nomenclature, transformation processes from forest keep a positive algebraic sign, indicating forest depletion. In contrast, transformation processes to forest get a negative algebraic sign, indicating forest recovery.

Novel entities

The process of novel entities considers the release of substances and modified life forms with unwanted geophysical or biological effects on the environment. As mentioned in the main article, neither control variables nor a SOS have yet been defined. However, to be of concern at global level, novel entities need to exhibit persistence, a widespread distribution, and potential impacts threatening the integrity of other vital Earth-system processes.42 Plastics certainly possess these characteristics, and recent literature indicated that plastic pollution has already exceeded the planetary boundary of novel entities.43 However, the indication is solely based on the fact that the annual production of plastics exceeds the capacity for safety assessments and monitoring. To enable the quantification, Persson et al. evaluate options to define control variables at several steps in the impact pathways of novel entities, from production-based to release and effect-focused variables.43 However, none of the proposed control variables comply with all defined criteria to assess novel entities, i.e., feasibility, relevance, and comprehensiveness. Furthermore, Kosnik et al. proposed a framework for determining levels of chemical pollution and the corresponding SOS for novel entities. Determining such levels, however, remains challenging due to limited data availability.44 Accordingly, despite the ongoing efforts, quantifying plastics' planetary footprint in novel entities is not yet possible.

Increasing plastic recycling, as suggested in the main article, would reduce the amount of plastics that enter the environment. Thus, plastic recycling reduces plastic pollution and the pressure on novel entities. In contrast, the pressure on novel entities due to plastic pollution is not reduced when switching from fossil to renewable feedstocks while producing the same plastics with the same use phase and end-of-life treatment. However, producing plastics that provide the same function but possess other end-of-life characteristics, such as biodegradable plastics, may be a viable option.

Planetary Boundaries

In this study, we apply the planetary boundaries proposed by Röckström et al., which define a safe operating space for all activities on Earth.45 From this safe operating space, we subtract the natural background level, i.e., the natural state of the Earth system processes. Thereby, we yield a safe operating space for human activities (SOS), according to Table S6. In this study, the SOS will exclusively refer to the safe operating space for human activities. The SOS can be allocated to the plastics industry based on downscaling principles.46 This corresponding share of the SOS will define the thresholds for absolute environmental sustainability of the plastics industry.

In contrast to the planetary boundary for the N-cycle defined by Steffen et al.42, we only assess the chemical fixation of nitrogen. Accordingly, the original value of 62 Tg is reduced to 39.7 Tg representing the share of chemical fixation according to Algunaibet et al.47

Table S6: Planetary boundaries: Planetary Boundaries define a safe operating space for all activities on Earth. Substracting the natural background level yields the safe operating space for human activities, to only which is referred to as SOS in this study. If the natural background level is higher than the planetary boundary, subtracting the planetary boundary from the natural background level yields the SOS for humanity.

| Earth-system process | Unit | Planetary boundary | Natural background level | Safe operating space for humanity |
| --- | --- | --- | --- | --- |
| Climate change – energy imbalance | W/m2 | 1 | 0 | 1 |
| Ocean acidification | Ωarag | 2.752 | 3.44 | 0.688 |
| Change in biosphere integrity | % | 90 | 100 | 10 |
| Nitrogen cycle | Tg | 39.7 | 0 | 39.7 |
| Phosphorus cycle | Tg | 11 | 1 | 10 |
| Atmospheric aerosol loading | Aerosol optical depth | 0.25 | ~0.15 | 0.11 |
| Freshwater use | km3 | 4000 | 0 | 4000 |
| Stratospheric ozone depletion | Dobson units | 275.5 | 290 | 14.5 |
| Land-system change | % | 75 | 100 | 25 |

## Inventory modeling

For the life cycle inventory, we adapted the model from Meys et al., which was originally designed to assess GHG emissions only, to consistently assess planetary footprints.1 To assess the impacts of plastic production on the N-cycle footprint, we follow the procedure from D'Angelo et al. and calculate the total amount of fixed nitrogen by fertilizer production and application in the supply chain of the plastics industry.29 The method allows for a direct calculation of the N-cycle footprint compared to the originally proposed characterization factors from Ryberg et al., which are considered to have higher uncertainty.29,30

The planetary boundary for freshwater use is based on the global consumption of blue water.45 In our model, blue water consumption consists of process water and cooling water consumption in the foreground system and other additional consumptions such as irrigation in the background system. We conservatively assume that process water consumption corresponds to a one-to-one use of blue water. To estimate the blue water consumption from cooling water applications, we use the average values for cooling water evaporation from BASF, a large manufacturer of chemicals and plastics.48 We assumed all evaporation as blue water loss as a conservative assumption for freshwater use.

## Downscaling

Downscaling principles

Previous research commonly applies downscaling principles based on four ethical concepts: Egalitarian, inegalitarian, prioritarian, and utilitarian.46,49 However, there is still no scientific consensus on which principle to choose for downscaling to an industrial sector level. In the main article, we applied utilitarian downscaling by using the economic indicator of consumption expenditure. We found utilitarian downscaling to best suit the goal of this study, which requires downscaling to a global industrial sector. The other downscaling principles were not considered due to the following reasons:

Egalitarian principles aim for an equal distribution of resources between all humans.50 Accordingly, egalitarian principles are particularly important for regionalized assessments.46 For instance, egalitarian principles could assign a share of the SOS to all human activities in a region according to the ratio between the region’s population and the global population. By assessing global plastic production, we consider the demand for plastics by all humans and thus implicitly apply an egalitarian principle. However, egalitarian principles must be combined with other downscaling principles to allocate the share of SOS to industrial sectors.

The egalitarian principle could be combined with an inegalitarian principle such as grandfathering. Grandfathering could be applied to allocate the SOS by the proportion of an industry's current contribution to the overall environmental impact.49 Thereby, grandfathering favors industrial sectors with high environmental impacts and disfavors industries that have already reduced their environmental impacts, which could be considered unjust. Moreover, grandfathering does incorporate any moral criteria, such as attention to human rights and an industry’s commitment to sustainable development.51 Therefore, we do not consider grandfathering as a downscaling principle in this study.

In contrast, downscaling by prioritarian principles such as historical debt favors historically disadvantaged entities, e.g., regions or industries, by allowing them to overcome their historical disadvantages.49 However, in addition to the data of the status quo, prioritarian downscaling requires historical data, which is not available in the case of the plastics industry. Thus, we do not consider a prioritarian principle.

Indicators for utilitarianism

Utilitarian downscaling aims to maximize welfare in society. In the main article, we approximate society’s welfare by the economic indicator of consumption expenditure. The calculation based on consumption expenditure yields a share of safe operating space (SOS) of 1.1 %.

To assess the accuracy of allocating the share of SOS by consumption expenditure, we applied gross value added as an additional economic indicator. The gross value added measures the value of goods and services and can be estimated by the total product output at basic product price minus the total intermediate consumption at purchasers' price.52 We use the gross value added of the plastics industry for the year 2020 from EXIOBASE.53 However, the gross value from EXIOBASE represents only the directly added value of the plastics industry and omits the indirectly added value generated by the supply chain of the plastics industry, e.g., the crude oil refinery. Thereby, the gross value corresponds to a gate-to-gate system boundary. In contrast, we apply a cradle-to-grave system boundary by assessing the environmental impacts of plastics' supply chain and disposal. To align both system boundaries, we derive the indirect gross value added from the plastic supply chain by using the ratio between the direct and indirect gross value added of the chemical industry from Oxford Economics.54 Furthermore, we use EXIOBASE to add the gross value added from plastic disposal. To determine the share of SOS of the plastics industry, we divide plastics’ total gross value added by the global gross value added of all industries.

The gross value added yields a share of SOS of 1.3 %. Being similar to the calculations based on consumption expenditure strengthens the reliability of the economic indicators. Actually, the gross value added is the more common economic indicator for welfare,55 and thus, is frequently used for calculating the share of SOS.51 However, we favor the consumption expenditure in the main article as it yields the lower and thus more conservative threshold for environmentally sustainable plastics.

## Results

## Sensitivity analysis for recycling technologies

Figure S1: Sensitivity analysis of the planetary footprint of the plastic industry for recycling technologies. The planetary footprints are calculated by minimizing the environmental impact of climate change (details in methods). The pathways include fossil-based (grey), bio-based (green), and CCU-based (blue) plastics with the current (upper value) and the maximum (lower value) recycling rates. Freshwater use for the pathway “bio-based, MR + PY” is the only exception, where the lower value represents the current recycling rate. Arrows indicate increasing recycling rates. For each of the three pathways, there are two scenarios, one with mechanical recycling and chemical recycling via pyrolysis (MR + PY) and one with mechanical recycling and chemical recycling to monomers (MR + CR).

Since we identify recycling as a key enabler towards absolute environmentally sustainable plastics, we conduct a sensitivity analysis on recycling technologies. In all cases, recycling is limited to maximum recycling rates of 94 % due to 6 % unavoidable landfilling, which is in line with the literature.56 In this study, recycling rates are defined as the ratio between all waste plastics sorted for mechanical recycling or recycled chemically divided by the total amount of plastic waste. In contrast, effective recycling rates additionally account for losses during the sorting and recycling processes. Accordingly, effective recycling rates of 70 %, which Meys et al. found to be required for net-zero GHG emission plastics, correspond to recycling rates of 94 %.

The reference case from the main article is mechanical recycling of packaging waste and pyrolysis to refinery feedstock of non-packaging wastes (MR+PY). Mechanical recycling and pyrolysis improve the sustainability of plastics with increasing recycling rates, thereby confirming the LCA results from recent literature for mechanical recycling.57 For chemical recycling via pyrolysis, however, the LCA literature indicates burden shifting from climate change to photochemical ozone formation due to NOx, SOx, and other direct emissions.57,58 These emissions also directly impact aerosol loading. In contrast, our results show higher direct emissions from plastic incineration than from pyrolysis, such that both mechanical and chemical recycling reduce impacts on aerosol loading. Future studies should therefore investigate the impact on aerosol loading for pyrolysis to avoid potential burden shifts.

Chemical recycling to monomers could boost circularity compared to current practice, albeit being not market-ready. Therefore, we apply the optimistic assumption of a 95 % waste plastic conversion to monomers to assess the potential of chemical monomer recycling.

Already at current recycling rates, chemical monomer recycling (MR+CR) would replace pyrolysis and reduce all planetary footprints compared to the reference case (MR+PY, see upper values in Figure S1). Chemical monomer recycling reduces virgin resource consumption, i.e., naphtha, biomass, and electricity, which are the main contributor to the planetary footprints (Figure 4 from the main article). The consumption is reduced due to the higher efficiency of chemical monomer recycling compared to pyrolysis. For pyrolysis, the overall efficiency from waste polymer to naphtha is set to 63 %1, which is already less efficient than the assumed efficiency of chemical monomer recycling (95 %). Moreover, pyrolysis requires reproducing monomers from naphtha, leading to additional losses. In contrast, chemical recycling directly yields monomers, which additionally enhances the efficiency of chemical monomer recycling. Thereby, increasing recycling efficiencies is most beneficial at maximum recycling rates, where planetary footprints decrease between 21 and 97 % compared to the reference case with pyrolysis (MR+PY).

In summary, chemical monomer recycling could enable absolute environmental sustainability for CCU-based plastics at maximum recycling rates and significantly improve the sustainability of bio-based plastics. Therefore, getting chemical monomer recycling market-ready is the key towards absolute environmentally sustainable plastics. In the meantime, increasing recycling rates and pyrolysis efficiency improve plastics' sustainability.

## Sensitivity analysis of biomass sources

Energy crops are the default biomass used in the main article. Using energy crops transgresses the plastic industry’s share of SOS in biosphere integrity and the N-cycle footprint by about four times even at maximum recycling rates (see Figure S2). Therefore, we analyze the planetary footprints of energy crops in the following. Furthermore, biomass cultivation, which is the main contributor to the planetary footprints of bio-based plastics, differs by biomass type. Thus, we conduct a sensitivity analysis for lignocellulosic biomass, including woody biomasses and two types of bagasse as industrial by-products. Bagasse is a by-product of sugar production from sugarcane and ethanol production from sweet sorghum. We exclude edible biomass from the sensitivity analysis to avoid competition with the food industry and compare the types of biomasses based on a dry mass basis. The following analysis focuses on bio-based plastics at maximum recycling rates (see Figure S2).

In this study, energy crops are represented by miscanthus. We found that miscanthus has a higher N-cycle footprint than other lignocellulosic biomasses resulting from higher nitrogen-containing fertilizer use (see Figure S2). In contrast, previous literature predicts lower nutrient requirements for energy crops when compared to other crops and little to no need for additional fertilizer when cultivated on arable land.59,60 Thus, the fertilizer demand for cultivating energy crops assumed in ecoinvent might be too conservative. The lower nutrient requirements of energy crops from literature align with the assumptions for sweet sorghum and sugarcane production from ecoinvent, where more nitrogen-containing fertilizer is used compared to miscanthus production. However, ecoinvent applies economic allocation between ethanol, bagasse, and other co-products for ethanol production from sweet sorghum and between sugar, sugarcane, and other co-products for sugar production from sugarcane. Since bagasse has a significantly lower price than the main products, the environmental impacts are mainly allocated to the main product. Thus, the N-cycle footprint of bagasse is lower than for miscanthus. However, using bagasse as a feedstock for plastics may increase the price of bagasse, resulting in higher environmental impacts. Furthermore, production volumes are limited by the demand for the bagasse’s main products. In contrast, woody biomass significantly reduces the N-cycle footprint compared to miscanthus since forestry demands less fertilizer than cultivating energy crops.5 By using woody biomass, the plastic industry could operate within the share of SOS for the N-cycle footprint.

In ecoinvent, miscanthus is assumed to grow mainly on arable land. However, Roncucci et al. found similar yields for energy crops on low-fertility land if about the same amount of nitrogen fertilizer (50 kg per hectare per year) is used as assumed in ecoinvent.61 Accordingly, the high impacts in biosphere integrity resulting from the occupation of arable land can be reduced if energy crops are cultivated on marginal land with lower fertility. In addition, using marginal land for biomass cultivation may increase its availability.

Considering the other biomass types, bagasse has the lowest impact on biosphere integrity due to the high yields of their corresponding crops (see Figure S2). Again, economic allocation favors the low environmental impact of bagasse. In contrast, except for bark chips, woody biomass worsens biosphere integrity compared to energy crops due to the lower yield of woody biomass per hectare resulting in higher land occupation.5 In addition, even plastics produced from bark chips transgress their share of SOS.

For GHG-related planetary footprints, bagasse and woody biomasses perform similar to energy crops, except for wood pellets. Wood pellets have higher impacts on climate change and ocean acidification due to additional efforts for pelletizing and drying. The additional efforts also cause higher impacts in aerosol loading compared to other biomass feedstocks. For the other biomass feedstocks, the largest share of aerosol loadings results from biomass cultivation, which originates from the construction of agricultural machinery. The machinery construction requires steel, which is currently predominantly produced by blast furnace processes using coal as a reduction agent. Thus, a switch to steel production based on alternative reducing agents may improve aerosol loading of coal-related processes.

In summary, energy crops may offer the highest additional availability and thus be the primary prospect for the large-scale production of bio-based plastics. The planetary footprint of energy crops can be reduced by deploying alternative biomass feedstocks. However, each feedstock comes with a trade-off, either by high biosphere integrity loss or limited availability. A balanced mix of feedstocks may exploit the advantages of each feedstock while keeping the trade-offs at a minimum. Thereby, building on various biomass feedstocks may enable sufficient availability and absolute environmental sustainability for the plastic industry if bio-based production is combined with high recycling rates.

Figure S2: Sensitivity analysis of the planetary footprint of the plastic industry for biomass feedstocks. Energy crops are used for bio-based plastics in the main article. The bio-based supply chain is calculated by minimizing the environmental impact of climate change (details in methods) when using energy crops. For all other biomass feedstocks the supply chain is the same and the energy crops are replaced on dry basis. All pathways show bio-based plastics with the current (upper value) and the maximum (lower value) recycling rates. Exception, where the lower value represents the current recycling rate, are the P-cycle footprint for bagasse from sugarcane and wood chips, and the freshwater use for all feedstocks except for both types of bagasse. Arrows indicate increasing recycling rates.

## Sensitivity analysis of renewable energy

CCU-based plastics strongly rely on renewable electricity generation. Even the most ambitious forecasts for the 2030’s grid mix25 would compromise the environmental sustainability of CCU-based plastics. Therefore, we further assess which electricity generation technologies could be promising for producing sustainable CCU-based plastics. For this purpose, we assess the planetary footprints of CCU-based plastics when entirely relying upon either geothermal, nuclear, water, solar, or wind power. The planetary footprints most sensitive to electricity generation are the P-cycle footprint, aerosol loading, and all GHG-related planetary footprints, namely climate change, ocean acidification, and biosphere integrity. In this analysis, we focus on CCU-based plastics at maximum recycling rates (Figure S3).

Currently, CCU-based plastics based on electricity from photovoltaic (PV) and geothermal power do not achieve absolute environmental sustainability due to the remaining GHG emissions from plant construction. Furthermore, PV and geothermal power exceed the planetary boundary for the P-cycle footprint and aerosol loading. However, the P-cycle footprint is reduced to 4.0 % of the SOS for PV with high full load hours and 3.0 % of the SOS for geothermal energy compared to 5.8 % of the SOS for the 2030’s grid mix (see Fig. 3 in main article). Aerosol loading increases to 6.0 % by PV and 7.8 % by geothermal power compared to 5.9 % of the SOS for the 2030’s grid mix. The P-cycle footprint results from construction, which for geothermal power plants and photovoltaic panels is linked to coal-based energy. Additionally, the construction of geothermal power plants is linked to coal-based steel production, while photovoltaic panels require copper for wiring. Along with direct emissions from steel production, the mining of copper and coal causes the P-cycle footprint. Aerosol loading is mostly linked to coal-based energy for the construction of the geothermal power plant and photovoltaic panels.

Reducing footprints from construction could make both technologies viable options for sustainable plastic production. For instance, decarbonizing the electricity supply used for construction decreases the construction’s climate change impact of PV and geothermal energy by 40 and 65 %, respectively.62 Increasing full load hours would further reduce the specific impact of the construction phase. In particular, for PV full load hours could double in regions with high solar intensity, such as the Middle East, when compared to regions with lower solar intensity like Germany. With higher full load hours (photovoltaic, high FLH in Figure S3), the electricity output per PV panel increases. The impacts from the construction phase remain unchanged and, thus, relatively decrease per energy output. However, even in solar-intense regions, current PV construction needs to be further decarbonized to comply with the planetary boundaries.

Considerably more sustainable are CCU-based plastics using either nuclear, wind, or hydro power. Hydro power even enables absolute environmental sustainability for CCU-based plastics with maximum recycling rates. However, hydro power is limited to regional topography.

In contrast, nuclear power is not limited to regional topography but exceeds the share of safe operating space for aerosol loading and freshwater use. Moreover, using nuclear power for CCU-based plastics increases the impact of ionizing radiation, which is strongly linked to human health impacts. Using nuclear power increases ionizing radiation by about 20-times compared to fossil-based and about 216-times compared to CCU-based plastics with wind power. Furthermore, the disposal of nuclear fuel rods is controversially discussed and could jeopardize absolute environmental sustainability.

For wind power, the construction is the main contributor to the remaining planetary footprints. In particular, for the P-cycle footprint and aerosol loading, improving steel and copper production will further reduce planetary footprints. Similar to PV, wind-intense regions enable high full load hours for wind power and, thereby, grant low planetary footprints. Full load hours would also tackle non-renewable resource depletion, which is often discussed for wind turbines and may be considered unsustainable from a general perspective. In the context of environmental sustainability, however, resource depletion does not directly threaten the stability of the Earth system.63 Thus, wind power may be the best choice for environmentally sustainable plastics.

In summary, wind, nuclear, and hydro power are the most promising prospects towards sustainable production of CCU-based plastics. However, including any of the assessed renewable electricity technologies in the grid mix will bring plastics industry closer towards absolute environmental sustainability. Maximizing full load hours and decreasing footprints from construction could go alongside with increasing recycling efficiency to enable absolute environmental sustainability.

Figure S3: Sensitivity analysis on the planetary footprint of CCU-based plastics for electricity generation technologies. Onshore wind power is used for CCU-based plastics in the main article. The CCU-based supply chain is calculated by minimizing the environmental impact of climate change (details in methods) when onshore wind power. For all other electricity generation technologies, the supply chain is the same, and wind-powered electricity is replaced on an energy basis. All pathways show CCU-based plastics with the current (upper value) and the maximum (lower value) recycling rates. Arrows indicate increasing recycling rates. For photovoltaic, we include two datasets: One from Germany and one representing solar-intense regions with higher full hours (high FLH), which we approximate by photovoltaic from Saudi Arabia.

## Planetary footprints of plastics in 2050

In the main article, we show that the plastics industry can achieve absolute environmental sustainability for production volumes of 2030 when applying optimistic assumptions of 95 % efficiency for chemical recycling to monomers and wind power as the source of electricity. From 2030 to 2050, production increases by 27-163 %, depending on the type of plastic (see Table 1 of the main article). The increasing production compromises absolute environmental sustainability in 2050 for any technology combination when applying best case assumptions (see Figure S4).

For bio-based plastics with maximum recycling rates, biosphere integrity and the N-cycle footprint are transgressed by 337 % and 353 % compared to the plastics industry’s share of SOS, while for CCU-based plastics with maximum recycling rates, aerosol loading is transgressed by 64 %. In addition, even a linear combination of both mitigation strategies does not allow for absolute environmentally sustainable plastics when applying a share of SOS of 1.1 %.

Figure S4: The planetary footprint of the plastic industry for three pathways in 2050. The planetary footprints are calculated by minimizing the environmental impact of climate change (details in methods in main article). The pathways include fossil-based (grey), bio-based (green), and CCU-based (light and dark blue) plastics with the current (upper value) and the maximum (lower value) recycling rates. The only exception is freshwater use of bio-based plastics, where the lower value represents the current recycling rate. Arrows indicate increasing recycling rates.

References

1	R. Meys, A. Kätelhön, M. Bachmann, B. Winter, C. Zibunas, S. Suh and A. Bardow, Science (New York, N.Y.), 2021, 374, 71–76.

2	IHS Markit Ed., Process Economics Program (PEP) Yearbook, 2018.

3	R. Meys, F. Frick, S. Westhues, A. Sternberg, J. Klankermayer and A. Bardow, Resources, Conservation and Recycling, 2020, 162, 105010.

4	M. Matzen, M. Alhajji and Y. Demirel, Yasar Demirel Publications, 2015.

5	G. Wernet, C. Bauer, B. Steubing, J. Reinhard, E. Moreno-Ruiz and B. Weidema, International Journal of Life Cycle Assessment, 2016, 1218–1230.

6	N. von der Assen, L. J. Müller, A. Steingrube, P. Voll and A. Bardow, Environmental Science & Technology, 2016, 50, 1093–1101.

7	I. Hannula and E. Kurkela, Bioresource Technology, 2010, 101, 4608–4615.

8	I. Hannula and E. Kurkela, Biomass and Bioenergy, 2012, 38, 58–67.

9	J. Isaksson, K. Pettersson, M. Mahmoudkhani, A. Åsblad and T. Berntsson, Energy, 2012, 44, 420–428.

10	M. Arvidsson, M. Morandin and S. Harvey, Energy & Fuels, 2014, 28, 4075–4087.

11	P. Spath, A. Aden, T. Eggeman, M. Ringer, B. Wallace and J. Jechura, Biomass to Hydrogen Production Detailed Design and Economics Utilizing the Battelle Columbus Laboratory Indirectly-Heated Gasifier, National Renewable Energy Lab., Golden, CO (US) NREL/TP-510-37408.

12	M. Haaf, R. Anantharaman, S. Roussanaly, J. Ströhle and B. Epple, Resources, Conservation and Recycling, 2020, 162, 104973.

13	M. Arvidsson, S. Heyne, M. Morandin and S. Harvey, PRES 2012, 2012, 29.

14	S. Deutz and A. Bardow, Nat Energy, 2021, 6, 203–213.

15	International Energy Agency (IEA), Energy technology perspectives 2020, 2020.

16	D. Humbird, R. Davis, L. Tao, C. Kinchin, D. Hsu, A. Aden, P. Schoen, J. Lukas, B. Olthof, M. Worley, D. Sexton and D. Dudgeon, Process Design and Economics for Biochemical Conversion of Lignocellulosic Biomass to Ethanol: Dilute-Acid Pretreatment and Enzymatic Hydrolysis of Corn Stover, National Renewable Energy Lab. (NREL), Golden, CO (United States) NREL/TP-5100-47764.

17	A. E. Agora Verkehrswende, The Future Cost of Electricity-Based Synthetic Fuels. Conclusions Drawn by Agora Verkehrswende and Agora Energiewende., Agora Verkehrswende, Agora Energiewende and Frontier Economics, 2018.

18	B. Müller, K. Müller, D. Teichmann and W. Arlt, Chemie Ingenieur Technik, 2011, 83, 2002–2013.

19	M. de Saint Jean, P. Baurens and C. Bouallou, International Journal of Hydrogen Energy, 2014, 39, 17024–17039.

20	M. Pérez-Fortes and E. Tzimas, Techno-economic and environmental evaluation of CO2 utilisation for fuel production. Synthesis of methanol and formic acid. JRC Science for Policy Report, 2016.

21	N. von der Assen, A. Sternberg, A. Kätelhön and A. Bardow, Faraday Discuss., 2015, 183, 291–307.

22	S. I. Pérez-Uresti, M. Martín and A. Jiménez-Gutiérrez, Applied Energy, 2019, 250, 1120–1131.

23	G. Doka, Life cycle inventories of waste treatment services. Ecoinvent report no. 13, 2003.

24	G. Doka, Updates to Life Cycle Inventories of Waste Treatment Services-part II: waste incineration., 2013.

25	IEA, World Energy Outlook 2021, Paris, 2021.

26	Á. Galán-Martín, V. Tulus, I. Díaz, C. Pozo, J. Pérez-Ramírez and G. Guillén-Gosálbez, One Earth, 2021, 4, 565–583.

27	S. Sala, A. K. Cerutti and R. Pant, Development of a weighting approach for the Environmental Footprint, Luxembourg, 2018.

28	S. Sala, E. Crenna, M. Secchi and R. Pant, Global normalisation factors for the Environmental Footprint and Life Cycle Assessment. EUR (28984), Luxembourg, 2017.

29	S. C. D’Angelo, S. Cobo, V. Tulus, A. Nabera, A. J. Martín, J. Pérez-Ramírez and G. Guillén-Gosálbez, ACS Sustainable Chemistry & Engineering, 2021, 9, 9740–9749.

30	M. W. Ryberg, M. Owsianiak, K. Richardson and M. Z. Hauschild, Ecological Indicators, 2018, 88, 250–262.

31	M. Meinshausen, S. J. Smith, K. Calvin, J. S. Daniel, M. L. T. Kainuma, J.-F. Lamarque, K. Matsumoto, S. A. Montzka, S. C. B. Raper, K. Riahi, A. Thomson, G. J. M. Velders and D. P.P. van Vuuren, Climatic Change, 2011, 109, 213–241.

32	EU Directive, Directive 2004/42/ce of the european parliament and of the council of 21 april 2004 on the limitation of emissions of volatile organic compounds due to the use of organic solvents in certain paints and varnishes and vehicle refinishing products amending directive 1999/13/ec, Official Journal of the European Union, 2004.

33	M. W. Ryberg, M. M. Andersen, M. Owsianiak and M. Z. Hauschild, Journal of Cleaner Production, 2020, 276, 123287.

34	European Commission - Joint Research Centre - Institute for Environment and Sustainability, International Reference Life Cycle Data System (ILCD) Handbook - General guide for Life Cycle Assessment - Detailed guidance. First edition March 2010, Publications Office of the European Union, EUR 24708 EN, Luxembourg, 2010.

35	M. M. Hanafiah, A. J. Hendriks and M. A.J. Huijbregts, Journal of Cleaner Production, 2012, 37, 107–114.

36	M. R. Hart, B. F. Quin and M. Long Nguyen, Journal of Environmental Quality, 2004, 33, 1954–1972.

37	M. Falkenmark and J. Rockström, Journal of Water Resources Planning and Management, 2006, 132, 129–132.

38	D. Gerten, J. Rockström, J. Heinke, W. Steffen, K. Richardson and S. Cornell, Science (New York, N.Y.), 2015, 348, 1217.

39	T. Gleeson, L. Wang-Erlandsson, S. C. Zipper, M. Porkka, F. Jaramillo, D. Gerten, I. Fetzer, S. E. Cornell, L. Piemontese, L. J. Gordon, J. Rockström, T. Oki, M. Sivapalan, Y. Wada, K. A. Brauman, M. Flörke, M. F.P. Bierkens, B. Lehner, P. Keys, M. Kummu, T. Wagener, S. Dadson, T. J. Troy, W. Steffen, M. Falkenmark and J. S. Famiglietti, One Earth, 2020, 2, 223–234.

40	United Nations Environment Programme, Montreal Protocol. Montreal protocol on substances that deplete the ozone layer, US Goverment Printing Office 128-136, Washington DC, 1987.

41	I. M. Algunaibet, C. Pozo, A. Galán-Martín, M.A.J. Huijbregts, N. Mac Dowell and G. Guillén-Gosálbez, Energy & Environmental Science, 2019, 12, 1890–1900.

42	W. Steffen, K. Richardson, J. Rockström, S. E. Cornell, I. Fetzer, E. M. Bennett, R. Biggs, S. R. Carpenter, W. de Vries, C. A. de Wit, C. Folke, D. Gerten, J. Heinke, G. M. Mace, L. M. Persson, V. Ramanathan, B. Reyers and S. Sörlin, Science (New York, N.Y.), 2015, 347, 1259855.

43	L. Persson, B. M. Carney Almroth, C. D. Collins, S. Cornell, C. A. de Wit, M. L. Diamond, P. Fantke, M. Hassellöv, M. MacLeod, M. W. Ryberg, P. Søgaard Jørgensen, P. Villarrubia-Gómez, Z. Wang and M. Z. Hauschild, Environmental Science & Technology, 2022, 56, 1510–1521.

44	M. B. Kosnik, M. Z. Hauschild and P. Fantke, Environmental Science & Technology, 2022.

45	J. Rockström, W. Steffen, K. Noone, Å. Persson, F. S. Chapin, E. Lambin, T. M. Lenton, M. Scheffer, C. Folke, H. J. Schellnhuber, B. Nykvist, C. A. de Wit, T. Hughes, S. van der Leeuw, H. Rodhe, S. Sörlin, P. K. Snyder, R. Costanza, U. Svedin, M. Falkenmark, L. Karlberg, R. W. Corell, V. J. Fabry, J. Hansen, B. Walker, D. Liverman, K. Richardson, P. Crutzen and J. Foley, Ecology and Society, 2009, 14.

46	A. W. Hjalsted, A. Laurent, M. M. Andersen, K. H. Olsen, M. W. Ryberg and M. Z. Hauschild, Journal of Industrial Ecology, 2021, 25, 6–19.

47	I. M. Algunaibet and G. Guillén-Gosálbez, Journal of Cleaner Production, 2019, 229, 886–901.

48	BASF, BASF Report 2020. Responsible Conduct Along the Value Chain, 2020.

49	A. Bjørn, C. Chandrakumar, A.-M. Boulay, G. Doka, K. Fang, N. Gondran, M. Z. Hauschild, A. Kerkhof, H. King, M. Margni, S. McLaren, C. Mueller, M. Owsianiak, G. Peters, S. Roos, S. Sala, G. Sandin, S. Sim, M. Vargas-Gonzalez and M. Ryberg, Environ. Res. Lett., 2020, 15, 83001.

50	R. Dworkin, Philosophy & Public Affairs, 1981, 283–345.

51	A. Bjørn and M. Z. Hauschild, Int J Life Cycle Assess, 2015, 20, 1005–1018.

52	T. Raa, Handbook of Input–Output Analysis, Edward Elgar Publishing Limited, 2017.

53	K. Stadler, R. Wood, T. Bulavskaya, C.-J. Södersten, M. Simas, S. Schmidt, A. Usubiaga, J. Acosta-Fernández, J. Kuenen, M. Bruckner, S. Giljum, S. Lutter, S. Merciai, J. H. Schmidt, M. C. Theurl, C. Plutzar, T. Kastner, N. Eisenmenger, K.-H. Erb, A. de Koning and A. Tukker, Journal of Industrial Ecology, 2018, 22, 502–515.

54	A. Gambarin and H. Galloway, The global chemical industry: Catalyzing growth and addressing our world’s sustainability challenges, Oxford Economics, 2019.

55	M. Fleurbaey and D. Blanchet, Beyond GDP: Measuring Welfare and Assessing Sustainability, Oxford University Press, 2013.

56	R. Geyer, J. R. Jambeck and K. L. Law, Science advances, 2017, 3, e1700782.

57	H. Jeswani, C. Krüger, M. Russ, M. Horlacher, F. Antony, S. Hann and A. Azapagic, Science of The Total Environment, 2021, 769, 144483.

58	G. Faraca, V. Martinez-Sanchez and T. F. Astrup, Resources, Conservation and Recycling, 2019, 143, 299–309.

59	S. Cadoux, A. B. Riche, N. E. Yates and J.-M. Machet, Biomass and Bioenergy, 2012, 38, 14–22.

60	J. P. McCalmont, A. Hastings, N. P. McNamara, G. M. Richter, P. Robson, I. S. Donnison and J. Clifton-Brown, GCB Bioenergy, 2017, 9, 489–507.

61	N. Roncucci, Nassi O Di Nasso, N., C. Tozzini, E. Bonari and G. Ragaglini, GCB Bioenergy, 2015, 7, 1009–1018.

62	C. Reinert, S. Deutz, H. Minten, L. Dörpinghaus, S. von Pfingsten, N. Baumgärtner and A. Bardow, Computers & Chemical Engineering, 2021, 153, 107406.

63	W. Steffen, K. Richardson, J. Rockström, H. J. Schellnhuber, O. P. Dube, S. Dutreuil, T. M. Lenton and J. Lubchenco, Nat Rev Earth Environ, 2020, 1, 54–63.