Independent Assessment of Global Temperature Rise Based Upon Approved Algorithms

Extended Data Addendum

This numerical Extended Data Addendum provides most of the information as necessary to validate the results provided in the subject climate-change article. It is presented in original detail to enable scientific reviewers to review each step of the lengthy analysis performed to obtain global temperature rise estimates at net-zero conditions. It has been limited to the ten Tables listed below. Four additional Tables and one Figure are included in Supplemental Data.

The information consists primarily of tabular data that calculates carbon sink amounts, determines atmospheric GHG concentrations, calculates radiative forcings for CO2, CH4 and N2O and computes the temperature rise for the interval before reaching net-zero. Recall the overall evaluation process is performed for three selected linear emission reduction pathways starting at 40 Gigatonne (Gt) CO2 in the year 2020. The selected pathways for evaluation were for annual reductions of 0.4, 0.5 and 0.6 GtCO2 per year until net-zero conditions are reached in years 2066, 2076 and 2100, respectively.

Table 8 – Calculation of Carbon Sink Values for 0.4 Gt CO2 per Year Emission Reduction Pathway

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2100 – 2098 | 3yrs of 25.2Gt X .070 | 1.76 |
| 2097 – 2095 | 3yrs of 28.8Gt X .030 | 0.86 |
| 2094 – 2091 | 4yrs of 44.0Gt X .015 | 0.66 |
| 2090 – 2081 | 10yrs of 148Gt X .005 | 0.74 |
| 2080 – 2061 | 20yrs of 400Gt X .004 | 1.60 |
| 2060 – 2041 | 20yrs of 560Gt X .003 | 1.68 |
| 2040 – 2000 | 40yrs of 1600Gt X .0015 | 2.40 |
|  | 2100 - Total | 9.70Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2080 – 2078 | 3yrs of 40.2Gt X .070 | 2.81 |
| 2077 – 2075 | 3yrs of 43.4Gt X .030 | 1.30 |
| 2074 – 2071 | 4yrs of 64.0Gt X .015 | 0.96 |
| 2070 – 2061 | 10yrs of 188Gt X .005 | 0.94 |
| 2060 – 2041 | 20yrs of 526GtX .004 | 2.10 |
| 2040 – 2021 | 20yrs of 716Gt X .003 | 2.15 |
| 2020 – 1980 | 40yrs of 1304Gt X .0015 | 1.96 |
|  | 2080 - Total | 12.22Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2060 – 2058 | 3yrs of 64.2Gt X .070 | 4.49 |
| 2057 – 2055 | 3yrs of 67.8Gt X .030 | 2.03 |
| 2054 – 2051 | 4yrs of 96.0Gt X .015 | 1.44 |
| 2050 – 2041 | 10yrs of 283Gt X .005 | 1.42 |
| 2040 – 2021 | 20yrs of 716Gt X .004 | 2.86 |
| 2020 – 2001 | 20yrs of 800Gt X .003 | 2.40 |
| 2000 – 1961 | 40yrs of 950Gt X .0015 | 1.43 |
|  | 2060 - Total | 16.07Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2040 – 2038 | 3yrs of 97.2Gt X .070 | 6.80 |
| 2037 – 2035 | 3yrs of 100.8Gt X .030 | 3.02 |
| 2034 – 2031 | 4yrs of 140.0Gt X .015 | 2.10 |
| 2030 – 2021 | 10yrs of 375Gt X .005 | 1.88 |
| 2020 – 2001 | 20yrs of 800Gt X .004 | 3.21 |
| 2000 – 1981 | 20yrs of 550Gt X .003 | 1.65 |
| 1980 – 1941 | 40yrs of 625Gt X .0015 | 0.93 |
|  | 2040 - Total | 19.59 Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2020 – 2018 | 3yrs of 125Gt X .070 | 8.75 |
| 2017 – 2015 | 3yrs of 124Gt X .030 | 3.92 |
| 2014 – 2011 | 4yrs of 165Gt X .015 | 2.48 |
| 2010 – 2001 | 10yrs of 340Gt X .005 | 1.70 |
| 2000 – 1981 | 20yrs of 550Gt X .004 | 2.20 |
| 1980 – 1961 | 20yrs of 400Gt X .003 | 1.20 |
| 1960 – 1920 | 40yrs of 415Gt X .0015 | 2.62 |
|  | 2020 - Total | 20.67 Gt |

Table 9 – Calculation of Atmospheric CO2 for 0.4 Gt Per Year Emission Reduction Pathway

| Year | New CO2 | CO2 Sink | CO2 Net | CO2 conc. | Year | New CO2 | CO2 Sink | CO2 Net | CO2 conc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2020 | 40.0 | 20.670 | 19.33 | 3239.33 | 2061 | 23.6 | 15.88 | 7.72 | 3759.87 |
| 2021 | 39.6 | 20.616 | 18.98 | 3258.31 | 2062 | 23.2 | 15.69 | 7.51 | 3767.38 |
| 2022 | 39.2 | 20.562 | 18.64 | 3276.93 | 2063 | 22.8 | 15.50 | 7.30 | 3774.68 |
| 2023 | 38.8 | 20.508 | 18.29 | 3295.24 | 2064 | 22.4 | 15.31 | 7.09 | 3781.77 |
| 2024 | 38.4 | 20.454 | 17.95 | 3313.19 | 2065 | 22.0 | 15.12 | 6.88 | 3788.65 |
| 2025 | 38.0 | 20.400 | 17.60 | 3330.79 | 2066 | 21.6 | 14.93 | 7.67 | 3795.72 |
| 2026 | 37.6 | 20.346 | 17.25 | 3348.04 | 2067 | 21.2 | 14.74 | 6.86 | 3802.58 |
| 2027 | 37.2 | 20.292 | 16.91 | 3364.93 | 2068 | 20.8 | 14.55 | 6.25 | 3808.83 |
| 2028 | 36.8 | 20.238 | 16.56 | 3381.51 | 2069 | 20.4 | 14.36 | 6.64 | 3814.87 |
| 2029 | 36.4 | 20.184 | 16.22 | 3397.73 | 2070 | 17.0 | 14.17 | 5.83 | 3820.70 |
| 2030 | 36.0 | 20.130 | 15.87 | 3413.16 | 2071 | 19.6 | 13.98 | 5.62 | 3826.32 |
| 2031 | 35.6 | 20.076 | 15.52 | 3429.12 | 2072 | 19.2 | 13.79 | 5.41 | 3831.73 |
| 2032 | 35.2 | 20.024 | 15.18 | 3444.30 | 2073 | 18.8 | 13.60 | 5.20 | 3836.93 |
| 2033 | 34.8 | 19.970 | 14.83 | 3459.13 | 2074 | 18.4 | 13.41 | 4.99 | 3841.92 |
| 2034 | 34.4 | 19.916 | 14.48 | 3473.61 | 2075 | 18.0 | 13.22 | 4.78 | 3846.70 |
| 2035 | 34.0 | 19.862 | 14.14 | 3482.75 | 2076 | 17.6 | 12.84 | 4.57 | 3851.27 |
| 2036 | 33.6 | 19.808 | 13.79 | 3501.54 | 2077 | 17.2 | 12.84 | 4.36 | 3855.63 |
| 2037 | 33.2 | 19.754 | 13.45 | 3514.99 | 2078 | 16.8 | 12.65 | 4.15 | 3859.78 |
| 2038 | 32.8 | 19.700 | 13.10 | 3528.09 | 2079 | 16.4 | 12.46 | 3.94 | 3863.72 |
| 2039 | 32.4 | 19.646 | 12.75 | 3540.84 | 2080 | 16.0 | 12.22 | 3.78 | 3867.50 |
| 2040 | 32.0 | 19.590 | 12.41 | 3553.25 | 2081 | 15.6 | 12.09 | 3.51 | 3871.01 |
| 2041 | 31.6 | 19.41 | 12.19 | 3565.44 | 2082 | 15.2 | 11.96 | 3.24 | 3874.25 |
| 2042 | 31.2 | 19.27 | 11.93 | 3577.37 | 2083 | 14.8 | 11.83 | 2.97 | 3877.22 |
| 2043 | 30.8 | 19.19 | 11.61 | 3588.38 | 2084 | 14.4 | 11.70 | 2.70 | 3879.92 |
| 2044 | 30.4 | 19.01 | 11.39 | 3599.77 | 2085 | 14.0 | 11.57 | 2.43 | 3882.33 |
| 2045 | 30.0 | 18.83 | 11.17 | 3610.94 | 2086 | 13.6 | 11.44 | 2.16 | 3884.51 |
| 2046 | 29.6 | 18.65 | 10.95 | 3621.89 | 2087 | 13.2 | 11.31 | 1.89 | 3886.40 |
| 2047 | 29.2 | 18.47 | 10.73 | 3632.62 | 2088 | 12.8 | 11.18 | 1.62 | 3888.02 |
| 2048 | 28.8 | 18.11 | 10.51 | 3643.13 | 2089 | 12.4 | 11.05 | 1.35 | 3889.37 |
| 2049 | 28.4 | 18.11 | 10.29 | 3653.42 | 2090 | 12.0 | 10.92 | 1.08 | 3890.45 |
| 2050 | 28.0 | 17.93 | 10.07 | 3663.49 | 2091 | 11.6 | 10.80 | 0.80 | 3891.25 |
| 2051 | 27.6 | 17.75 | 9.85 | 3673.84 | 2092 | 11.2 | 10.68 | 0.52 | 3891.77 |
| 2052 | 27.2 | 17.57 | 9.63 | 3682.97 | 2093 | 10.8 | 10.56 | 0.24 | 3892.01 |
| 2053 | 26.8 | 17.39 | 9.41 | 3692.38 | 2094 | 10.4 | 10.44 | -0.04 | 3891.97 |
| 2054 | 26.4 | 17.21 | 9.19 | 3701.57 | 2095 | 10.0 | 10.32 | -0.32 | 3891.65 |
| 2055 | 26.0 | 17.03 | 8.97 | 3710.54 | 2096 | 9.6 | 10.20 | -0.60 | 3891.05 |
| 2056 | 25.6 | 16.85 | 8.75 | 3719.29 | 2097 | 9.2 | 10.08 | -0.88 | 3890.17 |
| 2057 | 25.2 | 16.67 | 8.53 | 3727.82 | 2098 | 8.8 | 9.96 | -1.16 | 3889.67 |
| 2058 | 24.8 | 16.49 | 8.31 | 3736.13 | 2099 | 8.4 | 9.84 | -1.44 | 3887.57 |
| 2059 | 24.4 | 16.31 | 8.09 | 3744.22 | 2100 | 8.0 | 9.72 | -1.72 | 3885.85 |
| 2060 | 24.0 | 16.07 | 7.93 | 3752.15 |  |  |  |  |  |

Table 10 - Calculation of Carbon Sink Values for 0.5 Gt CO2 per Year Emission Reduction Pathway

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2100 – 2098 | 3yrs of 1.5Gt X .070 | 0.105 |
| 2097 – 2095 | 3yrs of 7.0Gt X .030 | 0.210 |
| 2094 – 2091 | 4yrs of 15.0Gt X .015 | 0.225 |
| 2090 – 2081 | 10yrs of 73.0Gt X .005 | 0.365 |
| 2080 – 2041 | 40yrs of 800Gt X .0035 | 2.80 |
| 2040 – 2000 | 40yrs of 1500 X .0015 | 2.25 |
|  | 2100 - Total | 5.96Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2080 – 2078 | 3yrs of 31.5Gt X .070 | 2.105 |
| 2077 – 2075 | 3yrs of 36.0Gt X .030 | 1.080 |
| 2074 – 2071 | 4yrs of 55.0Gt X .015 | 0.825 |
| 2070 – 2061 | 10yrs of 175Gt X .005 | 0.875 |
| 2060 – 2021 | 40yrs of 1200Gt X .0035 | 4.200 |
| 2020 – 1980 | 40yrs of 1350 X .0015 | 2.025 |
|  | 2080 - Total | 11.19Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2060 – 2058 | 3yrs of 61.5Gt X .070 | 4.285 |
| 2057 – 2055 | 3yrs of 66.0 Gt X .030 | 1.980 |
| 2054 – 2051 | 4yrs of 91.0Gt X .015 | 1.363 |
| 2050 – 2041 | 10yrs of 275Gt X .005 | 1.375 |
| 2040 – 2001 | 40yrs of 1500Gt X .0035 | 5.250 |
| 2000 – 1961 | 40yrs of 950Gt X .0015 | 1.425 |
|  | 2060 - Total | 16..68Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2040 – 2038 | 3yrs of 91.5Gt X .070 | 6.405 |
| 2037 – 2035 | 3yrs of 96.0Gt X .030 | 2.880 |
| 2034 – 2031 | 4yrs of 135Gt X .015 | 2.025 |
| 2030 – 2021 | 10yrs of 375Gt X .005 | 1.875 |
| 2020 – 1981 | 40yrs of 1350Gt X .003 | 4.725 |
| 1980 – 1941 | 40yrs of 625Gt X .0015 | 0.938 |
|  | 2040 - Total | 18.48Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2020 – 2018 | 3yrs of 125Gt X .070 | 8.75 |
| 2017 – 2015 | 3yrs of 124Gt X .030 | 3.72 |
| 2014 – 2011 | 4yrs of 165Gt X .015 | 2.98 |
| 2010 – 2001 | 10yrs of 340Gt X .005 | 1.70 |
| 2000 – 1961 | 40yrs of 950Gt X .0035 | 3.375 |
| 1960 – 1921 | 40yrs of 415Gt X .0015 | 0.623 |
|  | 2020 - Total | 20.59Gt |

Table 11 - Calculation of Atmospheric CO2 Concentrations for 0.5 Gt per Year Reduction Pathway

| Year | New CO2 | CO2 Sink | CO2 Net | CO2 conc. | Year | New CO2 | CO2 Sink | CO2 Net | CO2 conc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2020 | 40.0 | 20.59 | 19.41 | 3239.4 | 2061 | 19.5 | 15.47 | 4.63 | 3696.5 |
| 2021 | 39.5 | 20.50 | 19.41 | 3239.4 | 2062 | 19.0 | 15.25 | 3.73 | 3699.8 |
| 2022 | 39.0 | 20.42 | 18.58 | 3277.0 | 2063 | 18.5 | 15.62 | 3.48 | 3703.3 |
| 2023 | 38.5 | 20.33 | 18.17 | 3294.2 | 2064 | 18.0 | 14.80 | 3.20 | 3706.5 |
| 2024 | 38.0 | 20.24 | 17.76 | 3312.0 | 2065 | 17.5 | 14.57 | 2.93 | 3707.4 |
| 2025 | 37.5 | 20.16 | 17.34 | 3329.3 | 2066 | 17.0 | 14.35 | 2.65 | 3710.1 |
| 2026 | 37.0 | 20.07 | 16.93 | 3346.2 | 2067 | 16.5 | 14.12 | 2.38 | 3712.5 |
| 2027 | 36.5 | 19.98 | 16.52 | 3362.7 | 2068 | 16.0 | 13.90 | 2.10 | 3714.6 |
| 2028 | 36.0 | 19.89 | 16.16 | 3378.8 | 2069 | 15.5 | 13.67 | 1.88 | 3716.5 |
| 2029 | 35.5 | 19.81 | 15.69 | 3394.5 | 2070 | 15.0 | 13.45 | 1.53 | 3718.1 |
| 2030 | 35.0 | 19.72 | 15.28 | 3489.8 | 2071 | 14.5 | 13.22 | 1.28 | 3719.4 |
| 2031 | 34.5 | 19.63 | 14.89 | 3424.7 | 2072 | 14.0 | 13.00 | 1.00 | 3720.4 |
| 2032 | 34.0 | 19.55 | 14.43 | 3439.2 | 2073 | 13.5 | 12.77 | 0.73 | 3721.2 |
| 2033 | 33.5 | 19.46 | 14.04 | 3453.2 | 2074 | 13.0 | 12.54 | 0.46 | 3721.8 |
| 2034 | 33.0 | 19.37 | 13.63 | 3466.8 | 2075 | 12.5 | 12.32 | 0.18 | 3722.0 |
| 2035 | 32.5 | 19.29 | 13.21 | 3480.0 | 2076 | 12.0 | 12.09 | -0.09 | 3721.9 |
| 2036 | 32.0 | 19.20 | 12.80 | 3492.8 | 2077 | 11.5 | 11.87 | -0.31 | 3721.5 |
| 2037 | 31.5 | 19.11 | 12.39 | 3505.2 | 2078 | 11.0 | 11.64 | -0.64 | 3720.9 |
| 2038 | 31.0 | 19.02 | 11.98 | 3517.2 | 2079 | 10.5 | 11.42 | -0.92 | 3720.0 |
| 2039 | 30.5 | 18.94 | 11.56 | 3528.8 | 2080 | 10.0 | 11.19 | -1.19 | 3118.8 |
| 2040 | 30.0 | 18.85 | 11.15 | 3540.0 | 2081 | 9.5 | 10.93 | -1.43 | 3717.4 |
| 2041 | 29.5 | 18.68 | 18.72 | 3550.7 | 2082 | 9.0 | 10.67 | -1.67 | 3715.7 |
| 2042 | 29.0 | 18.52 | 10.48 | 3561.2 | 2083 | 8.5 | 10.41 | -1.91 | 3713.8 |
| 2043 | 28.5 | 18.37 | 10.13 | 3571.3 | 2084 | 8.0 | 10.14 | -2.14 | 3711.7 |
| 2044 | 28.0 | 18.21 | 9.79 | 3581.1 | 2085 | 7.5 | 9.88 | -2.38 | 3709.3 |
| 2045 | 27.5 | 18.05 | 9.45 | 3590.6 | 2086 | 7.0 | 9.62 | -2.62 | 3706.7 |
| 2046 | 27.0 | 17.89 | 9.11 | 3599.7 | 2087 | 6.5 | 9.36 | -2.86 | 3703.8 |
| 2047 | 26.5 | 17.73 | 8.77 | 3608.5 | 2088 | 6.0 | 9.10 | -3.10 | 3700.7 |
| 2048 | 26.0 | 17.58 | 8.42 | 3616.9 | 2089 | 5.5 | 8.83 | -3.33 | 3697.4 |
| 2049 | 25.5 | 17.42 | 8.08 | 3625.0 | 2090 | 5.0 | 8.57 | -3.57 | 3693.8 |
| 2050 | 25.0 | 17.26 | 7.74 | 3632.7 | 2091 | 4.5 | 8.31 | -3.81 | 3670.0 |
| 2051 | 24.5 | 17.10 | 7.35 | 3640.1 | 2092 | 4.0 | 8.05 | -4.03 | 3665.9 |
| 2052 | 24.0 | 16.94 | 7.06 | 3647.2 | 2093 | 3.5 | 7.79 | -4.29 | 3661.6 |
| 2053 | 23.5 | 16.79 | 6.71 | 3653.9 | 2094 | 3.0 | 7.53 | -4.53 | 3657.1 |
| 2054 | 23.0 | 16.63 | 6.37 | 3660.3 | 2095 | 2.5 | 7.26 | -4.76 | 3657.3 |
| 2055 | 22.5 | 16.47 | 6.03 | 3666.3 | 2096 | 2.0 | 7.00 | -5.00 | 3647.0 |
| 2056 | 22.0 | 16.31 | 5.69 | 3672.0 | 2097 | 1.5 | 6.74 | -5.24 | 3643.4 |
| 2057 | 21.5 | 16.15 | 5.45 | 3678.5 | 2098 | 1.0 | 6.48 | -5.48 | 3637.9 |
| 2058 | 21.0 | 16.00 | 5.00 | 3683.5 | 2099 | .5 | 6.22 | -3.72 | 3632.2 |
| 2059 | 20.5 | 15.84 | 4.66 | 3688.2 | 2100 | .0 | 5.96 | -5.96 | 3626.2 |
| 2060 | 20.0 | 15.68 | 4.32 | 3692.5 |  |  |  |  |  |

Table 12 - Calculation of Carbon Sink Values for 0.6 GtCO2 per Year Emission Reduction Pathway

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2100 – 2098 | 3yrs of 14.4Gt X .070 | 1.01 |
| 2097 – 2095 | 3yrs of 18.0Gt X .030 | 0.54 |
| 2094 – 2091 | 4yrs of 30.4Gt X .015 | 0.46 |
| 2090 – 2081 | 10yrs of 208Gt X .005 | 1.04 |
| 2080 – 2061 | 20yrs of 360Gt X .004 | 1.44 |
| 2060 – 2041 | 20yrs of 340Gt X .003 | 1.02 |
| 2040 – 2000 | 40yrs of 1516Gt X .0015 | 2.27 |
|  | 2100 - Total | 7.78Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2080 – 2078 | 3yrs of 13.8Gt X .070 | 0.97 |
| 2077 – 2075 | 3yrs of 19.2Gt X .030 | 0.58 |
| 2074 – 2071 | 4yrs of 34.0Gt X .015 | 0.51 |
| 2070 – 2061 | 10yrs of 127Gt X .005 | 0.64 |
| 2060 – 2041 | 20yrs of 434GtX .004 | 1.74 |
| 2040 – 2021 | 20yrs of 674Gt X .003 | 2.02 |
| 2020 – 1980 | 40yrs of 1304Gt X .0015 | 1.96 |
|  | 2080 - Total | 8.42Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2060 – 2058 | 3yrs of 49.8Gt X .070 | 3.49 |
| 2057 – 2055 | 3yrs of 55.2Gt X .030 | 1.66 |
| 2054 – 2051 | 4yrs of 82.0Gt X .015 | 1.73 |
| 2050 – 2041 | 10yrs of 247Gt X .005 | 1.24 |
| 2040 – 2021 | 20yrs of 674Gt X .004 | 2.70 |
| 2020 – 2001 | 20yrs of 800Gt X .003 | 2.40 |
| 2000 – 1961 | 40yrs of 950Gt X .0015 | 1.40 |
|  | 2060 - Total | 16.07Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2040 – 2038 | 3yrs of 85.8Gt X .070 | 6.01 |
| 2037 – 2035 | 3yrs of 91.2Gt X .030 | 2.74 |
| 2034 – 2031 | 4yrs of 130Gt X .015 | 1.95 |
| 2030 – 2021 | 10yrs of 367Gt X .005 | 1.84 |
| 2020 – 2001 | 20yrs of 800Gt X .004 | 3.20 |
| 2000 – 1981 | 20yrs of 550Gt X .003 | 1.65 |
| 1980 – 1941 | 40yrs of 625Gt X .0015 | 0.93 |
|  | 2040 - Total | 18.33Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2020 – 2018 | 3yrs of 125.2Gt X .070 | 8.75 |
| 2017 – 2015 | 3yrs of 124Gt X .030 | 3.71 |
| 2014 – 2011 | 4yrs of 165Gt X .015 | 2.48 |
| 2010 – 2001 | 10yrs of 340Gt X .005 | 1.70 |
| 2000 – 1981 | 20yrs of 550Gt X .004 | 2.20 |
| 1980 – 1961 | 20yrs of 400Gt X .003 | 1.20 |
| 1960 – 1920 | 40yrs of 415Gt X .0015 | 0.623 |
|  | 2020 - Total | 20.673 Gt |

Table 13 - Calculation of Carbon Sink Values for 0.6 GtCO2 per Year Emission Reduction Pathway

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2100 – 2098 | 3yrs of 14.4Gt X .070 | 1.01 |
| 2097 – 2095 | 3yrs of 18.0Gt X .030 | 0.54 |
| 2094 – 2091 | 4yrs of 30.4Gt X .015 | 0.46 |
| 2090 – 2081 | 10yrs of 208Gt X .005 | 1.04 |
| 2080 – 2061 | 20yrs of 360Gt X .004 | 1.44 |
| 2060 – 2041 | 20yrs of 340Gt X .003 | 1.02 |
| 2040 – 2000 | 40yrs of 1516Gt X .0015 | 2.27 |
|  | 2100 - Total | 7.78Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2080 – 2078 | 3yrs of 13.8Gt X .070 | 0.97 |
| 2077 – 2075 | 3yrs of 19.2Gt X .030 | 0.58 |
| 2074 – 2071 | 4yrs of 34.0Gt X .015 | 0.51 |
| 2070 – 2061 | 10yrs of 127Gt X .005 | 0.64 |
| 2060 – 2041 | 20yrs of 434GtX .004 | 1.74 |
| 2040 – 2021 | 20yrs of 674Gt X .003 | 2.02 |
| 2020 – 1980 | 40yrs of 1304Gt X .0015 | 1.96 |
|  | 2080 - Total | 8.42Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2060 – 2058 | 3yrs of 49.8Gt X .070 | 3.49 |
| 2057 – 2055 | 3yrs of 55.2Gt X .030 | 1.66 |
| 2054 – 2051 | 4yrs of 82.0Gt X .015 | 1.73 |
| 2050 – 2041 | 10yrs of 247Gt X .005 | 1.24 |
| 2040 – 2021 | 20yrs of 674Gt X .004 | 2.70 |
| 2020 – 2001 | 20yrs of 800Gt X .003 | 2.40 |
| 2000 – 1961 | 40yrs of 950Gt X .0015 | 1.40 |
|  | 2060 - Total | 16.07Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2040 – 2038 | 3yrs of 85.8Gt X .070 | 6.01 |
| 2037 – 2035 | 3yrs of 91.2Gt X .030 | 2.74 |
| 2034 – 2031 | 4yrs of 130Gt X .015 | 1.95 |
| 2030 – 2021 | 10yrs of 367Gt X .005 | 1.84 |
| 2020 – 2001 | 20yrs of 800Gt X .004 | 3.20 |
| 2000 – 1981 | 20yrs of 550Gt X .003 | 1.65 |
| 1980 – 1941 | 40yrs of 625Gt X .0015 | 0.93 |
|  | 2040 - Total | 18.33Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2020 – 2018 | 3yrs of 125.2Gt X .070 | 8.75 |
| 2017 – 2015 | 3yrs of 124Gt X .030 | 3.71 |
| 2014 – 2011 | 4yrs of 165Gt X .015 | 2.48 |
| 2010 – 2001 | 10yrs of 340Gt X .005 | 1.70 |
| 2000 – 1981 | 20yrs of 550Gt X .004 | 2.20 |
| 1980 – 1961 | 20yrs of 400Gt X .003 | 1.20 |
| 1960 – 1920 | 40yrs of 415Gt X .0015 | 0.623 |
|  | 2020 - Total | 20.673 Gt |

Table 14 - Calculation of Radiative Forcing for Year 2020

| CO2 |  |
| --- | --- |
| RF= | [-2.4 x 10-7(412-278) exp2 + 7.2 x 10-4(412-278) - 2.1 x 10-4 x 0.5(333+278) + 5.36] ln (412/278) |
| = | [-2.4 x 1.8 x 10-3+7.2 x 1.34 x 10-2 -0.0633+5.36]0.393 |
| = | [-0.0043+0.096-0.0633+5.36]0.393 = [.026+5.36] x 0.393 = 2.12 W/sqm |

| CH4 |  |
| --- | --- |
| RF= | [-1.3 x 10-6 x 0.5 x (1884+722) – 8.2 x 10-6 x .5 (333+270) + .043] (sqrt1884 - sqrt722) |
| = | [-7.35 x 10-3 – 4.94 x 10-3 + .043] (434-36.9) = 0.039 x 16.5= 0.64 W/sqm |

| N20 |  |
| --- | --- |
| RF= | [-8 x 10-6 x 0.5(412+278) + 4.2 x 10-6 x 0.5(333+270) – 4.9 x 10-6 x .5 (884 + 722) + |
|  | 0.017] (sqrt333 – sqrt270) |
| = | [-.00276 + .00127 - .00638 + 0.117] x 1.8 |
| = | [0.118 – .0091] x 1.8 = 0.109 x 1.8 = 0.20 W/sqm |

Breakdown of Temperature Components:

Total RF of 2.96 x 0.37C (old ECS value) = 1.1C; and 2.96 x 0.47˚C (for newly derived future EQS value) = 1.39C

Apportionment of year 2020 Temperature Rise:

CO2 = 2.12 W/sqm x .37 = 0.78C; CH4 = 0.54 W/sqm x .37 = 0.24C; N20 = 0.20 W/sqm x .37 = 0.074C

Table 15 - Calculation of Radiative Forcing by CO2 at Net-Zero for 0.4, 0.5, and 0.6 Pathways

Calculation for CO2

| 2066 | [-2.4 x 10-7 (466-412) exp2 + 7.2 x 10-4 (466 - 412) – 2.1 x 10-4 x 0.5 (333+379) + 5.36] ln (466/412) |
| --- | --- |
|  | [-.0007 + .039 - .075 +5.36] x 0.123 = (.037 + 5.36) x 0.123 = 5.32 x 0.123 = 0.654 W/sqm |
| 2076 | [-2.4 x 10-7 (477.2 – 412) exp2 + 7.2 x 10-4 (477.2 – 412) - 2.1 x 10-4 x 0.5 (333+389) + 5.36] ln (477/412) |
|  | [-.00102 + .0469 - .076 + 5.36] x 0.209 = (-0.030 + 5.36) x 0.209 = 5.33 x 0.147= 0.783 W/sqm |
| 2100 | [-2.4 x 10-7 (499.6- 412) exp2 + 7.2 x 10-4 (499.6 - 412) – 2.1 x 10-4 x 0.5 (333 + 413) + 5.36] ln (500/412) |
|  | [-.0018 + .063 - .078 + 5.36] x 0.192 = (-0.017 + 5.36) x 0.192 = 5.34 x 0.192 = 1.025 W/sqm |

Table 16 - Calculation of Radiative Forcing by CH4 at Net-Zero for 0.4, 0.5, and 0.6 Pathways

Calculation for CH4

| 2066 | [-1.3 x 10-6 (1884 + 2100)/2 - 8.2 x 10-6 (333 + 379) / 2 + 0.043] (sqrt2100 - sqrt1884) |
| --- | --- |
|  | [-.0026 - .0029 + 0.043] (45.8-43.4) = 0.037 x 2.4 = 0.089 W/sqm |
| 2076 | [-1.3 x 10-6 (1884 + 2100/2 – 8.2 x 10-6 (333 + 389)2 + .043] (sqrt2100 - sqrt1884) |
|  | [-.0026 - .0030 + .043] x 2.4 = .037 x 2.4 = .089 W/sqm |
| 2100 | [-1.3 x 10-6 (1884 + 2100/2 – 8.2 x 10-6 (333+413)2 + .043] (sqrt2100 - sqrt1884) |
|  | [-.0026 - .0031 + .043] x 2.4 = .037 x 2.4 = .089 W/sqm |

Table 17 - Calculation of Radiative Forcing by N20 at Net-Zero for 0.4, 0.5, and 0.6 Pathways

Calculation for N20

| 2066 | [-8.0 x 10-6 x 439 + 4.2 x 10-6 x 356 – 4.9 x 1992 x 10-6 + 0.117] (19.47 –18.25) |
| --- | --- |
|  | [-.0035 + .0015 - .0098 + .117] x 1.22 = [-.10118 + .117] +1.22 = 0.105 x 1.22 = 0.128 W/sqm |
| 2076 | [-8.0 x 10-6 x 439 + 4.2 x 10-6 x 361 – 4.9 x 10-6 x 1992 x 10-6 + .117] (19.72 - 18.25) |
|  | [-.0036 + .0015 - .0098 + .117] x 1.22 - .0118 + .117] x 1.47 x .105 = 0.154 W/sqm |
| 2100 | [-8.0 x 10-6 x 456 + 4.2 x 10-6 x 376 – 4.9 x 10-6 x 1992 x 10-6 + .117] (20.4 – 18.25) |
|  | [-.0036 + .0016 - .0098 + .117] x (2.15) = 0.105 x 2.15 = 0.226 W/sqm |

Independent Assessment of Global Temperature Rise Based Upon Approved Algorithms

Supplemental Data Addendum

This numerical Data Addendum provides the supplemental information as necessary to validate the results provided in the subject climate-change article. It is presented in original detail to enable scientific reviewers to review each step of the lengthy analysis performed to obtain global temperature rise estimates at net-zero conditions.

This Supplemental Data is as important as the Extended Data and represents a necessary overflow of data because of the ten Table limit specified for Extended.

The information consists primarily of tabular data that calculates past carbon sink amounts to verify what the algorithm predicts, calculates radiative forcing for the year 2020 to verify observed temperature and determines a new value to use for the Equilibrium Climate Sensitivity factor. Recall the overall evaluation process is performed for three selected linear emission reduction pathways starting at 40 Gigatonne (Gt) CO2 in the year 2020. The selected pathways for evaluation were for annual reductions of 0.4, 0.5 and 0.6 GtCO2 per year until net-zero conditions are reached in years 2066, 2076 and 2100, respectively.

The following List identifies four Tables, and one Figure that are included as Supplemental Data:

Table 18. Affirmation of Calculated Carbon Sink Values Prior to 2020

Table 19 Reconstruction of Idealized Temperature Profile from 1850 to 2020

Table 20. CO2 Carbon Sink Values from year 1980 to 20

Figure 2. Smoothed Actual Temperature Profile and Idealized Pathway from 1850 to 2020

Table 18 - Affirmation of Carbon Sink Values Prior for 2020 and Decades Earlier

| Year | CO2 ppm | ppm/year | Net Gt | Calc. Sink Gt | Total Gt | Emissions Gt |
| --- | --- | --- | --- | --- | --- | --- |
| 2020 | 414.24 | 2.5 | 19.5 | 20.7 | 40.2 | 40.3 |
| 2010 | 391.10 | 2.5 | 19.5 | 19.3 | 38.8 | 41.2 |
| 2000 | 369.71 | 1.8 | 14.0 | 14.7 | 28.7 | 30.0 |
| 1990 | 354.43 | 1.7 | 13.3 | 13.3 | 26.8 | 27.0 |
| 1980 | 338.76 | 1.6 | 12.5 | 10.1 | 23.5 | 25.0 |

Note: This Table shows that the calculated carbon sink values, when added to the Net measured increase in concentration, produces results very close to the estimated emissions.

Table 19 - Reconstruction of Idealized Temperature Profile from 1850 to 2020

| Year | CO2 ppm | ln(ppm/280) | Temperature C |
| --- | --- | --- | --- |
| 2020 | 414 | 0.391 | 1.400 |
| 2010 | 390 | 0.331 | 1.185 |
| 2000 | 370 | 0.279 | 1.000 |
| 1990 | 354 | 0.235 | 0.841 |
| 1980 | 339 | 0.191 | 0.684 |
| 1970 | 325 | 0.149 | 0.534 |
| 1960 | 317 | 0.124 | 0.444 |
| 1950 | 310 | 0.102 | 0.365 |
| 1940 | 105 | 0.086 | 0.308 |
| 1930 | 300 | 0.069 | 0.247 |
| 1920 | 295 | 0.052 | 0.186 |
| 1910 | 292 | 0.042 | 0.150 |
| 1900 | 290 | 0.035 | 0.125 |
| 1890 | 288 | 0.028 | 0.100 |
| 1880 | 286 | 0.021 | 0.075 |
| 1870 | 284 | 0.014 | 0.050 |
| 1860 | 283 | 0.011 | 0.039 |
| 1850 | 282 | 0.007 | 0.025 |

Temperature calculated as 3.58 In (CO2 ppm/280) where the value 3.58 was chosen so that the shape from 1980 forward matched the measured temperature. This place the end values at 1.4 degrees C in 2020 compared to 1.1 degrees C actual. Please see Figure 2 shaping mode for illustration.

The volcanic ash in the atmosphere during certain periods in the last century caused large negative radiative forcing that appreciably altered the global temperature during this period.  This means that only climate sensitivity values based upon measured temperatures will be biased towards a smaller value than that due to GHG forcing alone.

Table 20 - CO2 Carbon Sink Values from Year 1980 to 2010

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2010 – 2008 | 3yrs of 122.2Gt X .070 | 8.55 |
| 2007 – 2005 | 3yrs of 122.0Gt X .030 | 3.36 |
| 2004 – 2001 | 4yrs of 136.0Gt X .015 | 1.95 |
| 2000 – 1991 | 10yrs of 256.3Gt X .005 | 1.28 |
| 1990 – 1971 | 20yrs of 647.7Gt X .004 | 2.60 |
| 1970 – 1951 | 20yrs of 300Gt X .003 | 0.90 |
| 1950 – 1911 | 40yrs of 440Gt X .0015 | 0.66 |
|  | 2010 - Total | 19.3Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 2000 – 1978 | 3yrs of 89.1Gt X .070 | 6.24 |
| 1997– 1995 | 3yrs of 86.3Gt X .030 | 2.59 |
| 1994 – 1991 | 4yrs of 110.6Gt X .015 | 1.66 |
| 1990 – 1981 | 10yrs of 260Gt X .005 | 1.30 |
| 1980 – 1961 | 20yrs of 407GtX .004 | 1.63 |
| 1960 – 1941 | 20yrs of 234Gt X .003 | 0.70 |
| 1940 – 1901 | 40yrs of 400Gt X .0015 | 0.60 |
|  | 2000 - Total | 14.7Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 1990 –1988 | 3yrs of 80.4Gt X .070 | 5.63 |
| 1887 – 1985 | 3yrs of 78.6Gt X .030 | 2.36 |
| 1984 – 1981 | 4yrs of 102Gt X .015 | 1.53 |
| 1980 – 1971 | 10yrs of 225Gt X .005 | 1.13 |
| 1970 – 1951 | 20yrs of 300Gt X /004 | 1.20 |
| 1950 – 1931 | 20yrs of200Gt X .003 | 0.60 |
| 1930 – 1891 | 40yrs of 350Gt X .0015 | 0.53 |
|  | 1990 - Total | 13.5Gt |

| Years Past | Period Amount and % Factor | Gt Amount |
| --- | --- | --- |
| 1980 – 1978 | 3yrs of 73.5Gt X .070 | 6.01 |
| 1977 – 1975 | 3yrs of 63Gt X .030 | 7.74 |
| 1974 – 1971 | 4yrs of 85Gt X .015 | 1.95 |
| 1970 – 1961 | 10yrs of 175Gt X .005 | 1.84 |
| 1960 – 1941 | 20yrs of 234Gt X .004 | 3.20 |
| 1940 – 1921 | 20yrs of 140Gt X .003 | 1.65 |
| 1920 – 1880 | 40yrs of 230Gt X .0015 | 0.93 |
|  | 1980 - Total | 11.1Gt |