OATech Network Plus

Biomechanics Beyond the Lab: Remote Technology for Osteoarthritis Patient Data – Executive Summary

Technology for the gathering of biomechanical and functional data for OA is traditionally carried out using established lab-based systems: [1, 2] both fixed gold standard systems e.g. Motion Capture, or increasingly popular wearable systems incorporating IMUs.  [3-5]

The Covid-19 pandemic and desires to improve efficiency, reduce cost and flexibly collect ‘real-world’ data has driven an interest in technology suitable for the collection of data out of a laboratory environment. [6-10]

This scoping review aimed to identify technology suitable for gathering biomechanical parameters pertinent to OA, that had been validated against gold standard technology, was suitable for remote use, and commercially available, therefore presenting an alternative for OA researchers.

Table 1. A wide number of technologies were identified and classified by application / location of use with the majority categorised as Part Remote (65%).

| Portable | Requires fixed research environment e.g. laboratory or clinic | Requires specialist trained users to gather data |
| --- | --- | --- |
| Part Remote | Can operate in most environments with some restrictions e.g. connectivity, power | Set up and data harvest requires specialist, but data can be gathered without specialist present |
| Fully Remote | Able to operate in any environment e.g. home, outdoors | Capable of being used and managed unobserved by participant with minimal support |

Given the validation criteria, technology was not quality assessed for accuracy, cost or usability factors (e.g. battery life, range of use, method of data recovery and analysis) or the advantages/disadvantages for researchers such as training, or technology support/maintenance.

Embracing these new technologies offers OA researchers the potential to simplify methods, reduce the cost and necessary skills for data collection, and widen the locations and environments in which data can be collected. Technology that can operate remotely could facilitate the gathering of objective data, a better understanding of real-world OA, its impact on the patient and effective treatment monitoring.

Table 2. Commercially available technologies identified for remote use.

|  | Description | Metric | Location |
| --- | --- | --- | --- |
| BioStamp https://www.mc10inc.com/ [11, 12] | Skin adherent sensor patch with accelerometer and gyroscope (IMUs) | Tri-axial linear/angular motion, ROM, joint angles, gait SPTs. | Part Remote |
| Echo5d from Atlas5D https://atlas5d.com/our-technology/ [13] | Ambient measurement system – non wearable activity monitoring | Gait SPTs - ADLs | Part Remote |
| Encephalog from Mon4t https://mon4t.com/movement/ [14] | Smartphone app (integrated tri-axial accelerometers and gyroscopes). | Gait SPTs – specifically in relation to TUG parameters. | Remote |
| GaitSmart https://www.gaitsmart.com/ [15, 16] | IMU’s with proprietary software | Gait SPTs, ROM, kinematic parameters. | Part Remote |
| Loadsol https://www.novelusa.com/loadsol [17] | In shoe worn insole device | Plantar peak force. | Part Remote |
| McRoberts Dynaport MoveTest https://www.mcroberts.nl/products/movetest/ [18] | Single IMU belt worn device | Gait SPTs. | Portable |
| OpenGo by Moticon https://moticon.com/opengo [19] | OpenGO (wireless shoe insole) with Moticon smartphone application | Kinetic parameters (KAM), gait SPTs. | Portable |
| Physilog GaitUP https://research.gaitup.com/physilog/ [20] | Two Physilog IMU’s and proprietary Gaitup software system | Gait SPTs. | Part Remote |
| SensFloor https://future-shape.com/en/gait-recording/ [21] | Capacitive sensor embedded flooring with recording outputs | Gait SPTs. | Part Remote |
| Theai3D Markerless https://www.theiamarkerless.ca/ In conjunction with Qualisys (Qualisys AB, Sweden) Miqus cameras [22] | Markerless motion capture software for processing of camera generated video to produce 3D kinematic data (segments and rotation matrices) that is ready for analysis. | Kinematic SPT’s, segments, angles. | Portable |

1.Kressig, R.W. and O. Beauchet, Guidelines for clinical applications of spatio-temporal gait analysis in older adults. Aging Clinical and Experimental Research, 2006. 18(2): p. 174-176.   2.Wren, T.A.L., et al., Clinical efficacy of instrumented gait analysis: Systematic review 2020 update. Gait & Posture, 2020. 80: p. 274-279.

3. Lee, J., et al., Sensitivity comparison of inertial to optical motion capture during gait: implications for tracking recovery. IEEE International Conference on Rehabilitation Robotics, 2019. 2019: p. 139-144.4.Rudisch, J., et al., Agreement and consistency of five different clinical gait analysis systems in the assessment of spatiotemporal gait parameters. Gait Posture, 2021. 85: p. 55-64. 5.Allseits, E., et al., A practical step length algorithm using lower limb angular velocities. Journal of Biomechanics, 2018. 66: p. 137-144. 6.	Urbanek, J.K., et al., Validation of Gait Characteristics Extracted from Raw Accelerometry during Walking Against Measures of Physical Function, Mobility, Fatigability, and Fitness. Journals of Gerontology - Series A Biological Sciences and Medical Sciences, 2018. 73(5): p. 676-681.7.Hillel, I., et al., Is every-day walking in older adults more analogous to dual-task walking or to usual walking? Elucidating the gaps between gait performance in the lab and during 24/7 monitoring. European Review of Aging and Physical Activity, 2019. 16(1).8.Takayanagi, N., et al., Relationship between Daily and In-laboratory Gait Speed among Healthy Community-dwelling Older Adults. Scientific Reports, 2019. 9(1).9.Brodie, M.A.D., et al., Wearable pendant device monitoring using new wavelet-based methods shows daily life and laboratory gaits are different. Medical and Biological Engineering and Computing, 2016. 54(4): p. 663-674.10.Drazan, Moving outside the lab: Markerless motion capture accurately quantifies sagittal plane kinematics during the vertical jump. 2021.11. Ammann, K.R., et al., Human motion component and envelope characterization via wireless wearable sensors. BMC Biomedical Engineering, 2020. 2: p. 3.12. Moon, Y., et al., Monitoring gait in multiple sclerosis with novel wearable motion sensors. PLoS ONE, 2017. 12(2).13. Bethoux, F., et al., Walking speed measurement with an Ambient Measurement System (AMS) in patients with multiple sclerosis and walking impairment. Gait and Posture, 2018. 61: p. 393-397.14. Tchelet, K., A. Stark-Inbar, and Z. Yekutieli, Pilot study of the encephalog smartphone application for gait analysis. Sensors (Switzerland), 2019. 19(23).15. Van Helvoort, E.M., et al., Relationship between motion, using the GaitSmartTM system, and radiographic knee osteoarthritis: an explorative analysis in the IMI-APPROACH cohort. Rheumatology, 2021. 60(8): p. 3588-3597.16. Zügner, R., et al., Validation of inertial measurement units with optical tracking system in patients operated with Total hip arthroplasty. BMC Musculoskeletal Disorders, 2019. 20(1).17. Renner, K. and R. Queen, Detection of age and gender differences in walking using mobile wearable sensors. Gait and Posture, 2021. 87: p. 59-64.18. Werner, C., et al., Concurrent validity, test-retest reliability, and sensitivity to change of a single body-fixed sensor for gait analysis during rollator-assisted walking in acute geriatric patients. Sensors (Switzerland), 2020. 20(17): p. 1-18.19. He, J., et al., Unsupervised gait retraining using a wireless pressure-detecting shoe insole. Gait and Posture, 2019. 70: p. 408-413.20. Lefeber, N., et al., Validity and Reproducibility of Inertial Physilog Sensors for Spatiotemporal Gait Analysis in Patients With Stroke. IEEE transactions on neural systems and rehabilitation engineering : a publication of the IEEE Engineering in Medicine and Biology Society, 2019. 27(9): p. 1865-1874.21. Lanzola, G., et al., Bringing spatiotemporal gait analysis into clinical practice: Instrument validation and pilot study of a commercial sensorized carpet. Computer Methods and Programs in Biomedicine, 2020. 188.22. Kanko, R.M., et al., Concurrent assessment of gait kinematics using marker-based and markerless motion capture. Journal of Biomechanics, 2021. 127: p. 110665.