Appendix A

A numerical approach for preventing the dispersion of infectious disease in a meeting room

Mahdi Ahmadzadeh1, Mehrzad Shams1*

1Faculty of Mechanical Engineering, K.N.Toosi University of Technology, Pardis St., Vanak Sq., Tehran, Iran

* To whom all correspondence should be addressed, shams@kntu.ac.ir

## Droplet evaporation model

Droplet evaporation model equations that were used in validation (section 2.6) are discussed in this appendix. Droplet evaporation is modeled by diffusive molar flux of water vapor droplet into the air, Ni, via the following expression

| Ni=kcCi,d-Ci,∞ | (A.1) |
| --- | --- |

where Ni is evaporative molar flux of vapor, kc is mass transfer coefficient, Ci,d and Ci,∞ are vapor concentration at the droplet surface and surrounding medium, respectively. In this case, Ci,d is related to the saturated vapor pressure via:

| Ci,d=PsatRTd | (A.2) |
| --- | --- |

where R is the universal gas constant and Td is droplet surface temperature. Ci,∞ is then related to the partial vapor pressure,

| Ci,∞=xipRT∞ | (A.3) |
| --- | --- |

where xi is the local mole fraction of species i, p is local pressure, and T∞ is bulk temperature of the surrounding continuous phase. The mass transfer coefficient from droplet to the surrounding continuous phase, kc, is correlated with the Reynolds number and the Schmidt number by the following expression:

| kc=Di,mDd2.0+0.6Re0.5Sc0.33 | (A.4) |
| --- | --- |

where Di,m is the diffusion coefficient of vapor in the continuous phase.

The change in mass of the droplet due to evaporation at every time step obtains as

| mdt+∆t=mdt-NiAdMw,i∆t | (A.5) |
| --- | --- |

where md is the droplet mass, Mw,i is the molecular weight of species i, and Ad is the droplet surface area.

A simple heat balance equation which conveys the relationship heat transfer between the droplet and the continuous phase,

| mdcpdTddt=hAdT∞-Td-dmddthfg | (A.6) |
| --- | --- |

where cp is the specific heat of droplet, dmddt  is evaporation rate, and hfg is latent heat of droplet. The convective heat transfer coefficient, h, is calculated as,

| h=λa ln1+BTDdBT2.0+0.6Red0.5Pr0.33 | (A.7) |
| --- | --- |

where Pr is the Prandtl number and λa is the thermal conductivity of air. BT is the Spalding heat transfer number,

| BT=CpvTa-Tdhfg-qdmd | (A.8) |
| --- | --- |

where md is the droplet evaporation rate and qd is the heat energy transferred to the droplet, and Cpv is the specific heat of the droplet vapor.