| Table S1. Imaging parameters of prostatic Mp-MRI scan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sequence | T2 TSE sagittal | T2 TSE coronal | T2 TSE axial | DWI axial | DCE axial |
| TR (ms) | 3900 | 3500 | 6980 | 5000 | 4.20 |
| TE (ms) | 105 | 105 | 105 | 72 | 1.34 |
| Flip Angle (deg) | 160 | 160 | 160 | - | 14 |
| FOV (cm) | 25 | 25 | 20 | 20 | 24 |
| Matrix size | 384 | 384 | 384 | 128 | 224 |
| Thickness (mm) | 3 mm | 3 mm | 3 mm | 5 mm | 3 mm |
| Voxel size (mm) | 0.6×0.6×3 | 0.6×0.6×3 | 0.5×0.5×3 | 1.5×1.5×3 | 1.1×1.1×3 |
| b-value (sec/mm2) | - | - | - | 50, 700, 1500, 2000 | - |
| Time (min: sec) | 3:28 | 3:23 | 3:32 | 5:05 | 3:44 |

| Table S2. The detailed parameter sets of six embedders for image feature extraction |  |  |  |  |
| --- | --- | --- | --- | --- |
| Embedder | Vectors | No of vectors | Images | Total features |
| Pyradiomics * | intensity, shape, texture, wavelets | 851 | T2, DWI, ADC | 2, 553 |
| Inception v3 + | penultimate layer | 2048 | T2, DWI, ADC | 6, 144 |
| SqueezeNet + | flatten10 layer | 1000 | T2, DWI, ADC | 3, 000 |
| VGG16 + | penultimate layer fc-7 | 4096 | T2, DWI, ADC | 12, 288 |
| VGG19 + | penultimate layer fc-7 | 4096 | T2, DWI, ADC | 12, 288 |
| DeepLoc + | penultimate layer fc-2 | 512 | T2, DWI, ADC | 1, 536 |
| Sum | - | 12, 603 | T2, DWI, ADC | 37, 809 |
| *, features extracted using the Pyradiomics version 2.2.0. +, features extracted using the dedicative pretrained models from ImageNet. DWI = diffusion-weighted imaging; ADC = apparent diffusion coefficient. |  |  |  |  |