Supplementary material for

Title: Resting-state functional connectivity disruption between the left and right pallidum as a biomarker for subthreshold depression

Yosuke Sato, Go Okada, Satoshi Yokoyama, Naho Ichikawa, Masahiro Takamura, Yuki Mitsuyama, Ayaka Shimizu, Eri Itai, Hotaka Shinzato, Mitsuo Kawato, Noriaki Yahata, Yasumasa Okamoto

Contents

Supplementary Figure S1

Supplementary Table S1

Supplementary Table S2

Supplementary Figure S1. Accuracy of the subthreshold depression (StD) classifier.

Distribution of the weighted linear summations of functional connections based on the classification of StD and healthy controls for (a) the training dataset (StD: n = 30; healthy controls: n = 61) and (b) the test dataset from an independent cohort (StD: n = 16; healthy controls: n = 27). (c) Histogram of the permutation test (1,000 repetitions) for leave-one-out cross validation (LOOCV) for the training dataset. (d) Histogram of the permutation test (1,000 repetitions) for accuracy in the independent test dataset. The binomial distribution is presented as an orange curve in (d). The accuracy of the StD classifier trained and tested without permutations is presented as vertical red lines in (c–d). LOOCV of the training dataset and independent test dataset was significant at p = 0.001, as shown by the results of the permutation test.

Supplementary Table S1. Scanner information and resting-state functional magnetic imaging protocols.

| Parameter | Training dataset | Test dataset |
| --- | --- | --- |
| MRI scanner | Siemens Verio | Siemens Skyra-fit |
| Participants (StD/healthy controls) | 30/61 | 16/27 |
| Magnetic field (T) | 3 | 3 |
| Field of view (mm) | 212 | 206 |
| Matrix | 64 × 64 | 86 × 86 |
| Number of slices | 40 | 60 |
| Number of volumes | 244 | 375 |
| In-plane resolution (mm) | 3.3 × 3.3 | 2.4 × 2.4 |
| Slice thickness (mm) | 3.2 | 2.4 |
| Slice gap (mm) | 0.8 | 0 |
| TR (ms) | 2,500 | 800 |
| TE (ms) | 30 | 34.4 |
| Total scan time (mm:ss) | 10:10 | 5:08 |
| Flip angle (degrees) | 80 | 52 |
| Slice acquisition order | Ascending | Ascending (interleaved) |
| Instructions to participants and other imaging conditions | Please relax. Do not think of anything in particular, do not sleep, but keep looking at the crosshair mark. The lights in the scanning room were dimmed. | Same as for the Training dataset |

MRI, magnetic resonance imaging; StD, subthreshold depression.

Supplementary Table S2. Head motion of subjects with subthreshold depression (StD) and healthy controls in the training and test datasets.

|  |  | Training dataset |  |  | Test dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Healthy controls | StD | p | Healthy controls | StD | p |
| Translation | x | 0.017 ± 0.010 | 0.023 ± 0.018 | 0.054 | 0.007 ± 0.003 | 0.007 ± 0.005 | 0.910 |
| (mm) | y | 0.046 ± 0.015 | 0.038 ± 0.015 | 0.023 | 0.039 ± 0.013 | 0.042 ± 0.016 | 0.413 |
|  | z | 0.062 ± 0.034 | 0.064 ± 0.050 | 0.798 | 0.038 ± 0.015 | 0.051 ± 0.030 | 0.065 |
| Rotation | x | 0.036 ± 0.014 | 0.038 ± 0.025 | 0.565 | 0.025 ± 0.007 | 0.027 ± 0.008 | 0.594 |
| (mm) | y | 0.012 ± 0.007 | 0.015 ± 0.014 | 0.165 | 0.008 ± 0.003 | 0.008 ± 0.002 | 0.797 |
|  | z | 0.011 ± 0.006 | 0.014 ± 0.011 | 0.076 | 0.006 ± 0.002 | 0.006 ± 0.003 | 0.976 |

Note: head radius was assumed to be 50 mm.