Comprehensive assessment of left atrial and ventricular remodeling in paroxysmal atrial fibrillation by cardiovascular magnetic resonance myocardial extracellular volume fraction and feature tracking strain

Akimasa Yamada, Naoki Hashimoto, Hidesato Fujito, Takumi Hatta, Yuki Saito, Naoto Otsuka, Yuji Wakamatsu, Masaru Arai, Ryuta Watanabe, Sayaka Kurokawa, Daisuke Kitano, Koichi Nagashima, Shunichi Yoda,Yasuo Okumura*

Department of Cardiovascular Medicine, Nihon University Graduate School, Tokyo, Japan

*Corresponding author: Yasuo Okumura, MD

Department of Cardiovascular Medicine, Nihon University Graduate School

30-1 Oyaguchi kamimachi, Itabashi, Tokyo 173-8610, Japan

E-mail: okumura.yasuo@nihon-u.ac.jp

Tel: +81-3-3972-8111

Fax: +81-3-3972-1098

Supplementary materials

Detailed CMR sequence parameters.

The standard SSFP cine images were acquired with the typical parameters: slice thickness: 8 mm, slice gap: 1 mm, TR/TE: 3.4/1.7 ms, flip angle 60°, field of view: 250×250 mm2, acquisition matrix size: 156×156, reconstruction matrix size: 320×320 SENSE factor 2, temporal resolution < 40 ms. LGE images were obtained a T1-weighted inversion-recovery gradient echo technique (slice thickness 10 mm, slice gap: -5 mm, TR/TE = 4.6/2.2 ms, flip angle 15°, field of view 360×360 mm2, acquisition matrix size 266×180, reconstruction matrix size: 384×384, SENSE factor 2, inversion time 200 to 300 ms). Pre and post T1 measurements were acquired with the following parameters: slice thickness 10 mm, slice gap: 10 mm, TR/TE = 2.7/1.8 ms, flip angle 35°, field of view 300×300 mm2, acquisition matrix size 142×142, reconstruction matrix size: 256×256, SENSE factor 2, 11 images from three inversions (3+3+5) with three heartbeats pauses prior to the second and third inversions and an adiabatic prepulse.