Supplementary

The potential of emerging bio-based products to reduce environmental impacts

Emma A R Zuiderveen1,2, Koen J J Kuipers1, Carla Caldeira2, Steef Hanssen1, Mitchell van der Hulst1,3, Melinda de Jonge1, Anestis Vlysidis2,4, Rosalie van Zelm1, Serenella Sala2, Mark A J Huijbregts1

1 Department of Environmental Science, Radboud Institute for Biological and Environmental Sciences, Radboud University Nijmegen, P.O. Box 9010, 6500 GL Nijmegen, The Netherlands

2   European Commission, Joint Research Centre, Via Enrico Fermi 2749, 21027, Ispra, VA, Italy

3 Department of Climate, Air and Sustainability, TNO, Princetonlaan 6, 3584CB Utrecht, The Netherlands

4 School of Chemical Engineering, National Technical University of Athens, Iroon Polytechneiou 9, Zografou, 15780 Athens, Greece

Contents

S.1 Arithmetic average RRs per bio-based product with corresponding 95% CI: ordered from largest reduction to highest increase.	3

S.2 Additional calculations in ‘Greenhouse gas Footprints’	5

S.3 Predicted mean RR and 95% CI for the GHG emissions and if it in- or excluded Land Use Changes (LUC) related GHG emissions based on single linear mixed-effect models	5

S.4 Predicted mean RR and 95% CI per feedstock category based on single linear mixed-effect models for eutrophication, acidification, non-renewable energy-use, ozone depletion and photochemical ozone formation impacts.	6

S.5 Predicted mean RR and 95% CI per product category based on single linear mixed-effect models for eutrophication, acidification, non-renewable energy-use, ozone depletion and photochemical ozone formation impacts.	7

S.6 Overview of arithmetic average RRs and corresponding 95% CI for greenhouse gas emissions, eutrophication, acidification, non-renewable energy-use, ozone depletion and photochemical ozone formation impacts 	8

S.7 Overview of the predicted mean and 95% CI of the RRs across all product types and studies for all environmental impacts with n<20. Separate random-effects models were ran for each impact category.	9

S.8 Overview TRLs	10

References	11

Data	12

Table S.9.a,b	12

Table S.10	32

Data References	34

# S.1 Arithmetic average RRs per bio-based product with corresponding 95% CI: ordered from largest reduction to highest increase.

Table S.1: overview of the bio-based products, the number of data points (n), the arithmetic average RR and 95% CI; if n = 1, there is no 95% CI, which is indicated with 0; *indicates the bio-based product with their average and 95% CI below zero.

| Bio-based product | n | Arithmetic average (RR) | Lower end 95% CI | Higher end 95% CI |
| --- | --- | --- | --- | --- |
| WOOD FIBRE/PLA/TPS BIOCOMPOSITE | 1 | -2.75 | 0.00 | 0.00 |
| WOOD FIBRE/PLA BIOCOMPOSITE | 1 | -2.64 | 0.00 | 0.00 |
| SPIRO-BISPHENOL | 1 | -1.87 | 0.00 | 0.00 |
| BOPLA* | 3 | -1.87 | -2.24 | -1.50 |
| 1,3-PROPANEDIOL* | 4 | -1.68 | -2.34 | -1.02 |
| 3-HYDROXY PROPIONIC ACID | 1 | -1.55 | 0.00 | 0.00 |
| BIOLUBRICANT | 1 | -1.54 | 0.00 | 0.00 |
| POLYLIMONENE CARBONATE | 1 | -1.52 | 0.00 | 0.00 |
| CURED WOOD FLOORING COATING | 1 | -1.49 | 0.00 | 0.00 |
| BIOPOLYETHYLENE/BAGASSE FIBERS BIOCOMPOSITE* | 2 | -1.35 | -1.36 | -1.33 |
| DIOL V | 1 | -1.30 | 0.00 | 0.00 |
| POLYLACTIDE | 1 | -1.28 | 0.00 | 0.00 |
| BIOPOLYMER | 1 | -1.23 | 0.00 | 0.00 |
| WAX ESTERS | 1 | -1.19 | 0.00 | 0.00 |
| ADIPIC ACID* | 5 | -1.17 | -1.76 | -0.58 |
| NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC))* | 6 | -1.13 | -1.93 | -0.32 |
| POLYOLS* | 3 | -1.09 | -1.33 | -0.85 |
| FLAX MAT/PLA BIOCOMPOSITE LAMINATES | 1 | -1.09 | 0.00 | 0.00 |
| POLYESTER BINDERS* | 2 | -1.09 | -1.36 | -0.81 |
| FORMIC ACID* | 3 | -1.05 | -1.29 | -0.82 |
| WHEAT-GLUTEN-BASED PACKAGING FILM | 1 | -1.03 | 0.00 | 0.00 |
| CELLULOSE NANOWHISKERS | 2 | -1.01 | -3.15 | 1.13 |
| HEXANOIC ACID* | 7 | -0.97 | -1.47 | -0.47 |
| LACTIC ACID* | 18 | -0.97 | -1.64 | -0.29 |
| FLAX MAT/PLLA/BALSA BIO-SANDWICH PANELS | 1 | -0.94 | 0.00 | 0.00 |
| 3-PROPANEDIOL | 1 | -0.91 | 0.00 | 0.00 |
| POLYITACONIC ACID* | 2 | -0.90 | -1.40 | -0.41 |
| SPIROCYCLIC DIOL | 1 | -0.89 | 0.00 | 0.00 |
| ETHYLENE* | 14 | -0.85 | -1.31 | -0.39 |
| 1,3-BUTADIENE* | 6 | -0.85 | -1.24 | -0.46 |
| LEVOGLUCOSAN | 1 | -0.84 | 0.00 | 0.00 |
| PP | 3 | -0.80 | -2.01 | 0.40 |
| ISOBUTANOL* | 2 | -0.80 | -1.17 | -0.44 |
| CURAUÁ/PP COMPOSITE | 1 | -0.76 | 0.00 | 0.00 |
| PROPYLENE GLYCOL | 3 | -0.75 | -1.71 | 0.21 |
| 2,3-BUTANEDIOL* | 4 | -0.70 | -0.81 | -0.60 |
| PE | 1 | -0.70 | 0.00 | 0.00 |
| MULCH FILM* | 3 | -0.68 | -0.79 | -0.57 |
| 1,4-BUTANEDIOL* | 2 | -0.66 | -0.82 | -0.50 |
| ALKYL POLYGLYCOSIDES | 1 | -0.64 | 0.00 | 0.00 |
| STARCH-FILLED POLYPROPYLENE* | 2 | -0.64 | -0.65 | -0.63 |
| LIGNIN POWDER | 1 | -0.64 | 0.00 | 0.00 |
| PROPIONIC ACID* | 6 | -0.59 | -0.78 | -0.40 |
| SUCCINIC ACID | 20 | -0.55 | -1.10 | 0.00 |
| LACTIDE* | 8 | -0.53 | -0.56 | -0.50 |
| ACETONITRILE | 1 | -0.52 | 0.00 | 0.00 |
| OLEFINS | 1 | -0.52 | 0.00 | 0.00 |
| SODIUM POLYACRYLATE (NA-PA)* | 2 | -0.50 | -0.52 | -0.48 |
| PSA BIOADHESIVE | 1 | -0.47 | 0.00 | 0.00 |
| ETHYL LACTATE* | 2 | -0.46 | -0.83 | -0.09 |
| POLYETHYLENE | 1 | -0.44 | 0.00 | 0.00 |
| PHB | 7 | -0.40 | -0.97 | 0.17 |
| PU FOAMS | 1 | -0.37 | 0.00 | 0.00 |
| PHA* | 11 | -0.36 | -0.69 | -0.04 |
| PLA* | 4 | -0.36 | -0.46 | -0.26 |
| LINSEED OIL/FLAX FIBRE/MMP BIOCOMPOSITE | 1 | -0.33 | 0.00 | 0.00 |
| FDCA | 1 | -0.32 | 0.00 | 0.00 |
| PHB/KENAF COMPOSITE | 1 | -0.30 | 0.00 | 0.00 |
| PHENOLIC | 1 | -0.27 | 0.00 | 0.00 |
| P-XYLENE | 2 | -0.27 | -1.84 | 1.30 |
| HDPE | 6 | -0.23 | -0.48 | 0.01 |
| PAG BIOADHESIVE | 1 | -0.22 | 0.00 | 0.00 |
| MODAL ANTI BACTERIAL FABRIC* | 2 | -0.22 | -0.32 | -0.12 |
| BIOFILM | 1 | -0.19 | 0.00 | 0.00 |
| BUTYLCATECHOL | 1 | -0.19 | 0.00 | 0.00 |
| LLDPE | 2 | -0.18 | -0.95 | 0.58 |
| EPOXIDIZED SUCROSE SOYATE COMPOSITES | 1 | -0.17 | 0.00 | 0.00 |
| LDPE | 2 | -0.17 | -0.90 | 0.56 |
| HEMP SHIV/SOL-GEL COATING MATERIAL | 1 | -0.15 | 0.00 | 0.00 |
| EPOXY RESIN SUPERSAP | 3 | -0.13 | -0.30 | 0.03 |
| 1,3-DIHYDROXYACETONE | 3 | -0.12 | -0.46 | 0.22 |
| SOY BIOADHESIVE | 1 | -0.04 | 0.00 | 0.00 |
| HEXAMETHYLENEDIAMINE | 12 | -0.03 | -0.10 | 0.05 |
| METHANOL | 2 | 0.01 | -1.77 | 1.78 |
| TANNIN BIOADHESIVE | 1 | 0.03 | 0.00 | 0.00 |
| PET | 9 | 0.09 | -0.04 | 0.21 |
| ACETONE | 2 | 0.14 | -0.09 | 0.37 |
| PHLA | 1 | 0.16 | 0.00 | 0.00 |
| PBS | 6 | 0.19 | -0.14 | 0.52 |
| 2-METHYL TETRAHYDROFURAN | 3 | 0.22 | -0.19 | 0.63 |
| POLY(LACTIC ACID)/CELLULOSE NANOCRYSTAL/LIMONENE BIOFILM | 1 | 0.34 | 0.00 | 0.00 |
| NANOFIBRILLATED CELLULOSE (NFC)-REINFORCED EPOXY COMPOSITES | 1 | 0.49 | 0.00 | 0.00 |
| NANOCELLULOSE YARN | 1 | 0.55 | 0.00 | 0.00 |
| BACTERIAL CELLULOSE (BC)-REINFORCED EPOXY COMPOSITES | 1 | 0.81 | 0.00 | 0.00 |
| LIGNIN BIOADHESIVE | 2 | 1.37 | 0.76 | 1.98 |
| CNC FOAM | 3 | 1.44 | 1.05 | 1.83 |

# S.2 Additional calculations in ‘Greenhouse gas Footprints’

Table S.2: additional GHG emission calculations in the section ‘Greenhouse gas Footprints’.

|  | GHG emissions (Gt CO2 eq./yearly) | Percentage of global emissions yearly | Source |
| --- | --- | --- | --- |
| Total GHG emissions globally | 52 | 100% | Ritchie, H. and M. Roser. 2020. CO₂ and Greenhouse Gas Emissions. Our World in Data. https://ourworldindata.org/co2-and-other-greenhouse-gas-emissions. Accessed January 20, 2022. |
| Total GHG emissions Plastics (fossil-based) | 1.8 | 3% | Zheng, J. & Suh, S. Strategies to reduce the global carbon footprint of plastics. Nature Climate Change vol. 9 374–378 (2019) |
| Total GHG emissions Primary Industry (based on the six main platform chemicals) | 0.91 | 2% | Galán-Martín, Á. et al. Sustainability footprints of a renewable carbon transition for the petrochemical sector within planetary boundaries. One Earth 4, 565–583 (2021) |
|  | Ethylene | Butadiene | Plastics |
| Total amount produced yearly (kg) | 2.01E11 | 1.2E10 | - |
| GHG emissions fossil-based (kg CO2 eq. /kg chemical) | 1.45 | 1.2 | - |
| Bio-based potential reduction | 57% (95% CI: 32, 73%) | 57% (95% CI: 37, 71%) | 44% (95% CI: -28, 55%) |
| % of GHG emissions primary industry / global | 18.3% / 0.3% | 0.9% / 0.02% | - / 1.52% |
| Calculation | (RPBBP∙GHGfossil)∙ TproductGHGtotal ∙100 with RPBBP as biobased product reduction potential; GHGfossil as GHG emissions fossilbased product; Tproduct as total amount produced (kg/year); GHGtotal as total GHG emissions (Primary Industry/Global) |  |  |
| Sources | Reduction potentials taken from S.1 and Figure 2a; Levi, P.G. and J.M. Cullen. 2018. Mapping Global Flows of Chemicals: From Fossil Fuel Feedstocks to Chemical Products. Environmental Science and Technology 52(4); Ecoinvent Centre. Ecoinvent database (Version 3.7). https://v34.ecoquery.ecoinvent.org/ (2020) |  |  |

# S.3 Predicted mean RR and 95% CI for the GHG emissions and if it in- or excluded Land Use Changes (LUC) related GHG emissions based on single linear mixed-effect models.

Figure S.3: LMM on relationship RR GHG emissions and studies either in- or excluding LUC. Average estimate including LUC is -43% (95% CI: 6, 66%) and excluding LUC is -47% (95% CI: 38, 55%). Omnibus F:0.06, p-value: 0.79.

# S.4 Predicted mean RR and 95% CI per feedstock category based on single linear mixed-effect models for eutrophication, acidification, non-renewable energy-use, ozone depletion and photochemical ozone formation impacts.

Figure S.4: LMM on relationship RR environmental footprints and feedstock category: (a) Eutrophication: omnibus F: 0.24, p-value: 0.94; (b) Acidification: omnibus F: 0.69, p-value: 0.63; (c) NREU: omnibus F: 1.18, p-value: 0.34; (d) Ozone Depletion: omnibus F: 0.90, p-value: 0.48; (e) Photochemical Ozone Formation: omnibus F: 1.02, p-value: 0.41.

# S.5 Predicted mean RR and 95% CI per product category based on single linear mixed-effect models for eutrophication, acidification, non-renewable energy-use, ozone depletion and photochemical ozone formation impacts.

Figure S.5: LMM on relationship RR environmental footprints and product category: (a) Eutrophication: omnibus F: 0.35, p-value: 0.84; (b) Acidification: omnibus F: 11.86, p-value: 1.56E-02*; (c) NREU: omnibus F: 2.82, p-value: 0.097; (d) Ozone Depletion: omnibus F: 1.35, p-value: 0.29; (e) Photochemical Ozone Formation: omnibus F: 2.95, p-value: 0.03*.

# S.6 Overview of arithmetic average RRs and corresponding 95% CI for greenhouse gas emissions, eutrophication, acidification, non-renewable energy-use, ozone depletion and photochemical ozone formation impacts

Figure S.6: Plot showing overall arithmetic averages and 95% CI of GHG, Eutrophication, Acidification, NREU, Ozone Depletion and Photochemical Ozone Formation impacts. In percentages, on average the GHG footprint is reduced with 43% (95% CI: 37, 49%), eutrophication is increased with 256% (95% CI: 125, 468%), acidification is increased with 38% (95% CI: -15, 124%), NREU is reduced with 30% (95% CI: 13, 43%), ozone depletion is reduced with 27% (95% CI: -63, 41%) and photochemical ozone formation is increased with 2% (95% CI: -48, 103%).

# S.7 Overview of the predicted mean and 95% CI of the RRs across all product types and studies for all environmental impacts with n<20. Separate random-effects models were ran for each impact category.

Figure S.7: plot showing predicted mean and 95% CI of the other environmental impacts’ RRs (with n < 20) collected from the 112 studies (Human Ecotoxicity, Freshwater Ecotoxicity, Marine Aquatic Ecotoxicity, Land Use, Water Scarcity, Depletion of Abiotic Resources).

# S.8 Overview TRLs

Only in a few cases the TRL was addressed by the study itself. The TRL evaluation was therefore based on the TRL specification from Moni et al. (2020) and afterwards regrouped into broader category classes (TRL 1-3, TRL 4-5, TRL 6-9).

Figure S.8: Description of TRLs, based on Moni et al. (2020)

# References

Ecoinvent Centre. Ecoinvent database (Version 3.7). https://v34.ecoquery.ecoinvent.org/ (2020).

Zheng, J. & Suh, S. Strategies to reduce the global carbon footprint of plastics. Nature Climate Change vol. 9 374–378 (2019)

Galán-Martín, Á. et al. Sustainability footprints of a renewable carbon transition for the petrochemical sector within planetary boundaries. One Earth 4, 565–583 (2021)

Levi, P.G. and J.M. Cullen. 2018. Mapping Global Flows of Chemicals: From Fossil Fuel Feedstocks to Chemical    Products. Environmental Science and Technology 52(4)

Ritchie, H. and M. Roser. 2020. CO₂ and Greenhouse Gas Emissions. Our World in Data. https://ourworldindata.org/co2-and other-greenhouse-gas-emissions. Accessed January 20, 2022.

# Data

Table S.9.a: List of data sources used in analysis and main parameters, including study number; bio-based product; product category; emerging technology category (BC = biological (biochemical) conversion, M = mechanical conversion or TC = thermochemical conversion); specific feedstock; feedstock category; LUC impact (taken into account in GHG footprint); fossil-based product; the GHG emissions of both the bio-based product and fossil-based product and subsequently, its response ratio; biogenic carbon credit (if given in the cradle-to-gate studies, which is thus added again to the GHG emissions of the bio-based product); fossil incineration end-of-life emissions based on chemical structure, which was calculated for each of the fossil products; original TRL of the technology researched in the study and to which TRL it is up scaled; the RRs of acidification, eutrophication and NREU. 1,2,11,101–110,12,111,112,13–20,3,21–30,4,31–40,5,41–50,6,51–60,7,61–70,8,71–80,9,81–90,10,91–100

| Study number | Bio-based product | Product category | Category Emerging Technology | Feedstock | Feedstock Category | Land Use Change impact | Fossil-based product | GHG emissions bio-based product (kg CO2 eq./kg product), cradle-to-grave | GHG emissions fossil-based counter product (kg CO2 eq./kg product), cradle-to-grave | Response Ratio (RR) | Biogenic credit (kg CO2/ kg product) | Fossil end-of-life emissions based on chemical structure (kg CO2/kg product) | TRL grouped | trl_upscaled | Acidification RR | Eutrophication RR | NREU RR | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3-PROPANEDIOL | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | 1,3-propanediol | 3.2 | 7.988 | -0.9147896 | 0 | 1.488 | 4 and 5 | 9 | -0.457425 | 0.169076 |  | Urban_&_Bakshi |
| 2 | ADIPIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | adipic acid | 4 | 27.41 | -1.9246135 | 0 | 1.81 | 1 to 3 | 9 | 0.693147 | -0.659246 | -0.980829 | Aryapratama_&_Janssen |
| 3 | 2,3-BUTANEDIOL | Biochemical | BC | Corn stover | Agricultural & Forestry residues | No | 2,3-butanediol | 2.3 | 4.1 | -0.5780779 | 0 | 0 | 4 and 5 | 9 |  |  | -1.15268 | Benalcazar_et_al |
| 3 | 2,3-BUTANEDIOL | Biochemical | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | 2,3-butanediol | 1.95 | 4.1 | -0.7431576 | 0 | 0 | 4 and 5 | 9 |  |  | -1.25804 | Benalcazar_et_al |
| 3 | 2,3-BUTANEDIOL | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | 2,3-butanediol | 2.1 | 4.1 | -0.6690496 | 0 | 0 | 4 and 5 | 9 |  |  | -1.315198 | Benalcazar_et_al |
| 3 | 2,3-BUTANEDIOL | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | 2,3-butanediol | 1.8 | 4.1 | -0.8232003 | 0 | 0 | 4 and 5 | 9 |  |  | -1.375823 | Benalcazar_et_al |
| 3 | HEXANOIC ACID | Biochemical | BC | Corn stover | Agricultural & Forestry residues | No | adipic acid | 3 | 11.5 | -1.3437347 | 0 | 0 | 4 and 5 | 9 |  |  | 1.120591 | Benalcazar_et_al |
| 3 | HEXANOIC ACID | Biochemical | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | adipic acid | 2.5 | 11.5 | -1.5260563 | 0 | 0 | 4 and 5 | 9 |  |  | 0.980829 | Benalcazar_et_al |
| 3 | HEXANOIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | adipic acid | 3 | 11.5 | -1.3437347 | 0 | 0 | 4 and 5 | 9 |  |  | 1.05315 | Benalcazar_et_al |
| 3 | HEXANOIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | adipic acid | 2.05 | 11.5 | -1.7245072 | 0 | 0 | 4 and 5 | 9 |  |  | 1.163151 | Benalcazar_et_al |
| 4 | PROPIONIC ACID | Biochemical | BC | Potato Pulp & Rapeseed | Industrial Side Stream | No | propionic acid | 1.3 | 3.45 | -0.97601 | 0 | 0 | 1 to 3 | 9 |  | 2.590267 |  | Ekman_&_Börjesson |
| 5 | POLYETHYLENE | Biochemical | BC | Sugarcane | Pure feedstock (1st generation) | Yes, iLUC | polyethylene | 3.9 | 6.04 | -0.4374275 | 0 | 3.14 | 6 to 9 | 9 | -2.302585 | 0.559616 |  | Liptow_&_Tillman |
| 6 | ETHYLENE | Biochemical | BC | Sawmill residues | Agricultural & Forestry residues | No | ethylene | 8.63712 | 4.59 | 0.6321892 | 3.14 | 3.14 | 6 to 9 | 9 | 2.508513 | 9.881557 |  | Liptow_et_al |
| 7 | PROPIONIC ACID | Biochemical | BC | Glycerol from rapeseed oil | Industrial Side Stream | Yes, dLUC | propionic acid | 3.302 | 6.136 | -0.6196447 | 0 | 1.78 | 4 and 5 | 9 |  |  |  | Tufvesson_et_al |
| 7 | PROPIONIC ACID | Biochemical | BC | Sugarbeet | Pure feedstock (1st generation) | Yes, dLUC | propionic acid | 3.654 | 6.136 | -0.5183506 | 0 | 1.78 | 4 and 5 | 9 |  |  |  | Tufvesson_et_al |
| 7 | PROPIONIC ACID | Biochemical | BC | Molasses | Agricultural & Forestry residues | Yes, dLUC | propionic acid | 3.093 | 6.136 | -0.6850316 | 0 | 1.78 | 4 and 5 | 9 |  |  |  | Tufvesson_et_al |
| 7 | PROPIONIC ACID | Biochemical | BC | J Artichoke | Pure feedstock (2nd generation) | No, pure feedstock | propionic acid | 3.809 | 6.136 | -0.4768064 | 0 | 1.78 | 4 and 5 | 9 |  |  |  | Tufvesson_et_al |
| 8 | OLEFINS | Biochemical | TC | Forest residues | Agricultural & Forestry residues | No | aromatics & olefins | 8.86 | 14.83 | -0.5151054 | 8.33 | 8.33 | 4 and 5 | 9 |  |  |  | Zhang_et_al |
| 9 | FORMIC ACID | Biorefinery | TC | Brassica carinata | Pure feedstock (2nd generation) | No, but only marginal land is considered | formic acid | 1.2041317 | 4.25 | -1.2611602 | 0.96 | 0.96 | 6 to 9 | 9 | -2.197225 | -0.635045 |  | Fiorentino_et_al |
| 10 | SUCCINIC ACID | Biochemical | BC | Corn stover | Agricultural & Forestry residues | No | succinic acid | 1.9 | 21 | -2.4026686 | 0 | 0 | 4 and 5 | 9 |  |  | -1.976063 | Adom_et_al |
| 10 | 1,4-BUTANEDIOL | Biochemical | TC | Corn stover | Agricultural & Forestry residues | No | 1,4-butanediol | 3 | 6.3 | -0.7419373 | 0 | 0 | 4 and 5 | 9 |  |  | -0.620827 | Adom_et_al |
| 10 | ISOBUTANOL | Biochemical | BC | Corn stover | Agricultural & Forestry residues | No | isobutanol | 2.7 | 5 | -0.6161861 | 0 | 0 | 4 and 5 | 9 |  |  | -0.300105 | Adom_et_al |
| 11 | SUCCINIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | succinic acid | 2.38 | 3.46 | -0.3741681 | 1.5 | 1.5 | 6 to 9 | 7 |  |  | -0.602763 | Cok_et_al |
| 11 | SUCCINIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | succinic acid | 3.2 | 3.46 | -0.0781178 | 1.5 | 1.5 | 6 to 9 | 7 |  |  | -0.24578 | Cok_et_al |
| 12 | 2-METHYL TETRAHYDROFURAN | Biochemical | TC | Corn stover | Agricultural & Forestry residues | No | THF | 8.18 | 8.55 | -0.0442391 | 2.56 | 2.56 | 1 to 3 | 3 | 0.581652 | 3.244194 |  | Khoo_et_al |
| 12 | 2-METHYL TETRAHYDROFURAN | Biochemical | TC | Sugarcane bagasse | Agricultural & Forestry residues | No | THF | 9.2 | 8.55 | 0.0732722 | 2.56 | 2.56 | 1 to 3 | 3 | 0.053584 | 2.684578 |  | Khoo_et_al |
| 12 | 2-METHYL TETRAHYDROFURAN | Biochemical | TC | Rice straw | Agricultural & Forestry residues | No | THF | 16.13 | 8.55 | 0.6347496 | 2.56 | 2.56 | 1 to 3 | 3 | 0.631662 | 3.244194 |  | Khoo_et_al |
| 13 | PROPYLENE GLYCOL | Biochemical | TC | soybean oil | Industrial Side Stream | No | propylene glycol | 2 | 6.74 | -1.2149127 | 0 | 1.74 | 4 and 5 | 9 | -0.875469 | -1.241713 |  | Gonzalez-Garay_et_al |
| 13 | PROPYLENE GLYCOL | Biochemical | TC | soybean oil | Industrial Side Stream | No | propylene glycol | 1.9 | 6.74 | -1.266206 | 0 | 1.74 | 4 and 5 | 9 | -0.875469 | -1.261131 |  | Gonzalez-Garay_et_al |
| 13 | PROPYLENE GLYCOL | Biochemical | TC | soybean oil | Industrial Side Stream | No | propylene glycol | 8.5 | 6.74 | 0.2320062 | 0 | 1.74 | 4 and 5 | 9 | 0 | -0.156004 |  | Gonzalez-Garay_et_al |
| 14 | SUCCINIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | succinic acid | 2 | 13.49 | -1.9088015 | 0 | 1.49 | 1 to 3 | 5 |  |  |  | Gunukula_et_al |
| 14 | ADIPIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | adipic acid | 2 | 10.81 | -1.6873245 | 0 | 1.81 | 1 to 3 | 5 |  |  |  | Gunukula_et_al |
| 14 | ISOBUTANOL | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | isobutanol | 2 | 5.38 | -0.9895412 | 0 | 2.38 | 1 to 3 | 5 |  |  |  | Gunukula_et_al |
| 14 | 1,3-PROPANEDIOL | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | 1,3-propanediol | 2 | 13.49 | -1.9088015 | 0 | 1.49 | 1 to 3 | 5 |  |  |  | Gunukula_et_al |
| 14 | 3-HYDROXY PROPIONIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | 3-hydroxy propionic acid | 1.8 | 8.47 | -1.5487438 | 0 | 1.47 | 1 to 3 | 5 |  |  |  | Gunukula_et_al |
| 15 | LACTIC ACID | Biorefinery | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | lactic acid | 3.8 | 5.77 | -0.417671 | 1.47 | 1.47 | 4 and 5 | 9 | -1.571028 | -1.246009 |  | Mandegari_et_al |
| 15 | LACTIC ACID | Biorefinery | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | lactic acid | 0.0861 | 5.77 | -4.2049179 | 1.47 | 1.47 | 4 and 5 | 9 | -10.3278 | -5.107 |  | Mandegari_et_al |
| 15 | LACTIC ACID | Biorefinery | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | lactic acid | 0.0752 | 5.77 | -4.3402761 | 1.47 | 1.47 | 4 and 5 | 9 | -5.713468 | -5.222919 |  | Mandegari_et_al |
| 16 | SUCCINIC ACID | Biochemical | BC | Giant reed | Pure feedstock (2nd generation) | No, but only marginal land is considered | succinic acid | 3.86 | 6.74 | -0.5573927 | 1.91 | 1.49 | 1 to 3 | 9 |  |  |  | Zucaro_et_al |
| 17 | HEXANOIC ACID | Biochemical | TC | Organic fraction of municipal solid waste (OFMSW) | Municipal waste | No | adipic acid | 14.9 | 15.51 | -0.0401238 | 0 | 1.81 | 1 to 3 | 7 | 1.218898 | 1.171183 |  | Chen_et_al |
| 17 | HEXANOIC ACID | Biochemical | TC | Supermarket food waste (SFW) | Food Processing waste | No | adipic acid | 12.34 | 15.51 | -0.228639 | 0 | 1.81 | 1 to 3 | 7 | 0.967584 | 0.948039 |  | Chen_et_al |
| 17 | HEXANOIC ACID | Biochemical | TC | Supermarket food waste (SFW) | Food Processing waste | No | adipic acid | 8.66 | 15.51 | -0.5827703 | 0 | 1.81 | 6 to 9 | 7 | 0.407968 | 0.660357 |  | Chen_et_al |
| 18 | ADIPIC ACID | Biochemical | TC | Cotton straw | Agricultural & Forestry residues | No | adipic acid | 6.68 | 9.66 | -0.3688757 | 1.81 | 1.81 | 4 and 5 | 9 |  |  |  | Zheng_et_al |
| 19 | ACETONITRILE | Biochemical | TC | Corn | Pure feedstock (1st generation) | No, pure feedstock | acetonitrile | 3.55 | 5.95 | -0.5164436 | 2.15 | 2.15 | 4 and 5 | 9 | 0.805625 |  | -0.391933 | Tripodi_et_al |
| 20 | ETHYLENE | Biochemical | TC | Corn stover | Agricultural & Forestry residues | No | ethylene | 2.1 | 4.5 | -0.7621401 | 3.1 | 3.1 | 4 and 5 | 9 |  |  |  | Yang_et_al |
| 20 | ETHYLENE | Biochemical | TC | Corn | Pure feedstock (1st generation) | No, pure feedstock | ethylene | 2.6 | 4.5 | -0.548566 | 3.1 | 3.1 | 4 and 5 | 9 |  |  |  | Yang_et_al |
| 21 | ETHYLENE | Biochemical | TC | Popular | Pure feedstock (2nd generation) | No, pure feedstock | ethylene | 0.48 | 4.5 | -2.2380466 | 0 | 3.1 | 6 to 9 | 9 | -0.026668 | 2.564949 |  | Alonso- Farinas_et_al |
| 21 | ETHYLENE | Biochemical | TC | Popular | Pure feedstock (2nd generation) | No, pure feedstock | ethylene | 0.79 | 4.5 | -1.7397997 | 0 | 3.1 | 4 and 5 | 9 | -0.054067 | 2.397895 |  | Alonso- Farinas_et_al |
| 21 | ETHYLENE | Biochemical | TC | Popular | Pure feedstock (2nd generation) | No, pure feedstock | ethylene | 1.3 | 4.5 | -1.2417131 | 0 | 3.1 | 4 and 5 | 9 | 0.332706 | 2.961831 |  | Alonso- Farinas_et_al |
| 22 | SUCCINIC ACID | Biochemical | BC | Food waste | Municipal waste | No | succinic acid | 0.667 | 4.57 | -1.9244784 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Brunklause_et_al |
| 22 | SUCCINIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | succinic acid | 2.34 | 4.57 | -0.6693623 | 0 | 1.49 | 6 to 9 | 9 |  |  |  | Brunklause_et_al |
| 23 | SUCCINIC ACID | Biochemical | BC | Apple pomace | Food Processing waste | No | succinic acid | 5.3 | 4.57 | 0.1481936 | 0 | 1.49 | 4 and 5 | 9 | 4.341753 | 5.131996 | 1.221452 | González-Garciá_et_al |
| 24 | ETHYLENE | Biochemical | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | ethylene | 0.55 | 3.4 | -1.8216124 | 3.09 | 3.14 | 6 to 9 | 9 |  |  |  | Jonker_et_al |
| 24 | ETHYLENE | Biochemical | BC | Woody biomass | Pure feedstock (2nd generation) | No, pure feedstock | ethylene | 0.35 | 3.4 | -2.2735976 | 3.09 | 3.14 | 4 and 5 | 9 |  |  |  | Jonker_et_al |
| 24 | 1,3-PROPANEDIOL | Biochemical | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | 1,3-propanediol | 0.9 | 6.474 | -1.9731547 | 1.736 | 1.49 | 4 and 5 | 9 |  |  |  | Jonker_et_al |
| 24 | 1,3-PROPANEDIOL | Biochemical | BC | Woody biomass | Pure feedstock (2nd generation) | No, pure feedstock | 1,3-propanediol | 0.75 | 6.474 | -2.1554762 | 1.736 | 1.49 | 4 and 5 | 9 |  |  |  | Jonker_et_al |
| 24 | SUCCINIC ACID | Biochemical | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | succinic acid | 0.95 | 6.01 | -1.844718 | 1.49 | 1.49 | 4 and 5 | 9 |  |  |  | Jonker_et_al |
| 24 | SUCCINIC ACID | Biochemical | BC | Woody biomass | Pure feedstock (2nd generation) | No, pure feedstock | succinic acid | 0.9 | 6.01 | -1.8987853 | 1.49 | 1.49 | 4 and 5 | 9 |  |  |  | Jonker_et_al |
| 25 | FDCA | Biochemical | TC | Woody biomass | Agricultural & Forestry residues | No | TPA | 2.52 | 3.48 | -0.3227734 | 0 | 1.69 | 4 and 5 | 9 |  | 0.626611 |  | Kim_et_al_1 |
| 26 | LEVOGLUCOSAN | Biochemical | TC | Cotton straw | Agricultural & Forestry residues | No | levoglucosan | 4.57 | 10.55 | -0.8366127 | 0 | 1.639 | 4 and 5 | 9 |  |  |  | Wang_et_al |
| 27 | BOPLA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | BoPP | 0.7992832 | 3.5644754 | -1.4950569 | 0 | 1.83 | 1 to 3 | 9 |  | 0.217921 |  | Lokesh_et_al |
| 27 | BOPLA | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | BoPP | 0.4301075 | 3.5644754 | -2.1147369 | 0 | 1.83 | 1 to 3 | 9 |  | 0.067587 |  | Lokesh_et_al |
| 27 | BOPLA | Bioplast/biopolymer | BC | Sugarbeet pulp | Industrial Side Stream | No | BoPP | 0.483871 | 3.5644754 | -1.9969539 | 0 | 1.83 | 1 to 3 | 9 |  | 0.373436 |  | Lokesh_et_al |
| 27 | MULCH FILM | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | LLDPE mulch film | 2.3185526 | 4.1027027 | -0.5707028 | 0 | 1 | 1 to 3 | 9 |  | 0.062944 |  | Lokesh_et_al |
| 27 | MULCH FILM | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | LLDPE mulch film | 1.9241447 | 4.1027027 | -0.7571644 | 0 | 1 | 1 to 3 | 9 |  | -0.109899 |  | Lokesh_et_al |
| 27 | MULCH FILM | Bioplast/biopolymer | BC | Sugarbeet pulp | Industrial Side Stream | No | LLDPE mulch film | 2 | 4.1027027 | -0.7184988 | 0 | 1 | 1 to 3 | 9 |  | -0.148365 |  | Lokesh_et_al |
| 28 | ADIPIC ACID | Biochemical | TC | Lignin | Agricultural & Forestry residues | No | adipic acid | 13.794835 | 24.8196 | -0.5873394 | 0 | 1.81 | 4 and 5 | 9 |  |  |  | Van_Duuren_et_al |
| 29 | LACTIC ACID | Biochemical | BC | Corn | Pure feedstock (1st generation) | Yes, iLUC | lactic acid | 4.19 | 5.5 | -0.2720474 | 2.55 | 1.47 | 6 to 9 | 9 | 1.858899 | -0.997516 |  | Ögmundarson_et_al |
| 29 | LACTIC ACID | Biochemical | BC | Corn stover | Agricultural & Forestry residues | No | lactic acid | 7.9 | 5.5 | 0.3621147 | 2.91 | 1.47 | 4 and 5 | 9 | 1.673976 | 3.211758 |  | Ögmundarson_et_al |
| 30 | PAG BIOADHESIVE | Bioadhesive | TC | Glycerol from soybean | Industrial Side Stream | No | bioadhesive | 12 | 15 | -0.2231436 | 0 | 0 | 1 to 3 | 9 |  |  |  | Yang_and_Rosentrater |
| 31 | PSA BIOADHESIVE | Bioadhesive | TC | Glycerol from soybean | Industrial Side Stream | No | bioadhesive | 3.8 | 6.1 | -0.4732877 | 0 | 0 | 4 and 5 | 9 |  |  |  | Yang_and_Rosentrater_1 |
| 32 | SOY BIOADHESIVE | Bioadhesive | TC | soybean meal | Food Processing waste | No | bioadhesive | 2.79 | 2.895 | -0.0369435 | 0 | 0 | 4 and 5 | 9 |  |  |  | Arias_et_al |
| 32 | LIGNIN BIOADHESIVE | Bioadhesive | TC | Kraft lignin | Industrial Side Stream | No | bioadhesive | 8.34 | 2.895 | 1.0580781 | 0 | 0 | 4 and 5 | 9 |  |  |  | Arias_et_al |
| 32 | LIGNIN BIOADHESIVE | Bioadhesive | TC | Organosolv lignin | Industrial Side Stream | No | bioadhesive | 15.54 | 2.895 | 1.6804322 | 0 | 0 | 4 and 5 | 9 |  |  |  | Arias_et_al |
| 32 | TANNIN BIOADHESIVE | Bioadhesive | TC | Bark chips | Industrial Side Stream | No | bioadhesive | 2.97 | 2.895 | 0.0255768 | 0 | 0 | 4 and 5 | 9 |  |  |  | Arias_et_al |
| 33 | FLAX MAT/PLA BIOCOMPOSITE LAMINATES | Biocomposite | M | Flax fiber | Pure feedstock (1st generation) | No, pure feedstock | glass/polyester composites | 3.3828714 | 10.053583 | -1.0892043 | 0.47025 | 0.47025 | 1 to 3 | 9 |  |  | -0.954798 | Le_Duigou_et_al |
| 33 | FLAX MAT/PLLA/BALSA BIO-SANDWICH PANELS | Biocomposite | M | Flax fiber, balsa | Pure feedstock (1st generation) | No, pure feedstock | glass/polyester/balsa sandwich | 2.6399703 | 6.7380563 | -0.9370038 | 0.47025 | 0.47025 | 1 to 3 | 9 |  |  | -0.849228 | Le_Duigou_et_al |
| 34 | WOOD FIBRE/PLA BIOCOMPOSITE | Biocomposite | M | Maize, wood fibre | Pure feedstock (1st generation) | No, pure feedstock | PP | 0.227017 | 3.18 | -2.6396116 | 0.637017 | 1.73 | 1 to 3 | 9 |  |  | -1.527123 | Mahalle_et_al |
| 34 | WOOD FIBRE/PLA/TPS BIOCOMPOSITE | Biocomposite | M | Maize, wood fibre | Pure feedstock (1st generation) | No, pure feedstock | PP | 0.203114 | 3.18 | -2.7508691 | 0.613114 | 1.73 | 1 to 3 | 9 |  |  | -1.218333 | Mahalle_et_al |
| 35 | LINSEED OIL/FLAX FIBRE/MMP BIOCOMPOSITE | Biocomposite | M | Flax fibres and linseed oil | Pure feedstock (1st generation) | No, pure feedstock | Epoxy resin reinforced by woven glass fibre fabric and flame retardant TBBPA | 7.9843478 | 11.141935 | -0.3332329 | 3.69 | 2.64 | 1 to 3 | 9 | -0.394654 | 0.521637 |  | Deng_et_al |
| 36 | EPOXY RESIN SUPERSAP | Biocomposite | M | Flax | Pure feedstock (1st generation) | No, pure feedstock | Gypsum plasterboard | 2.2612333 | 2.9678029 | -0.2719115 | 0 | 0 | 1 to 3 | 9 |  |  |  | Quintana_et_al |
| 36 | EPOXY RESIN SUPERSAP | Biocomposite | M | Hemp | Pure feedstock (1st generation) | No, pure feedstock | Gypsum plasterboard | 3.0118402 | 2.9678029 | 0.0147293 | 0 | 0 | 1 to 3 | 9 |  |  |  | Quintana_et_al |
| 36 | EPOXY RESIN SUPERSAP | Biocomposite | M | Jute | Pure feedstock (1st generation) | No, pure feedstock | Gypsum plasterboard | 2.5695227 | 2.9678029 | -0.1441018 | 0 | 0 | 1 to 3 | 9 |  |  |  | Quintana_et_al |
| 37 | BIOPOLYETHYLENE/BAGASSE FIBERS BIOCOMPOSITE | Biocomposite | TC | Sugarcane bagasse | Agricultural & Forestry residues | Yes, dLUC | PE | 1.35 | 5.15 | -1.3388921 | 3.14 | 3.14 | 1 to 3 | 9 | 1.293367 | 2.694821 |  | Ita-Nagy_et_al |
| 37 | BIOPOLYETHYLENE/BAGASSE FIBERS BIOCOMPOSITE | Biocomposite | TC | Sugarcane bagasse | Agricultural & Forestry residues | Yes, dLUC | PE | 1.33 | 5.15 | -1.3538178 | 3.14 | 3.14 | 1 to 3 | 9 | 1.256029 | 2.660335 |  | Ita-Nagy_et_al |
| 38 | PHA | Bioplast/biopolymer | BC | Soybean | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 3.52 | 3.75 | -0.0632949 | 3.26 | 2.05 | 4 and 5 | 9 |  |  |  | Akiyama_et_al |
| 38 | PHA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 3.22 | 3.75 | -0.1523745 | 2.77 | 2.05 | 4 and 5 | 9 |  |  |  | Akiyama_et_al |
| 39 | PHA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | Polystyrene | 5.5 | 4.95 | 0.1053605 | 2 | 2.05 | 1 to 3 | 9 |  |  |  | Kim_and_Dale |
| 39 | PHA | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | Polystyrene | 1.3 | 4.95 | -1.3370233 | 2 | 2.05 | 1 to 3 | 9 |  |  |  | Kim_and_Dale |
| 40 | PHB | Bioplast/biopolymer | BC | corn starch | Pure feedstock (1st generation) | No, pure feedstock | LDPE | 4.69 | 5.03 | -0.0699874 | 1.69 | 3.13 | 4 and 5 | 9 |  | 1.504077 | -0.756863 | Broeren_et_al |
| 40 | PHB | Bioplast/biopolymer | BC | Wastewater | Municipal waste | No | LDPE | 3.19 | 5.03 | -0.4553991 | 1.69 | 3.13 | 4 and 5 | 9 |  | 1.252763 | -0.783531 | Broeren_et_al |
| 41 | NANOCELLULOSE YARN | Biofiber | BC | Carrot Pomace | Food Processing waste | No | Carbon fiber | 13.3 | 7.69 | 0.5478433 | 0 | 3.66 | 1 to 3 | 9 |  |  |  | Piccinno_et_al |
| 42 | CNC FOAM | Biofiber | TC | Wood pulp | Industrial Side Stream | No | Carbon fiber | 42.5 | 7.69 | 1.7095833 | 1.49 | 3.66 | 1 to 3 | 5 |  |  |  | Tan_et_al |
| 42 | CNC FOAM | Biofiber | TC | Wood pulp | Industrial Side Stream | No | Carbon fiber | 36.49 | 7.69 | 1.5571175 | 1.49 | 3.66 | 4 and 5 | 7 |  |  |  | Tan_et_al |
| 42 | CNC FOAM | Biofiber | TC | Wood pulp | Industrial Side Stream | No | Carbon fiber | 21.99 | 7.69 | 1.050667 | 1.49 | 3.66 | 6 to 9 | 9 |  |  |  | Tan_et_al |
| 43 | BIOLUBRICANT | Bioadhesive | TC | Rapeseed | Pure feedstock (1st generation) | No, but only marginal land is considered | Mineral oil-based base fluid (TMP oleate) | 1.426 | 6.632 | -1.5370331 | 0 | 2.54 | 1 to 3 | 9 | 0.743829 | 3.088974 |  | Ekman_&_Borjesson |
| 44 | ALKYL POLYGLYCOSIDES | Bioadhesive | TC | Wheat straw | Agricultural & Forestry residues | No | APG (via Fischer glycosidation) | 1.870856 | 3.55956 | -0.6432409 | 0 | 1.31882 | 1 to 3 | 9 | -0.348342 | -0.310812 |  | Brière_et_al |
| 45 | CURED WOOD FLOORING COATING | Bioadhesive | TC | Corn and Soy | Pure feedstock (1st generation) | Yes, dLUC | low-VOC UV- cured wood flooring coating | 2.64 | 11.7 | -1.4888099 | 0 | 0 | 1 to 3 | 9 | -0.297738 | 0.020203 |  | Montazeri_et_al |
| 46 | LACTIC ACID | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | lactic acid | 2.1 | 3.26 | -0.4397899 | 1.45 | 1.46 | 4 and 5 | 9 |  |  |  | Adom_&_Dunn |
| 46 | ETHYL LACTATE | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | ethyl lactate | 2.4 | 4.57 | -0.6440445 | 1.87 | 1.87 | 4 and 5 | 9 |  |  |  | Adom_&_Dunn |
| 46 | LACTIC ACID | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | lactic acid | 2.9 | 3.26 | -0.1170165 | 1.45 | 1.46 | 4 and 5 | 9 |  |  |  | Adom_&_Dunn |
| 46 | ETHYL LACTATE | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | ethyl lactate | 3.5 | 4.57 | -0.2667502 | 1.87 | 1.87 | 4 and 5 | 9 |  |  |  | Adom_&_Dunn |
| 47 | POLYESTER BINDERS | Bioplast/biopolymer | TC | Corn | Pure feedstock (1st generation) | No, pure feedstock | Polyester binders | 3.1 | 10.56 | -1.2256712 | 0 | 2.46 | 6 to 9 | 9 |  |  |  | Garcia_Gonzalez_et_al |
| 48 | PE | Bioplast/biopolymer | BC | Sugarcane molasses | Agricultural & Forestry residues | No | PE | 2.3 | 4.63 | -0.6996477 | 0 | 3.14 | 6 to 9 | 9 |  |  |  | Kikuchi_et_al |
| 48 | PP | Bioplast/biopolymer | BC | Sugarcane molasses | Agricultural & Forestry residues | No | PP | 4.21 | 4.66 | -0.1015528 | 0 | 3.13 | 6 to 9 | 9 |  |  |  | Kikuchi_et_al |
| 48 | PP | Bioplast/biopolymer | TC | Woody biomass | Agricultural & Forestry residues | No | PP | 3.5 | 4.66 | -0.2862525 | 0 | 3.13 | 6 to 9 | 9 |  |  |  | Kikuchi_et_al |
| 49 | BIOFILM | Bioplast/biopolymer | M | Cassava starch | Pure feedstock (1st generation) | No, pure feedstock | LDPE | 4.3 | 5.2 | -0.1900436 | 0 | 3.13 | 1 to 3 | 9 | 1.261471 | 7.205909 |  | de_Leis_et_al |
| 50 | POLY(LACTIC ACID)/CELLULOSE NANOCRYSTAL/LIMONENE BIOFILM | Bioplast/biopolymer | TC | Citrus waste | Agricultural & Forestry residues | No | PLA/3OMMT/16.5ATBC (PLA/organomodified montmorillonite reference film plasticized with ATBC) | 5.3 | 3.79 | 0.3353408 | 0 | 0 | 1 to 3 | 9 | 0.300105 | -0.011542 |  | Petrucci_et_al |
| 51 | POLYLIMONENE CARBONATE | Bioplast/biopolymer | TC | Citrus waste | Agricultural & Forestry residues | No | Polycarbonate | 2.48 | 11.37 | -1.5227197 | 0 | 2.77 | 4 and 5 | 9 |  |  |  | Zhang_et_al_1 |
| 52 | POLYESTER BINDERS | Bioplast/biopolymer | TC | Sugarbeet | Pure feedstock (1st generation) | Yes, dLUC | Polyester binders | 4.21 | 10.86 | -0.9476237 | 2.46 | 2.46 | 1 to 3 | 9 |  |  | -0.900787 | Garcia_Gonzalez_et_al_1 |
| 53 | PBS | Bioplast/biopolymer | TC | Sugarcane and corn | Pure feedstock (1st generation) | No, pure feedstock | Polystyrene | 4.43 | 3.8 | 0.1533985 | 1.4 | 3.3 | 4 and 5 | 9 |  |  |  | Cheroennet_et_al |
| 54 | PBS | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | PP | 5.05 | 5.67 | -0.1158009 | 0 | 3.13 | 1 to 3 | 9 |  |  | 0.182322 | Patel_et_al |
| 54 | PBS | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | PP | 3.56 | 5.67 | -0.4654286 | 0 | 3.13 | 1 to 3 | 9 |  |  | -0.510826 | Patel_et_al |
| 55 | SPIROCYCLIC DIOL | Bioplast/biopolymer | TC | Sugarbeet | Pure feedstock (1st generation) | No, pure feedstock | 1,3-propanediol | 2.67 | 6.49 | -0.8881841 | 1.49 | 1.49 | 1 to 3 | 9 |  |  |  | Warlin_et_al |
| 56 | PHB | Bioplast/biopolymer | BC | soybean oil | Pure feedstock (1st generation) | No, pure feedstock | High density polyethylene (HDPE) | 0.6 | 4.93 | -2.1061646 | 2.05 | 3.13 | 4 and 5 | 9 | 1.444061 | 3.111964 | -0.263499 | Kookos_et_al |
| 57 | PU FOAMS | Bioplast/biopolymer | TC | Lignin | Industrial Side Stream | No | polyurethane (PU) foams | 4.0095 | 5.79744 | -0.3687499 | 0 | 0 | 1 to 3 | 9 |  | -0.566238 |  | Manzardo_et_al |
| 58 | SPIRO-BISPHENOL | Bioplast/biopolymer | TC | vanillin, biomass | Pure feedstock (1st generation) | No, pure feedstock | bisphenol A | 1.08 | 7.01 | -1.8703767 | 0 | 2.89 | 1 to 3 | 2 |  |  |  | Mankar_et_al |
| 58 | DIOL V | Bioplast/biopolymer | TC | vanillin, biomass | Pure feedstock (1st generation) | No, pure feedstock | 1,3-propanediol | 1.76 | 6.49 | -1.3049487 | 0 | 1.49 | 1 to 3 | 2 |  |  |  | Mankar_et_al |
| 59 | PP | Bioplast/biopolymer | TC | used cooking oil (UCO) | Food Processing waste | No | PP | 0.63 | 4.78 | -2.026476 | 0 | 3.13 | 6 to 9 | 9 |  |  |  | Moretti_et_al |
| 60 | POLYOLS | Biochemical | TC | Rapeseed | Pure feedstock (1st generation) | No, pure feedstock | polyols | 2.035 | 5.45 | -0.9851198 | 0 | 1.45 | 6 to 9 | 9 |  |  |  | Fridrihsone_et_al |
| 60 | POLYOLS | Biochemical | TC | Rapeseed | Pure feedstock (1st generation) | No, pure feedstock | polyols | 2.095 | 5.45 | -0.9560621 | 0 | 1.45 | 6 to 9 | 9 |  |  |  | Fridrihsone_et_al |
| 61 | LACTIC ACID | Biorefinery | BC | alfalfa | Pure feedstock (1st generation) | Yes, dLUC and iLUC | lactic acid | 4.9 | 5.5 | -0.1155129 | 0.2 | 1.47 | 6 to 9 | 9 |  | 4.598053 |  | Parajuli_et_al |
| 61 | LACTIC ACID | Biorefinery | BC | alfalfa | Pure feedstock (1st generation) | Yes, dLUC and iLUC | lactic acid | 1.22 | 5.5 | -1.5058972 | 0.11 | 1.47 | 6 to 9 | 9 |  | 2.443504 |  | Parajuli_et_al |
| 62 | SUCCINIC ACID | Biorefinery | BC | corn stover (60 wt%), switchgrass (35 wt%), and municipal solid wastes (MSW, 5 wt%) | Agricultural & Forestry residues | No | maleic anhydride | 4.61565 | 7.89 | -0.5361434 | 0 | 1.79 | 4 and 5 | 9 |  |  |  | Cai_and_Davis |
| 62 | ADIPIC ACID | Biorefinery | BC | corn stover (60 wt%), switchgrass (35 wt%), and municipal solid wastes (MSW, 5 wt%) | Agricultural & Forestry residues | No | adipic acid | 3.9228 | 14.01 | -1.2729657 | 0 | 1.81 | 4 and 5 | 9 |  |  |  | Cai_and_Davis |
| 63 | LACTIC ACID | Biochemical | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | lactic acid | 3.67 | 5.5 | -0.4045564 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Fernandez-Dacosta_et_al |
| 64 | BIOPOLYMER | Bioplast/biopolymer | TC | corn starch | Pure feedstock (1st generation) | No, pure feedstock | LDPE | 1.524 | 5.2 | -1.2273202 | 0 | 3.13 | 1 to 3 | 9 |  |  |  | Mercado_et_al |
| 65 | POLYLACTIDE | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | LDPE | 1.44 | 5.2 | -1.2840155 | 3.13 | 3.13 | 1 to 3 | 9 |  |  |  | Vink_et_al |
| 66 | PHB | Bioplast/biopolymer | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 5.09 | 5.64 | -0.1026062 | 3.13 | 3.13 | 4 and 5 | 9 | 0.101352 | 1.856221 |  | Harding_et_al |
| 67 | LACTIC ACID | Biorefinery | BC | vine shoots | Agricultural & Forestry residues | No | lactic acid | 3.26 | 5.5 | -0.5230209 | 0 | 1.47 | 4 and 5 | 9 |  | 1.616505 |  | Páchon_et_al |
| 68 | LACTIDE | Biorefinery | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | PET | 2.753 | 4.49 | -0.4891615 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Sugarbeet | Pure feedstock (1st generation) | No, pure feedstock | PET | 2.793 | 4.49 | -0.4747364 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Woody biomass | Pure feedstock (2nd generation) | No, pure feedstock | PET | 2.595 | 4.49 | -0.5482662 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Sawmill residues | Agricultural & Forestry residues | No | PET | 2.518 | 4.49 | -0.5783878 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Forest residues | Agricultural & Forestry residues | No | PET | 2.579 | 4.49 | -0.554451 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Woody biomass | Pure feedstock (2nd generation) | No, pure feedstock | PET | 2.724 | 4.49 | -0.4997513 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Sawmill residues | Agricultural & Forestry residues | No | PET | 2.558 | 4.49 | -0.562627 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 68 | LACTIDE | Biorefinery | BC | Forest residues | Agricultural & Forestry residues | No | PET | 2.631 | 4.49 | -0.5344887 | 1.825 | 2.29 | 4 and 5 | 9 |  |  |  | Vera_et_al |
| 69 | CURAUÁ/PP COMPOSITE | Biocomposite | M | Curauá fiber | Pure feedstock (2nd generation) | No, pure feedstock | glass fiber/PP composite | 1.98 | 4.2162 | -0.7558374 | 0 | 1.0962 | 4 and 5 | 9 | -0.293348 | 1.058607 |  | Zah_et_al |
| 70 | PHB/KENAF COMPOSITE | Biocomposite | TC | Kenaf fiber, corn | Pure feedstock (1st generation) | No, pure feedstock | glass fiber/PP composite | 3.95 | 5.3462 | -0.3026704 | 0 | 1.0962 | 1 to 3 | 9 |  |  | -0.245122 | Kim_et_al |
| 71 | CELLULOSE NANOWHISKERS | Biofiber | TC | coconut | Pure feedstock (1st generation) | No, pure feedstock | carbon nanofibers | 1086.412 | 1003.66 | 0.0792272 | 0 | 3.66 | 1 to 3 | 3 |  |  |  | Britode_Figueirêdo_et_al |
| 71 | CELLULOSE NANOWHISKERS | Biofiber | TC | cotton | Pure feedstock (1st generation) | No, pure feedstock | carbon nanofibers | 122.171 | 1003.66 | -2.1059869 | 0 | 3.66 | 1 to 3 | 3 |  |  |  | Britode_Figueirêdo_et_al |
| 72 | NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC)) | Biofiber | TC | Wood pulp | Industrial Side Stream | No | carbon nanofibers | 980 | 1003.66 | -0.023856 | 0 | 3.66 | 1 to 3 | 9 |  |  |  | Li_et_al |
| 72 | NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC)) | Biofiber | TC | Wood pulp | Industrial Side Stream | No | carbon nanofibers | 190 | 1003.66 | -1.6643845 | 0 | 3.66 | 1 to 3 | 9 |  |  |  | Li_et_al |
| 72 | NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC)) | Biofiber | TC | Wood pulp | Industrial Side Stream | No | carbon nanofibers | 1160 | 1003.66 | 0.1447667 | 0 | 3.66 | 1 to 3 | 9 |  |  |  | Li_et_al |
| 72 | NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC)) | Biofiber | TC | Wood pulp | Industrial Side Stream | No | carbon nanofibers | 360 | 1003.66 | -1.0253046 | 0 | 3.66 | 1 to 3 | 9 |  |  |  | Li_et_al |
| 73 | MODAL ANTI BACTERIAL FABRIC | Biofiber | TC | Wood pulp | Industrial Side Stream | No | polyester (NanoAg coated polyester) | 10.826772 | 12.841181 | -0.1706354 | 0 | 2.29 | 6 to 9 | 9 |  |  |  | Manda_et_al |
| 73 | MODAL ANTI BACTERIAL FABRIC | Biofiber | TC | Wood pulp | Industrial Side Stream | No | polyester (NanoAg coated polyester) | 9.7637795 | 12.841181 | -0.2739777 | 0 | 2.29 | 6 to 9 | 9 |  |  |  | Manda_et_al |
| 74 | NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC)) | Biofiber | BC | Carrot Pomace | Industrial Side Stream | No | carbon nanofibers | 106.8 | 1003.66 | -2.2404507 | 0 | 3.66 | 1 to 3 | 3 |  |  |  | Piccinno_et_al_1 |
| 74 | NANOCELLULOSE (MICROFIBRILLATED CELLULOSE (MFC)) | Biofiber | BC | Carrot Pomace | Industrial Side Stream | No | carbon nanofibers | 141.6 | 1003.66 | -1.9584024 | 0 | 3.66 | 1 to 3 | 3 |  |  |  | Piccinno_et_al_1 |
| 75 | BACTERIAL CELLULOSE (BC)-REINFORCED EPOXY COMPOSITES | Biocomposite | BC | Kraft pulp | Pure feedstock (1st generation) | No, pure feedstock | glass fiber/PP composite | 11.82906 | 5.2825161 | 0.8061567 | 0 | 1.218 | 1 to 3 | 9 |  |  |  | Hervy_et_al |
| 75 | NANOFIBRILLATED CELLULOSE (NFC)-REINFORCED EPOXY COMPOSITES | Biocomposite | TC | Kraft pulp | Pure feedstock (1st generation) | No, pure feedstock | glass fiber/PP composite | 8.66 | 5.2825161 | 0.4943122 | 0 | 1.218 | 1 to 3 | 9 |  |  |  | Hervy_et_al |
| 76 | WAX ESTERS | Bioadhesive | BC | Rapeseed | Pure feedstock (1st generation) | No, pure feedstock | paraffin wax | 1.15 | 3.778 | -1.1894328 | 0 | 3.12 | 1 to 3 | 9 | 1.309063 | 3.854933 |  | Tufvesson_and_Borjesson |
| 77 | LIGNIN POWDER | Biochemical | TC | Woody biomass | Agricultural & Forestry residues | No | polyol (polyethylene glycol) | 2.87 | 5.43 | -0.6376271 | 2.3 | 1.42 | 1 to 3 | 9 |  |  | -0.616311 | Bernier_et_al |
| 78 | WHEAT-GLUTEN-BASED PACKAGING FILM | Bioplast/biopolymer | M | Wheat | Pure feedstock (1st generation) | No, pure feedstock | LDPE | 2.0214286 | 5.6502899 | -1.0279024 | 0 | 3.13 | 1 to 3 | 9 | 0.678758 |  |  | Deng_et_al_1 |
| 79 | STARCH-FILLED POLYPROPYLENE | Biocomposite | M | waste seed (AWS) | Agricultural & Forestry residues | No | talc-filled PP thermoformed tray | 3.64 | 6.934 | -0.6444532 | 0 | 2.504 | 1 to 3 | 9 |  |  |  | Pang_et_al |
| 79 | STARCH-FILLED POLYPROPYLENE | Biocomposite | M | agricultural waste tuber (AWT) | Agricultural & Forestry residues | No | talc-filled PP thermoformed tray | 3.68 | 6.934 | -0.6335241 | 0 | 2.504 | 1 to 3 | 9 |  |  |  | Pang_et_al |
| 80 | HEMP SHIV/SOL-GEL COATING MATERIAL | Bioadhesive | TC | Hemp | Pure feedstock (1st generation) | No, pure feedstock | reference wall | 0.25 | 0.29 | -0.14842 | 1.84 | 0 | 1 to 3 | 9 |  |  |  | Heidari_et_al |
| 81 | PHENOLIC | Biochemical | TC | Olive mill wastewaters (OMWs) | Food Processing waste | No | phenol | 4.488 | 5.9 | -0.2735452 | 0 | 2.8 | 1 to 3 | 9 |  |  |  | Frascari_et_al |
| 82 | EPOXIDIZED SUCROSE SOYATE COMPOSITES | Biocomposite | TC | Soybean | Pure feedstock (1st generation) | No, pure feedstock | bisphenol A (BPA)-based resins | 3.93 | 4.67 | -0.1725196 | 2.75 | 2.89 | 4 and 5 | 9 |  |  |  | Ghasemi_et_al |
| 83 | SUCCINIC ACID | Biorefinery | BC | marcoalgae | 3rd generation | No | succinic acid | 0.645 | 4.57 | -1.9580182 | 0 | 1.49 | 1 to 3 | 9 |  |  |  | Sadhukan_et_al |
| 83 | LACTIC ACID | Biorefinery | BC | marcoalgae | 3rd generation | No | lactic acid | 0.57 | 4.94 | -2.1594842 | 0 | 1.47 | 1 to 3 | 9 |  |  |  | Sadhukan_et_al |
| 84 | P-XYLENE | Biochemical | TC | red oak | Pure feedstock (2nd generation) | No, pure feedstock | p-xylene | 1.7314 | 5.04 | -1.0684758 | 0 | 3.32 | 4 and 5 | 9 |  |  |  | Lin_et_al |
| 84 | P-XYLENE | Biochemical | TC | maize starch | Pure feedstock (1st generation) | No, pure feedstock | p-xylene | 8.5665 | 5.04 | 0.5304532 | 0 | 3.32 | 4 and 5 | 9 |  |  |  | Lin_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Potatoes | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 7 | 7.8 | -0.1082136 | 0 | 2.3 | 1 to 3 | 9 |  | 1.237874 | 0.04879 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Grain maize | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 8.1 | 7.8 | 0.0377403 | 0 | 2.3 | 1 to 3 | 9 |  | 1.420196 | 0.139762 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Potatoes | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 6 | 7.8 | -0.2623643 | 0 | 2.3 | 1 to 3 | 9 |  | 1.132514 | -0.097455 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Grain maize | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 7 | 7.8 | -0.1082136 | 0 | 2.3 | 1 to 3 | 9 |  | 1.237874 | 0 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Potatoes | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 6.5 | 7.8 | -0.1823216 | 0 | 2.3 | 1 to 3 | 9 |  | 1.175999 | -0.028988 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Grain maize | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 7.5 | 7.8 | -0.0392207 | 0 | 2.3 | 1 to 3 | 9 |  | 1.333185 | 0.062304 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Potatoes | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 8.7 | 8.1 | 0.071459 | 0 | 2.3 | 1 to 3 | 9 |  | 1.564986 | 0.035091 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Grain maize | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 9.9 | 8.1 | 0.2006707 | 0 | 2.3 | 1 to 3 | 9 |  | 0.633428 | 0.127262 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Potatoes | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 7.5 | 8.1 | -0.076961 | 0 | 2.3 | 1 to 3 | 9 |  | 1.435774 | -0.113329 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Grain maize | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 8.6 | 8.1 | 0.0598981 | 0 | 2.3 | 1 to 3 | 9 |  | 1.534214 | -0.014389 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Potatoes | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 8 | 8.1 | -0.0124225 | 0 | 2.3 | 1 to 3 | 9 |  | 1.435774 | -0.051293 | Dros_et_al |
| 85 | HEXAMETHYLENEDIAMINE | Biochemical | TC | Grain maize | Pure feedstock (1st generation) | No, pure feedstock | Hexamethylenediamine (HMDA) | 9 | 8.1 | 0.1053605 | 0 | 2.3 | 1 to 3 | 9 |  | 1.534214 | 0.04879 | Dros_et_al |
| 86 | ETHYLENE | Biochemical | TC | Forest residues | Agricultural & Forestry residues | No | ethylene | 6.064 | 10.11 | -0.5111554 | 5.78 | 5.78 | 4 and 5 | 9 | 0.149212 | 0.689569 |  | Liptow_et_al |
| 86 | ETHYLENE | Biochemical | TC | Forest residues | Agricultural & Forestry residues | No | ethylene | 6.09 | 10.15 | -0.5108256 | 5.82 | 5.82 | 4 and 5 | 9 | 0.175273 | 0.721318 |  | Liptow_et_al |
| 86 | ETHYLENE | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | ethylene | 7.968 | 10.99 | -0.3215522 | 6.66 | 6.66 | 4 and 5 | 9 | 0.650088 | 0.998529 |  | Liptow_et_al |
| 86 | ETHYLENE | Biochemical | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | ethylene | 16.72 | 19.45 | -0.1512415 | 15.12 | 15.12 | 4 and 5 | 9 | 0.964898 | 1.178655 |  | Liptow_et_al |
| 87 | ACETONE | Biochemical | BC | Wheat straw | Agricultural & Forestry residues | Yes, dLUC | Acetone | 5.88 | 4.5346212 | 0.2598152 | 0 | 2.2746212 | 4 and 5 | 9 | 1.696449 | 2.847312 |  | Khoo_et_al |
| 87 | ACETONE | Biochemical | BC | Rice straw | Agricultural & Forestry residues | Yes, dLUC | Acetone | 4.64 | 4.5346212 | 0.0229728 | 0 | 2.2746212 | 4 and 5 | 9 | 1.408767 | 1.991646 |  | Khoo_et_al |
| 87 | FORMIC ACID | Biochemical | BC | Wheat straw | Agricultural & Forestry residues | Yes, dLUC | Formic acid | 1.47 | 3.4167988 | -0.8434417 | 0 | 0.9567988 | 4 and 5 | 9 | 0.310155 | 0.541916 |  | Khoo_et_al |
| 87 | FORMIC ACID | Biochemical | BC | Corn stover | Agricultural & Forestry residues | Yes, dLUC | Formic acid | 1.19 | 3.4167988 | -1.0547508 | 0 | 0.9567988 | 4 and 5 | 9 | 1.430746 | 1.57735 |  | Khoo_et_al |
| 87 | METHANOL | Biochemical | BC | Rice straw | Agricultural & Forestry residues | Yes, dLUC | Methanol | 4.72 | 1.8944278 | 0.912892 | 0 | 1.3744278 | 4 and 5 | 9 | 3.829331 | 3.247836 |  | Khoo_et_al |
| 87 | METHANOL | Biochemical | BC | Sugarcane bagasse | Agricultural & Forestry residues | Yes, dLUC | Methanol | 0.77 | 1.8944278 | -0.9002816 | 0 | 1.3744278 | 4 and 5 | 9 | 0.308884 | 1.927461 |  | Khoo_et_al |
| 88 | 1,4-BUTANEDIOL | Biochemical | BC | Wheat straw | Pure feedstock (1st generation) | No, pure feedstock | 1,4-butanediol | 3.55 | 6.35 | -0.5815072 | 1.95 | 1.95 | 4 and 5 | 9 | -0.182322 | 4.248495 |  | Forte_et_al |
| 89 | BUTYLCATECHOL | Biochemical | TC | Candlenut shell | Agricultural & Forestry residues | No | tert-butylcatechol | 13.4 | 16.15 | -0.1866653 | 0 | 2.65 | 4 and 5 | 9 | 0.617161 |  |  | Montazeri_&_Eckelman |
| 90 | LACTIC ACID | Biochemical | BC | Sugarcane bagasse | Agricultural & Forestry residues | No | Lactic acid | 0.4125 | 5.808 | -2.6447554 | 0 | 1.47 | 4 and 5 | 9 | -2.874963 | -3.089064 |  | Daful_et_al |
| 91 | SUCCINIC ACID | Biochemical | BC | Sorghum grains | Pure feedstock (2nd generation) | No, pure feedstock | succinic acid | 0.87 | 4.8395 | -1.7160735 | 0 | 1.49 | 6 to 9 | 9 |  |  |  | Moussa_et_al |
| 91 | SUCCINIC ACID | Biochemical | BC | Dextrose | Pure feedstock (2nd generation) | No, pure feedstock | succinic acid | 1.4964 | 4.8395 | -1.1737492 | 0 | 1.49 | 6 to 9 | 9 |  |  |  | Moussa_et_al |
| 92 | SUCCINIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | succinic acid | 12 | 4.96 | 0.8835009 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 92 | SUCCINIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | succinic acid | 29.6 | 4.96 | 1.7863686 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 92 | SUCCINIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | succinic acid | 14.4 | 4.96 | 1.0658225 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 92 | SUCCINIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | succinic acid | 10.18 | 4.96 | 0.7190193 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 92 | SUCCINIC ACID | Biochemical | BC | Forest residues | Agricultural & Forestry residues | No | succinic acid | 10.5 | 4.96 | 0.7499695 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 92 | SUCCINIC ACID | Biochemical | BC | Sugarbeet | Pure feedstock (1st generation) | No, pure feedstock | succinic acid | 10 | 4.96 | 0.7011794 | 0 | 1.49 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 93 | 1,3-BUTADIENE | Biochemical | TC | Corn, wheat, rye sugar | Pure feedstock (1st generation) | No, pure feedstock | 1,3 butadine | 2.02 | 5.37 | -0.9777304 | 0 | 3.26 | 4 and 5 | 9 |  |  |  | Cespi_et_al |
| 93 | 1,3-BUTADIENE | Biochemical | TC | Surgarcane | Pure feedstock (1st generation) | No, pure feedstock | 1,3 butadine | 1.02 | 5.37 | -1.6610253 | 0 | 3.26 | 4 and 5 | 9 |  |  |  | Cespi_et_al |
| 93 | 1,3-BUTADIENE | Biochemical | TC | Corn | Pure feedstock (1st generation) | No, pure feedstock | 1,3 butadine | 2.29 | 5.37 | -0.8522761 | 0 | 3.26 | 4 and 5 | 9 |  |  |  | Cespi_et_al |
| 93 | 1,3-BUTADIENE | Biochemical | TC | Corn, wheat, rye sugar | Pure feedstock (1st generation) | No, pure feedstock | 1,3 butadine | 3.61 | 5.37 | -0.3971201 | 0 | 3.26 | 4 and 5 | 9 |  |  |  | Cespi_et_al |
| 93 | 1,3-BUTADIENE | Biochemical | TC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | 1,3 butadine | 2.18 | 5.37 | -0.901503 | 0 | 3.26 | 4 and 5 | 9 |  |  |  | Cespi_et_al |
| 93 | 1,3-BUTADIENE | Biochemical | TC | Corn | Pure feedstock (1st generation) | No, pure feedstock | 1,3 butadine | 4 | 5.37 | -0.2945335 | 0 | 3.26 | 4 and 5 | 9 |  |  |  | Cespi_et_al |
| 94 | LACTIC ACID | Biochemical | BC | glycerol from palm oil | Industrial Side Stream | No | Lactic acid | 1.9 | 5.77 | -1.1108182 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Gargalo_et_al |
| 94 | PROPIONIC ACID | Biochemical | BC | glycerol from palm oil | Industrial Side Stream | No | propionic acid | 2.84 | 3.68 | -0.2591087 | 0 | 1.78 | 4 and 5 | 9 |  |  |  | Gargalo_et_al |
| 94 | 1,3-PROPANEDIOL | Biochemical | BC | glycerol from palm oil | Industrial Side Stream | No | 1,3 propanediol | 3.3 | 6.5 | -0.6778797 | 0 | 1.74 | 4 and 5 | 9 |  |  |  | Gargalo_et_al |
| 95 | 1,3-DIHYDROXYACETONE | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 7.01 | 5.77 | 0.1946656 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 95 | 1,3-DIHYDROXYACETONE | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 4.91 | 5.77 | -0.1613981 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 95 | 1,3-DIHYDROXYACETONE | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 3.86 | 5.77 | -0.4020049 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 95 | LACTIC ACID | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 7.32 | 5.77 | 0.2379382 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 95 | LACTIC ACID | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 6.91 | 5.77 | 0.1802976 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 95 | LACTIC ACID | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 6.29 | 5.77 | 0.086289 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 95 | LACTIC ACID | Biochemical | TC | glycerol from biodiesel | Industrial Side Stream | No | Lactic acid | 5.79 | 5.77 | 0.0034602 | 0 | 1.47 | 4 and 5 | 9 |  |  |  | Morales_et_al |
| 96 | PHA | Bioplast/biopolymer | BC | Corn stover | Agricultural & Forestry residues | No | PET | 2 | 5.59 | -1.0278321 | 1.51 | 2.29 | 4 and 5 | 9 |  |  |  | Yu_and_Chen |
| 97 | PHA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | PET | 1.61 | 4.19 | -0.9564666 | 0 | 2.29 | 4 and 5 | 9 |  |  |  | Kim_and_Dale |
| 98 | PHA | Bioplast/biopolymer | BC | Cellulosic fraction of organic residuals from material recovery facilities | Industrial Side Stream | No | PET | 4.7 | 4.49 | 0.0457098 | 0 | 2.29 | 4 and 5 | 9 |  |  |  | Kendall_et_al |
| 99 | PHA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | PET | 2.3 | 4.44 | -0.6577453 | 0 | 0 | 4 and 5 | 9 |  |  |  | Eerhart_et_al |
| 100 | POLYITACONIC ACID | Bioplast/biopolymer | BC | Woody biomass | Pure feedstock (2nd generation) | No, pure feedstock | Polyacrylic acid | 1.32 | 4.19 | -1.155069 | 0 | 1.45 | 4 and 5 | 9 |  |  |  | Nuss_and_Gardner |
| 100 | POLYITACONIC ACID | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | Polyacrylic acid | 2.19 | 4.19 | -0.6487992 | 0 | 1.45 | 4 and 5 | 9 |  |  |  | Nuss_and_Gardner |
| 101 | PET | Bioplast/biopolymer | BC | Wheat stover | Pure feedstock (1st generation) | No, pure feedstock | PET | 7.1 | 5.59 | 0.2391155 | 0 | 2.29 | 4 and 5 | 9 | 1.128179 | 0.908259 | -0.105361 | Akanuma_et_al |
| 101 | PET | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | PET | 6.81 | 5.59 | 0.1974128 | 0 | 2.29 | 4 and 5 | 9 | 1.203973 | 1.660891 | -0.303186 | Akanuma_et_al |
| 101 | PET | Bioplast/biopolymer | TC | Poplar | Pure feedstock (2nd generation) | No, pure feedstock | PET | 4.31 | 5.59 | -0.2600414 | 0 | 2.29 | 4 and 5 | 9 | 0.509365 | 0.416603 | -0.643834 | Akanuma_et_al |
| 102 | PLA | Bioplast/biopolymer | BC | Cassava | Pure feedstock (1st generation) | No, pure feedstock | PET | 5.2276876 | 6.4551376 | -0.2109073 | 1.47 | 2.29 | 4 and 5 | 9 | 0.210743 | 1.89637 | -0.97689 | Papong_et_al |
| 102 | PLA | Bioplast/biopolymer | BC | Cassava | Pure feedstock (1st generation) | No, pure feedstock | PET | 4.274428 | 6.4551376 | -0.412226 | 1.47 | 2.29 | 4 and 5 | 9 | 0.125446 | 1.017121 | -1.105012 | Papong_et_al |
| 103 | PHB | Bioplast/biopolymer | TC | Poplar | Pure feedstock (2nd generation) | No, pure feedstock | PP | 4.73316 | 5 | -0.0548449 | 2 | 3.14 | 4 and 5 | 9 |  |  |  | Hohenschuh_et_al |
| 104 | PHA | Bioplast/biopolymer | BC | Wastewater | Municipal waste | No | PET | 8.42 | 5.59 | 0.4096305 | 2 | 2.29 | 4 and 5 | 9 |  |  |  | Heimersson_et_al |
| 104 | PHA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | PET | 5.35 | 5.59 | -0.0438827 | 2 | 2.29 | 4 and 5 | 9 |  |  |  | Heimersson_et_al |
| 105 | POLYOLS | Bioplast/biopolymer | BC | Palm fruit branches | Pure feedstock (2nd generation) | No, pure feedstock | polyol | 1.308 | 4.95 | -1.3308883 | 0 | 1.45 | 6 to 9 | 9 |  |  |  | Zolkarinain_et_al |
| 106 | PHLA | Bioplast/biopolymer | BC | Sweet sorghum | Pure feedstock (2nd generation) | No, pure feedstock | PBT | 8.6 | 7.3 | 0.1638879 | 2.4 | 2.4 | 4 and 5 | 9 |  |  |  | Sun_et_al |
| 107 | PBS | Bioplast/biopolymer | BC | Maze starch | Pure feedstock (1st generation) | No, pure feedstock | PET | 8.64 | 4.79 | 0.5898722 | 2.05 | 2.29 | 4 and 5 | 9 |  |  |  | Tecchio_et_al |
| 107 | PBS | Bioplast/biopolymer | BC | Sugarcane | Pure feedstock (1st generation) | No, pure feedstock | PET | 7.63 | 4.79 | 0.4655574 | 2.05 | 2.29 | 4 and 5 | 9 |  |  |  | Tecchio_et_al |
| 107 | PBS | Bioplast/biopolymer | BC | Lignocellulosics | Pure feedstock (2nd generation) | No, pure feedstock | PET | 7.94 | 4.79 | 0.5053829 | 2.05 | 2.29 | 4 and 5 | 9 |  |  |  | Tecchio_et_al |
| 108 | SODIUM POLYACRYLATE (NA-PA) | Bioplast/biopolymer | BC | Wood pulp | Industrial Side Stream | No | Sodium polyacrylate (Na-PA) | 5.1 | 8.5 | -0.5108256 | 1.4 | 1.4 | 4 and 5 | 9 | 0.995428 | 0.52501 | -0.220062 | Gontia_and_Janssen |
| 108 | SODIUM POLYACRYLATE (NA-PA) | Bioplast/biopolymer | BC | Wood pulp | Industrial Side Stream | No | Sodium polyacrylate (Na-PA) | 5.2 | 8.5 | -0.4914075 | 1.4 | 1.4 | 4 and 5 | 9 | 0.567984 | 0.389465 | -0.405465 | Gontia_and_Janssen |
| 109 | PET | Bioplast/biopolymer | BC | Woody biomass and corn | Pure feedstock (1st generation) | No, pure feedstock | PET | 6.5 | 7.19 | -0.100889 | 0 | 2.29 | 6 to 9 | 9 | 0.927987 |  |  | Chen_et_al |
| 109 | PET | Bioplast/biopolymer | BC | Woody biomass and switchgrass | Pure feedstock (2nd generation) | No, pure feedstock | PET | 7.6 | 7.19 | 0.0554571 | 0 | 2.29 | 6 to 9 | 9 | 1.137079 |  |  | Chen_et_al |
| 109 | PET | Bioplast/biopolymer | BC | Woody biomass and wheat straw | Pure feedstock (2nd generation) | No, pure feedstock | PET | 6.7 | 7.19 | -0.0705836 | 0 | 2.29 | 6 to 9 | 9 | 1.371479 |  |  | Chen_et_al |
| 109 | PET | Bioplast/biopolymer | BC | Corn stover and corn | Pure feedstock (1st generation) | No, pure feedstock | PET | 9.5 | 7.19 | 0.2786006 | 0 | 2.29 | 6 to 9 | 9 | 1.523495 |  |  | Chen_et_al |
| 109 | PET | Bioplast/biopolymer | BC | Corn stover and switchgrass | Pure feedstock (2nd generation) | No, pure feedstock | PET | 9.5 | 7.19 | 0.2786006 | 0 | 2.29 | 6 to 9 | 9 | 1.644123 |  |  | Chen_et_al |
| 109 | PET | Bioplast/biopolymer | BC | Corn stover and wheat straw | Pure feedstock (1st generation) | No, pure feedstock | PET | 8.6 | 7.19 | 0.179071 | 0 | 2.29 | 6 to 9 | 9 | 1.573506 |  |  | Chen_et_al |
| 110 | PLA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | Yes, dLUC | PET | 3.2 | 4.76 | -0.3970969 | 1.8 | 2.29 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | PLA | Bioplast/biopolymer | BC | Switchgrass | Pure feedstock (2nd generation) | Yes, dLUC | PET | 3.11 | 4.76 | -0.4256249 | 1.8 | 2.29 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | PHB | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | Yes, dLUC | HDPE | 5.41 | 5.25 | 0.030021 | 2 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | PHB | Bioplast/biopolymer | BC | Switchgrass | Pure feedstock (2nd generation) | Yes, dLUC | HDPE | 5.02 | 5.25 | -0.0447981 | 2 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | ETHYLENE | Biochemical | BC | Corn | Pure feedstock (1st generation) | Yes, dLUC | ethylene | 2.603 | 4.25 | -0.4902544 | 3.1 | 3.14 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | ETHYLENE | Biochemical | BC | Switchgrass | Pure feedstock (2nd generation) | Yes, dLUC | ethylene | 4.7 | 4.25 | 0.1006435 | 3.1 | 3.14 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | HDPE | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | Yes, dLUC | HDPE | 2.98 | 5.25 | -0.5663048 | 3.13 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | LDPE | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | Yes, dLUC | LDPE | 3.22 | 5.54 | -0.5426131 | 3.13 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | LLDPE | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | Yes, dLUC | LLDPE | 2.97 | 5.26 | -0.5715691 | 3.13 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | HDPE | Bioplast/biopolymer | BC | Switchgrass | Pure feedstock (2nd generation) | Yes, dLUC | HDPE | 6.46 | 5.25 | 0.2074012 | 3.13 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | LDPE | Bioplast/biopolymer | BC | Switchgrass | Pure feedstock (2nd generation) | Yes, dLUC | LDPE | 6.77 | 5.54 | 0.2005066 | 3.13 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 110 | LLDPE | Bioplast/biopolymer | BC | Switchgrass | Pure feedstock (2nd generation) | Yes, dLUC | LLDPE | 6.48 | 5.26 | 0.2085895 | 3.13 | 3.13 | 6 to 9 | 9 |  |  |  | Posen_et_al |
| 111 | HDPE | Bioplast/biopolymer | BC | Sugarbeet | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 5.2 | 7.23 | -0.3295804 | 0 | 3.13 | 4 and 5 | 9 |  |  |  | Belboom_and_Léonard |
| 111 | HDPE | Bioplast/biopolymer | BC | Wheat stover | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 6.78 | 7.23 | -0.0642619 | 0 | 3.13 | 4 and 5 | 9 |  |  |  | Belboom_and_Léonard |
| 111 | HDPE | Bioplast/biopolymer | BC | Sugarbeet | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 2.73 | 4.77 | -0.5580447 | 0 | 3.13 | 4 and 5 | 9 |  |  |  | Belboom_and_Léonard |
| 111 | HDPE | Bioplast/biopolymer | BC | Wheat stover | Pure feedstock (1st generation) | No, pure feedstock | HDPE | 4.32 | 4.77 | -0.0990909 | 0 | 3.13 | 4 and 5 | 9 |  |  |  | Belboom_and_Léonard |
| 112 | PHA | Bioplast/biopolymer | BC | Corn | Pure feedstock (1st generation) | No, pure feedstock | LDPE | 3.68 | 5.03 | -0.3125072 | 0 | 3.13 | 4 and 5 | 9 |  |  |  | Kim_and_Dale |

Table S.9.b: Continuation data-table; additional background information on different upscaling processes, impact assessment method, year(s) of collected foreground data, location of biomass cultivation and location of the production of both the fossil- and bio-products, including LCI source/database for the fossil-product. Not for all categories the information was available or could be retrieved, indicated with NA.

| Study number | Recovery and re-use of heat (heat integration) | Recovery of solvents | Treatment of waste including energy recovery | Recycling of flows (waste streams) | Impact Assessment method | Uncertainty Analysis? | Year of foreground data | Location foreground data | Location biomass cultivation | Location production of fossil alternative | Fossil product LCI database/source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Yes | No | No | No | CML 2.03 | Yes | 2006 | USA | USA | Yes | Own modelling |
| 2 | No | No | No | Yes | CML | No | 2012 - 2014 | Europe | Europe | No | Ecoinvent 3.1 |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | Brazil | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | Brazil | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | Brazil | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | USA | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | USA | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | Brazil | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | USA | No | literature |
| 3 | Yes | No | No | No | na | No | 1991 - 2016 | Europe | USA | No | literature |
| 4 | No | No | Yes | No | na | No | 1993 - 2010 | Europe | Europe | No | Ullmann’s Encyclopedia of Industrial Chemistry |
| 5 | Yes | Yes | Yes | Yes | na | No | 2004 - 2009 | Europe | Brazil | No | StatoilHydro and NTM |
| 6 | No | No | Yes | No | own calculations | No | 2007-2011 | Europe | Europe | No | Simapro |
| 7 | No | Yes | No | No | IPCC GWP 100a | No | 2010-2013 | Europe | Europe | No | Literature (Samel et al. (2005)) |
| 7 | No | Yes | No | No | IPCC GWP 100a | No | 2010-2013 | Europe | Europe | No | Literature (Samel et al. (2005)) |
| 7 | No | Yes | No | No | IPCC GWP 100a | No | 2010-2013 | Europe | Europe | No | Literature (Samel et al. (2005)) |
| 7 | No | Yes | No | No | IPCC GWP 100a | No | 2010-2013 | Europe | Europe | No | Literature (Samel et al. (2005)) |
| 8 | Yes | No | Yes | No | IPCC GWP 100a | No | 2010-2014 | Global | USA | No | Literature |
| 9 | Yes | Yes | Yes | Yes | CML | Yes | 2013-2014 | Europe | Europe | Yes | SimaPro |
| 10 | Yes | No | Yes | No | GREET | No | 2009 | USA | USA | No | literature |
| 10 | Yes | No | Yes | No | GREET | No | 2014 | USA | USA | No | literature |
| 10 | Yes | No | Yes | No | GREET | No | 2012-2013 | USA | USA | No | literature |
| 11 | Yes | No | Yes | No | IPCC GWP 100a | Yes | 2013 | Europe | Europe | Yes | Ecoinvent |
| 11 | No | No | Yes | No | IPCC GWP 100a | Yes | 2013 | Europe | Europe | Yes | Ecoinvent |
| 12 | No | No | No | No | own calculations | No | 1997 - 2013 | USA | USA | No | Ecoinvent 3.7 (Tetrahydrofuran {GLO}¦ market for ¦ APOS, U) |
| 12 | No | No | No | No | own calculations | No | 1997 - 2013 | Brazil | Brazil | No | Ecoinvent 3.7 (Tetrahydrofuran {GLO}¦ market for ¦ APOS, U) |
| 12 | No | No | No | No | own calculations | No | 1997 - 2013 | China | China | No | Ecoinvent 3.7 (Tetrahydrofuran {GLO}¦ market for ¦ APOS, U) |
| 13 | Yes | No | No | No | CML 2001 | Yes | 2012 - 2016 | USA | USA | Yes | Encyclopedia of chemical processing and design |
| 13 | Yes | No | No | No | CML 2001 | Yes | 2012 - 2016 | USA | USA | Yes | Encyclopedia of chemical processing and design |
| 13 | Yes | No | No | No | CML 2001 | Yes | 2012 - 2016 | USA | USA | Yes | Encyclopedia of chemical processing and design |
| 14 | No | No | No | No | IPCC GWP 100a | No | 2014 - 2016 | USA | USA | No | Literature |
| 14 | No | No | No | No | IPCC GWP 100a | No | 2014 | USA | USA | No | Literature |
| 14 | No | No | No | No | IPCC GWP 100a | No | 2009 | USA | USA | No | Literature |
| 14 | No | No | No | No | IPCC GWP 100a | No | 2011 | USA | USA | No | Literature |
| 14 | No | No | No | No | IPCC GWP 100a | No | 2003 | USA | USA | No | Literature |
| 15 | Yes | Yes | Yes | No | CML 3.02 | No | 2015 | South-Africa | South-Africa | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 15 | Yes | Yes | Yes | No | CML 3.02 | No | 2015 | South-Africa | South-Africa | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 15 | Yes | Yes | Yes | No | CML 3.02 | No | 2015 | South-Africa | South-Africa | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 16 | No | No | Yes | No | IPCC GWP 100a | No | 2010 | Europe | Europe | No | Literature |
| 17 | Yes | Yes | Yes | Yes | CML 3.02 | No | 2014 - 2016 | Europe | Europe | No | Ecoinvent 3.7 adipic acid {GLO}¦ market for ¦ APOS, U) |
| 17 | No | No | No | No | CML 3.02 | No | 2014 - 2016 | Europe | Europe | No | Ecoinvent 3.7 adipic acid {GLO}¦ market for ¦ APOS, U) |
| 17 | No | No | No | No | CML 3.02 | No | 2014 - 2016 | Europe | Europe | No | Ecoinvent 3.7 adipic acid {GLO}¦ market for ¦ APOS, U) |
| 18 | No | No | No | No | TRACI 2.1 Midpoint | No | 2012-2017 | USA | USA | No | Literature |
| 19 | No | Yes | No | No | ReCiPe 2016 | No | 2017 | USA | na | No | Ecoinvent 3.3 |
| 20 | Yes | No | Yes | Yes | na | No | 1999 - 2017 | USA | USA | No | Ullmann’s Encyclopedia of Industrial Chemistry |
| 20 | Yes | No | Yes | Yes | na | No | 1999 - 2017 | USA | USA | No | Ullmann’s Encyclopedia of Industrial Chemistry |
| 21 | Yes | No | Yes | No | CML 2 | No | 2017 | Europe | Europe | No | Ullmann’s Encyclopedia of Industrial Chemistry |
| 21 | Yes | No | Yes | No | CML 2 | No | 2011 | Europe | Europe | No | Ullmann’s Encyclopedia of Industrial Chemistry |
| 21 | Yes | No | Yes | No | CML 2 | No | 2012 | Europe | Europe | No | Ullmann’s Encyclopedia of Industrial Chemistry |
| 22 | No | No | No | No | na OpenLCA | No | 2010 - 2014 | Europe | Europe | No | Ecoinvent 3.7 Succinic acid {GLO}¦ market for succinic acid ¦ APOS, U |
| 22 | Yes | Yes | Yes | Yes | na OpenLCA | No | 2013 | Europe | Europe | No | Ecoinvent 3.7 Succinic acid {GLO}¦ market for succinic acid ¦ APOS, U |
| 23 | No | Yes | No | No | CML 2.05 | No | 2009 - 2016 | Europe | Europe | No | Ecoinvent 3.7 Succinic acid {GLO}¦ market for succinic acid ¦ APOS, U |
| 24 | Yes | No | Yes | No | na | Yes | 2013 | Brazil | Brazil | Yes | Literature |
| 24 | Yes | No | Yes | No | na | Yes | 2013 | Brazil | Brazil | Yes | Literature |
| 24 | Yes | No | Yes | No | na | Yes | 2006, 2015 | Brazil | Brazil | Yes | Literature |
| 24 | Yes | No | Yes | No | na | Yes | 2006, 2015 | Brazil | Brazil | Yes | Literature |
| 24 | Yes | No | Yes | No | na | Yes | 2013, 2016 | Brazil | Brazil | Yes | Literature |
| 24 | Yes | No | Yes | No | na | Yes | 2013, 2016 | Brazil | Brazil | Yes | Literature |
| 25 | Yes | Yes | Yes | Yes | ReCiPe | No | 2013 -2020 | Global | na | No | Literature |
| 26 | Yes | No | No | No | IPCC GWP 100a | No | 2019 | China | China | No | Literature |
| 27 | No | No | No | Yes | IPCC GWP 100a | No | 2009 - 2014 | na | na | No | Literature |
| 27 | No | No | No | Yes | IPCC GWP 100a | No | 2009 - 2014 | na | na | No | Literature |
| 27 | No | No | No | Yes | IPCC GWP 100a | No | 2009 - 2014 | na | na | No | Literature |
| 27 | No | No | No | Yes | IPCC GWP 100a | No | 2018 - 2019 | na | na | No | Literature |
| 27 | No | No | No | Yes | IPCC GWP 100a | No | 2018 - 2019 | na | na | No | Literature |
| 27 | No | No | No | Yes | IPCC GWP 100a | No | 2018 - 2019 | na | na | No | Literature |
| 28 | Yes | No | Yes | Yes | IPCC GWP 100a | No | 2015 - 2018 | Europe | Europe | No | Literature |
| 29 | Yes | No | Yes | Yes | ReCiPe 2016 | Yes | 1997 - 2018 | USA | USA | Yes | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 29 | No | No | Yes | Yes | ReCiPe 2016 | Yes | 2011 - 2018 | USA | USA | Yes | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 30 | No | No | No | No | TRACI 2.1 Midpoint | No | na | USA | USA | No | Databases |
| 31 | No | No | No | No | TRACI 2.1 Midpoint | No | na | USA | USA | No | Databases |
| 32 | No | No | No | No | ReCiPe 2016 | No | 2015 | na | na | No | Econinvent 3.5 (Melamine-urea-formaldehyde resin, at plant/US, max: Phenol-resorcinol-formaldehyde resin, at plant/US) |
| 32 | No | No | No | No | ReCiPe 2016 | No | 2016 - 2019 | na | na | No | Econinvent 3.5 (Melamine-urea-formaldehyde resin, at plant/US, max: Phenol-resorcinol-formaldehyde resin, at plant/US) |
| 32 | No | No | No | No | ReCiPe 2016 | No | 2006 - 2013 | na | na | No | Econinvent 3.5 (Melamine-urea-formaldehyde resin, at plant/US, max: Phenol-resorcinol-formaldehyde resin, at plant/US) |
| 32 | No | No | No | No | ReCiPe 2016 | No | 2007 - 2019 | na | na | No | Econinvent 3.5 (Melamine-urea-formaldehyde resin, at plant/US, max: Phenol-resorcinol-formaldehyde resin, at plant/US) |
| 33 | No | No | No | No | CML 2000 | No | 2011 | Europe | Europe | No | Ecoinvent |
| 33 | No | No | No | No | CML 2000 | No | 2011 | Europe | Europe | No | Ecoinvent |
| 34 | No | No | No | Yes | TRACI 2.1 Midpoint | No | 2009 - 2013 | USA | USA | No | US-EI database |
| 34 | No | No | No | Yes | TRACI 2.1 Midpoint | No | 2009 - 2013 | USA | USA | No | US-EI database |
| 35 | No | No | No | No | ReCiPe | Yes | 2012 | Europe | na | Yes | Ecoinvent 2.1 & industrial data |
| 36 | No | No | No | No | IPCC GWP 100a | No | na | Europe | Europe | No | Ecoinvent 3 |
| 36 | No | No | No | No | IPCC GWP 100a | No | na | Europe | na | No | Ecoinvent 3 |
| 36 | No | No | No | No | IPCC GWP 100a | No | na | Europe | Europe | No | Ecoinvent 3 |
| 37 | No | No | No | Yes | IPCC GWP 100a | Yes | 2018 | Brazil | Brazil | Yes | Ecoinvent 3.4 |
| 37 | No | No | No | Yes | IPCC GWP 100a | yes | 2018 | Brazil | Brazil | yes | Ecoinvent 3.4 |
| 38 | No | No | No | No | na | No | 1997 - 2003 | Japan | USA | No | Association of Plastic Manufacturers in Europe (APME) |
| 38 | No | No | No | No | na | No | 1997 - 2003 | Japan | USA | No | Association of Plastic Manufacturers in Europe (APME) |
| 39 | No | No | Yes | No | TRACI 2003 | No | 2001 -2003 | USA | USA | No | DEAM Databases |
| 39 | No | No | Yes | No | TRACI 2003 | No | 2001 -2003 | USA | USA | No | DEAM Databases |
| 40 | No | No | No | No | IPCC GWP 100a | No | 2007, 2014 | Europe | na | No | PlasticsEurope |
| 40 | No | No | No | No | IPCC GWP 100a | No | 2007, 2014 | Europe | Europe | No | PlasticsEurope |
| 41 | Yes | No | Yes | No | ReCiPe | No | 2015 | Europe | na | No | Ecoinvent 3.3 |
| 42 | Yes | Yes | Yes | No | ReCiPe | No | 2013 - 2016 | Europe | Europe | No | Ecoinvent 3.3 |
| 42 | Yes | Yes | No | No | ReCiPe | No | 2013 - 2016 | Europe | Europe | No | Ecoinvent 3.3 |
| 42 | No | Yes | No | No | ReCiPe | No | 2013 - 2016 | Europe | na | No | Ecoinvent 3.3 |
| 43 | No | No | No | No | IPCC GWP 100a | No | 2011 | Europe | Europe | No | Literature |
| 44 | No | No | Yes | No | ReCiPe 2016 | No | 2015 | Europe | Europe | No | Literature |
| 45 | No | No | No | No | TRACI 2.1 Midpoint | No | 2012 - 2018 | USA | USA | No | Database |
| 46 | Yes | No | No | No | GREET | No | 2015 - 2017 | USA | USA | No | Database |
| 46 | Yes | No | No | No | GREET | No | 2015 - 2017 | USA | USA | No | Database |
| 46 | Yes | No | No | No | GREET | No | 2015 - 2017 | USA | USA | No | Database |
| 46 | Yes | No | No | No | GREET | No | 2015 - 2017 | USA | USA | No | Database |
| 47 | na | na | na | na | IPCC GWP 100a | No | 2009 - 2014 | Europe | USA | No | Econinvent 3.2 |
| 48 | Yes | No | No | Yes | IPCC GWP 100a | No | 2016, 2017 | Japan | Japan | No | Plant data |
| 48 | Yes | No | No | Yes | IPCC GWP 100a | No | 2005, 2017 | Japan | Japan | No | Plant data |
| 48 | Yes | No | No | Yes | IPCC GWP 100a | No | 2016, 2017 | Japan | Japan | No | Plant data |
| 49 | No | No | No | No | IPCC GWP 100a | No | 2014, 2015 | Brazil | Brazil | No | Ecoinvent 3 Polyethylene, linear low density, granulate {GLO}¦ market for ¦ APOS, U |
| 50 | No | No | No | No | Eco-indicator 99 | No | 2013, 2014 | Europe | na | No | Plant data |
| 51 | Yes | Yes | No | No | ReCiPe 2016 | No | 2012 - 2017 | na | na | No | Ecoinvent 3 Polycarbonate {GLO}¦ market for ¦ APOS, U |
| 52 | No | No | No | No | IPCC GWP 100a | No | 2014 - 2017 | Europe | Europe | No | Ecoinvent 3.2 |
| 53 | No | No | No | No | IPCC GWP 100a | No | 2011 - 2014 | Southeast Asia | Southeast Asia | No | Literature |
| 54 | Yes | No | Yes | No | IPCC GWP 100a | No | 2010, 2016 | Europe | Europe | No | Ecoinvent |
| 54 | Yes | No | Yes | No | IPCC GWP 100a | No | 2010, 2016 | Europe | Europe | No | Ecoinvent |
| 55 | No | No | No | No | IPCC GWP 100a | No | 2007, 2018 | Europe | Europe | No | Industrial data |
| 56 | No | Yes | Yes | No | CML 2 | No | 2004, 2018 | USA | USA | No | PlasticsEurope |
| 57 | No | No | No | No | IPCC GWP 100a | Yes | 2019 | Europe | USA | Yes | Ecoinvent 3.5 |
| 58 | No | No | No | No | IPCC GWP 100a | No | 2019 | Europe | na | No | Ecoinvent 3.5 |
| 58 | No | No | No | No | IPCC GWP 100a | No | 2019 | Europe | na | No | Industrial data |
| 59 | na | na | na | na | IPCC GWP 100a | Yes | 2018 | Europe | na | Yes | PlasticsEurope |
| 60 | na | na | na | na | ReCiPe 2016 | No | 2020 | Europe | Europe | No | Ecoinvent 3.5 |
| 60 | na | na | na | na | ReCiPe 2016 | No | 2020 | Europe | Europe | No | Ecoinvent 3.5 |
| 61 | Yes | na | Yes | Yes | EPD method | No | 2016 | Europe | Europe | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 61 | Yes | na | Yes | Yes | EPD method | No | 2016 | Europe | Europe | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 62 | No | No | Yes | No | GREET | No | 2016 | USA | USA | No | Ecoinvent |
| 62 | No | No | Yes | No | GREET | No | 2016 | USA | USA | No | Ecoinvent |
| 63 | No | No | Yes | Yes | IPCC GWP 100a | No | 2014, 2015 | Brazil | Brazil | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 64 | No | No | No | No | TRACI 2.1 Midpoint | No | 2013 - 2016 | Mexico | na | No | Ecoinvent 3 Polyethylene, linear low density, granulate {GLO}¦ market for ¦ APOS, U |
| 65 | No | No | Yes | No | own calculations | No | 2002 - 2004 | USA | USA | No | Ecoinvent 3 Polyethylene, linear low density, granulate {GLO}¦ market for ¦ APOS, U |
| 66 | No | Yes | No | Yes | CML 2000 | No | 1990 - 2006 | South-Africa | South-Africa | No | Literature |
| 67 | No | No | Yes | No | ReCiPe 2016 | No | 2002, 2018 | Europe | Europe | No | Ecoinvent 3.7 lactic acid {GLO}¦ market for ¦ APOS, U) |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | Europe | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | USA | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | Europe | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | Europe | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | USA | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | USA | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | Brazil | No | PlasticsEurope |
| 68 | No | No | Yes | Yes | IPCC GWP 100a | No | 2008 - 2019 | Europe | Europe | No | PlasticsEurope |
| 69 | Yes | Yes | Yes | Yes | CML 2001 | No | 2004, 2005 | Brazil | Brazil | No | Literature |
| 70 | na | na | na | na | TRACI | Yes | 2002 - 2005 | USA | USA | Yes | Database |
| 71 | No | No | No | No | ReCiPe | Yes | 2010 | Brazil | Brazil | Yes | Literature |
| 71 | No | No | No | No | ReCiPe | Yes | 2010 | Brazil | Brazil | Yes | Literature |
| 72 | No | No | No | No | IPCC GWP 100a | No | 2010 | USA | na | No | Literature |
| 72 | No | No | No | No | IPCC GWP 100a | No | 2010 | USA | na | No | Literature |
| 72 | No | No | No | No | IPCC GWP 100a | No | 2010 | USA | na | No | Literature |
| 72 | No | No | No | No | IPCC GWP 100a | No | 2010 | USA | na | No | Literature |
| 73 | na | na | na | na | ReCiPe 2010 | No | 2010 - 2015 | Europe | Europe | No | Literature |
| 73 | na | na | na | na | ReCiPe 2010 | No | 2010 - 2015 | Europe | Europe | No | Literature |
| 74 | No | No | No | No | ReCiPe | No | 2015 | Europe | Europe | No | Literature |
| 74 | No | No | No | No | ReCiPe | No | 2015 | Europe | Europe | No | Literature |
| 75 | No | No | No | No | CML 2001 | No | 2012 | Europe | Europe | No | Literature |
| 75 | No | No | No | No | CML 2001 | No | 2012 | Europe | Europe | No | Literature |
| 76 | No | No | No | No | IPCC GWP 100a | No | 2005 | Europe | Europe | No | Literature |
| 77 | No | Yes | Yes | No | IMPACT 2000+ | No | 2010, 2011 | Europe | Europe | No | Ecoinvent 3.7 Polyol {RER}¦ market for polyol ¦ APOS, U |
| 78 | No | No | No | No | ReCiPe | Yes | 2004 - 2011 | Europe | Europe | Yes | Ecoinvent 2.1 |
| 79 | No | No | No | No | IPCC GWP 100a | No | 2012 - 2014 | Southeast Asia | Southeast Asia | No | PlasticsEurope |
| 79 | No | No | No | No | IPCC GWP 100a | No | 2012 - 2014 | Southeast Asia | Southeast Asia | No | PlasticsEurope |
| 80 | No | No | No | No | IPCC GWP 100a | No | 2018 | Europe | Europe | No | Ecoinvent 3.4 |
| 81 | No | Yes | No | Yes | IPCC GWP 100a | No | 2016 - 2019 | Europe | na | No | Ecoinvent 3.7 Phenol {RER}¦ production ¦ APOS, U |
| 82 | No | No | No | No | TRACI | Yes | 2016 - 2017 | USA | USA | Yes | Own LCA based on literature |
| 83 | No | No | No | No | CML | No | 2016 - 2017 | Europe | Europe | No | Ecoinvent 3.3 |
| 83 | No | No | No | No | CML | No | 2016 - 2017 | Europe | Europe | No | Ecoinvent 3.7 Succinic acid {GLO}¦ market for succinic acid ¦ APOS, U |
| 84 | Yes | No | No | No | ReCiPe | Yes | Na | USA | USA | Yes | Ecoinvent v2.2 |
| 84 | Yes | No | No | No | ReCiPe | Yes | Na | USA | USA | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 85 | No | No | No | No | ILCD 2011 | Yes | 2013 | Europe | Europe | Yes | Ecoinvent v2.2 |
| 86 | No | No | No | No | Na | No | 2010 | Europe | Europe | No | Literature |
| 86 | No | No | No | No | Na | No | 2010 | Europe | Europe | No | Literature |
| 86 | No | No | No | No | Na | No | 2010 | Europe | Europe | No | Literature |
| 86 | No | No | No | No | Na | No | 2010 | Europe | Brazil | No | Literature |
| 87 | No | No | No | No | Na | No | na | na | na | No | Literature/Ecoinvent |
| 87 | No | No | No | No | Na | No | na | na | na | No | NREL/literature |
| 87 | No | No | No | No | Na | No | na | na | na | No | NREL/Ecoinvent |
| 87 | No | No | No | No | Na | No | na | na | China | No | NREL |
| 87 | No | No | No | No | Na | No | na | na | Brazil | No | NREL/literature |
| 87 | No | No | No | No | na | No | na | na | na | No | NREL |
| 88 | No | No | No | No | ReCiPe | Yes | na | Europe | Europe | Yes | Ecoinvent v2.0 |
| 89 | No | Yes | No | No | TRACI 2.1 | No | na | na | Southeast Asia | No | Ecoinvent v3.1/NREL |
| 90 | No | Yes | No | No | ReCiPe | Yes | na | South-Africa | South-Africa | Yes | Ecoinvent |
| 91 | No | No | No | No | IPCC GWP 100a | No | 2011 | USA | USA | No | Ecoinvent/US LCI database |
| 91 | No | No | No | No | IPCC GWP 100a | No | 2011 | USA | USA | No | Ecoinvent/US LCI database |
| 92 | No | No | No | No | eco-indicator 99 | No | 2015 | Europe | Europe | No | Ecoinvent v2.2 |
| 92 | No | No | No | No | eco-indicator 99 | No | 2015 | Europe | Europe | No | Ecoinvent v2.2 |
| 92 | No | No | No | No | eco-indicator 99 | No | 2015 | Europe | Europe | No | Ecoinvent v2.2 |
| 92 | No | No | No | No | eco-indicator 99 | No | 2015 | Europe | Europe | No | Ecoinvent v2.2 |
| 92 | No | No | No | No | eco-indicator 99 | No | 2015 | Europe | Europe | No | Ecoinvent v2.2 |
| 92 | No | No | No | No | eco-indicator 99 | No | 2015 | Europe | Europe | No | Ecoinvent v2.2 |
| 93 | No | No | No | No | IPCC GWP 20a | No | 2013 | Brazil | Brazil | No | Ecoinvent v3.1 |
| 93 | No | No | No | No | IPCC GWP 20a | No | 2013 | Europe | Europe | No | Ecoinvent v3.1 |
| 93 | No | No | No | No | IPCC GWP 20a | No | 2013 | Brazil | Brazil | No | Ecoinvent v3.1 |
| 93 | No | No | No | No | IPCC GWP 20a | No | 2013 | USA | USA | No | Ecoinvent v3.1 |
| 93 | No | No | No | No | IPCC GWP 20a | No | 2013 | Europe | Europe | No | Ecoinvent v3.1 |
| 93 | No | No | No | No | IPCC GWP 20a | No | 2013 | USA | USA | No | Ecoinvent v3.1 |
| 94 | No | No | No | No | ReCiPe | Yes | na | na | na | Yes | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 94 | No | No | No | No | ReCiPe | Yes | na | na | na | Yes | Ecoinvent v3.7 (proponic acid {GLO}¦ market for ¦ APOS, U) |
| 94 | No | No | No | No | ReCiPe | Yes | na | na | na | Yes | Urban_&_Bakshi (2009) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2016 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2015 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2020 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2019 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2018 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2014 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 95 | No | Yes | No | No | IPCC GWP 100a | No | 2006-2017 | na | na | No | Ecoinvent 3.7 (lactic acid {GLO}¦ market for ¦ APOS, U) |
| 96 | No | No | No | No | na | No | na | na | na | No | na |
| 97 | No | No | No | No | na | Yes | na | USA | USA | Yes | PlasticsEurope |
| 98 | No | No | No | No | IPCC GWP 100a | Yes | na | USA | USA | Yes | PlasticsEurope |
| 99 | No | No | No | No | IPCC GWP 100a | Yes | na | na | na | Yes | Ecoinvent |
| 100 | No | No | No | No | ReCiPe | No | na | USA | USA | No | Ecoinvent + NREL |
| 100 | No | No | No | No | ReCiPe | No | na | USA | USA | No | Ecoinvent + NREL |
| 101 | No | No | No | No | IMPACT 2002+ v2.1 | Yes | na | USA | USA | Yes | Ecoinvent 2.0 |
| 101 | No | No | No | No | IMPACT 2002+ v2.1 | Yes | na | USA | USA | Yes | Ecoinvent 2.0 |
| 101 | No | No | No | No | IMPACT 2002+ v2.1 | Yes | na | USA | USA | Yes | Ecoinvent 2.0 |
| 102 | Yes | No | Yes | No | CML 2 | No | na | Southeast Asia | Southeast Asia | No | Ecoinvent 2.2 |
| 102 | No | No | No | No | CML 2 | No | na | Southeast Asia | Southeast Asia | No | Ecoinvent 2.2 |
| 103 | No | No | No | No | na | No | na | USA | USA | No | na |
| 104 | No | No | No | No | CML2001 | No | na | Europe | Europe | No | Ecoinvent 2.0 |
| 104 | No | No | No | No | CML2001 | No | na | Europe | Europe | No | Ecoinvent 2.0 |
| 105 | No | No | No | No | Eco-indicator 99 | No | na | Southeast Asia | Southeast Asia | No | Literature |
| 106 | No | No | No | No | IPCC GWP 100a | Yes | na | Japan | Japan | Yes | na |
| 107 | No | No | No | No | IPCC GWP 100a | Yes | na | na | na | Yes | na |
| 107 | No | No | No | No | IPCC GWP 100a | Yes | na | na | na | Yes | na |
| 107 | No | No | No | No | IPCC GWP 100a | Yes | na | na | na | Yes | na |
| 108 | No | No | No | Yes | CML2001 | Yes | na | Europe | Europe | Yes | Ecoinvent |
| 108 | No | No | No | Yes | CML2001 | Yes | na | Europe | Europe | Yes | Ecoinvent |
| 109 | No | No | No | No | ReCiPe v1.08 | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 109 | No | No | No | No | ReCiPe v1.08 | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 109 | No | No | No | No | ReCiPe v1.08 | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 109 | No | No | No | No | ReCiPe v1.08 | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 109 | No | No | No | No | ReCiPe v1.08 | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 109 | No | No | No | No | ReCiPe v1.08 | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 110 | No | No | No | No | na | Yes | na | USA | USA | Yes | Ecoinvent v3 |
| 111 | No | No | No | No | ReCiPe 2008 | Yes | na | Europe | Europe | Yes | Ecoinvent v3 |
| 111 | No | No | No | No | ReCiPe 2008 | Yes | na | Europe | Europe | Yes | Ecoinvent v3 |
| 111 | No | No | No | No | ReCiPe 2008 | Yes | na | Europe | Europe | Yes | Ecoinvent v3 |
| 111 | No | No | No | No | ReCiPe 2008 | Yes | na | Europe | Europe | Yes | Ecoinvent v3 |
| 112 | No | No | No | No | TRACI | Yes | na | USA | USA | Yes | PlasticsEurope |

Table S.10:  Response ratios (RR) of the other impact categories, including Ozone Depletion; Photochemical Ozone Formation; Fine Particulate Matter Formation; Human Toxicity; Freshwater Ecotoxcity; Marine Aquatic Ecotoxicity; Terrestial Ecotoxciity; Land Use; Water Scarcity; Depletion of Abiotic Resources.

| RR Ozone Depletion | RR Photochemical Ozone Formation | RR Fine Particulate Matter Formation | RR Human Toxicity | RR Freshwater Ecotoxicty | RR Marine Aquatic Ecotoxicty | RR Terrestial Ecotoxicty | RR Land Use | RR Water Scarcity | RR Depletion of Abiotic Resources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -1.89712 | -0.40547 | 0.589963 | -0.55354 | -1.60944 | -1.64866 | -1.79176 | 4.171534 | 1.578666 | 9.454537 |
| -1.89712 | -0.09531 | 0.36346 | -1.15924 | -1.94591 | -1.44238 | -1.89712 | 2.063105 | 2.041894 | -1.28093 |
| -1.38629 | -0.40547 | -3.44129 | -1.04145 | -1.94591 | -1.4816 | -1.89712 | 6.294419 | 0.921303 | -1.28093 |
| -6.34116 | 2.289513 | 0.723178 | -0.06062 | -1.07044 | -0.60614 | -1.38629 | 9.69118 | 0.135887 | -0.11778 |
| -6.21583 | -2.12493 |  | -4.48211 | -5.31909 | -6.24662 | -5.76407 | -0.12045 | -0.01036 | -7.34307 |
| -2.28317 | -5.85024 |  | -4.33551 | -5.21411 | -6.19427 | -5.70912 |  |  | -7.51635 |
| -0.65678 | -6.14965 |  | -0.47957 | -1.31593 | -2.20355 | -1.72988 |  |  | -3.18395 |
| -1.81529 | 2.673917 |  | 2.70805 | 2.662588 | 1.988928 | 4.382027 |  |  | 5.214936 |
| -3.71241 | 1.942582 |  | 2.484907 | 1.791759 | 1.072637 | 3.850148 |  |  | 3.178054 |
| -5.37064 | 0.953838 |  | 3.028522 | 3.028522 | 2.628007 | 4.543295 |  |  | 5.928258 |
| 3.32751 | -2.87313 |  | 0.341891 | 0.494381 | 0.411769 | 0.174135 |  |  | -0.10681 |
| 0.388223 | 0.488847 |  | -0.39465 | 6.746487 | 5.488197 | 6.907755 |  |  | -0.11231 |
| 3.026191 | 4.350851 |  | -1.10598 | 7.094794 | 6.483625 | 7.513891 |  |  | -0.47629 |
| 1.224382 | 0.044717 |  | -2.69464 | 0.521637 | -2.69724 | -2.00409 |  |  | -1.94061 |
| -1.34373 | -0.79851 |  | -0.57042 | -0.16376 | -0.22677 | -0.38394 |  |  |  |
| -1.4929 | -0.40547 |  | 1.496323 | -0.50704 | 0.047628 | -4.3305 |  |  |  |
| -0.25783 | -0.21233 |  | -2.34733 | 2.933697 | -0.88986 | -1.3225 |  |  |  |
| -0.74924 | -1.9521 |  | -3.19386 | -0.4097 | -0.21963 | -1.25804 |  |  |  |
| -0.21233 | -0.73584 |  | -2.54297 | -1.80586 |  | -3.38684 |  |  |  |
| -0.69315 | -0.37098 |  |  |  |  |  |  |  |  |
| -0.3902 | -3.11066 |  |  |  |  |  |  |  |  |
| 0.156141 | -0.40547 |  |  |  |  |  |  |  |  |
| -1.50538 | -1.61498 |  |  |  |  |  |  |  |  |
| -1.44836 | -0.59006 |  |  |  |  |  |  |  |  |
| 3.158251 | -2.12026 |  |  |  |  |  |  |  |  |
| 0.056353 | -2.13425 |  |  |  |  |  |  |  |  |
| 0.728738 | 3.973432 |  |  |  |  |  |  |  |  |
| 0.172613 | 3.957634 |  |  |  |  |  |  |  |  |
| 0.19671 | -0.05349 |  |  |  |  |  |  |  |  |
| 0.106768 | 2.079442 |  |  |  |  |  |  |  |  |
| 0.789774 | -0.70678 |  |  |  |  |  |  |  |  |
| 0.747955 | 0.976915 |  |  |  |  |  |  |  |  |
| 0.309188 | 0.67475 |  |  |  |  |  |  |  |  |
| 0.214657 | 3.273364 |  |  |  |  |  |  |  |  |
| 0.24818 | 0.566492 |  |  |  |  |  |  |  |  |
| 0.806224 | -1.67909 |  |  |  |  |  |  |  |  |
| 0.362293 | 1.275069 |  |  |  |  |  |  |  |  |
| -0.82668 | 1.332227 |  |  |  |  |  |  |  |  |
| 5.182516 | 1.486378 |  |  |  |  |  |  |  |  |
| -4.31096 | 1.098612 |  |  |  |  |  |  |  |  |
| 1.185624 | 1.289668 |  |  |  |  |  |  |  |  |
| 1.157453 | 1.025853 |  |  |  |  |  |  |  |  |
| 1.363305 |  |  |  |  |  |  |  |  |  |
| 1.493925 |  |  |  |  |  |  |  |  |  |
| 1.627456 |  |  |  |  |  |  |  |  |  |
| 2.090741 |  |  |  |  |  |  |  |  |  |

# Data References

1.	Urban, R. A. & Bakshi, B. R. 1,3-Propanediol from fossils versus biomass: A life cycle evaluation of emissions and ecological resources. Ind. Eng. Chem. Res. 48, 8068–8082 (2009).

2.	Aryapratama, R. & Janssen, M. Prospective life cycle assessment of bio-based adipic acid production from forest residues. J. Clean. Prod. 164, 434–443 (2017).

3.	Benalcázar, E. A., Deynoot, B. G., Noorman, H., Osseweijer, P. & Posada, J. A. Production of bulk chemicals from lignocellulosic biomass via thermochemical conversion and syngas fermentation: a comparative techno‐economic and environmental assessment of different site‐specific supply chain configurations. Biofuels, Bioprod. Biorefining 11, 861–886 (2017).

4.	Ekman, A. & Börjesson, P. Environmental assessment of propionic acid produced in an agricultural biomass-based biorefinery system. J. Clean. Prod. 19, 1257–1265 (2011).

5.	Liptow, C. & Tillman, A.-M. A Comparative Life Cycle Assessment Study of Polyethylene Based on Sugarcane and Crude Oil. J. Ind. Ecol. 16, 420–435 (2012).

6.	Liptow, C., Tillman, A. M., Janssen, M., Wallberg, O. & Taylor, G. A. Ethylene based on woody biomass - What are environmental key issues of a possible future Swedish production on industrial scale. Int. J. Life Cycle Assess. 18, 1071–1081 (2013).

7.	Tufvesson, P., Ekman, A., Sardari, R. R. R., Engdahl, K. & Tufvesson, L. Economic and environmental assessment of propionic acid production by fermentation using different renewable raw materials. Bioresour. Technol. 149, 556–564 (2013).

8.	Zhang, Y., Hu, G. & Brown, R. C. Life cycle assessment of commodity chemical production from forest residue via fast pyrolysis. Int. J. Life Cycle Assess. 19, 1371–1381 (2014).

9.	Fiorentino, G., Ripa, M., Mellino, S., Fahd, S. & Ulgiati, S. Life cycle assessment of Brassica carinata biomass conversion to bioenergy and platform chemicals. J. Clean. Prod. 66, 174–187 (2014).

10.	Adom, F., Dunn, J. B., Han, J. & Sather, N. Life-cycle fossil energy consumption and greenhouse gas emissions of bioderived chemicals and their conventional counterparts. Environ. Sci. Technol. 48, 14624–14631 (2014).

11.	Cok, B., Tsiropoulos, I., Roes, A. L. & Patel, M. K. Succinic acid production derived from carbohydrates: An energy and greenhouse gas assessment of a platform chemical toward a bio-based economy. Biofuels, Bioprod. Biorefining 8, 16–29 (2014).

12.	Khoo, H. H., Wong, L. L., Tan, J., Isoni, V. & Sharratt, P. Synthesis of 2-methyl tetrahydrofuran from various lignocellulosic feedstocks: Sustainability assessment via LCA. Resour. Conserv. Recycl. 95, 174–182 (2015).

13.	Gonzalez-Garay, A., Gonzalez-Miquel, M. & Guillen-Gosalbez, G. High-Value Propylene Glycol from Low-Value Biodiesel Glycerol: A Techno-Economic and Environmental Assessment under Uncertainty. ACS Publ. 5, 5723–5732 (2017).

14.	Gunukula, S., Runge, T. & Anex, R. Assessment of Biocatalytic Production Parameters to Determine Economic and Environmental Viability. ACS Sustain. Chem. Eng. 5, 8119–8126 (2017).

15.	Mandegari, M. A., Farzad, S., van Rensburg, E. & Görgens, J. F. Multi-criteria analysis of a biorefinery for co-production of lactic acid and ethanol from sugarcane lignocellulose. Biofuels, Bioprod. Biorefining 11, 971–990 (2017).

16.	Zucaro, A., Forte, A. & Fierro, A. Greenhouse gas emissions and non-renewable energy use profiles of bio-based succinic acid from Arundo donax L. lignocellulosic feedstock. Clean Technol. Environ. Policy 19, 2129–2143 (2017).

17.	Chen, W. S., Strik, D. P. B. T. B., Buisman, C. J. N. & Kroeze, C. Production of Caproic Acid from Mixed Organic Waste: An Environmental Life Cycle Perspective. Environ. Sci. Technol. 51, 7159–7168 (2017).

18.	Zheng, J. L., Zhu, Y. H., Zhu, M. Q., Sun, G. T. & Sun, R. C. Life-cycle assessment and techno-economic analysis of the utilization of bio-oil components for the production of three chemicals. Green Chem. 20, 3287–3301 (2018).

19.	Tripodi, A. et al. Acetonitrile from Bioethanol Ammoxidation: Process Design from the Grass-Roots and Life Cycle Analysis. ACS Sustain. Chem. Eng. 6, 5441–5451 (2018).

20.	Yang, M., Tian, X. & You, F. Manufacturing Ethylene from Wet Shale Gas and Biomass: Comparative Technoeconomic Analysis and Environmental Life Cycle Assessment. Ind. Eng. Chem. Res. 57, 5980–5998 (2018).

21.	Alonso-Fariñas, B., Gallego-Schmid, A., Haro, P. & Azapagic, A. Environmental assessment of thermo-chemical processes for bio-ethylene production in comparison with bio-chemical and fossil-based ethylene. J. Clean. Prod. 202, 817–829 (2018).

22.	Brunklaus, B., Rex, E., Carlsson, E. & Berlin, J. The future of Swedish food waste: An environmental assessment of existing and prospective valorization techniques. J. Clean. Prod. 202, 1–10 (2018).

23.	González-García, S., Argiz, L., Míguez, P. & Gullón, B. Exploring the production of bio-succinic acid from apple pomace using an environmental approach. Chem. Eng. J. 350, 982–991 (2018).

24.	Jonker, J. G. G. et al. Economic performance and GHG emission intensity of sugarcane‐ and eucalyptus‐derived biofuels and biobased chemicals in Brazil. Biofuels, Bioprod. Biorefining 13, 950–977 (2019).

25.	Kim, H., Choi, J., Park, J. & Won, W. Production of a sustainable and renewable biomass-derived monomer: conceptual process design and techno-economic analysis. Green Chem. 22, 7070–7079 (2020).

26.	Wang, J., You, S., Lu, Z., Chen, R. & Xu, F. Life cycle assessment of bio-based levoglucosan production from cotton straw through fast pyrolysis. Bioresour. Technol. 307, 123179 (2020).

27.	Lokesh, K. et al. Hybridised sustainability metrics for use in life cycle assessment of bio-based products: Resource efficiency and circularity. Green Chem. 22, 803–813 (2020).

28.	van Duuren, J. B. J. H. et al. A limited LCA of bio-adipic acid: Manufacturing the nylon-6,6 precursor adipic acid using the benzoic acid degradation pathway from different feedstocks. Biotechnol. Bioeng. 108, 1298–1306 (2011).

29.	Ögmundarson, Ó., Sukumara, S., Laurent, A. & Fantke, P. Environmental hotspots of lactic acid production systems. GCB Bioenergy 12, 19–38 (2020).

30.	Yang, M. & Rosentrater, K. A. Life Cycle Assessment and Techno-Economic Analysis of Pressure Sensitive Bio-Adhesive Production. Energies 2019, Vol. 12, Page 4502 12, 4502 (2019).

31.	Yang, M. & Rosentrater, K. A. Cradle-to-gate life cycle assessment of structural bio-adhesives derived from glycerol. Int. J. Life Cycle Assess. 26, 799–806 (2021).

32.	Arias, A., González-García, S., González-Rodríguez, S., Feijoo, G. & Moreira, M. T. Cradle-to-gate Life Cycle Assessment of bio-adhesives for the wood panel industry. A comparison with petrochemical alternatives. Sci. Total Environ. 738, 140357 (2020).

33.	Le Duigou, A., Deux, J. M., Davies, P. & Baley, C. PLLA/flax mat/balsa bio-sandwich-environmental impact and simplified life cycle analysis. Appl. Compos. Mater. 19, 363–378 (2012).

34.	Mahalle, L., Alemdar, A., Mihai, M. & Legros, N. A cradle-to-gate life cycle assessment of wood fibre-reinforced polylactic acid (PLA) and polylactic acid/thermoplastic starch (PLA/TPS) biocomposites. Int. J. Life Cycle Assess. 19, 1305–1315 (2014).

35.	Deng, Y. et al. Life cycle assessment of flax-fibre reinforced epoxidized linseed oil composite with a flame retardant for electronic applications. J. Clean. Prod. 133, 427–438 (2016).

36.	Quintana, A., Alba, J., del Rey, R. & Guillén-Guillamón, I. Comparative Life Cycle Assessment of gypsum plasterboard and a new kind of bio-based epoxy composite containing different natural fibers. J. Clean. Prod. 185, 408–420 (2018).

37.	Ita-Nagy, D., Vázquez-Rowe, I., Kahhat, R., Chinga-Carrasco, G. & Quispe, I. Reviewing environmental life cycle impacts of biobased polymers: current trends and methodological challenges. International Journal of Life Cycle Assessment vol. 25 2169–2189 (2020).

38.	Akiyama, M., Tsuge, T. & Doi, Y. Environmental life cycle comparison of polyhydroxyalkanoates produced from renewable carbon resources by bacterial fermentation. Polym. Degrad. Stab. 80, 183–194 (2003).

39.	Kim, S. & Dale, B. E. Environmental aspects of ethanol derived from no-tilled corn grain: nonrenewable energy consumption and greenhouse gas emissions. Biomass and Bioenergy 28, 475–489 (2005).

40.	Broeren, M. L. M. et al. Environmental assessment of bio‐based chemicals in early‐stage development: a review of methods and indicators. Biofuels, Bioprod. Biorefining 11, 701–718 (2017).

41.	Piccinno, F., Hischier, R., Seeger, S. & Som, C. Predicting the environmental impact of a future nanocellulose production at industrial scale: Application of the life cycle assessment scale-up framework. J. Clean. Prod. 174, 283–295 (2018).

42.	Tan, L. et al. Combining ex-ante LCA and EHS screening to assist green design: A case study of cellulose nanocrystal foam. J. Clean. Prod. 178, 494–506 (2018).

43.	Ekman, A. & Börjesson, P. Life cycle assessment of mineral oil-based and vegetable oil-based hydraulic fluids including comparison of biocatalytic and conventional production methods. Int. J. Life Cycle Assess. 16, 297–305 (2011).

44.	Brière, R. et al. Life cycle assessment of the production of surface-active alkyl polyglycosides from acid-assisted ball-milled wheat straw compared to the conventional production based on corn-starch. Green Chem. 20, 2135–2141 (2018).

45.	Montazeri, M. & Eckelman, M. J. Life cycle assessment of UV-Curable bio-based wood flooring coatings. J. Clean. Prod. 192, 932–939 (2018).

46.	Adom, F. K. & Dunn, J. B. Life cycle analysis of corn-stover-derived polymer-grade l-lactic acid and ethyl lactate: greenhouse gas emissions and fossil energy consumption. Biofuels, Bioprod. Biorefining 11, 258–268 (2017).

47.	Garcia Gonzalez, M. N., Levi, M. & Turri, S. Development of polyester binders for the production of sustainable polyurethane coatings: Technological characterization and life cycle assessment. J. Clean. Prod. 164, 171–178 (2017).

48.	Kikuchi, Y., Oshita, Y., Mayumi, K. & Hirao, M. Greenhouse gas emissions and socioeconomic effects of biomass-derived products based on structural path and life cycle analyses: A case study of polyethylene and polypropylene in Japan. J. Clean. Prod. 167, 289–305 (2017).

49.	de Léis, C. M., Nogueira, A. R., Kulay, L. & Tadini, C. C. Environmental and energy analysis of biopolymer film based on cassava starch in Brazil. J. Clean. Prod. 143, 76–89 (2017).

50.	Petrucci, R. et al. Life Cycle Analysis of Extruded Films Based on Poly(lactic acid)/Cellulose Nanocrystal/Limonene: A Comparative Study with ATBC Plasticized PLA/OMMT Systems. J. Polym. Environ. 26, 1891–1902 (2018).

51.	Zhang, D., del Rio-Chanona, E. A., Wagner, J. L. & Shah, N. Life cycle assessments of bio-based sustainable polylimonene carbonate production processes. Sustain. Prod. Consum. 14, 152–160 (2018).

52.	García González, M. N., Börjesson, P., Levi, M. & Turri, S. Development and Life Cycle Assessment of Polyester Binders Containing 2,5-Furandicarboxylic Acid and Their Polyurethane Coatings. J. Polym. Environ. 26, 3626–3637 (2018).

53.	Cheroennet, N., Pongpinyopap, S., Leejarkpai, T. & Suwanmanee, U. A trade-off between carbon and water impacts in bio-based box production chains in Thailand: A case study of PS, PLAS, PLAS/starch, and PBS. J. Clean. Prod. 167, 987–1001 (2017).

54.	Patel, M. K. et al. Second-generation bio-based plastics are becoming a reality - Non-renewable energy and greenhouse gas (GHG) balance of succinic acid-based plastic end products made from lignocellulosic biomass. Biofuels, Bioprod. Biorefining 12, 426–441 (2018).

55.	Warlin, N. et al. A rigid spirocyclic diol from fructose-based 5-hydroxymethylfurfural: synthesis, life-cycle assessment, and polymerization for renewable polyesters and poly(urethane-urea)s. Green Chem. 21, 6667–6684 (2019).

56.	Kookos, I. K., Koutinas, A. & Vlysidis, A. Life cycle assessment of bioprocessing schemes for poly(3-hydroxybutyrate) production using soybean oil and sucrose as carbon sources. Resour. Conserv. Recycl. 141, 317–328 (2019).

57.	Manzardo, A. et al. Life Cycle Assessment Framework To Support the Design of Biobased Rigid Polyurethane Foams. ACS Omega 4, 14114–14123 (2019).

58.	Mankar, S. V. et al. Synthesis, Life Cycle Assessment, and Polymerization of a Vanillin-Based Spirocyclic Diol toward Polyesters with Increased Glass-Transition Temperature. ACS Sustain. Chem. Eng. 7, 19090–19103 (2019).

59.	Moretti, C., Junginger, M. & Shen, L. Environmental life cycle assessment of polypropylene made from used cooking oil. Resour. Conserv. Recycl. 157, 104750 (2020).

60.	Fridrihsone, A., Romagnoli, F., Kirsanovs, V. & Cabulis, U. Life Cycle Assessment of vegetable oil based polyols for polyurethane production. J. Clean. Prod. 266, 121403 (2020).

61.	Parajuli, R. et al. Environmental impacts of producing bioethanol and biobased lactic acid from standalone and integrated biorefineries using a consequential and an attributional life cycle assessment approach. Sci. Total Environ. 598, 497–512 (2017).

62.	Cai, H. et al. Life-cycle analysis of integrated biorefineries with co-production of biofuels and bio-based chemicals: co-product handling methods and implications. Biofuels, Bioprod. Biorefining 12, 815–833 (2018).

63.	Fernandez-Dacosta, C. et al. Can we assess innovative bio-based chemicals in their early development stage? A comparison between early-stage and life cycle assessments. J. Clean. Prod. 230, 137–149 (2019).

64.	Mercado, G., Dominguez, M., Herrera, I. & Melgoza, R. M. Are Polymers Toxic? Case Study: Environmental Impact of a Biopolymer. J. Environ. Sci. Eng. B 6, 121–126 (2017).

65.	Vink, E. T. H. et al. The Sustainability of NatureWorksTM Polylactide Polymers and IngeoTM Polylactide Fibers: an Update of the Future. Macromol. Biosci. 4, 551–564 (2004).

66.	Harding, K. G., Dennis, J. S., von Blottnitz, H. & Harrison, S. T. L. Environmental analysis of plastic production processes: Comparing petroleum-based polypropylene and polyethylene with biologically-based poly-β-hydroxybutyric acid using life cycle analysis. J. Biotechnol. 130, 57–66 (2007).

67.	Pachón, E. R., Mandade, P. & Gnansounou, E. Conversion of vine shoots into bioethanol and chemicals: Prospective LCA of biorefinery concept. Bioresour. Technol. 303, 122946 (2020).

68.	Vera, I., Hoefnagels, R., van der Kooij, A., Moretti, C. & Junginger, M. A carbon footprint assessment of multi-output biorefineries with international biomass supply: a case study for the Netherlands. Biofuels, Bioprod. Biorefining 14, 198–224 (2020).

69.	Zah, R., Hischier, R., Leão, A. L. & Braun, I. Curauá fibers in the automobile industry – a sustainability assessment. J. Clean. Prod. 15, 1032–1040 (2007).

70.	Kim, S., Dale, B. E., Drzal, L. T. & Misra, M. Life Cycle Assessment of Kenaf Fiber Reinforced Biocomposite. J. Biobased Mater. Bioenergy 2, 85–93 (2008).

71.	De Figueirêdo, M. C. B. et al. Life cycle assessment of cellulose nanowhiskers. J. Clean. Prod. 35, 130–139 (2012).

72.	Li, Q., McGinnis, S., Sydnor, C., Wong, A. & Renneckar, S. Nanocellulose life cycle assessment. ACS Sustain. Chem. Eng. 1, 919–928 (2013).

73.	Manda, B. M. K., Worrell, E. & Patel, M. K. Prospective life cycle assessment of an antibacterial T-shirt and supporting business decisions to create value. Resour. Conserv. Recycl. 103, 47–57 (2015).

74.	Piccinno, F., Hischier, R., Seeger, S. & Som, C. Life cycle assessment of a new technology to extract, functionalize and orient cellulose nanofibers from food waste. ACS Sustain. Chem. Eng. 3, 1047–1055 (2015).

75.	Hervy, M., Evangelisti, S., Lettieri, P. & Lee, K. Y. Life cycle assessment of nanocellulose-reinforced advanced fibre composites. Compos. Sci. Technol. 118, 154–162 (2015).

76.	Tufvesson, L. M. & Börjesson, P. Wax production from renewable feedstock using biocatalysts instead of fossil feedstock and conventional methods. Int. J. Life Cycle Assess. 13, 328–338 (2008).

77.	Bernier, E., Lavigne, C. & Robidoux, P. Y. Life cycle assessment of kraft lignin for polymer applications. Int. J. Life Cycle Assess. 18, 520–528 (2013).

78.	Deng, Y., Achten, W. M. J., Van Acker, K. & Duflou, J. R. Life cycle assessment of wheat gluten powder and derived packaging film. Biofuels, Bioprod. Biorefining 7, 429–458 (2013).

79.	Pang, M. M., Pun, M. Y., Chow, W. S. & Ishak, Z. A. M. Carbon footprint calculation for thermoformed starch-filled polypropylene biobased materials. J. Clean. Prod. 64, 602–608 (2014).

80.	Heidari, M. D., Lawrence, M., Blanchet, P. & Amor, B. Regionalised Life Cycle Assessment of Bio-Based Materials in Construction; the Case of Hemp Shiv Treated with Sol-Gel Coatings. Mater. 2019, Vol. 12, Page 2987 12, 2987 (2019).

81.	Frascari, D., Molina Bacca, A. E., Wardenaar, T., Oertlé, E. & Pinelli, D. Continuous flow adsorption of phenolic compounds from olive mill wastewater with resin XAD16N: life cycle assessment, cost–benefit analysis and process optimization. J. Chem. Technol. Biotechnol. 94, 1968–1981 (2019).

82.	Ghasemi, S., Sibi, M. P., Ulven, C. A., Webster, D. C. & Pourhashem, G. A Preliminary Environmental Assessment of Epoxidized Sucrose Soyate (ESS)-Based Biocomposite. Mol. 2020, Vol. 25, Page 2797 25, 2797 (2020).

83.	Sadhukhan, J. et al. Novel macroalgae (seaweed) biorefinery systems for integrated chemical, protein, salt, nutrient and mineral extractions and environmental protection by green synthesis and life cycle sustainability assessments. Green Chem. 21, 2635–2655 (2019).

84.	Lin, Z., Nikolakis, V. & Ierapetritou, M. Life cycle assessment of biobased p -xylene production. Ind. Eng. Chem. Res. 54, 2366–2378 (2015).

85.	Dros, A. B., Larue, O., Reimond, A., De Campo, F. & Pera-Titus, M. Hexamethylenediamine (HMDA) from fossil- vs. bio-based routes: an economic and life cycle assessment comparative study. Green Chem. 17, 4760–4772 (2015).

86.	Liptow, C., Tillman, A. M. & Janssen, M. Life cycle assessment of biomass-based ethylene production in Sweden — is gasification or fermentation the environmentally preferable route? Int. J. Life Cycle Assess. 20, 632–644 (2015).

87.	Khoo, H. H., Ee, W. L. & Isoni, V. Bio-chemicals from lignocellulose feedstock: Sustainability, LCA and the green conundrum. Green Chem. 18, 1912–1922 (2016).

88.	Forte, A., Zucaro, A., Basosi, R. & Fierro, A. LCA of 1,4-Butanediol Produced via Direct Fermentation of Sugars from Wheat Straw Feedstock within a Territorial Biorefinery. Mater. 2016, Vol. 9, Page 563 9, 563 (2016).

89.	Montazeri, M. & Eckelman, M. J. Life Cycle Assessment of Catechols from Lignin Depolymerization. ACS Sustain. Chem. Eng. 4, 708–718 (2016).

90.	Gezae Daful, A. & Görgens, J. F. Techno-economic analysis and environmental impact assessment of lignocellulosic lactic acid production. Chem. Eng. Sci. 162, 53–65 (2017).

91.	Moussa, H. I., Elkamel, A. & Young, S. B. Assessing energy performance of bio-based succinic acid production using LCA. J. Clean. Prod. 139, 761–769 (2016).

92.	Morales, M. et al. Sustainability assessment of succinic acid production technologies from biomass using metabolic engineering. Energy Environ. Sci. 9, 2794–2805 (2016).

93.	Cespi, D., Passarini, F., Vassura, I. & Cavani, F. Butadiene from biomass, a life cycle perspective to address sustainability in the chemical industry. Green Chem. 18, 1625–1638 (2016).

94.	Gargalo, C. L. et al. Assessing the environmental sustainability of early stage design for bioprocesses under uncertainties: An analysis of glycerol bioconversion. J. Clean. Prod. 139, 1245–1260 (2016).

95.	Morales, M. et al. Environmental and economic assessment of lactic acid production from glycerol using cascade bio- and chemocatalysis. Energy Environ. Sci. 8, 558–567 (2015).

96.	Yu, J. & Chen, L. X. L. The Greenhouse Gas Emissions and Fossil Energy Requirement of Bioplastics from Cradle to Gate of a Biomass Refinery. Environ. Sci. Technol. 42, 6961–6966 (2008).

97.	Kim, S. & Dale, B. E. Energy and greenhouse gas profiles of polyhydroxybutyrates derived from corn grain: A life cycle perspective. Environ. Sci. Technol. 42, 7690–7695 (2008).

98.	Kendall, A. A life cycle assessment of biopolymer production from material recovery facility residuals. Resour. Conserv. Recycl. 61, 69–74 (2012).

99.	Eerhart, A. J. J. E., Faaij, A. P. C. & Patel, M. K. Replacing fossil based PET with biobased PEF; Process analysis, energy and GHG balance. Energy Environ. Sci. 5, 6407–6422 (2012).

100.	Nuss, P. & Gardner, K. H. Attributional life cycle assessment (ALCA) of polyitaconic acid production from northeast US softwood biomass. Int. J. Life Cycle Assess. 18, 603–612 (2013).

101.	Akanuma, Y., Selke, S. E. M. & Auras, R. A preliminary LCA case study: Comparison of different pathways to produce purified terephthalic acid suitable for synthesis of 100 % bio-based PET. Int. J. Life Cycle Assess. 19, 1238–1246 (2014).

102.	Papong, S. et al. Comparative assessment of the environmental profile of PLA and PET drinking water bottles from a life cycle perspective. J. Clean. Prod. 65, 539–550 (2014).

103.	Hohenschuh, W., Kumar, D. & Murthy, G. S. Economic and cradle-to-gate life cycle assessment of poly-3-hydroxybutyrate production from plastic producing, genetically modified hybrid poplar leaves. J. Renew. Sustain. Energy 6, 063113 (2014).

104.	Heimersson, S., Morgan-Sagastume, F., Peters, G. M., Werker, A. & Svanström, M. Methodological issues in life cycle assessment of mixed-culture polyhydroxyalkanoate production utilising waste as feedstock. N. Biotechnol. 31, 383–393 (2014).

105.	Zolkarnain, N. et al. Evaluation of environmental impacts and GHG of palm polyol production using life cycle assessment approach. J. oil Palm Res. 27, 144–155 (2015).

106.	Sun, X. Z., Minowa, T., Yamaguchi, K. & Genchi, Y. Evaluation of energy consumption and greenhouse gas emissions from poly(phenyllactic acid) production using sweet sorghum. J. Clean. Prod. 87, 208–215 (2015).

107.	Tecchio, P., Freni, P., De Benedetti, B. & Fenouillot, F. Ex-ante Life Cycle Assessment approach developed for a case study on bio-based polybutylene succinate. J. Clean. Prod. 112, 316–325 (2016).

108.	Gontia, P. & Janssen, M. Life cycle assessment of bio-based sodium polyacrylate production from pulp mill side streams: case study of thermo-mechanical and sulfite pulp mills. J. Clean. Prod. 131, 475–484 (2016).

109.	Chen, L., Pelton, R. E. O. & Smith, T. M. Comparative life cycle assessment of fossil and bio-based polyethylene terephthalate (PET) bottles. J. Clean. Prod. 137, 667–676 (2016).

110.	Posen, I. D., Jaramillo, P. & Griffin, W. M. Uncertainty in the Life Cycle Greenhouse Gas Emissions from U.S. Production of Three Biobased Polymer Families. Environ. Sci. Technol. 50, 2846–2858 (2016).

111.	Belboom, S. & Léonard, A. Does biobased polymer achieve better environmental impacts than fossil polymer? Comparison of fossil HDPE and biobased HDPE produced from sugar beet and wheat. Biomass and Bioenergy 85, 159–167 (2016).

112.	Kim, S. & Dale, B. E. Life Cycle Assessment Study of Biopolymers (Polyhydroxyalkanoates) - Derived from No-Tilled Corn (11 pp). Int. J. Life Cycle Assess. 2005 103 10, 200–210 (2004).