Supplemental Table 1. Cost breakdown of point-of-use fluoride biosensors compared to operating cost for a fluoride photometer.

| Item | Vendor | Catalog Number | Cost (USD) | Cost per kit (USD) | Notes |
| --- | --- | --- | --- | --- | --- |
| Cell-Free Reaction | N/A | N/A | $4.66 | $0.56 | 3 tests, 3 positive controls |
| Catechol | Alfa Aesar | A10164 | $284 | $0 | Price is a fraction of a cent |
| PCR Tubes | BrandTech Scientific | 781332 | $77.12 | $0.48 | 6 tubes |
| 20 µL micropipette | Safe-Tec LLC | 1020 | $1,500 | $0.90 | 6 pipettes |
| Dessicant | Uline | S-19581 | $47 | $0.06 | 1 card |
| Vacuum bag | Amazon | B075KKWFYN | $18 | $0.09 | 6 inches |
| Light-protectant bag | Uline | S-11661 | $85 | $0.09 | 1 bag |
| Total (per test kit) |  |  |  | $2.17 | 3 samples |
| Total (per test) |  |  |  | $0.72 |  |
| Photometer Reagents | Hanna Instruments | HI93739-03 |  | $266 | 300 samples |
| Total (per test) |  |  |  | $0.89 |  |

Supplemental Table 2. Sources and test results for water tests used to determine point-of-use fluoride biosensor accuracy (n=57).

| Sample Number | Water Source | Sample Treated? | Positive Control On? | Fluoride (ppm) | Test On? | Time to Result | Test Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 107-1 | Rainwater Collection | No | Yes | 0 | No | No color change | True Negative |
| 107-2 | Mixed Rain/Borehole | No | Yes | 5.4 | Yes | 3-4h | True Positive |
| 107-3 | Borehole | No | Yes | 6.5 | Yes | 2h | True Positive |
| 108-1 | Rainwater Collection | No | Yes | 0 | No | No color change | True Negative |
| 108-2 | Borehole | No | Yes | 6.5 | Yes | 4-5h | True Positive |
| 109-1 | Borehole | No | Yes | 5.7 | Yes | 5h | True Positive |
| 110-1 | Borehole | No | Yes | 6.6 | Yes | 1-2h | True Positive |
| 110-2 | Borehole | No | Yes | 4.9 | Yes | 1-2h | True Positive |
| 111-1 | Borehole | No | Yes | 5.8 | Yes | 2-3h | True Positive |
| 112-1 | Borehole | No | Yes | 5.6 | Yes | 2-3h | True Positive |
| 113-1 | Rainwater Collection | No | Yes | 0 | No | No color change | True Negative |
| 113-2 | Mixed Rain/Borehole | No | Yes | 7.9 | Yes | 2-3h | True Positive |
| 114-1 | Borehole | No | Yes | 6.2 | Yes | 1-2h | True Positive |
| 114-2 | Borehole | No | Yes | 5.5 | Yes | 3-4h | True Positive |
| 114-3 | Mixed Rain/Borehole | No | No | 5.3 | Yes | 5h | True Positive |
| 115-1 | Borehole | No | Yes | 6.1 | Yes | 2-3h | True Positive |
| 116-1 | Borehole | No | Yes | 6 | Yes | 4-5h | True Positive |
| 117-1 | Mixed Rain/Borehole | No | Yes | 3.9 | Yes | 3-4h | True Positive |
| 118-1 | Protected Dug Well | No | Yes | 8.8 | Yes | 1-2h | True Positive |
| 118-2 | Borehole | Unknown | Yes | 5.8 | Yes | 3-4h | True Positive |
| 119-1 | Rainwater Collection | No | No | 0 | Yes | 5h | False Positive |
| 119-2 | Protected Dug Well | No | Yes | 7.4 | Yes | 2-3h | True Positive |
| 120-1 | Borehole | Unknown | Yes | 5.7 | Yes | 5h | True Positive |
| 121-1 | Borehole | No | Yes | 5.8 | Yes | 3-4h | True Positive |
| 121-2 | Rainwater Collection | No | No | 1.1 | Yes | 3-4h | False Positive |
| 122-1 | Borehole | No | No | 6 | Yes | 3-4h | True Positive |
| 123-1 | Protected Dug Well | No | Yes | 7.7 | Yes | 2-3h | True Positive |
| 124-1 | NAWASCO | Yes | Yes | 5 | Yes | 3-4h | True Positive |
| 124-2 | Protected Dug Well | No | Yes | 15.8 | Yes | 1-2h | True Positive |
| 125-1 | Protected Dug Well | No | Yes | 18.7 | Yes | 3-4h | True Positive |
| 125-2 | Borehole | No | Yes | 15.8 | Yes | 2-3h | True Positive |
| 126-1 | Rainwater Collection | No | Yes | 0.4 | No | No color change | True Negative |
| 126-2 | Protected Dug Well | No | Yes | 17 | Yes | 1-2h | True Positive |
| 127-1 | Borehole | No | Yes | 5.1 | Yes | 5h | True Positive |
| 128-1 | Borehole | No | Yes | 5 | Yes | 3-4h | True Positive |
| 129-1 | Borehole | No | Yes | 5.4 | Yes | 2-3h | True Positive |
| 130-1 | Rainwater Collection | No | Yes | 0.4 | No | No color change | True Negative |
| 130-2 | Borehole | No | Yes | 7.2 | Yes | 2-3h | True Positive |
| 131-1 | Protected Dug Well | No | Yes | 8.8 | Yes | 4-5h | True Positive |
| 132-1 | Rainwater Collection | No | Yes | 0 | Yes | 5h | False Positive |
| 132-2 | Protected Dug Well | No | Yes | 10.8 | Yes | 3-4h | True Positive |
| 133-1 | Rainwater Collection | Yes | Yes | 0.5 | No | No color change | True Negative |
| 133-2 | Protected Dug Well | No | Yes | 8.8 | Yes | 5h | True Positive |
| 134-1 | Rainwater Collection | Yes | Yes | 0.8 | No | No color change | True Negative |
| 134-2 | Protected Dug Well | No | Yes | 8.7 | Yes | 1-2h | True Positive |
| 135-1 | Surface Water | No | Yes | 20 | No | No color change | False Negative |
| 135-2 | Borehole | No | Yes | 6 | Yes | 5h | True Positive |
| 136-1 | Borehole | No | Yes | 6.3 | Yes | 5h | True Positive |
| 137-1 | Borehole | No | No | 6.6 | No | No color change | False Negative |
| 137-2 | Bagged Water | Yes | No | 0.1 | No | No color change | True Negative |
| 138-1 | Rainwater Collection | No | No | 5.5 | No | No color change | False Negative |
| 138-2 | Borehole | No | No | 0.3 | No | No color change | True Negative |
| 139-1 | Borehole | No | Yes | 5.9 | Yes | 5h | True Positive |
| 139-2 | Mixed Rain/Borehole | No | No | 5.8 | Yes | 3-4h | True Positive |
| 140-1 | Borehole | No | Yes | 5.5 | Yes | 3-4h | True Positive |
| 141-1 | Borehole | No | Yes | 6.4 | Yes | 2-3h | True Positive |
| 142-1 | Borehole | No | No | 5.4 | Yes | 2-3h | True Positive |