Supplementary Table and Figure titles and captions

Supplementary Table 1. List of key optomechanical components.

| Optomechanical components | Manufacturers | Product Identifier |
| --- | --- | --- |
| Fluorescent beads | ThermoFisher Scientific | T7282 |
| Rotation stage unit: Motorized rotation stage | ThorLabs | DDR100/M |
| Rotation stage unit: Motor controller | ThorLabs | BBD201 |
| Rotation stage unit: Stage adaptor plate | ThorLabs | NR360SP8 |
| Objective lens unit: Kinematic mirror mount | ThorLabs | KCB1EC/M |
| Objective lens unit: Elliptical mirror | ThorLabs | PFE10-P01 |
| Objective lens unit: Z translation lens positioner | ThorLabs | SM1Z |
| Objective lens unit: Objective lens (20×, N.A. 0.8) | Olympus | UPLXAPO20X |
| Objective lens unit: Objective lens (40×, N.A. 0.8) | Olympus | LUMPLFLN40XW |
| Objective lens unit: Cage mount | OptoSigma | C30-CMP-H32 |
| Objective lens unit: XY translation mount | ThorLabs | CXY1 |
| Objective lens unit: Rotation mount | ThorLabs | CRM05 |
| Tube lens unit: Camera port with a tube lens (f = 200 mm) | ThorLabs | WFA4100 (tube lens: TTL200) |
| Tube lens unit: Cage adaptor plate | ThorLabs | CSA1003 |
| Tube lens unit: Cage adaptor plate | ThorLabs | LCP02/M |
| Confocal scanner unit | Yokogawa | CSU-X1 |
| Confocal scanner unit: Dichroic mirror | Semrock | Di01-T405/488/561-13x15x0.5 |
| Image splitting optics: W-VIEW GEMINI | Hamamatsu | A12801-01 |
| Image splitting optics: Dichroic mirror for image splitting optics | Semrock | FF560-FDi01-25x36 |
| Image splitting optics: Emission filter | Semrock | FF01-512/25-25 |
| Image splitting optics: Emission filter | Semrock | FF02-617/73-25 |
| Digital sCMOS camera ORCA-Flash 4.0 V3 | Hamamatsu | C13440-20CU |
| Laser | Coherent | Sapphire 488-50 |
| Specimen chamber: FEP sheet: thickness 50 μm | Flonchemical | NR0538-002 |
| Accelerometer | MicroStone | MA3-04AD |
| Digitizer | Molecular Devices | Digidata 1440A |

Supplementary Table 2. List of key software and algorithms.

| Software and algorithms | Manufacturers/Source | Uniform Resource Locator |
| --- | --- | --- |
| HCImage Live | Hamamatsu Photonics | https://hcimage.com/hcimage-overview/hcimage-live/ |
| Kinesis | ThorLabs | https://www.thorlabs.com/newgrouppage9.cfm?objectgroup_id=10285 |
| MATLAB | MathWorks | https://www.mathworks.com/products/matlab.html |
| Image Processing Toolbox | MathWorks | https://www.mathworks.com/products/image.html |
| dftregistration | Guizar-Sicairos et al., 2008 | https://www.mathworks.com/matlabcentral/fileexchange/18401-efficient-subpixel-image-registration-by-cross-correlation |
| ImageJ | National Institute for Health | https://imagej.nih.gov/ij/ |
| Python | Python Software Foundation | https://www.python.org/ |
| SciPy | Virtanen et al., 2020 | https://scipy.org/ |
| pClamp | Molecular Devices | https://www.moleculardevices.com/products/axon-patch-clamp-system/acquisition-and-analysis-software/pclamp-software-suite#gref |

Supplementary Figure 1. Characterization of tiltable objective microscope.

a Schematic showing axes of acceleration in fish and otolith organs during tilt. Static tilt with an angle, θ, produces vector components of gravitational acceleration (g, gray arrow): g · cos θ (blue arrow) in the centripetal axis (X) and g · sin θ (magenta arrow) in the tangential axis (Y) that is orthogonal to the centripetal axis X.

b-d Acceleration during 90° static tilt (b), 360° static tilt (c), and vibration (d) stimulus. Gravitational acceleration simulated by vector components in (a) (top). The Z axis is orthogonal to the X and Y axes. Measured inertial acceleration (middle). Time course of tilt angle (bottom). Scale bar: 0.5 g.

e Close up view of the rectangle area in (d). Vertical bar: 0.5 g. Horizontal bar: 100 ms.

f Frequency power spectrum density of acceleration in the Y axis during vibration stimulus. Arrowhead indicates 2.2 Hz, which corresponds to the cycle frequency of angle changes (bottom in [d]).

g Time course of fluorescence intensity in beads during 360° static tilt.

h Time course of fluorescence intensity in Kaede-expressing neurons during 360° static tilt.

i Changes in fluorescence intensity ratio (ΔR/R0) in Kaede-expressing neurons during 90° static tilt.

j Distribution of ΔR/R0 values during 90° static tilt.

k ΔR/R0 in Kaede-expressing neurons during vibration stimulus.

l Distribution of ΔR/R0 values during vibration stimulus.

Supplementary Figure 2. Utricular HC responses to static tilt.

a Utricular HC responses to 90° static tilt. Mean ΔR/R0 during the period “i” and “ii” showing positive (yellow) and negative (blue) responses during nose-down, tail-down, lateral-down, and medial-down tilt. Dashed lines: utricle contours. Color scale range is chosen to show small-amplitude negative responses. Average of 6 utricles. Scale bar: 10 μm.

b, c HC responses to 360° static tilt in the pitch (b) and roll (c) axes. The same utricle shown in Figure 2d. Tilt angle: 90° and 270° in the pitch axis indicate nose down and tail down, respectively; 90° and 270° in the roll axis indicate lateral down and medial down, respectively (top row). Gravitational acceleration in the tangential direction (Y in Supplementary Fig. 1a) that provides hair bundles with shearing force during the tilt (second row). A positive value indicates acceleration to the nose down in the pitch axis and lateral down in the roll axis. ΔR/R0 responses grouped by preferred tilt direction and biphasic/monophasic response patterns, both of which are determined by responses to 90° tilt (Fig. 2d). Color bar applies to (b) and (c).

d, e Mean HC responses to 360° tilt in the pitch (d) and roll (e) axes. Top and second rows are the same as in (b, c). Mean ΔR/R0 responses of each functional group shown in (b, c) (third row). Mean ΔR/R0 responses of 6 utricles for pitch and 3 left utricles for roll (bottom row). Deviation (SEM) is shown in gray. Number of cells in each group: Nose-down preferred group: 11 ± 2.9 biphasic cells, 25 ± 4.5 monophasic cells; Tail-down preferred group: 7.3 ± 2.0 biphasic cells, 27 ± 3.0 monophasic cells; Lateral-down preferred group: 34 ± 4.5 biphasic cells, 27 ± 7.3 monophasic cells; Medial-down preferred group: 3.3 ± 0.2 biphasic cells, 9.5 ± 2.1 monophasic cells (mean ± SEM, 6 utricles). Scale bar: 100% ΔR/R0.

f Mean HC responses to 90° roll tilt in 3 right utricles. Tilt angle: positive value indicates lateral down (top row). Mean ΔR/R0 responses (bottom). Deviation (SEM) is shown in gray. Scale bar: 100% ΔR/R0.

Supplementary Figure 3. Activity map and vector length of static tilt and vibration responses.

a Activity maps showing direction-selective responses to static tilt (top) and vibration (bottom) stimulus in utricular HCs in Tg(myo6b:jGCaMP7f; s100s-hs:tdTomato) larvae. Similar to Figure 2c, tail-down response image subtracted from nose-down response image for the pitch axis (top left). Medial-down response image subtracted from lateral-down response image for the roll axis (top right). Similar to Figure 6a, changes of mean ΔR/R0 during the time period “iv” from those during the pre-stimulus period are shown (bottom). Average of 6 utricles. Scale bar: 10 μm. Rostral is to the top. Lateral is to the left.

b Box plots of the ΔR/R0 response vector length in 90° static tilt responses (Fig. 4c) compared between striolar and extrastriolar HCs. Magenta bars indicate median. The top and bottom of the boxes indicate 75th and 25th percentiles, respectively. Top ends of upper whiskers indicate the 99th percentile. Bottom ends of lower whiskers indicate the 1st percentile. Dots indicate outliers above the 99th percentile. Striolar HCs: 297 cells. Extrastriolar HCs: 229 cells. Mann–Whitney U test, p = 9.3 × e-14.

c Box plots of the vector sum length of ΔR/R0 bars in vibration responses (Fig. 4d) compared between striolar and extrastriolar HCs. Parameters of box plots and number of cells are the same as in (b). Mann–Whitney U test, p = 1.3 × e-54.

Supplementary Figure 4. VGN responses to static tilt.

a, b VGN responses to 360° tilt in pitch (a) and roll (b) axes. The same VGNs shown in Figure 5e. Tilt angle time course (top row). Gravitational acceleration vector component that provides HCs with shearing force in the tangential direction (Y in Supplementary Fig. 1a) during tilt (second row). Heat map showing ΔR/R0 responses grouped by preferred tilt direction that is determined by responses to 90° tilt. Color bar applies to (a) and (b).

c, d Mean VGN responses to 360° tilt in pitch (c) and roll (d) axes. Tilt angle time course (top row). Gravitational acceleration vector component in the tangential direction (Y in Supplementary Fig. 1a) during tilt (second row). Mean ΔR/R0 responses of each VGN group shown in (a) and (b) (third row). Nose-down preferred: 57 cells; tail-down preferred: 47 cells; lateral-down preferred: 64 cells. Mean ΔR/R0 responses of 6 ganglia for pitch and 3 left ganglia for roll (bottom row). Deviation (SEM) is shown in gray. Scale bar: 50% ΔR/R0.

e Mean VGN responses to 90° roll tilt in 3 right ganglia. Tilt angle: a positive value indicates lateral down (top row). Mean ΔR/R0 responses (bottom). Deviation (SEM) is shown in gray. Scale bar: 50% ΔR/R0.

Supplementary Figure 5. Topographic pattern in VGN innervation of utricular HCs.

a Maximum intensity projection image of VGNs expressing Kaede in a Tg(hspGFF53A; UAS:Kaede) larva at 5 dpf. Green-Kaede (green) in the peripheral neural processes is photoconverted to red-Kaede (magenta) only in the rostral half in the utricle. The red-Kaede diffuses from the neural processes to VGN somata. Dashed line encloses the utricle. Arrowhead: horizontal canal nerve. Asterisk: ectopic Kaede-expressing cell. R: rostral. L: lateral. Scale bar: 10 μm.

b-d Distribution of fluorescence intensity ratio (red/green) in VGN somata in the rostrocaudal (RC) (b), mediolateral (ML) (c), and dorsoventral (DV) (d) axes. Mean and SEM from 5 utricles. Large ratio values indicate a large amount of photoconverted Kaede fluorescence in the somata.

Supplementary Movie 1. Movement of tiltable objective microscope during 360° static tilt and vibration stimulus.

Movement of the objective lens unit on the rotation stage during 360° static tilt and vibration stimulus imaged from the front side of the microscope. ×1 playback speed.

Supplementary Movie 2. Raw images, tracking beads position, and registered images.

Three sequential movie clips at ×4 playback speed. First clip: sequential raw images of fluorescent beads in green and red channels. Second clip: beads position tracked in color-merged channel. Third clip: registered sequential images.

Supplementary Movie 3. Utricular HC responses to 90° static tilt.

Two sequential movie clips at ×4 playback speed showing ΔR/R0 utricular HC responses to 90° static tilt. First clip: pitch axis. Second clip: roll axis. Average of 5 trials.

Supplementary Movie 4. Utricular HC responses to vibration stimulus.

Two sequential movie clips at ×4 playback speed showing ΔR/R0 utricular HC responses to vibration stimulus. First clip: pitch axis. Second clip: roll axis. Average of 8 trials. Baseline ratio image was the mean of ratio image frames ±2.5 s from the beginning of the stimulus onset.

Supplementary Movie 5. VGN responses to 90° static tilt.

Two sequential movie clips at ×4 playback speed showing ΔR/R0 VGN responses to 90° static tilt. First clip: pitch axis. Second clip: roll axis. Maximum intensity projection images. Single trial. Dashed line indicates VG contour.

Supplementary Movie 6. VGN responses to vibration stimulus.

Two sequential movie clips at ×4 playback speed showing ΔR/R0 VGN responses to vibration stimulus. First clip: roll axis. Second clip: pitch axis. Maximum intensity projection images. Single trial. A VGN responds to both stimuli (arrowhead) whereas another VGN responds to only pitch vibration (arrow). Dashed line indicates VG contour.