Supplementary materials

Spatial and temporal trends in polychlorinated naphthalenes in sediment from industrialized coastal environments of Korea: Potential sources and ecotoxicological concerns

Ha-Hyun Leea, Sunggyu Leeb, Moonjin Leec, Hyo-Bang Moon*,a

a Department of Marine Science and Convergent Technology, College of Science and Convergence Technology, Hanyang University, Ansan 15588, Republic of Korea

b Marine Environment Research Division, National Institute of Fisheries Science (NIFS), Busan 46083, Republic of Korea

c Maritime Safety and Environmental Research Division, Korea Research Institute of Ships and Ocean Engineering, Daejeon 34103, Republic of Korea

Table S1. Detailed information on retention time and transition ions of multiple reaction monitoring (MRM) mode for the determination of polychlorinated naphthalenes (PCNs) in our study

|  | Retention time (min) | Quantifier (m/z) | Qualifier (m/z) |
| --- | --- | --- | --- |
| Mono-CN |  |  |  |
| CN 2 | 13.8 | 162.1→127.0 | 162.1→76.9 |
| Di-CN |  |  |  |
| CN 6 | 16.7 | 196.0→126.0 | 196.0→160.9 |
| Tri-CN |  |  |  |
| CN 13 | 20.9 | 230.0→159.9 | 230.0→194.9 |
| Tetra-CN |  |  |  |
| CN 28/36 | 25.0 | 266.0→195.9 | 266.0→230.9 |
| CN 27 | 25.5 | 266.0→195.9 | 266.0→230.9 |
| CN 48 | 26.3 | 266.0→195.9 | 266.0→230.9 |
| CN 46 | 27.7 | 266.0→195.9 | 266.0→230.9 |
| Penta-CN |  |  |  |
| CN 52 | 29.0 | 300.0→229.8 | 300.0→264.6 |
| CN 50 | 30.3 | 300.0→229.8 | 300.0→264.6 |
| CN 54 | 31.4 | 300.0→229.8 | 300.0→264.6 |
| CN 53 | 32.1 | 300.0→229.8 | 300.0→264.6 |
| Hexa-CN |  |  |  |
| CN 66/67 | 36.2 | 334.2→263.8 | 334.2→298.8 |
| CN 64/68 | 37.3 | 334.2→263.8 | 334.2→298.8 |
| CN 69 | 37.6 | 334.2→263.8 | 334.2→298.8 |
| CN 63 | 39.5 | 334.2→263.8 | 334.2→298.8 |
| CN 72 | 45.3 | 334.2→263.8 | 334.2→298.8 |
| Hepta-CN |  |  |  |
| CN 73 | 45.4 | 367.9→297.8 | 367.9→332.6 |
| Octa-CN |  |  |  |
| CN 75 | 53.0 | 403.9→333.6 | 403.9→368.6 |

Table S2. Results of quality control for the determination of PCN congeners in sediment samples using GC-MS/MS

|  | iLOQ a (ng/g) | Cali. range (ng/mL) | Linearity | PB b (ng/mL) | RSD d (%) | Recovery (n=49) (Average ± SD e, %) |
| --- | --- | --- | --- | --- | --- | --- |
| Target compounds |  |  |  |  |  |  |
| CN 2 | 0.0001 | 0.1-500 | 0.999 | 0.14 ± 0.04 | 0.43 |  |
| CN 6 | 0.0001 | 0.1-500 | 0.999 | 0.28 ± 0.11 | 0.89 |  |
| CN 13 | 0.0001 | 0.1-500 | 0.999 | NDc | 2.11 |  |
| CN 36/28 | 0.0003 | 0.1-500 | 0.999 | 0.12 ± 0.02 | 1.13 |  |
| CN 27 | 0.0002 | 0.1-500 | 0.997 | ND | 2.14 |  |
| CN 48 | 0.0002 | 0.1-500 | 0.998 | ND | 1.01 |  |
| CN 46 | 0.0002 | 0.1-500 | 0.997 | 0.10 ± 0.02 | 0.85 |  |
| CN 52 | 0.0003 | 0.1-500 | 0.999 | ND | 1.23 |  |
| CN 50 | 0.0003 | 0.1-500 | 0.999 | ND | 1.43 |  |
| CN 54 | 0.0002 | 0.1-500 | 0.999 | ND | 2.14 |  |
| CN 53 | 0.0004 | 0.1-500 | 0.995 | ND | 2.34 |  |
| CN 66/67 | 0.0004 | 0.1-500 | 0.999 | ND | 3.16 |  |
| CN 64/68 | 0.0004 | 0.1-500 | 0.999 | ND | 2.36 |  |
| CN 69 | 0.0004 | 0.1-500 | 0.991 | ND | 2.43 |  |
| CN 63 | 0.0003 | 0.1-500 | 0.999 | ND | 4.23 |  |
| CN 72 | 0.0003 | 0.1-500 | 0.998 | ND | 5.01 |  |
| CN 73 | 0.0006 | 0.1-500 | 0.999 | ND | 5.32 |  |
| CN 75 | 0.0014 | 0.1-500 | 0.998 | ND | 6.77 |  |
| Internal Standards |  |  |  |  |  |  |
| 13C10-CN 27 |  |  |  |  |  | 102 ± 17 |
| 13C10-CN 42 |  |  |  |  |  | 109 ± 20 |
| 13C10-CN 52 |  |  |  |  |  | 85 ± 13 |
| 13C10-CN 67 |  |  |  |  |  | 79 ± 15 |
| 13C10-CN 73 |  |  |  |  |  | 68 ± 12 |
| 13C10-CN 75 |  |  |  |  |  | 62 ± 10 |

a iLOQ = Instrumental limit of quantitation.

b PB = Procedural blank.

c ND = Not detected.

d RSD = Relative standard deviation of check standards.

e SD = Standard deviation.

Table S3. Comparison of mean concentrations (pg/g dry weight) of total polychlorinated naphthalenes (PCNs) in sediment from Ulsan and Onsan Bays and their inflowing streams and rivers with those reported for other studies

| Sampling locations | Sampling years | na | Targeted PCN congeners | Min-Max | Mean | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Liaohe River, China | 2010 | 45 | Tri- to octa-CNs | 330-12,500 | 4,400 | Li et al. (2016b) |
| Great Lakes | 2012 | 23 | Tri- to octa-CNs | 400-28,500 | 10,340 | McGoldrick et al. (2018) |
| Chenab River, Pakistan | 2013 | 28 | Tri- to octa-CNs | 8940-414,000 | 8,930 | Mahmood et al. (2014) |
| Yellow River, China | 2014 | 60 | Tri- to octa-CNs | 618-130,000 | 7,100 | Li et al. (2017) |
| East China Sea, China | 2014 | 75 | Mono- to octa-CNs | <LOQb-262,000 | 77,000 | Liu et al. (2018) |
| Laojie River, Taiwan | 2017 | 3 | Tri- to octa-CNs | 408-1,470 | 987 | Dat et al. (2019) |
| Feitsui Reservoir, Taiwan | 2017 | 6 | Tri- to octa-CNs | 12.0-55.0 | 29.2 | Dat et al. (2019) |
| Catalonian coast, Spain | 2017 | 5 | Mono- to octa-CNs | 350-5,000 | 2622 | Ayala-Cabrera et al. (2021) |
| Yangtze River, China | 2019 | 31 | Mono- to octa-CNs | 10-1,590 | -c | Lei et al. (2022) |
| Ulsan and Onsan Bays, Korea | 2014 | 42 | Tri- to octa-CNs | 1.10-404 | 75.4 | This study |

an = sample numbers analyzed.

b<LOQ = less than limit of quantification.

c- = not available.

Table S4. Pearson’s correlation coefficents of PCN congeners in sediment from Ulsan and Onsan Bays and their inflowing rivers and streams of Korea

|  | CN 27 | CN 36/28 | CN 48 | CN 52 | CN 54 | CN 63 | CN 64/68 | CN 66/67 | CN 69 | CN 72 | CN 73 | ƩPCN | TOC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CN 36/28 | 0.615*** |  |  |  |  |  |  |  |  |  |  |  |  |
| CN 48 | 0.477** | 0.501** |  |  |  |  |  |  |  |  |  |  |  |
| CN 52 | 0.511** | 0.557*** | 0.922*** |  |  |  |  |  |  |  |  |  |  |
| CN 54 | 0.567*** | 0.419** | 0.934*** | 0.935*** |  |  |  |  |  |  |  |  |  |
| CN 63 | 0.697*** | 0.666*** | 0.886*** | 0.822*** | 0.836*** |  |  |  |  |  |  |  |  |
| CN 64/68 | 0.530*** | 0.491** | 0.933*** | 0.905*** | 0.908*** | 0.909*** |  |  |  |  |  |  |  |
| CN 66/67 | 0.533*** | 0.501** | 0.932*** | 0.906*** | 0.912*** | 0.909*** | 0.999*** |  |  |  |  |  |  |
| CN 69 | 0.532*** | 0.482** | 0.930*** | 0.904*** | 0.912*** | 0.906*** | 0.999*** | 0.999*** |  |  |  |  |  |
| CN 72 | 0.534*** | 0.491** | 0.943*** | 0.914*** | 0.922*** | 0.911*** | 0.996*** | 0.995*** | 0.995*** |  |  |  |  |
| CN 73 | 0.530*** | 0.314* | 0.626*** | 0.436** | 0.604*** | 0.718*** | 0.612** | 0.607** | 0.610** | 0.617*** |  |  |  |
| ƩPCN | 0.622*** | 0.456** | 0.913*** | 0.820*** | 0.896*** | 0.931*** | 0.922*** | 0.920*** | 0.921*** | 0.926*** | 0.858*** |  |  |
| TOCa | 0.084 | –0.127 | 0.319* | 0.126 | 0.256 | 0.328* | 0.255 | 0.247 | 0.254 | 0.255 | 0.625*** | 0.446** |  |
| GSb | 0.391* | –0.052 | 0.259 | 0.166 | 0.327* | 0.377* | 0.287 | 0.278 | 0.298 | 0.289 | 0.484** | 0.406** | 0.521*** |

*p < 0.05;**p < 0.01; ***p < 0.001.

aTOC=total organic carbon (%); bGS=grain size (µm).

Figure S1. Diagnostic ratios of the concentrations of PCN congeners in sediment from Ulsan and Onsan Bays and their inflowing streams and rivers, Korea.