Ct= 1 (1 – Hct)  ×-1rGd × TR×TRT10+Ln (StS0 × 1-E1-cosαE -1)S(t)S0×cosα×1-E1-cosαE -1 ,

where S0 and S(t) are the signals measured  before contrast-enhancement  and at time t after contrast injection, Hct is the hematocrit level fixed at 0.45 [44] , rGd = 3.4 s-1.mM-1 is the relaxivity of Dotarem at 3T [45], E = e-TR/T10 where T10 is the estimated pre-contrast T1 value in the voxel of interest, and α is the flip angle of the 3D fast-SPGR pulse sequence, set to 15° in our imaging protocol.