Supplementary Information

| Table 1 Average Off-Peak Locational Marginal Price (LMP)1 |  |
| --- | --- |
| Region | Day-Ahead LMP |
|  | US$/MW |
| California ISO | 33.54 |
| ERCOT | 26.18 |
| New England ISO | 29.49 |
| New York ISO | 30.67 |
| PJM | 28.81 |

| Table 2 Summary of inputs to total cost of reliability-driven investments model |  |  |
| --- | --- | --- |
| Parameter | Value | Units |
| Financial Assumptions |  |  |
| Discount rate2 | 7 | % |
| Battery economic lifetime3 | 15 | years |
| Transmission line economic lifetime4 | 50 | years |
| Inverter economic lifetime2 | 20 | years |
| Boxcar economic lifetime2 | 50 | years |
| Battery Storage Assumptions |  |  |
| Storage duration | 4 | hours |
| NREL ATB storage cost year | 2025 |  |
| NREL ATB storage cost decline rate | Moderate |  |
| Capital cost of battery3 | 582 | US$/kW |
| Fixed operation & maintenance (FOM) cost3 | 24 | US$/kW-yr |
| Siting, development, interconnection (SDI) cost3 | 533 | US$/kW |
| Round-trip efficiency (RTE) 3 | 85 | % |
| Size of battery to power one locomotive2 | 9 | MWh |
| Weight per 9 MWh battery and inverter2 | 89 | t |
| Transmission Assumptions |  |  |
| Transmission line losses4 | 0.62 | % per 100 km |
| Per-km capital cost4 | 1.63 | US$/kW-km |
| Transmission line voltage | 345 | kV |
| Transmission line current type | AC |  |
| Rail Sector Assumptions |  |  |
| Inverter |  |  |
| Inverter cost5 | 70 | US$/kW |
| Inverter size2 | 3,300 | kW |
| Inverters per boxcar2 | 1 |  |
| Boxcar |  |  |
| Boxcar cost6 | 135,000 | US$ |
| Variable Costs and Revenues |  |  |
| Per-locomotive cost of diesel | 1142 | US$/day |
| Per-locomotive cost of maintenance2 | 100 | US$/day |
| Battery leasing rate at cost of switching from diesel | 1,242 | US$/day |
| Average freight shipping rate7 | 0.03 | US$/t-km |
| Charging Infrastructure Assumptions |  |  |
| Cost of a single 72 MW charging station2 | 8 | $USD million |
| Total number of chargers on rail network2 | 933 |  |
| Per-kW cost of charging infrastructure | 119 | $USD/kW-yr |
| Average off-peak electricity price (ERCOT)1 | 26 | $USD/MWh |

| Table 3 Battery Capital Cost Decline3 |  |  |  |
| --- | --- | --- | --- |
| Year | Advanced | Moderate | Conservative |
| 2020 | 1.00 | 1.00 | 1.00 |
| 2021 | 0.92 | 0.94 | 0.97 |
| 2022 | 0.85 | 0.88 | 0.94 |
| 2023 | 0.77 | 0.82 | 0.92 |
| 2024 | 0.69 | 0.76 | 0.89 |
| 2025 | 0.62 | 0.70 | 0.86 |
| 2026 | 0.58 | 0.68 | 0.83 |
| 2027 | 0.54 | 0.65 | 0.81 |
| 2028 | 0.50 | 0.63 | 0.78 |
| 2029 | 0.46 | 0.60 | 0.75 |
| 2030 | 0.42 | 0.58 | 0.72 |
| 2031 | 0.41 | 0.57 | 0.72 |
| 2032 | 0.40 | 0.56 | 0.72 |
| 2033 | 0.39 | 0.55 | 0.72 |
| 2034 | 0.39 | 0.55 | 0.72 |
| 2035 | 0.38 | 0.54 | 0.72 |
| 2036 | 0.37 | 0.53 | 0.72 |
| 2037 | 0.36 | 0.52 | 0.72 |
| 2038 | 0.35 | 0.52 | 0.72 |
| 2039 | 0.34 | 0.51 | 0.72 |
| 2040 | 0.34 | 0.50 | 0.72 |
| 2041 | 0.33 | 0.50 | 0.72 |
| 2042 | 0.32 | 0.49 | 0.72 |
| 2043 | 0.31 | 0.48 | 0.72 |
| 2044 | 0.30 | 0.47 | 0.72 |
| 2045 | 0.30 | 0.47 | 0.72 |
| 2046 | 0.29 | 0.46 | 0.72 |
| 2047 | 0.28 | 0.45 | 0.72 |
| 2048 | 0.27 | 0.45 | 0.72 |
| 2049 | 0.26 | 0.44 | 0.72 |
| 2050 | 0.25 | 0.43 | 0.72 |

| Table 4 Battery Investment Cost3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| component | cost in 2020 | cost with reduction | units | cost category |
| Battery | 192 | 134 | US$/kWh | capital |
| Sales tax | 16 | 11 | US$/kWh | capital |
| Central Inverter | 59 | 59 | US$/kW | SDI |
| Structural BOS | 52 | 52 | US$/kW | SDI |
| Electrical BOS | 142 | 142 | US$/kW | SDI |
| Installation labor and Equipment | 77 | 77 | US$/kW | SDI |
| EPC overhead | 39 | 39 | US$/kW | SDI |
| Land acquisition | 0 | 0 | US$/kW | SDI |
| Permitting fee | 5 | 5 | US$/kW | SDI |
| Interconnection fee | 31 | 31 | US$/kW | SDI |
| Contingency | 37 | 37 | US$/kW | SDI |
| Developer overhead | 26 | 26 | US$/kW | SDI |
| EPC/developer net profit | 65 | 65 | US$/kW | SDI |

| Table 5 Transmission Line Costs4 |  |  |  |
| --- | --- | --- | --- |
| Voltage | Type of current | Variable CAPEX | Fixed CAPEX* |
| kV |  | US$/kW-km | US$/kW |
| 230 | AC | 2.56 | 0 |
| 345 | AC | 1.63 | 0 |
| 500 | AC | 0.94 | 0 |
| 765 | AC | 0.98 | 0 |
| DC | DC | 1.05 | 233 |
| * AC/DC converter for DC lines only |  |  |  |

References

1.	ABB. Velocity Suite. ISO Real Time & Day Ahead LMP Pricing - Hourly. (2022).

2.	Popovich, N. D. et al. Economic, environmental, and grid resilience benefits of converting diesel trains to battery-electric. Nat. Energy 6, 1017–1025 (2021).

3.	National Renewable Energy Laboratory. Annual Technology Baseline Utility-Scale Battery Storage. (2021). Available at: https://atb.nrel.gov/electricity/2021/utility-scale_battery_storage. (Accessed: 23rd May 2022)

4.	Brown, P. R. & Botterud, A. The Value of Inter-Regional Coordination and Transmission in Decarbonizing the US Electricity System. Joule 5, 115–134 (2021).

5.	Fu, R. et al. 2018 U.S. Utility-Scale Photovoltaics-Plus-Energy Storage System Costs Benchmark. Natl. Renew. Energy Lab. 32 (2018).

6.	Julian Turner. Can the US railroad freight sector avoid a shortage of boxcars? Railway Technology Available at: https://www.railway-technology.com/features/us-railroad-freight-boxcars/. (Accessed: 14th December 2020)

7.	Railroads, A. of A. Average U.S. Freight Rail Rates. Available at: https://www.aar.org/data/average-u-s-freight-rail-rates-since-deregulation/. (Accessed: 23rd May 2022)