| Features | SSHD | SSD | HDD |
| --- | --- | --- | --- |
| Mechanism | Magnetic + NAND Flash Memory | NAND NOR Flash Memory | Magnetic rotating platters |
| Capacity | Up to 2TB (notebooks) + upto 8 Up to 10TB (desktops) GB SSD | Up to 1TB (notebooks) Up to 4TB (desktops) | Up to 2TB (notebooks) Up to 10TB (desktops) |
| Durability | ---- | Shock-resistant | Fragile |
| Power consumption | Same as SSD | Average 2W | Average 10W |
| Endurance | ----- | MTBF > 2 million hours | MTBF < 700,000 Hours |
| Noise | ----- | None | Present |
| Operating System Boot-Time | Same as SSD | Average of 10–15 seconds | Average of 30–40 seconds |
| File Opening Speed | Faster Compare to HDD | 30% faster than HDD | Slower than SSD |
| Speed | Faster than HDD | >200 MB/s | 50–120 MB/s |
| Vibration | While HDD in use | No Vibration | Moving parts cause vibration |
| Affected by Magnetism | Sometime affected | No effect | Can erase data |
| Full Drive Encryption | Supported | Supported | Supported |
| Cost | Cheap compare to SSD | Costly | Cheap compare to SSD |
| Heating | Moderate | less | High |
| Size | Large | Compact | Large |
| Figure |  |  |  |

Table 1. SSHD Vs SSD Vs HDD [18]