Appendix A. Observation with ONC-W2 during the re-entry phase of the return capsule

Since ONC-W2 was rarely used during the return cruise, even during the proximity phase after TD2, it has not been confirmed that any significant optical degradation occurred. We attempted to use the unique installation angle of this camera to image Earth and the capsule during the Earth swing-by operation. We decided to include it in this study as part of a qualitative optical degradation assessment.

Based on the observational result of the maximum brightness of Hayabusa’s return capsule during the re-entry phase of its 2010 mission, the maximum temperature of the return capsule was expected to be around 3000 °C from considerations of blackbody emission, and the altitude during maximum brightness was 55 – 60 km (Sato et al., 2012). We assumed that the return capsule of Hayabusa2 would also use the brightness at 55 – 60 km to determine its temperature. The estimated distance between the capsule and the Hayabusa2 spacecraft would be around 350–400 km at the time of maximum brightness.

Although the size of the capsule (40 cm diameter) was much smaller than the corresponding spatial scale of a ONC-W2 pixel (400 m), even from the minimum distance of 350 km, we confirmed that the capsule would be bright enough to be observed (i.e., the S/N ratio would be 10) within the possible exposure times, assuming a 3,000 °C capsule temperature (Figure A.1). If W2 observes the capsule from a distance of 350 km, then 1 s of exposure would be enough for unambiguous detection if the object were moving with respect to Hayabusa2.

However, it was also expected that the motion of the capsule in the W2 FOV would be approximately 10 pixels within the 1-s exposure owing to the velocity difference between Hayabusa2 and the capsule. Therefore, an effective exposure time for the capsule observation would be 0.1 s, and the possible S/N ratio would be around 5. Although the expected S/N ratio could be marginal, we conducted observations using 1-s exposures with an interval of 2 s during the re-entry phase of the return capsule.

Figure A.2a shows an example of the images obtained on December 5, 2020 during this phase. Since there were clear periodic noise patterns (due to electromagnetic noise), we performed Fourier analysis for each image (Figure A.2b) and reduced the noise by subtracting the top-10 Fourier components for each line, thus enhancing their visibility (Figure A.2c). The formula for doing this can be given by

gix=j=110ajsin(2πfjx-bj),

where aj indicates the amplitude of the j-th largest Fourier component, and bj and fj are its phase and wavenumber, respectively.

Figure A.3 shows a close-up image that includes the region where the capsule was expected to be observed. In this region, there was no clear bright spot that exceeded a S/N level of 3 or 5, compared with the surrounding region. The standard deviation of counts in the image was 1.7 DN; the S/N levels of 3 and 5 corresponded to 5.1 DN and 8.5 DN, respectively.

It is difficult to discuss the implications of an observation result in which no significant signal was detected. However, the unique nature of this particular observation allows us to infer the significance of the fact that ONC-W2 did not detect light from the aerodynamically heated side of the capsule. From the sensitivity and spectral response performance of ONC-W2 (Suzuki et al., 2019; Kouyama et al., 2021), the brightness temperatures that provide 5.1 DN and 8.5 DN for ONC-W2 from a 350 km distance are 2,125 K and 2,224 K, respectively (Figure A.4). This means that the apparent brightness temperature from ONC-W2 might not reach 3,000 °C.

It should be noted from the investigation of the return capsule of the first mission that the back side of the capsule experienced only approximately 650 K heating (Sato et al., 2012), while the front side of the capsule experienced 3,000 °C. Therefore, if ONC-W2 observed both the front and back sides of the capsule, then the apparent brightness of it should be much darker than the brightness for 3,000 °C.

Figure A.5 shows a schematic view of a geometric configuration for the capsule observation by ONC-W2 based on the planned trajectories of the capsule and Hayabusa2 during the re-entry phase. Assuming the shape of the capsule to be a sphere for simplicity, the front side of it (higher temperature) occupies only 6.7% of the capsule’s silhouette seen from ONC-W2. In this case, the apparent brightness of the capsule corresponds to 2,150 K, even if the front side of the capsule reached 3,000 °C. A temperature of 2,150 K is almost the same as that for a S/N level of 3, and thus, the geometric configuration and relatively low temperature of the capsule’s backside can be reasons why ONC-W2 could not capture a clear signal from the capsule.

It has been noted that post-flight analysis of the onboard accelerometers indicates that the attitude of the return capsule was very stable at high altitudes, where high aerodynamic heating took place. A large-amplitude rotation only took place at low altitudes ≲ 20 km (Yamada and Yoshihara, 2022). This is consistent with our result that ONC-W2 did not detect significant light during the atmospheric re-entry of the capsule.