Individualizing mechanical ventilation:

Titration of driving pressure to pulmonary elastance through Young’s module in an acute respiratory distress syndrome animal model.

Álvaro Mingote,1,6, Ramsés Marrero García2, Martín Santos González3, Raquel Castejón Díaz4, Clara Salas Antón5,6, Juan Antonio Vargas Nuñez4,6, Javier García Fernández1,6.

Online Data Supplement

Suplementary material and methods

Animals and ethical approval

The animals were allowed to acclimatize under veterinary supervision for at least one week before the experiments. Rats were housed in groups of 8 to 10 in U-TEMP polyetherimide cages, with bedding, environmental enrichment, and free access to food and water, and maintained on a 12-hour light/12-hour dark cycle under controlled environmental conditions (50-70% relative humidity, 20 ± 2 oC temperature). Before the study, the health status of the animals was assessed, and any alteration was considered a reason for exclusion. Each experiment was performed in the morning (09:00-14:00 hours). On each study day, one unmedicated animal was transported to an operating room close to the housing room, where the animal was weighted.  Before the beginning of the experiments, we routinely checked the condition of the anesthesia machine, as well as we calibrated pressure sensors and established a room temperature of 24 – 25 ºC.

Experimental protocol

General anesthesia was induced with Sevoflurane 8% vaporized in a mixture of air and oxygen with an inspired fraction (FiO2) of 0.5. After obtaining adequate anesthetic depth, we monitored the animal with a pulse oximeter (Infinity® Delta; Dräger, Lübeck, Germany) in the lower extremity to record hemoglobin oxygen saturation (SpO2). A peripheral 24-gauge polyethylene catheter was then introduced in one of the lateral caudal veins in the tail to get peripheral venous access. Then, 0.1 mg/kg of morphine chlorydrate was administrated.

After we checked the anesthetic depth again, we dissected the anterior cervical space to expose the trachea and the left carotid artery, isolated from the vague nerve. We inserted a 24-gauge polyethylene catheter in the left carotid artery, connected it to a pressure sensor, and fixed it to the subcutaneous tissue through silk sutures to continuously monitor arterial pressure. Then we inserted a 16-gauge polyethylene catheter in the trachea and fixed it to the trachea with silk sutures to avoid potential leaks.

Following endotracheal intubation, we administered 0.6 mg/kg of rocuronium, and the 16-G catheter was connected to an anesthesia Flow-i C20 (Getinge, Solna, Sweden) machine through a “Y connector” with a minimum dead space (< 0.2 mL) and a three way stopcock (figure E1). Then we started MV with the following basal parameters: PC – CMV with PEEP = 5 cm H2O, DP = 14 cm H2O (hence, inspiratory peak pressure 19 cm H2O), FiO2 = 0.5 Sevoflurane 3%, and RR = 40 bpm (these parameters were then adjusted if necessary to obtain an end-tidal CO2 between 40-45 mmHg). We also monitored body temperature through a rectal probe to maintain normothermia (37-39 ºC) with a hot air blanket (EQUATOR® Convective Warmer; Smiths Medical ASD, Minneapolis, USA). Once the general anesthesia was correctly induced, and after assessing adequate hemodynamic stability, we established the ARDS.

The ARDS was established following the validated model proposed by Germann and Häffner in 1998 [1]. To summarize, it is shown through repeated bronchoalveolar lavages (BAL). In our case, we performed 5 to 7 BAL with 10 mL/kg of warmed saline solution 0.9% to generate a severe ARDS according to the Berlin definition of ARDS [2]. After the BAL, we waited at least 10 minutes with the basal parameters of our PC – CMV but with FiO2 = 0.8 to ensure post-ARDS oxygenation (SpO2 > 92%). To verify the severity of the ARDS, we obtained a blood sample from the carotid artery and measured blood gas to check that the PaO2/FiO2 ratio was less than 100. If this criterion was not met, we repeated the BAL as often as needed to accomplish it. The material obtained in the first two BALs was kept in a test tube and transported to the Internal Medicine Lab, where it was processed as described below.

For the maintenance of anesthesia, sevoflurane at 2.5% was used, and we administered 0.3 mg/kg of rocuronium every 30 minutes. In addition, rats received 10mL/kg/h of normal saline 0.9%. If the mean arterial pressure was less than 60 mmHg, we administrated 10 mL/kg of saline solution (NS 0,9%). There were no differences in the amount of fluid therapy received between both groups. An extra bolus of morphine chlorydrate of 0.1 mg/kg was administrated if the heart rate was higher than 200 bpm.

Individualized PEEP assessment

In both groups, after 15 minutes from the ARDS, we calculated the individualized PEEP for each animal through the super syringe method: after a recruitment maneuver (with a constant DP of 15 cm H2O, increasing PEEP 5 cm H2O at each step of 5 respiratory cycles until reaching 20 cm H2O of PEEP), starting with PEEP of 0 cm H2O (to obtain the inspiratory pressure-volume curve) and closing the port from the Flow-I machine in the three-way stopcock, we introduced 6 mL/kg of oxygen through the three-way stopcock connected to the intra-tracheal catheter (to avoid dead space between the anesthesia machine and the catheter). Then we registered mean airway pressure (Pmean). Then we opened the port from the Flow-I machine and increased PEEP by one cmH2O for 10 respiratory cycles. During expiration, we closed the port from the Flow-I machine in the three way stopcock and introduced 6 mL/kg of oxygen, registering Pmean. We repeated this procedure until reaching PEEP of 20 cm H2O by increases in PEEP of 1 cm H2O, and we repeated the whole process but decreasing PEEP from 20 cm H2O in steps of 1 cm H2O and reaching to PEEP 0 cm H2O (to obtain the espiratory pressure-volume curve), registering Pmean for each reduction of PEEP. We introduced these data in the software Excel 16.59 for Mac to individualize PEEP (the PEEP with the best compliance in the espiratory pressure-volume curve). A new recruitment maneuver (as described previously) was performed, and PEEP was decreased to the level of PEEP that we obtained to be the individualized PEEP.

Individualized DP titration

After setting an individualized PEEP for each animal, in the individualized DP group, we calculated for each animal their individualized DP (DP adjusted to pulmonary elastance). We changed the ventilatory mode to CPAP with the individualized PEEP for the DP titration. We interrupted the connection between the anesthesia machine and the rat by closing the Flow - I port from the 3-way stopcock. Then we introduced a known volume of 14 mL of oxygen (separately in steps of 1mL each step) through the stopcock connected to the intratracheal catheter, and we measured Pmean for each mL that we introduced. We introduced the data in the software Excel 16.59 for Mac, and we obtained a pressure–volume graph. An example of one of the animal’s graph is presented in figure 1 – B.  According to Young’s curve, the maximum slope of the graph would represent the elastic limit, followed by the flat zone that indicates the transition zone. Hence, we considered this point as the individualized driving pressure. It is important to emphasize that the first part of the theoretical graph (‘elastic behavior’) corresponds to a flat zone in the animal’s graph because the individualized PEEP already opens the lung.

Experimental groups

A computer-generated random list (Microsoft Excel 2016) selected one of the two groups. In the Standard DP group 10 animals received 120 minutes of MV with the standard ventilation parameters described previously (PEEP = individualized PEEP, FiO2 = 0,8, RR = 55 bpm, DP = 14 cm H2O always checking that TV was < 6mL/kg). In experimental group, 10 animals received MV 120 minutes with the following parameters: PEEP = individualized PEEP, DP = individualized DP, FiO2 = 0.8, RR = 55 bpm) (TABLE E1). Hence, the total MV time was 150 minutes: 30 minutes during the induction and calculation of the parameters and 120 minutes with the parameters we described. After this time, we finished the experiment, we obtained a blood sample from the carotid artery for blood gas analysis, and we performed 2 BALs with the same solution and volume as the previous one, and the material obtained was kept in a test tube, and transported to the Lab. The animals were sacrificed by increasing inspired Sevoflurane to 8% and through the administration of 150 mg/kg of sodic pentobarbital. Once asystole was established, we dissected the thorax, collected the lungs, and introduced them into a tube with 10% formaldehyde.

BAL analysis

BAL Processing

Bronchoalveolar lavage fluid (BALF) samples, kept cold, were processed in the following four h as follows: first, the BALF was filtered through a 70-mm cell strainer (BD Biosciences, USA) to remove mucus; then, the cellular content in the BALF was recovered by centrifugation at 300g for 10 min. The supernatant was collected and stored at -80°C for further analysis (ELISA). The cell pellet was resuspended in 2 mL of RPMI-1640 (Bio-Whittaker Europe, Belgium) to analyze the cellular influx in the lungs.

Analysis of cellular subsets in the BALF by Flow Cytometry

BALF cells were stained for 30 min at four °C with an anti-rat CD11b/CD11c monoclonal antibody coupled to fluorescein isothiocyanate (FITC) (clone REA325, Miltenyi Biotec, Germany). Anti-rat IgG irrelevant antibody was used as a negative control of staining. Then samples were washed in PBS to remove unbound antibodies and immediately analyzed in a FACSort flow cytometer with the CellQuest and the Paint-a-Gate Pro software (BD Biosciences, USA).

The staining and forward scatter component (FSC), and side scatters component (SSC) criteria were used to analyze leukocyte populations. Lymphocytes were identified using forward, side scatter as CD11b/CD11c negative cells, neutrophils were the CD11b/CD11clow population, and macrophages were identified as CD11b/CD11chigh cells.

Measurements of inflammatory cytokines in BALF

Concentrations of tumor necrosis factor (TNF)-α, interleukin (IL)-6, and IL-1β were detected with enzyme-linked immunosorbent assays (ELISAs) (Elabscience, China) according to the manufacturer's protocol. Briefly, standards or samples were added to each micro-ELISA plate well that were pre-coated with an antibody specific to the corresponding cytokine and combined with the specific antibody. Then a biotinylated detection antibody specific for each cytokine and Avidin-Horseradish Peroxidase (HRP) conjugate were added successively to each micro plate well and incubated. After washing away free components, the substrate solution was added to each well. Wells containing any of the cytokines, biotinylated detection antibody, and Avidin-HRP conjugate appeared in color. The optical density (OD) was measured spectrophotometrically at a wavelength of 450 nm. The OD value is proportional to the concentration of cytokines, therefore the concentration of cytokines in the samples could be calculated by comparing the OD of the samples to the standard curve. Detection range was 15.63-1000 pg/mL for TNFα, 12.5-800 pg/mL for IL-6 and 31.25-2000 pg/mL for IL-1β.

Histopathological grading of VILI

After processing the lungs (S1) two pathologists experts in interstitial pneumopathies blindly analyzed lung injury through the most used score to analyze VILI: the ALI score [3 – 4]. This score has four items: a) alveolar capillary congestion, b) hemorrhage, c) inﬁltration of neutrophils into the airspace or the vessel wall and thickness of the alveolar wall, and d) alveolar wall thickness/hyaline membrane formation. Every item receives punctuation ranged 0 to 4, corresponding 0 to normal findings, 1 to mild (< 25% of the surface), 2 to moderate (25 – 50% of the surface), three severe (50 – 75% of the surface) and 4 to very severe (> 75% of the surface) lung involvement, respectively. The score used for the results was the mean score for each sample from both pathologists.

| Animal | 001 | 002 | 003 | 004 | 005 | 006 | 007 | 008 | 009 | 010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Individualized DP (cmH2O) | 11 | 11 | 11 | 11 | 11 | 12 | 12 | 14 | 14 | 14 |
| Standard DP (cmH2O) | 14 | 14 | 14 | 14 | 14 | 14 | 14 | 14 | 14 | 14 |

Supplementary Table E1. DP received by each animal of both groups (p = 0.01).

References.

- Hafner D, Germann PG: A rat model of acute respiratory distress syndrome (ARDS) Part 2, influence of lavage volume, lavage repetition, and therapeutic treatment with rSP-C surfactant. J Pharmacol Toxicol Methods 1999, 41(2-3):97-106

- Force ADT, Ranieri VM, Rubenfeld GD, Thompson BT, Ferguson ND, Caldwell E, Fan E, Camporota L, Slutsky AS: Acute respiratory distress syndrome: the Berlin Definition. JAMA 2012, 307(23):2526-2533.

- Fan E, Del Sorbo L, Goligher EC, Hodgson CL, Munshi L, Walkey AJ, Adhikari NKJ, Amato MBP, Branson R, Brower RG et al: An Official American Thoracic Society/European Society of Intensive Care Medicine/Society of Critical Care Medicine Clinical Practice Guideline: Mechanical Ventilation in Adult Patients with Acute Respiratory Distress Syndrome. Am J Respir Crit Care Med 2017, 195(9):1253-1263.

- Combes A, Schmidt M, Hodgson CL, Fan E, Ferguson ND, Fraser JF, Jaber S, Pesenti A, Ranieri M, Rowan K et al: Extracorporeal life support for adults with acute respiratory distress syndrome. Intensive Care Med 2020, 46(12):2464-2476

Figure Legends.

Figure E1. A. Tracheal cannulation with 16G Abocath. B. Configuration of the connection from the Flow I Machine to the tracheal cannula through a 3-way stopcock and a pressure sensor for Pmean.