Appendix A.  Analytical Predictions for Paralinear Features.

W= ΔWmax at maximum weight gain and t= tmax (at maximum weight gain) in COSP) can be obtained from the original paralinear solution espoused by Wajszel,[12] as utilized by Barrett/Presler;[5]

W=∆Wmax=AB∙;where=1a-lnbaA1

t=tmax=AB2∙β;whereβ=-1b+lnbaA2

And where, from Barrett/Pressler,[5] p.3:

A = (a2/2) kp				       		A3

B = (a/b) kl (or kv)					       	A4

with stoichiometric ratios for gas and metal reactants in the scale:

a = Me/gas (= b - 1)			        		A5

b = scale product/gas (i.e., S or Seff in COSP)     		A6

Using these conversions and kp, kl(or v) from the published paralinear and fitted COSP parameters, ΔWmax and tmax were calculated and compared in Table 3.

Lastly, the terminal or final slope can be obtained from kl(v), as stated in eqn. 4:

kl = g (T.S.)					          		A7

where g  is the MW scale/MW compound stoichiometric ratio.  Thus, the steady state slopes of the weight change curve predicted from scale volatility rates are shown for the published paralinear and fitted COSP kl(v) in Table 3.

Also, the A, B, a, b Wajszel parameters above were used to obtain conventional paralinear kp, kl(v) for Pb/PbCl2 in Table 2 according to:

| A = 4.2E-11 g2/cm4/s |  | = 0.1512 | mg2/cm4h |
| --- | --- | --- | --- |
| B = 2.8E-9 g/cm2/s |  | = 0.01008 | mg/cm2h |
| kp = 2A/a2 |  | = 0.0356 | mg2/cm4h |
| kl,v) = bB/a |  | = 0.0135 | mg/cm2h |
| a = metal/gas (MW) |  | = 2.922 |  |
| b = scale/gas (MW) |  | = 3.922 |  |

In general, the paralinear rate constants can also be determined from knowing the stoichiometric factor b of the oxide, (S or Seff), and the data point coordinates at maximum, (ΔWmax, tmax).  By combining eqn. A1-A6, one obtains the paralinear rate constants as simple functions of the coordinates at maximum:

kp=2a2βα2∆Wmax2tmax=Fp∆Wmax2tmax≅4.06∆Wmax2tmax					A8

kv=baβα∆Wmaxtmax=Fv∆Wmaxtmax≅1.16∆Wmaxtmax					A9

The stoichiometric factor coefficients, Fp, Fv, preceding the max point quotients are tabulated in Table A1.  A regular progression of model oxide stoichiometries and those for a selection of actual oxides are listed (for S only, not Seff).   It is found that the Fp, Fv factors remain relatively tight over the range considered in the present analyses.  An average factor of Fp ~4.1 is obtained for the parabolic rate constant, kp in eqn. A8, and Fv ~ 1.2 for the linear volatility constant, kv in eqn. A9.  (More precisely, Fp and Fv are equal to 4.13 and 1.28, respectively, for low MW oxides with S ≤ 3, and equal to 4.01, 1.07 for high MW oxides with S ³3).  The overall consistency of both A8 and A9 relations is displayed in Figures A1, A2 for all the cases examined.  A log-log presentation was used to illustrate details across the full range of paralinear parameters examined.  It is seen that the dependencies do not differ appreciably between relations using exact and average F coefficients.   Furthermore, these F factors do not vary at all with kp or kl.  Regression analyses of model cases (log-log plots) yielded:

Fp∆Wmax2tmax=1.00 kp1.0046 ; 	r2 = 1.000		A10

Fp,avg∆Wmax2tmax=1.01 kp1.0049; 	r2 = 1.000		A11

Fv∆Wmaxtmax=0.96kV0.9958 ; 	 r2 = 0.999		A12

Fv,avg∆Wmaxtmax=1.11 kV1.0196;      	  r2 = 0.997		A13

thus, verifying that kp and kv are linearly equivalent to, and can be reasonably estimated from, Fp∆Wmax2tmax and Fv∆Wmaxtmax, respectively.

Appendix B.  COSP and the Tedmon Study.

The original Tedmon paralinear study [1] derived the analytical, transcendental solution and compared it to experimental data for Fe-Cr and Cr alloys.  The data was presented as log (weight gain) vs log (time), or weight change vs log (time).  These are less-familiar presentations than typical weight change vs time linear plots.  Such data is shown for Cr-0.2%Y, Figure B1.  The weight gain data exhibits a maximum, then decreases, in units of mg/cm2 vs log time (s).  Parabolic kinetics were extracted from the initial portion of the curve, then combined with kv from the Hagel 1963 volatility study for bulk Cr2O3 :[19]

kp = 1.5 x 10-10 g2/cm4s (0.54 mg2/cm4h), and

kv = 0.214 exp (- 48 kcal/RT) g/cm2s (yielding 0.058 mg/cm2h at 1200°C),

The calculated paralinear curve shown was similar in form but did not fit the data closely.  Unlike the data, the curve produced a lower maximum in weight gain, at shorter times and approached 0 mg/cm2 at the end of test.

Appropriate COSP model curves are provided in a similar mg/cm2 vs log time (s) format, Figure B1.  They were based on Tedmon/Hagel parameters or specific characteristics of the plots in Tedmon [1], with stoichiometric factors given in Table 1.  The shape of the COSP model curves is consistent with the Tedmon plot, but the characteristic parameters show some discrepancies.  The results are summarized in Table B1.   Curve 1 just represents parabolic growth with no volatility based on the initial portion of the curve.  Curve 2, open circles, is the original data.  Curve 3 (small, filled circles, teal) is the best COSP fit to the experimental data, but with lower kp = 0.34 mg2/cm4h and kv = 0.0273 mg/cm2h than those stated in Tedmon.  COSP curve 4, large dash, blue) matched the paralinear curve in Tedmon, but needed kv = 0.048 mg/cm2h; and the lowest curve 5, small dash, red) used the kp and kv prescribed by Tedmon (0.540 mg2/cm4h, 0.058 mg/cm2h), which underpredicts the data.

This suggests inconsistencies in the original analysis.  It appears that the experimental data corresponds to slightly lower kp and kv parameters:

- An effective reduction in kp can be generated by cubic-linear behavior, where grain growth over time simultaneously decreases scale growth controlled by short circuit paths.  This produces sub-parabolic behavior.  Attempts at a COSP cubic-linear fit produced improved an improved fit but did not match the entire curve, nor did the ‘perturbation’ solution offered by Chen et al., in their Figure 5.[18]  Barrett and Presler [5] found best fits for the Tedmon kp to vary from 0.29 – 0.44 mg2/cm4h, depending on the portion of the curve being fit, in general agreement with present findings (0.34 mg2/cm4h).

- The kv for the scale may be different than that measured by Hagel for bulk Cr2O3.[19]  This can occur for test conditions containing different gas flow rates or moisture content, both of which affect kv.  With little experimental detail available, no further speculation is offered.  The COSP model curves parallels the Tedmon plot, but implies kp and kv parameters ~ 50% less than those quoted.   Barrett and Presler [5] found best fits for the Tedmon kv  to vary from 0.023 – 0.034 mg/cm2h, depending on the portion of the curve being fit, in general agreement with present findings (kv = 0.027 mg/cm2h).