Supplementary materials

OCT use criteria for patients with ST-segment elevation myocardial infarction

- Stable hemodynamic conditions without cardiogenic shock, congestive heart failure, or other conditions requiring inotropes infusion

- TIMI flow grade ≥ 2 obtained at OCT image acquisition

- Preserved renal function with an estimated glomerular filtration rate (eGFR) ≥ 45 mL/min/1.73m2 or absence of known chronic kidney disease history

- Non aorto-ostial lesions

- Absence of angiographic findings suggestive of spontaneous coronary artery dissection

Procedural protocol for OCT/OFDI-guided and IVUS-guided PCI [1]

|  | OCT/OFDI-guided PCI | IVUS-guided PCI |
| --- | --- | --- |
| 1st. Pre-dilatation | - Pre-dilatation with a 2.0-mm semi-compliant balloon is allowed only when catheter passage through the lesion was unsuccessful or blood removal was insufficient potentially resulting in poor image quality for analyses. - Not specified for subsequent pre-dilatation after imaging investigation. | - Pre-dilatation with a 2.0-mm semi-compliant balloon is allowed only when catheter passage through the lesion was unsuccessful. - Not specified for subsequent pre-dilatation after imaging investigation. |
| 2nd. Stent landing zone (i.e. reference sites) | - Ideally the most normal looking sites that had the largest lumen without lipidic plaque >2 quadrants or suggestive thin-cap fibroatheroma (TCFA). - Length will be measured between the proximal and the distal reference sites. | - Ideally the most normal looking sites that had the largest lumen with a percentage plaque area of ≤50%. - Length will be measured between the proximal and the distal reference sites. |
| 3rd. Stent sizing | - By measuring lumen area at the proximal and the distal reference sites. - Nominal size will be selected by taking 10% or 0.25-mm larger than mean lumen diameter. | - By measuring vessel area (external elastic lamina) at the proximal and the distal reference sites. - In case with diseased reference sites (e.g. diffuse lesions), lumen diameter can also be referred. |
| 4th. Post-dilatation | - A non-compliant balloon with a nominal diameter 0.25-mm larger than that of selected stent is recommended. | - Not specified |
| 5th. Stent optimization | - Stent expansion index ≥80% - Malapposed stent strut <20% - Absence of major edge dissection |  |

OCT = optical coherence tomography; OFDI = optical frequency domain imaging; IVUS = intravascular ultrasound; PCI = percutaneous coronary intervention.

References

[1] Muramatsu T, Ozaki Y, Nanasato M, Ishikawa M, Nagasaka R, Ohota M, et al. Comparison Between Optical Frequency Domain Imaging and Intravascular Ultrasound for Percutaneous Coronary Intervention Guidance in Biolimus A9-Eluting Stent Implantation: A Randomized MISTIC-1 Non-Inferiority Trial. Circ Cardiovasc Interv. 2020;13:e009314.