ONLINE RESOURCE

TECHNO-ECONOMIC STUDY AND ENVIRONMENTAL ANALYSIS FOR THE PRODUCTION OF BIO-METHANOL USING A SOLAR-AIDED DUAL-BED GASIFIER

Christian Yakan a Nwaia

Bilal Patelb

aDepartment of Chemical Engineering, University of South Africa, Rooderport 1709, South Africa. Email: yakanchristian@gmail.com (corresponding author)

bDepartment of Chemical Engineering, University of South Africa, Rooderport 1709, South Africa. Email: patelb@unisa.ac.za

RBPD

RBPD

Fig. S1 Illustration of corn stover transportation operation

Table S1: Kinetic data constants used for methanol synthesis [1]

|  | Unit | Kinetic parameter |
| --- | --- | --- |
| k5a'K2'K3'K4'KH2PCO2 | mol∙kgcat-1s-1Pa-2 | 1.07×10-3exp36696R.T |
| k1' | mol∙kgcat-1s-1Pa-2 | 1.22×109exp94765R.T |
| KH2OK8K9KH2 | - | 3.45338 |
| KH2PH2 | Pa-0.5 | 31.578×10-3exp17197R.T |
| KH2O | Pa-1 | 6.62×10-16exp124119R.T |

Table S2: Capital cost breakdown of major equipment used

|  |  |  | COST DATA OBTAINED FROM LITERATURE |  |  |  |  |  |  |  |  |  |  |  |  | COST DATA ESCALATION DATA IN THE PRESENT STUDY |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Base cost | Base currency |  | Currency Base Year |  | Base scale |  | Base unit scale |  | Scale factor |  | Reference |  | Scale in the present study | Cost after scaling |  | Cost adjusted to 2019 using CEPCI |  |
| GASIFICATION AREA |  | Indirect gasifier & combustor | 6.7 |  | M Euro |  | 2007 |  | 20 |  | MWth input |  | 0.72 |  | [2] | 387 |  | 56.61 |  | 72.00 |
|  |  | Tar reformer | 93.7 |  | M $ |  | 2007 |  | 31 000 |  | Syngas flow at exit kmol/hr |  | 0.9 |  | [3] | 2 430 |  | 9.48 |  | 10.96 |
|  |  | Fabric filter | 68.8 |  | k $ |  | 2002 |  | 15.6 |  | m3 / s |  | 1 |  | [4] | 32.78 |  | 144.57 |  | 222.01 |
|  |  | water scrubber | 3 |  | M $ |  | 2002 |  | 12 |  | m3 / s |  | 0.7 |  | [5] | 32.78 |  | 6.03 |  | 9.26 |
|  |  | water gas shift | 3.36 |  | M $ |  | 2007 |  | 815 |  | MWLHV (dried biomass) |  | 0.67 |  | [3] | 387 |  | 2.04 |  | 2.36 |
| CO2 REMOVAL |  | MEA | 5.19 |  | M Euro |  | 2010 |  | 17 |  | kg CO2 esp / s |  | 1 |  | [4] | 10.1592 |  | 3.14 |  | 3.81 |
| METHANOL SYNTHESIS |  | MeOH reactor | 3.5 |  | M Euro |  | 2001 |  | 87.5 |  | ton MeOH / hr |  | 0.72 |  | [6]) | 5.92 |  | 0.50 |  | 0.85 |
|  |  | recycle compressor | 12.9 |  | M Euro |  | 2002 |  | 13.2 |  | MWel |  | 0.85 |  | [6] | 2.2 |  | 2.81 |  | 4.75 |
|  |  | syngas compressor (before MeOH synthesis | 12.9 |  | M Euro |  | 2002 |  | 13.2 |  | MWel |  | 0.85 |  | [6] | 36 |  | 30.27 |  | 51.13 |
|  |  | MeOH product separation and purification | 1.72 |  | M $ |  | 2002 |  | 4.66 |  | kg / s |  | 0.291 |  | [7] | 4.3 |  | 1.68 |  | 2.58 |
| MEOH PURIFICA-TION |  | MeOH refining | 15.1 |  | M $ |  | 2001 |  | 87.5 |  | ton MeOH / hr |  | 0.7 |  | [7] | 5.92 |  | 2.29 |  | 3.53 |
| POWER GENERATION AREA |  | Boiler/steam generator/ductwork/stack | 52 |  | M $ |  | 2007 |  | 355 |  | Boiler duty MWth |  | 1 |  | [3] | 146 |  | 21.39 |  | 24.73 |
|  |  | Steam cycle (steam turbine/condenser/piping/auxiliaries) | 66.7 |  | M $ |  | 2007 |  | 275 |  | Gross power production MWel |  | 1 |  | [3] | 17 |  | 4.12 |  | 4.77 |

Table S3: Salary structure of biorefinery’ s employees

| Plant manager | R850 000 | 1 | 850 000 |
| --- | --- | --- | --- |
| Plant engineer | R750 000 | 2 | 1 500 000 |
| Maintenance supervisor | R500 000 | 1 | 500 000 |
| Maintenance technician | R400 000 | 12 | 4 800 000 |
| Lab manager | R600 000 | 1 | 600 000 |
| Lab technician | R500 000 | 2 | 1 000 000 |
| Shift supervisor | R450 000 | 4 | 1 800 000 |
| Shift operator | R350 000 | 20 | 7 000 000 |
| Yard employees | R126 254 | 4 | 505 016 |
| Clerks and secretaries | R300 000 | 3 | 900 000 |
| TOTAL |  | 60 | R19 455 016 |
|  |  |  | $1 111 715 |

*Converted to USD using a USD/ZAR exchange rate of 17.5

# References

| [1] | K. V. Bussche and G. Froment, "A Steady-State Kinetic Model for Methanol Synthesis and the Water Gas Shift Reaction on a Commercial Cu / ZnO / Al 2 O 3 Catalyst," Journal of Catlysis, vol. 161, no. ARTICLE NO. 0156, pp. 1-10, 1996. |
| --- | --- |
| [2] | L. Tock, M. Gassner and F. Maréchal, "Thermochemical production of liquid fuels from biomass: Thermo-economic modeling, process design and process integration analysis," Biomass and Bioenergy, vol. 34, no. 12, pp. 1838-1854, 2010. |
| [3] | G. Liu, E. D. Larson, R. H. Williams, T. G. Kreutz and X. Guo, "Making Fischer−Tropsch Fuels and Electricity from Coal and Biomass: Performance and Cost Analysis," Energy & Fuels, vol. 25, no. 1, pp. 415-437, 2011. |
| [4] | S. Heyne and S. Harvey, "Impact of choice of CO2 separation technology on thermo-economic performance of Bio-SNG production processes," International Journal of Energy Research, vol. 38, pp. 299-318, 2013. |
| [5] | C. N. Hamelinck, A. P. Faaij and H. d. Uil, "Production of FT transportation fuels from biomass;technical options, process analysis and optimisation, and development potential," Energy, vol. 29, p. 1743 – 1771, 2004. |
| [6] | C. N. Hamelinck and A. P. Faaij, "Future prospects for production of methanol and hydrogen from biomass," Journal of Power Sources, vol. 111, no. 1, pp. 1-22, 2002. |
| [7] | E. D. Larson and R. Tingjin, "Synthetic fuel production by indirect coal liquefaction," Energy for Sustainable Development, vol. 7, no. 4, pp. 79-102, 2003. |