Supplementary File 2

The Effect of Arterial Cannula Tip Position on Differential Hypoxemia During Venoarterial Extracorporeal Membrane Oxygenation, Physical and Engineering Sciences in Medicine

Avishka Wickramarachchi*, Aidan J.C. Burrell, Andrew F. Stephens, Michael Šeman, Ashkan Vatani, Mehrdad Khamooshi, Jaishankar Raman, Rinaldo Bellomo, Shaun D. Gregory.

*Corresponding Author:

avishka.wickramarachchi@monash.edu, Cardio-respiratory Engineering and Technology Laboratory, Baker Heart and Diabetes Institute, Melbourne, Australia

# Windkessel Parameters

The 3-element Windkessel functions lump the systemic circulation into peripheral and distal resistances whilst also modeling compliance effects. The relationships between the proximal and distal arterial resistances (Rp and Rd, respectively), compliance (C), pressure (P) and flow (Q) result in the following ordinary differential equation which was solved iteratively at each branch outlet:

dPdt+PCRd=QC1+RpRd+RpdQdt #1

The parameters for resistance and compliance were obtained from literature and are outlined in Table 1 (1).

Table 1: Resistance and compliance values for each branch (1).

BCA – Brachiocephalic artery, LCCA – Left Common Carotid artery, LSCA – Left Subclavian artery, SMA – Superior Mesenteric artery, L-Renal – Left Renal artery, R-Renal – Right Renal artery, IMA – Inferior Mesenteric artery, LCI – Left Common Iliac artery and RCI – Right Common Iliac artery.

| Artery | Peripheral Resistance (mmHg ∙ min ∙ L-1) | Distal Resistance (mmHg ∙ min ∙ L-1) | Compliance (10-5 ∙ mmHg-1 ∙ L) |
| --- | --- | --- | --- |
| BCA | 6.49 | 132.61 | 11.60 |
| LCCA | 23.94 | 652.72 | 2.36 |
| LSCA | 12.35 | 162.74 | 9.45 |
| Coeliac | 14.70 | 94.67 | 16.24 |
| SMA | 21.80 | 68.88 | 22.33 |
| L-Renal | 42.68 | 67.44 | 22.80 |
| R-Renal | 42.68 | 67.44 | 22.80 |
| IMA | 92.53 | 577.86 | 2.66 |
| LCI | 7.39 | 127.19 | 12.09 |
| RCI | 7.39 | 127.19 | 12.09 |

1.	Xiao N, Alastruey J, Alberto Figueroa C. A systematic comparison between 1‐D and 3‐D hemodynamics in compliant arterial models. International journal for numerical methods in biomedical engineering. 2014;30(2):204-31.https://www.doi.org/10.1002/cnm.2598