Supporting Information for the article:

Fully-Automated SPE Coupled to UPLC-MS/MS Method for Simultaneous Detection of Sulfonamides, Quinolones, and Macrolide antibiotics in Water

Ming Zhenga,* ,1, Suwen Tanga, 1, Yangyang Baob, Kevin D. Danielsc Zuo Tong Howd, Mohamed Gamal El-Dind, Liang Tanga, *

a Key Laboratory of Organic Compound Pollution Control Engineering, School of Environmental and Chemical Engineering, Shanghai University, Shanghai 200444, China

b Pudong New Area Environmental Monitoring Station, No.51 Lingshan Road.Pudong New Area, Shanghai, China.

c Hazen and Sawyer, 1400 E. Southern Ave, Tempe, Arizona, 85282, USA

d Department of Civil & Environmental Engineering, University of Alberta, Edmonton, Alberta, T6G 1H9, Canada

1 These authors contributed equally to this work.

Suwen Tang: ytdxtangsuwen@163.com

Yangyang Bao: byy_0625@yeah.net

Kevin D. Daniels: kddaniels@outlook.com

Zuo Tong How: how@ualberta.ca

Mohamed Gamal El-Din: mgamalel-din@ualberta.ca

*Corresponding authors

E-mail addresses: 1010zhengming@tongji.edu.cn (M. Zheng). Tel.: +86 21 66137508; fax: +86 21 66137787；

E-mail addresses: tang1liang@shu.edu.cn (L. Tang).

# Tables

Table S1 Longitude and latitude of sampling stations and related parameters

| Sampling | Description | Longitude | Latitude |
| --- | --- | --- | --- |
| WWE1 | Outlet 1 of Bailongang STP | 121°44'35’’ | 31°14'52’’ |
| WWE2 | Outlet 2 of Bailongang STP | 121°44'35’’ | 31°14'52’’ |
| SW1 | The water of the Yangtze River near WWTP outlet | 121°45’9.6’’E | 31°15’31’’N |
| SW2 | 5 km upstream of Yangtze River from SR1-section | 121°42’37’’E | 31°17’39’’N |
| SW3 | 4 km downstream of Yangtze River from SR1 section | 121°45’03’’E | 31°8’47’’N |

Table S2 Compound-dependent parameters in MS/MS analysis

| Category | Compound | Precursor Ion(m/z) | Product Ion (m/z) | Fragmentor (v) | Collision Energy (v) | Retention time (min) |
| --- | --- | --- | --- | --- | --- | --- |
| Sulfonamides | SD | 251.06 | 156 | 110 | 14 | 3.436 |
|  |  |  | 92 | 110 | 26 |  |
|  | SDMD | 265.08 | 155.9 | 126 | 14 | 5.123 |
|  |  |  | 92.1 | 126 | 34 |  |
|  | SM2 | 279.09 | 186 | 138 | 18 | 7.068 |
|  |  |  | 92 | 138 | 34 |  |
|  | SMT | 271.03 | 156 | 118 | 14 | 7.414 |
|  |  |  | 92 | 118 | 34 |  |
|  | SMX | 254.06 | 155.9 | 86 | 10 | 9.624 |
|  |  |  | 92.1 | 86 | 26 |  |
|  | SDM | 311.08 | 156 | 134 | 20 | 15.719 |
|  |  |  | 92.1 | 134 | 36 |  |
| Quinolones | MAR | 363.37 | 320 | 122 | 14 | 7.461 |
|  |  |  | 72.2 | 122 | 20 |  |
|  | ENR | 360.41 | 342.1 | 128 | 18 | 10.672 |
|  |  |  | 316.1 | 128 | 18 |  |
|  | DAM | 358.51 | 340.1 | 132 | 22 | 10.988 |
|  |  |  | 82.1 | 132 | 46 |  |
|  | DFLX | 400.31 | 382.1 | 132 | 22 | 11.413 |
|  |  |  | 356.1 | 132 | 18 |  |
|  | OFL | 362.38 | 318.1 | 138 | 18 | 8.78 |
|  |  |  | 261.1 | 138 | 26 |  |
|  | NOR | 320.31 | 302.1 | 108 | 22 | 9.415 |
|  |  |  | 276.1 | 108 | 14 |  |
|  | LOM | 352.51 | 314 | 129 | 42 | 10.817 |
|  |  |  | 231 | 129 | 42 |  |
|  | NAL | 233.25 | 215.1 | 96 | 13 | 21.115 |
|  |  |  | 187 | 96 | 25 |  |
|  | FLU | 262.26 | 244.1 | 98 | 18 | 22.458 |
|  |  |  | 202 | 98 | 34 |  |
|  | CIP | 332.51 | 314 | 129 | 18 | 10.073 |
|  |  |  | 231 | 129 | 42 |  |
| Macrolides | LIN | 407.22 | 359.1 | 128 | 18 | 5.881 |
|  |  |  | 126.1 | 138 | 30 |  |
|  | SPM | 843.52 | 174.1 | 204 | 42 | 16.705 |
|  |  |  | 101 | 204 | 54 |  |
|  | TILM | 869.61 | 174 | 108 | 48 | 20.974 |
|  |  |  | 88.1 | 108 | 70 |  |
|  | TYL | 916.53 | 174 | 238 | 44 | 26.159 |
|  |  |  | 101 | 238 | 50 |  |
|  | LM | 772.4 | 174 | 146 | 32 | 26.162 |
|  |  |  | 109 | 146 | 48 |  |
|  | ETM-H2O | 716.46 | 558.3 | 150 | 14 | 26.65 |
|  |  |  | 158.1 | 132 | 30 |  |
|  | CTM | 748.51 | 158.1 | 162 | 29 | 27.338 |
|  |  |  | 83.1 | 156 | 60 |  |
|  | AZM·2H2O | 749.41 | 591.3 | 130 | 30 | 27.337 |
|  |  |  | 83.1 | 138 | 60 |  |
|  | ROM | 837.51 | 679.4 | 158 | 18 | 27.565 |
|  |  |  | 158.1 | 158 | 38 |  |
|  | CLI | 425.19 | 377.1 | 150 | 18 | 19.605 |
|  |  |  | 126.1 | 150 | 34 |  |

Table S3 Absolute Recoveries of target antibiotics on six different SPE cartridges/disks. (Analytes of 100 ug L-1)

| Category | Compound | Absolute Recovery (%) and the corresponding RSD (%) (n=5) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HLB disk | RSD (%) | C18 disk | RSD (%) | HLB cartridge | RSD (%) | C18 cartridge (Lab tech) | RSD (%) | C18 cartridge (IST) | RSD (%) | Florisil cartridge | RSD (%) |
| Sulfonamides | SD | 68.7 | 1.3 | 98.4 | 1.2 |  |  |  |  |  |  |  |  |
|  | SDMD | 71.2 | 5.2 | 97 | 4.1 | 6.8 | 20.4 |  |  | 5.85 | 17.3 |  |  |
|  | SM2 | 84.2 | 7.5 | 94.1 | 2.2 | 10.3 | 9.7 |  |  | 12.8 | 9.3 | 9.9 | 23.7 |
|  | SMT | 69.5 | 11.9 | 88.8 | 7.6 | 21.7 | 6.2 |  |  | 21.8 | 3.3 |  |  |
|  | SMX | 60.2 | 7.7 | 90.3 | 8.4 | 20.5 | 2.3 |  |  | 18.8 | 8.9 |  |  |
|  | SDM | 74 | 8.6 | 102.4 | 5.2 | 82.8 | 5.2 |  |  | 192.3 | 17.6 | 18.2 | 11.2 |
| Quinolones | MAR | 50.2 | 6.7 | 7.8 | 5.6 | 35.4 | 12.6 | 2.0 | 19.4 | 34.6 | 13.3 |  |  |
|  | ENR | 19.8 | 4.4 | 2.5 | 6.6 | 13.5 | 15.8 | 1.1 | 11.3 | 27.4 | 9.3 |  |  |
|  | DAM | 19.7 | 7.5 |  |  | - |  | - |  | 13.4 | 6.4 |  |  |
|  | DFLX | 37.5 | 10.4 | 21.5 | 2.4 | 50.8 | 5.1 | 8.8 | 8.4 | 60.2 | 7.4 |  |  |
|  | OFL | 35.4 | 14.1 | 7.7 | 4.2 | 10.0 | 10.7 | 2.5 | 19.2 | 38.5 | 8.6 | 1.5 | 7.7 |
|  | NOR | 89.1 | 7.9 | 54.8 | 12.5 | 34.0 | 4.1 |  |  | 60.2 | 2.6 |  |  |
|  | LOM | 30.3 | 5.3 | 21.9 | 20.4 | 52.0 | 7.9 |  |  | 52.0 | 2.3 |  |  |
|  | NAL | 47.4 | 9.6 | 57 | 9.2 | 50.3 | 20.9 |  |  |  |  |  |  |
|  | FLU | 84.6 | 5.5 | 157.1 | 7.5 | 243.6 | 26.4 | 26.0 | 24.6 | 130.7 | 3.1 | 12.1 | 8.9 |
|  | CIP | 39 | 6.5 | 26.5 | 8.1 | 28.2 | 10.3 | 16.0 | 17.9 | 51.5 | 8.3 | 14.2 | 9.1 |
| Macrolides | LIN | 12 | 14.4 | 29.3 | 6.4 | 89.4 | 4.6 | 39.2 | 9.3 | 59.8 | 12.9 | 12.0 | 10.5 |
|  | SPM | 55.5 | 7.8 |  |  | 6.9 | 2.9 |  |  |  |  | 31.5 | 12.5 |
|  | TILM | 27.4 | 2.1 | 9.9 | 9.1 | 28.8 | 6.7 | 7.0 | 5.1 | 27.4 | 9.4 | 2.4 | 7.5 |
|  | TYL | 106.8 | 4.7 | 96.1 | 13.5 | 153.5 | 5.4 | 112.8 | 6.5 | 153.9 | 12.2 | 2.4 | 6.2 |
|  | LM | 90.5 | 11.6 | 60.8 | 3.6 | 20.4 | 7.6 | 31.7 | 10.4 | 44.6 | 7.5 | 37.8 | 15.7 |
|  | ETM-H2O | 51.8 | 16.2 | 42 | 4.2 | 28.7 | 3.3 | 41.1 | 9.4 | 35.1 | 3.2 | 31.2 | 12.2 |
|  | CTM | 63.6 | 9.6 | 36.3 | 20.3 | 30.4 | 4.1 | 10.2 | 15.6 | 26.1 | 1.9 | 7.0 | 8.5 |
|  | AZM·2H2O | 75.3 | 7.4 | 119.7 | 6.1 | 93.7 | 6.6 | 37.2 | 12.8 | 93.1 | 5.1 | 37.1 | 7.6 |
|  | ROM | 86.5 | 5.6 | 128.2 | 5.3 | 84.4 | 6.4 | 40.6 | 78.1 | 115.1 | 9.3 | 38.2 | 9.7 |
|  | CLI | 81 | 2.4 | 119.4 | 7.2 | 87.3 | 11.2 | 69.9 | 7.3 | 109.4 | 3.7 | 43.8 | 15.6 |

RSD: relative standard deviation. The eluent consist of 40 mL mixed solvent of methanol and acetone.

Table S4 Optimal selection of elution solvents on the recoveries for antibiotics.

| Category | Compound | Methanol | Acetone | Methanol/Methylene chloride | Methanol/Acetone | Methylene chloride |
| --- | --- | --- | --- | --- | --- | --- |
| Sulfonamides | SD | - | √ | - | - | - |
|  | SDMD | - | √ | - | - | - |
|  | SM2 | - | √ | - | - | - |
|  | SMT | √ | - | - | - | - |
|  | SMX | - | - | - | - | √ |
|  | SDM | - | - | √ | - | - |
| Quinolones | MAR | √ | - | - | - | - |
|  | ENR | - | - | √ | - | - |
|  | DAM | - | - | - | - | √ |
|  | DFLX | √ | - | - | - | - |
|  | OFL | - | - | - | - | √ |
|  | NOR | - | - | - | √ | - |
|  | LOM | - | √ | - | - | - |
|  | NAL | - | - | √ | - | - |
|  | FLU | √ | - | - | - | - |
|  | CIP | √ | - | - | - | - |
| Macrolides | LIN | √ | - | - | - | - |
|  | SPM |  | √ | - | - | - |
|  | TILM | √ | - | - | - | - |
|  | TYL | - | - | - | √ | - |
|  | LM | - | √ | - |  | - |
|  | ETM-H2O | - |  | - | √ | - |
|  | CTM | - | √ | - | - | - |
|  | AZM.·2H2O | - | - | - | - | √ |
|  | ROM | √ | - | - | - | - |
|  | CLI | - | √ | - | - | - |

The elution effect were all calculated at a time of 6 mL eluent.

Table S5 Optimal selection of elution volume on the recoveries for antibiotics using MeOH and acetone.

| Category | Compound | Elution volume (mL) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3 6 | 4 6 | 5 6 |
| Sulfonamides | SD |  | MeOH |  |
|  | SDMD |  | MeOH |  |
|  | SM2 |  | MeOH |  |
|  | SMT |  | MeOH |  |
|  | SMX |  | MeOH |  |
|  | SDM |  | MeOH |  |
| Quinolones | MAR |  | MeOH |  |
|  | ENR |  |  | MeOH |
|  | DAM |  | Acetone |  |
|  | DFLX |  | Acetone |  |
|  | OFL |  | Acetone |  |
|  | NOR | MeOH |  |  |
|  | NAL | Acetone |  |  |
|  | FLU |  | MeOH |  |
|  | CIP | MeOH |  |  |
| Macrolides | LIN |  | MeOH |  |
|  | SPM |  |  | MeOH |
|  | TILM |  | Acetone |  |
|  | TYL |  | MeOH |  |
|  | LM | MeOH |  |  |
|  | ETM-H2O |  | MeOH |  |
|  | CTM |  | MeOH |  |
|  | AZM.·2H2O | Acetone |  |  |
|  | ROM |  | MeOH |  |
|  | CLI |  | MeOH |  |

Table S6 Matrix spike recoveries of target antibiotics in PW, SW and WWE and the corresponding matrix effects (MEs). (Target analytes and isotopically-labeled surrogates (ISs) were set at 50 μg L-1)

| Category | Compound | PW |  | SW (n=5) |  |  | WWE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Recovery (%) | RSD (%) | Recovery (%) | RSD (%) | ME (%) | Recovery (%) | RSD (%) | ME (%) |
| Sulfonamides | SD | 96.3 | 2.9 | 80.0 | 2.3 | 17.0 | 96.5 | 2.4 | -0.2 |
|  | SDMD | 99.2 | 1.7 | 79.3 | 4.5 | 20.0 | 88.1 | 2.6 | 11.2 |
|  | SM2 | 89.8 | 7.1 | 73.4 | 4.3 | 18.3 | 80.3 | 4.7 | 10.5 |
|  | SMT | 95.0 | 5.4 | 80.4 | 1.3 | 15.4 | 87.0 | 2.9 | 8.4 |
|  | SMX | 95.7 | 3.2 | 82.8 | 2.2 | 13.5 | 102.8 | 5.0 | -7.4 |
|  | SDM | 94.0 | 4.3 | 91.3 | 3.6 | 2.8 | 94.0 | 4.1 | 0.0 |
| Quinolones | MAR | 76.6 | 2.5 | 71.5 | 2.4 | 6.7 | 65.3 | 6.8 | 14.8 |
|  | ENR | 83.8 | 2.9 | 75.0 | 4.6 | 10.6 | 67.2 | 8.3 | 19.8 |
|  | DAM | 64.6 | 4.2 | 69.8 | 2.4 | -8.0 | 65.8 | 6.8 | -1.9 |
|  | DFLX | 87.2 | 4.1 | 76.9 | 4.6 | 11.8 | 76.2 | 8.3 | 12.6 |
|  | OFL | 74.3 | 3.1 | 69.8 | 5.8 | 6.1 | 67.8 | 6.9 | 8.8 |
|  | NOR | 79.9 | 2.4 | 75.6 | 3.8 | 5.3 | 77.7 | 6.0 | 2.7 |
|  | LOM | 72.7 | 6.1 | 66.9 | 5.2 | 8.0 | 70.5 | 4.6 | 3.1 |
|  | NAL | 97.0 | 3.4 | 72.0 | 2.0 | 25.8 | 73.7 | 6.0 | 24.0 |
|  | FLU | 88.2 | 2.4 | 80.3 | 3.4 | 8.9 | 82.0 | 4.8 | 7.0 |
|  | CIP | 84.4 | 11.0 | 79.1 | 4.6 | 6.3 | 74.0 | 2.4 | 12.3 |
| Macrolides | LIN | 74.6 | 2.1 | 68.5 | 4.5 | 8.2 | 71.5 | 4.4 | 4.1 |
|  | SPM | 76.3 | 3.4 | 63.5 | 3.5 | 16.7 | 65.1 | 2.3 | 14.6 |
|  | TILM | 69.4 | 2.1 | 69.0 | 8.6 | 0.6 | 64.7 | 4.4 | 6.9 |
|  | TYL | 64.4 | 4.4 | 60.8 | 4.5 | 5.5 | 59.9 | 4.4 | 7.0 |
|  | LM | 63.3 | 2.3 | 62.7 | 4.3 | 0.9 | 61.9 | 4.4 | 2.3 |
|  | ETM-H2O | 70.0 | 4.6 | 69.5 | 5.6 | 0.8 | 68.4 | 4.9 | 2.3 |
|  | CTM | 74.5 | 6.7 | 74.5 | 7.8 | 0.0 | 74.0 | 1.8 | 0.7 |
|  | AZM.·2H2O | 73.7 | 7.8 | 70.7 | 6.9 | 4.1 | 70.7 | 5.5 | 4.1 |
|  | ROM | 86.0 | 5.5 | 88.9 | 5.2 | -3.3 | 75.5 | 5.7 | 12.2 |
|  | CLI | 69.1 | 2.4 | 64.0 | 10.2 | 7.4 | 69.7 | 6.0 | -0.9 |

PW: pure water; SW: surface water; WWE: wastewater effluent; ME: matrix effect

# Figures

B

A2

A1

B

A2

A1

Figure S1 (A) Automatic cartridge-disk universal solid phase extraction system (LabTech, China), (B) MultiVap-8 channel parallel concentrator (LabTech, China).

Figure S2 Total ion chromatogram of target compounds with mixing standards.

(a) Sulfadiazine

(a) Sulfadiazine

(b) Sulfamerazine

(b) Sulfamerazine

(c) Sulfamethazine

(c) Sulfamethazine

(d) Sulfamethoxadiazole

(d) Sulfamethoxadiazole

(e) Sulfamethoxazole

(e) Sulfamethoxazole

(f) Sulfamethoxine

(f) Sulfamethoxine

(g) Mabofloxacin

(g) Mabofloxacin

(h) Enrofloxacin

(h) Enrofloxacin

(h) Enrofloxacin

(h) Enrofloxacin

Figure S3 (a)(z) Mass spectra of 26 antibiotics standards.

(i) Dafloxacin mesylate

(i) Dafloxacin mesylate

(j) Difloxacin

(j) Difloxacin

(k) Ofloxacin

(k) Ofloxacin

(l) Norfloxacin

(l) Norfloxacin

(m) Lomefloxacin hydrochloride

(m) Lomefloxacin hydrochloride

(n) Nalidixic acid

(n) Nalidixic acid

(o) Flumequine

(o) Flumequine

Figure S3 (a)(z) Mass spectra of 26 antibiotics standards (continued)..

(p) Ciprofloxacin Hydrochloride

(p) Ciprofloxacin Hydrochloride

(q) (7S)-Lincomycin

(q) (7S)-Lincomycin

(r) Spiramycin

(r) Spiramycin

(s) Tilmicaoxin

(s) Tilmicaoxin

(t) Tylosin

(t) Tylosin

(u) Leucomycin A5

(u) Leucomycin A5

(v) Anhydroerythromycin A

(v) Anhydroerythromycin A

(w) Clarithromycin

(w) Clarithromycin

(x) Azithromycin Dihydrate

(x) Azithromycin Dihydrate

(y) Roxithromycin

(y) Roxithromycin

(z) Clindamycin hydrochloride

(z) Clindamycin hydrochloride

Figure S3 (a)(z) Mass spectra of 26 antibiotics standards (continued).

pH 5.0

pH 4.0

pH 3.0

pH 5.0

pH 4.0

pH 3.0

Figure S4 The eluent of different pH.

Figure S5 Absolute recoveries in pure water (PW), surface water (SW), and wastewater effluent (WWE). (Target analytes and isotopically-labeled surrogates (ISs) were set at 50 μg L-1).