Supplementary Information

#### Covariates used in modelling geographic access

The accessibility model corrects for various landscape factors that were barriers towards travelling to health facilities. The Table 1 shows where these datasets were obtained including their spatial resolution.

SI Table1. Covariates used in modelling spatial accessibility including their type, sources and spatial resolution.

| Data Layer | Format | Spatial resolution | Purpose | Source |
| --- | --- | --- | --- | --- |
| Land Use | Raster | 20m | Provide land feature classes (i.e., forestland, grassland, cropland, settlement, wetland) | sentinel 2 satellite sensor was downloaded from (https://www.rcmrd.org/) |
| Elevation | Raster | 30m | To define slope used for adjustment of walking speed | Shuttle Radar Topography Mission downloaded from (http://gdex.cr.usgs.gov/gdex/) |
| Roads | Vector | NA | Provide road networks where motorized transport is enabled. With road classed defined in Appendix 4 | Ministry of Transport, Infrastructure, Housing & Urban Development maps digitised |
| Protected Areas | Vector | NA | Provide a layer of barrier to travel | KWS and KFS obtained from (http://biodiversityatlaskenya.org/bio-geoportal/) |
| Water bodies (Rivers and lakes) | Vector | NA | Provide layers of barriers to travel | Global Lakes and Wetlands Database from (http://www.worldwildlife.org/pages/global-lakes-and-wetlandsdatabase) |

Table2: Transport speeds used in creating the cost friction surface. For roads, only those in poor condition were deemed impassable in the wet period.

| Data | Classification | Speed (Km/hr.) -Dry | Model |
| --- | --- | --- | --- |
| Land Use | Cultivated Land | 5 | Walking |
|  | Tree cover | 5 | Walking |
|  | Grassland | 5 | Walking |
|  | Shrubland | 4 | Walking |
|  | Wetland | 2 | Walking |
|  | Urban Areas | 5 | Walking |
|  | Water Bodies | 0 | Walking |
|  | Forest | 0 | Walking |
| Roads | Class S | 110 | Driving |
|  | Class A | 80 | Driving |
|  | Class B | 60 | Driving |
|  | Class H | 50 | Driving |
|  | Class J | 50 | Driving |
|  | Class C | 50 | Driving |
|  | Class D | 40 | Driving |
|  | Class E | 30 | Driving |
|  | Class F | 30 | Driving |
|  | Class G | 20 | Driving |
|  | Class K | 20 | Driving |
|  | Class L | 20 | Driving |
|  | Class M | 15 | Driving |
|  | Class R | 5 | Walking |
|  | Class T | 5 | Walking |
| DEM | Slope | W = 6 * exp {-3.5 * abs (S + 0.05)} | Walking |

Table3: Capacity of health facilities used in the modelling.

| min | max | label | capacity |
| --- | --- | --- | --- |
| 0 | 1000 | Outreach | 999 |
| 1001 | 10000 | Small facility | 9500 |
| 10001 | 100000 | Medium Facility | 99000 |
| 100001 | 500000 | Large facility | 450000 |