Appendix—Calculation of carbon metabolism

Carbon sequestration and emission mainly refer to the sequestration and emission of greenhouse gases (CO2, CH4, N2O). Due to data limitations, the calculation of carbon sequestration and emission in this paper only considers the CO2 from different land-use types and converts them into C uniformly. Typically, the natural land-use type such as forests, grassland, and waters exert carbon sequestration functions, built-up land with concentrated human activities mainly emits carbon emissions, and arable land absorbs carbon through photosynthesis and emits it through agricultural production (Wang et al., 2021). Therefore, this paper, mainly includes carbon metabolism, arable land, crop carbon absorption, forest, and grassland vegetation carbon sequestration, the waters of the carbon sequestration, dry wet sediment in water from arable land soil carbon and carbon emissions, and human carbon emissions, energy, industrial and construction land humans, animals, breathe carbon emissions and forest meadow soil respiration carbon emissions, And volatile carbon emissions from water bodies.

1 Carbon sequestration

1.1 Arable land

Crop carbon sequestration refers to the net primary production (NPP) formed by crop photosynthesis, that is, biological yield (Han et al., 2012), and the calculation formula is as follows:

CScrop=iCcrop-i×(1-Pwater-i)×Yeco-iHcrop-i

where the CScrop is the total carbon sequestration of the crop during the growth period (t), the Ci is the carbon sequestration rate of the crop, the Yeco-i is the economic yield of the crop (ten thousand t), the Hcrop-i is the economic coefficient of the crop, and the Pwater-i is the moisture content of the crop. The moisture content, carbon sequestration rate, and economic coefficient of major crops were based on data from existing studies (Wang, 1996; Han et al., 2012).

Table 1 Coefficients for various types of crops in Henan Province

| The main crops | Economic coefficient | Moisture content | Carbon uptake rate | Economic output in 1995 (unit: 10,000 tons) | Economic output in 2015 (unit: 10,000 tons) | 1995 NPP (in 10,000 tons t) | 2015 NPP (in 10,000 tons t) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| wheat | 0.4 | 0.12 | 0.485 | 1754.18 | 3501 | 1871.71 | 3735.57 |
| paddy | 0.45 | 0.12 | 0.414 | 295.82 | 531.52 | 239.5 | 430.32 |
| corn | 0.4 | 0.13 | 0.471 | 757.77 | 1853.65 | 776.28 | 1898.93 |
| soybean | 0.34 | 0.13 | 0.45 | 106.69 | 49.9 | 122.85 | 57.46 |
| sweet potato | 0.7 | 0.7 | 0.423 | 286.04 | 110.79 | 51.85 | 20.08 |
| peanut | 0.43 | 0.1 | 0.45 | 233.65 | 485.31 | 220.07 | 457.09 |
| fruit | 0.7 | 0.9 | 0.45 | 211.66 | 915.76 | 13.61 | 58.87 |
| vegetable | 0.6 | 0.9 | 0.45 | 1660.8 | 7456.52 | 124.56 | 559.24 |
| total | —— | —— | —— | —— | —— | 3420.42 | 7217.56 |

1.2 Forest and grassland

Forest and grassland can achieve carbon sequestration as natural ingredients (Wang et al., 2021). For carbon sequestration in forests, and grassland, this study mainly considered the net primary production (NPP) generated by vegetation photosynthesis. Carbon sequestration is estimated based on the area of forests and grasslands and the corresponding NPP factors (Zhao et al., 2015), as follows:

CSveg=iCveg-i×Areaveg-i

where the CSveg represents vegetation carbon uptake (t C), the Cveg-i is the NPP factor of the plantation quilt type (t C/(hm2·yr)), and the Areaveg-i is the area of the planting quilt type (hm2). Among them, the average NPP factor references different years of woodland and grassland in the study area (Li et al., 2011). The values of forest and grassland in 1995 were 4.571 and 2.204 t C/(hm2·yr), respectively. The values of forest and grassland in 2015 were 7.852 and 2.576 t C/(hm2·yr), respectively.

1.3 Waters

Water carbon sequestration mainly includes carbon sequestration in water areas and carbon absorption from dry and wet sedimentation in waters (Zhao et al., 2012), and the calculation formula is:

CSwater=Cunit-water×Areawater+Csub×Areawater

Where, CSwater is the total carbon uptake of the water (t); Cunit-water is the carbon sequestration rate of the river and lake, 0.567 t C/ (hm2·yr) (the value is taken from the eastern plain river) (Duan et al., 2008); the Areawater is the area of the river and lake; the Csub is the carbon input coefficient of dry and wet settlement per unit area, taking 0.116t C/(hm2·yr) (the value point is Henan Province) (Ye and Chen, 1992).

2 Carbon emission

2.1 Arable land

CEarable=CEsoil-arable+CEcombined-arable

CO2 emissions from soil respiration in arable land, calculated according to the relevant literature (Ye and Chen, 1992):

CEsoil-arable=EFsoil-arable×Areaarable

Where the CEsoil-arable is the soil respiratory CO2 emission from cultivated land; the EFarable is the annual respiration flux of arable soil (t CO2/hm2). In this paper, the annual soil respiration flux of arable land was 15.96 (t CO2/hm2) using the cumulative calculation of the average soil respiration rate of cultivated land of 182.21 mg CO2/(m2·h) in the existing study (Wang et al., 2007). Area (hm2) represents the area of arable land.

Refer to Oak Ridge National Laboratory and some Chinese literature (West and Marland, 2002； Tian and Zhang, 2013；Wang et al., 2021), based on the specific situation of Henan Province, this paper divides the carbon emissions of arable land into CO2 agricultural material inputs (fertilizers, pesticides, agricultural films), agricultural machinery uses energy (agricultural diesel), and irrigation energy consumption, as follows:

CEcombined-arable=CEfertilizer+CEmachinery+CEirrigation

CEfertilizer=EFi×ADi

CEmachinery=EFj×ADj

CEirrigation=EFirrigation×Area

Wherein, CEcombined-arableis the anthropogenic carbon emissions of cultivated land; CEfertilizer, CEmachinery, CEirrigation represent the carbon emissions generated by agricultural material inputs, mechanical energy, and irrigation (t); EFi(t C/t)，EFj(t C/t)and EF(t C/hm2) represent Class i agricultural materials, respectively. Class j energy used in agricultural machinery, as well as carbon emission coefficients for irrigation; ADi, ADj represent Class i materials and Class j energy inputs (t); Area (hm2) stands for irrigation area. Among them, the carbon emission coefficients of fertilizers, pesticides, agricultural films, diesel, and irrigation are referred to in the relevant literature (West and Marland, 2002; Eggleston, 2006; Duan et al., 2011; Tian and Zhang, 2013).

Table 2 Input volume and carbon emission coefficient of major agricultural materials in Henan Province

| Item | Inputs in 1995 | Inputs in 2015 | Carbon Emission Factors (t C/t)、(t C/hm2) | Carbon Emissions in 1995 (ten thousand Tons C) | Carbon Emissions in 2015 (ten thousand Tons C) |
| --- | --- | --- | --- | --- | --- |
| Fertilizer (ten thousand tons) | 322.21 | 716.09 | 0.8956 | 288.57 | 641.33 |
| Pesticides (ten thousand tons) | 7.56 | 12.87 | 4.9341 | 37.3 | 63.5 |
| Agricultural film (ten thousand tons) | 5.32 | 16.2 | 5.18 | 27.56 | 83.92 |
| Diesel (ten thousand tons) | 20.1 | 114.7 | 0.5927 | 11.91 | 67.98 |
| Irrigated area (1,000 hectares) | 4044.19 | 5210.64 | 0.2665 | 107.78 | 138.86 |
| total | —— | —— | —— | 473.12 | 995.59 |

2.2 Built-up land

The CO2 emissions from construction land in this paper consider carbon emissions from energy consumption, industrial production, and human and livestock respiratory carbons.

CEbuilt-up=CEenergy+CEindustrial+CEbreathe

2.2.1 Energy

Energy production and consumption activities are important sources of greenhouse gas emissions in our country. The carbon emissions from energy consumption in this study mainly refer to carbon dioxide produced by fossil fuel combustion activities. Drawing on the IPCC's calculation methodology, the CO2 emissions from various energy consumptions are calculated as follows:

CEenergy=iQi×Hi×Ci

Where the CEenergy is the CO2 emissions (Mt) of the energy, the consumption of the Qi energy (ten thousand t), the net calorific value of the Hi energy source (TJ/Gg), and the carbon emission coefficient (Kg/GJ) of the Ci energy. where Ci=Ai×Bi, the Ai is the default carbon content (Kg/GJ), and the Bi is the default carbon factor. Among them, the net calorific value, default carbon content, default carbon factor, etc. of various energy sources are from the IPCC (2006 IPCC National Greenhouse Gas Inventory Guidelines), and the energy consumption comes from the Henan Statistical Yearbook.

Table 3 CO2 emissions from energy consumption in Henan Province

| Item | Energy net calorific value NCVs (TJ/Gg) | Default carbon oxide factor | Default carbon content (Kg/GJ) | Energy consumption in 1995 (10 000t) | Energy consumption in 2015 (10 000t) | Energy CO2 emissions in 1995 (10 000t) | Energy CO2 emissions in 2015 (10 000t) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coal (10 000 tons) | 28.2 | 1 | 25.8 | 8516.48 | 25808.48 | 6196.25 | 18777.22 |
| Coke (10 000 tons) | 28.2 | 1 | 29.2 | 393.19 | 2926.1 | 323.77 | 2409.47 |
| Crude Oil (10 000 tons) | 42.3 | 1 | 20 | 401.96 | 847.26 | 340.06 | 716.78 |
| Gasoline (10 000 tons) | 43 | 1 | 18.9 | 142.55 | 676.63 | 115.85 | 549.9 |
| Kerosene (10 000 tons) | 43.8 | 1 | 19.5 | 16.42 | 68.16 | 14.02 | 58.22 |
| Diesel Oil (10 000 tons) | 43 | 1 | 20.2 | 134.81 | 835.3 | 117.1 | 725.54 |
| Fuel Oil (10 000 tons) | 40.4 | 1 | 26.6 | 52.89 | 70.66 | 56.84 | 75.93 |
| Natural Gas (100 million cu.m) | 48 | 1 | 15.3 | 9.37 | 77.02 | 49.37 | 405.79 |
| Total | —— | —— | —— | —— | —— | 7213.25 | 23718.84 |

2.2.2 Industry

Greenhouse gas (GHG) emissions from industrial production processes refer to greenhouse gas emissions from chemical reaction processes or physical changes in industrial production other than greenhouse gas emissions from energy activities (Guidelines for compiling provincial greenhouse gas inventories, 2011). Combined with the collectability of relevant data and the actual situation in Henan Province, this study selected CO2 generated by cement and calcium carbide production processes.

CEindustry=iADi×EFi

Wherein: CEindustry is the CO2 emissions of industrial production processes (Mt); ADi the output of the industrial product (ten thousand t); EFi the emission factor of the industrial process. Among them, the emission factor data refer to the Provincial Greenhouse Gas Emission Guide, the cement clinker data are from the references (Gao et al., 2012) and the China Cement Yearbook (China Cement Association, 2009), and the calcium carbide data is from the Henan Provincial Statistical Yearbook (http://tjj.henan.gov.cn/tjfw/tjcbw/tjnj/).

Table 4 CO2 emissions from industrial production in Henan Province

| Item | Production Process Emission Factors | 1995 Production (ten thousand tons) | 2015 Production (ten thousand tons) | Industrial Emissions in 1995 (ten thousand Tons CO2) | Industrial Emissions in 2015 (ten thousand tons CO2) |
| --- | --- | --- | --- | --- | --- |
| Cement clinker (t CO2/ t clinker) | 0.538 | 2411 | 8099.23 | 1297.12 | 4357.39 |
| Calcium carbide (t CO2/t calcium carbide) | 1.154 | 15.183 | 127.76 | 17.52 | 147.44 |
| total | —— | —— | —— | 1314.64 | 4504.82 |

2.2.3 Breathing

Human respiration and livestock rearing generate CO2 emissions in urban and rural settlements, which are calculated below.

CEbreath=CEhuman+CElivestock

CEhuman=Q×N

CElivestock=iQhi×Nli

where the CEhuman and the CElivestock (t CO2) represent the carbon emissions produced by human respiration and livestock rearing; Nh (t CO2/ (person∙yr)) and Nli (t CO2/(head∙yr)) represent the carbon emission factors of human respiration and i livestock rearing, respectively. Qh (person) and Qhi (head) represent the number of people and livestock. Among them, the reference to the carbon emission coefficient of human respiratory carbon (Matthews et al., 2000) is determined to be 0.3t CO2 / (person∙yr). Referring to the relevant literature (Tao, 2003), the CO2 emission coefficients of different animals were selected, and the emission coefficients of pigs, cattle, sheep, and poultry were selected: 0.301, 2.92, 0.237, and 0.013, respectively. The population and livestock numbers are from the Statistical Yearbook of Henan Province.

Table 5 CO2 emissions from respiration

| Item | Carbon emission coefficient (t CO2/ (person.yr)、t CO2/(head∙yr)) | 1995 quantity (ten thousand) | 2015 quantity (ten thousand) | 1995 CO2 emissions (ten thousand tons) | 2015 CO2 emissions (ten thousand tons) |
| --- | --- | --- | --- | --- | --- |
| Urban permanent population | 0.3 | 1564 | 5023 | 469.2 | 1506.9 |
| Rural permanent population | 0.3 | 7536 | 5699 | 2260.8 | 1709.7 |
| pig | 0.301 | 2667.7 | 4376 | 802.98 | 1317.18 |
| cattle | 2.92 | 1209 | 934 | 3530.19 | 2727.28 |
| sheep | 0.237 | 2298.3 | 1926 | 544.69 | 456.46 |
| poultry | 0.013 | 36504 | 70020 | 474.56 | 910.26 |
| total | —— | —— | —— | 8082.42 | 8627.78 |

2.3 Forests and grasslands

In this paper, carbon emissions from forests and grassland are mainly considered soil respiration. Soil carbon emissions are calculated from the average respiration rates of different soil types, which are determined by the litter ratio of different vegetation types (Fang et al., 1996). CO2 emissions from soil respiration in forests and grassland were calculated using the relevant literature (Ye and Chen, 1992):

CEforest/grassland=CEsoil-i=iEFi× Areai

where the CEsoil is soil respiratory CO2 emissions; EF is the annual soil respiratory flux (t/hm2). Areai(hm2) represents the area of woodland and grassland. In this paper, the average soil respiration rates of woodland and grassland were 264.68 and 242.91 mg CO2/(m2·h), respectively, and the cumulative calculations determined that the annual soil respiration flux of woodland and grassland was 23.19 and 21.28 t CO2/(hm2·yr), respectively.

2.4 Waters

Carbon emissions of water areas mainly refer to the volatile carbon of water bodies, and the volatile carbon of waters is measured by multiplying the area of the water area by the carbon volatility coefficient (Zhao et al., 2015).

CEwater=EFwater×Areawater

where the CEwater is the water carbon emission (t); the EFwater is the water carbon volatilization coefficient t C/ (hm2·yr); Areawater is the area of water. According to the relevant literature (Ye and Chen, 1992), the EFwater in this article takes 0.00041 t/ (hm2·yr) (the value is taken in the eastern plains)

3 References

China Cement Association, 2009. China Cement Yearbook. Jiangsu People’s Publishing House.

Duan, X., Wang, X., Lu, F., Ouyang, Z.Y., 2008. Carbon sequestration and its potential by wetland ecosystems in China. Acta Ecol. Sin. 28, 463–469.

Eggleston, S., 2006. IPCC Guidelines for National Greenhouse Gas Inventories. Energy 2.

Fang, J.Y., Liu, G.F., Xu, S.L., 1996. The carbon cycle of terrestrial ecosystems in China and its global significance in china. In: Monitoring and relevant process of greenhouse gas concentration and emission. eds.

Gao, C., Gao, G., Zhang, H., Cai, C., 2012. Calculation on CO2 Emissions from Cement Manufacturing Process in Henan Province. Resour. Dev. ＆Market 28, 696–698.

Guidelines for compiling provincial greenhouse gas inventories, 2011.

Han, Z., Meng, Y., Xu, J., Wu, Y., Zhou, Z., 2012. Temporal and spatial difference in carbon footprint of regional farmland ecosystem in Guangdong province. J. Agro-Environment Sci. 31, 1034–1041.

Li, J., Chen, H., Zou, C., Liu, Z., Guo, Q., Guo, P., 2011. Vegetation net primary productivity and its temporal-spatial changes in Henan province during 1994—2008. Ecol. Environ. Sci. 20, 1424–1429. https://doi.org/10.16258/j.cnki.1674-5906.2011.10.001

Matthews, E., Amann, C., Bringezu, S., 2000. The weight of nations: material outflows from industrial economies, 2000, World Resources Institute, Washington, DC.

Tao, Z., 2003. Ecological Burden and Ecological Footprint: The Concept of Weight and Area for Sustainable Development. Beijing :Economic Science Press.

Tian, Y., Zhang, J., 2013. Regional Differentiation Research on Net Carbon Effect of Agricultural Production in China. J. Nat. Resour. 28, 1298–1309. https://doi.org/10. 11849 /zrzyxb. 2013. 08． 003

Wang, C., Zhan, J., Zhang, F., Liu, W., Twumasi-Ankrah, M.J., 2021. Analysis of urban carbon balance based on land use dynamics in the Beijing-Tianjin-Hebei region, China. J. Clean. Prod. 281. https://doi.org/10.1016/j.jclepro.2020.125138

Wang, X., 1996. Carbon Dioxide,Climate Change,and Agriculture, Beijing: China Meteorological Press.

Wang, X., Zhu, B., Wang, Y., Zheng, X., 2007. Soil respiration and its sensitivity to temperature under different land use conditions. Acta Ecol. Sin. 27, 0–8.

West, T.O., Marland, G., 2002. A synthesis of carbon sequestration, carbon emissions, and net carbon flux in agriculture: comparing tillage practices in the United States. Agric. Ecosyst. Environ. 91, 217–232. https://doi.org/https://doi.org/10.1016/S0167-8809(01)00233-X

Ye, D., Chen, P., 1992. Study on Prediction of Global Change in China (Part 2). Beijing: Seismological Press.

Zhao, R., Huang, X., Liu, Y., Zhong, T., Ding, M., Chuai, X., 2015. Carbon emission of regional land use and its decomposition analysis: Case study of Nanjing City, China. Chinese Geogr. Sci. 25, 198–212. https://doi.org/10.1007/s11769-014-0714-1

Zhao, R., Huang, X., Peng, B., 2012. Research on Carbon Cycle and Carbon Balance of Nanjing Urban System. Acta Geogr. Sin. 67, 758–770.