| Table 2 Laser characteristics table. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authors | Type of laser | Wavelength (nm) | Mode | Output power (W) | Total exposure time (s) (n × t) Exposure time per point/session | Total energy (J) | Beam diameter (µm-mm)/ Probe spot size (cm2) | Energy density (Jcm2) | Distance to intervention site | Method of application | Frequency of laser treatment/ Number of irradiation sessions |
| Lingamaneni et al. 2019 | Diode (λ =810 nm Picasso diode laser, AMD lasers, Indianapolis, USA) | 810 | Continuous | 0.1 | 1200 (4×300) 300 | 120 | NM | NM | Noncontact (not mentioned the number) | Test site was exposed with a diode laser at a with the power of 0.1W used in a continuous wave, non-contact mode | Instantly post-surgery, and 7, and 14 days later/ 4 |
| Isler et al. 2018 | Diode ( λ = 970 ±15 nm, 14 W source power; SIRO Laser Xtend, Sirona Dental Systems GmbH, Bensheim, Germany) | 970 | Continuous | 2 | 120 (4×30) 30 | 240 | 4mm | 5.25 per point (5 points) | 1 mm | A diode laser biostimulation (using a biostimulation probe) on the test site was done | Instantly post-surgery, first, third and seventh days later/ 4 |
| Kohale et al. 2018 | Diode InGaAsP (Biolase - iLase‑7400040‑0XX) | 940 | Continuous | 0.1 | 360 (120 x3) (40 per point x 3 points 120 sec in each session) | 12 | 1 cm2 | 4 per teeth (3 points) | 1-2 cm | 1‐2 cm away from the intervention site in a circular motion 3 points were irradiated in each session 1cm2 area over each tooth | Instantly post-surgery and, 3, 7 days later/ 3 |
| Heidari et al. 2017 | Diode (THOR® Laser, London, UK) | 660 | Continuous | 0.2 | 160 (5×32) 32 | 32 | 5mm | 4 per point (8 points) | Slight contact | The laser was directed vertically with minor contact with the intervention site. The laser was used instantly post-surgery and at 1, 2, 4, and 7 days later at the donor and recipient sites. | Instantly post-surgery, and 1,2,4 and 7 days later/ 5 |
| Ustaoglu et al. 2017 | Diode GaAlAs (Calibrated by Ezlase; Biolase_ Technology, Inc., _Istanbul, Turkey). | 940 | Continuous; the pulse interval size was 0.10 msec, and the pulse length was 0.05 msec | 3 | 32 (4×8) 8 | Total applied energy was 24 J )energy over the actual wound area was 7.2J( | 2.8 cm2 | 8.6 | 1 mm | The handpiece was situated at a distance of 1mm at the top of the intervention area, vertical, in non-contact mode | 4 times (48h interval) |
| Chawla et al. 2016 | Diode (Details not mentioned) | 810 | Continuous | 1 | 1200 (4×300) | 1200 | NM | NM | 1mm | To avoid the scattering of the beam to the contrary side, stents were prepared prior to the operation using putty impression material. The stent was used to conceal the control site during the PBM procedure | LLLT procedure was repeated each day until the 7th day. |
| Keskiner et al. 2016 | Nd:YAG (Fotona Fidelis III, Ljubljana, Slovenia) | 1064 | Continuous | 0.25 | 50 (5×10) | 12 | 0.28cm2 | 1.6 per point (5 points) | 1 cm | A, Nd:YAG laser biostimulation (using a biostimulation probe) on the surgical site was carried out | Instantly post-surgery, first, second, third, and fourth days later/5 |
| Vieira et al. 2010 | LED (Details not mentioned) | 650 | NM | 5 | NM | NM | NM | 4 per point (2 points) | NM | A, LED laser biostimulation (using a biostimulation probe) on the surgical site was carried out | Instantly post-surgery, 48h, and 72h later/3 |
| Ozcelik et al. 2008 | Diode (ULOCKS, VSMA Lab, Voronezh, Russia) | 588 | Continuous | 0.12 | 2400 (8×300) | 288 | NM | 4 | Contact | PBM was used on one side of the surgical area with minor contact with the intervention area | Instantly post-surgery, and daily for 7 days later /8 |
| Amorim et al. 2006 | Diode (Model IR 500; Laser Beam, Rio de Janeiro, Brazil) | 685 | Continuous | 0.05 | 320 (4×80) | 16 | 2mm | 4 | Contact | The irradiation was made in contact mode while holding the delivery tip perpendicular to the tissue surface | Instantly post-surgery, and at 24 h, 3, and 7 days/4 |
| Damante et al. 2004 (A) | Diode GaAlAs | 670 | NM | 0.015 | NM | NM | 4mm | 4 per point (3 points) | Contact | Diode laser was used in the punctual mode with slight contact to three points. The focal spot diameter was 4 mm. Three points of 4 mm each were sufficient to conceal the intervention site. | Every 48 h for 1 week, for a total of four sessions/4 |
| Damante et al. 2004 (B) | Diode GaAlAs | 670 | NM | NM | NM | NM | 4mm | 4 per point (3 points | Contact | Diode laser was used in the punctual mode with slight contact to three points. The focal spot diameter was 4 mm. Three points of 4 mm each were sufficient to conceal the intervention site. | Every 48 h for 1 week, for a total of four sessions/4 |
| Damante et al. published two articles in the year 2004 and they are marked with (A) and (B) to be distinguished. NM not mentioned, nm nanometer, W watt, s seconds, n number, t times, J joules, µm micrometer, cm2 square centimeter, mm millimeter, LED light-emitting diode, LLLT low-level laser therapy, PBM photobiomodulation |  |  |  |  |  |  |  |  |  |  |  |