Supporting Information

Cellulose Derivatives Functionalized with Multidentate N‑Donor Atoms: Comparative Adsorption of Cadmium(II) and Lead(II) Ions from Water

Baohong Ding, Lu Yang, Hongtao Fan*, Nan You*

(College of Petrochemical Engineering, Liaoning Petrochemical University, Fushun 113001, China)

& The first two authors contributed equally to this paper

* Corresponding author

E-mail: httyf_77@163.com and younan_77@163.com

Preparation of aldehyde cellulose

The aldehyde cellulose was obtained by an oxidation reaction of microcrystalline cellulose by sodium metaperiodate which is a gentle oxidant and can oxidized hydroxide, methyl to aldehyde group. 4 g sodium metaperiodate was dissolved into 50 mL H2O, and then 2 g of the microcrystalline cellulose was added. The resulting suspension mixture with the solution pH adjusted to 3 was stirred at 45 ◦C for 180 min. The main water-insoluble oxidized products were obtained by filtration, washed 5 times with deionized water, and freeze-dried for further grafting.

Determination of aldehyde content

The aldehyde content of aldehyde cellulose was determined by the hydroxylamine hydrochloride method. Excessive hydroxylamine hydrochloride solution (5 %) was added into a suspension with a known amount of aldehyde cellulose. The H+ concentration was released through a Schiff base reaction between the aldehyde groups of the aldehyde cellulose and the amino groups of hydroxylamine hydrochloride. The stoichiometry between H+ ion and aldehyde group is 1. The H+ concentration was determined by the consumption of the 0.0100 mol L−1 of NaOH standard solution using methyl orange as indicator.

Calculation of adsorption capacity

q = (C0 − Ct) V/m                          (S1)

where q (mg g−1) is the adsorption capacity, C0 (mg L−1) is the initial concentration of metals, Ct (mg L−1) is the remnant concentration of metals, V (mL) is the volume of solution and m (g) is the mass of the Schiff base functionalized cellulose.

Adsorption isotherms

The linearized equations of Langmuir (L), Freundlich (F) and Dubinin–Radushkevich (D–R) isotherm can be expressed as Eqs. (S3) and (S4), respectively.

Ce/qe= 1/(qmax b) + Ce/qmax                     (S2)

log qe = log kF + (1/n) log Ce                   (S3)

ln qe = ln qs – kadε2                                   (S4)

where qe (mg g−1) is the amount of metals adsorbed per unit mass of the Schiff base functionalized cellulose at equilibrium; Ce (mg L−1) is the concentration at equilibrium; qmax (mg g−1) is the maximum adsorption at monolayer coverage; b (L mg−1) is the adsorption equilibrium constant; KF (L g−1) is a Freundlich constant; n is a constant; kad (mol2 kJ−2) is the D–R isotherm constant; qs (mg g−1) is the saturation capacity from D–R isotherm; ε is the Polanyi potential and calculated as follows:

ε= RT ln (1+1/Ceq)                          (S5)

where R (8.314 J mol−1 K−1) is universal gas constant; T (K) is the absolute temperature. Ceq (mol L−1) is the equilibrium concentration of metal ions.

E (kJ mol−1) is the change of free energy transforming 1 mol of metal ions from solution to the surfaces and is conducive to the estimation of adsorption reaction type, and is obtained from kad as follows:

E= –(2kad) −1/2                                            (S6)

Kinetic analysis

The pseudo-first-order and pseudo-second-order rate expressions are linearly expressed as:

log(qe−qt) = log qe − k1t/2.303                  (S7)

t/qt= 1/k2qe 2 + t/qe                                         (S8)

where k1 (min−1) is the rate constant of the pseudo-first-order adsorption. qe and qt (mg g−1) are the adsorption capacity at equilibrium and the adsorption amount at time t (min), respectively. k2 (g mg−1 min−1) is the rate constant of the pseudo-second-order equation.

Thermodynamic analysis

Thermodynamic parameters such as standard Gibbs free energy change (ΔGº), enthalpy change (ΔHº) and entropy change (ΔSº) at equilibrium at different temperatures can be calculated from the constant (b, L mol−1) of L isotherm equation as the following equations:

ΔGo = −RT lnb                              (S9)

In order to use b in the thermodynamic calculations, the value of b expressed in L mg−1 in L isotherm equation can be multiplied by 1000 to convert the units in L g−1, and then multiplied by the atom weight of the metals, to transform b in L mol−1.

ΔHº and ΔSº were obtained from Eq.S11.

lnb=ΔS0R-ΔH0RT                              (S10)

Scheme S1 Preparation process of the N3-CL.

Figure S1 SEM images of the microcrystalline cellulose.

Figure S2 EDS mapping of the N elements in the N2-CL, N3-CL and N4-CL

Figure S3 Solid state CNMR spectra of dialdehyde cellulose.

Table S1 The N contents of the N3-CL at different experimental conditions

| Molar ratios of aldehyde groups/primary amino groups | N Contents (wt%) |  |
| --- | --- | --- |
|  | Element analysis | Kjeldahl method |
| 1:0.5 | 2.76 | 2.57 |
| 1:1 | 4.43 | 4.37 |
| 1:2 | 4.31 | 4.39 |

Table S2 Isotherms parameters for the adsorption of both the ions by the N2-CL

| Ions | Temperature | Freundlich | Langmuir | D–R |
| --- | --- | --- | --- | --- |
| Cd2+ |  | KF = 60.4 L mg−1 | qmax = 210.7 mg g−1 | E = 14.7 mol kJ−1 |
|  | 15°C | n = 4.63 | b = 0.0630 L mg−1 | qs = 338.0 mg g−1 |
|  |  | R2 = 0.951 | R2 = 0.999 | R2 = 0.975 |
|  |  | KF = 67.4 L mg−1 | qmax = 225.6 mg g−1 | E = 15.0 mol kJ−1 |
|  | 30°C | n = 4.71 | b = 0.0742 L mg−1 | qs = 359.3 mg g−1 |
|  |  | R2 = 0.953 | R2 = 0.999 | R2 = 0.978 |
|  |  | KF = 71.7 L mg−1 | qmax = 231.3 mg g−1 | E = 15.2 mol kJ−1 |
|  | 45°C | n = 4.79 | b = 0.0884 L mg−1 | qs = 370.6 mg g−1 |
|  |  | R2 = 0.944 | R2 = 0.999 | R2 = 0.974 |
| Pb2+ |  | KF = 59.6 L mg−1 | qmax = 355.9 mg g−1 | qs = 947.3 mg g−1 |
|  | 15°C | n = 2.95 | b = 0.0593 L mg−1 | E = 11.9 mol kJ-1 |
|  |  | R2 = 0.885 | R2 = 0.995 | R2 = 0.919 |
|  |  | KF = 64.3 L mg−1 | qmax = 370.8 mg g−1 | qs = 1029.8 mg g−1 |
|  | 30°C | n = 2.94 | b = 0.0700 L mg−1 | E = 11.9 mol kJ-1 |
|  |  | R2 = 0.875 | R2 = 0.998 | R2 = 0.915 |
|  |  | KF = 69.7 L mg−1 | qmax = 376.9 mg g−1 | qs =1020.8 mg g−1 |
|  | 45°C | n = 3.02 | b = 0.0765 L mg−1 | E = 12.1 mol kJ-1 |
|  |  | R2 = 0.857 | R2 = 0.997 | R2 = 0.899 |

Table S3 Isotherms parameters for the adsorption of both the ions by the N3-CL

| Ions | Temperature | Freundlich | Langmuir | D–R |
| --- | --- | --- | --- | --- |
| Cd2+ |  | KF = 70.9 L mg−1 | qmax = 233.7 mg g−1 | E = 15.1 mol kJ−1 |
|  | 15°C | n = 4.68 | b = 0.0910 L mg−1 | qs = 381.6 mg g−1 |
|  |  | R2 = 0.928 | R2 = 0.999 | R2 = 0.960 |
|  |  | KF = 86.7 L mg−1 | qmax = 248.9 mg g−1 | E = 16.2 mol kJ−1 |
|  | 30°C | n = 5.23 | b = 0.115 L mg−1 | qs = 377.6 mg g−1 |
|  |  | R2 = 0.947 | R2 = 0.999 | R2 = 0.973 |
|  |  | KF = 93.3 L mg−1 | qmax = 256.1 mg g−1 | E = 17.1 mol kJ−1 |
|  | 45°C | n = 5.45 | b = 0.124 L mg−1 | qs = 377.4 mg g−1 |
|  |  | R2 = 0.953 | R2 = 0.999 | R2 = 0.976 |
| Pb2+ |  | KF = 58.0 L mg−1 | qmax = 400.0 mg g−1 | qs = 1305.2 mg g−1 |
|  | 15°C | n = 2.63 | b = 0.0568 L mg−1 | E = 11.2 mol kJ-1 |
|  |  | R2 = 0.819 | R2 = 0.996 | R2 = 0.863 |
|  |  | KF = 71.8 L mg−1 | qmax = 416.7 mg g−1 | qs = 1288.3 mg g−1 |
|  | 30°C | n = 2.83 | b = 0.0723 L mg−1 | E = 11.6 mol kJ-1 |
|  |  | R2 = 0.746 | R2 = 0.996 | R2 = 0.790 |
|  |  | KF = 76.9 L mg−1 | qmax = 420.5 mg g−1 | qs = 1275.9 mg g−1 |
|  | 45°C | n = 2.89 | b = 0.0855 L mg−1 | E = 11.8 mol kJ-1 |
|  |  | R2 = 0.801 | R2 = 0.998 | R2 = 0.849 |